| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0052-v1 |
| --- | --- |

| Title: | AHG9: Overhead reduction for picture header and slice header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Lulin Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, lulin.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to modify the high-level syntax in the VVC Draft 8 for a further overhead reduction in the picture header (PH) and the slice header. The proposed methods signal a new syntax element ph_multiple_slice_types_in_pic_flag in the PH for specifying whether more than one slice types may be present in the coded picture. When ph_multiple_slice_types_in_pic_flag is equal to 0, a new syntax element ph_slice_type is further signalled to specify the slice type for all slices of the picture. The syntax element slice_type in the slice header is not coded and is inferred to be equal to the value of ph_slice_type.

In Method 1, the possible presence of inter/intra slices is derived from related syntax elements signalled in the PH. It is more efficient in signalling the selected slice type for slices of the picture when only one slice type is allowed to be present in the picture.

In Method 2, the possible presence of each specific slice type is derived from related syntax elements signalled in the PH. It can further reduce overhead by skipping coding the PH syntax elements only applied to the B slice type when B slices are not allowed in the picture.

In addition, a new syntax element pps_multiple_slice_types_in_pic_flag can be further signalled in the PPS as the enabling flag for the methods.

Introduction

In the Versatile Video Coding (VVC) Draft 8 as specified in [1], the new syntax elements ph_inter_slice_allowed_flag and ph_intra_slice_allowed_flag are signalled to indicate whether inter slices and intra slices are allowed to be present in the picture, respectively. When ph_inter_slice_allowed_flag is equal to 0, the syntax elements only related to inter coding tools are not signalled. When ph_intra_slice_allowed_flag is equal to 0, the syntax elements only related to intra coding tools are not signalled. When ph_inter_slice_allowed_flag is equal to 0, the value of slice_type is inferred to be equal to 2 and is not coded in the slice header.

Proposed modifications

In the proposed methods, a new syntax element ph_multiple_slice_types_in_pic_flag is added to the PH structure for specifying whether more than one slice types may be present in the current picture. ph_multiple_slice_types_in_pic_flag equal to 1 specifies that coded slices of the picture may have different values of slice_type. ph_multiple_slice_types_in_pic_flag equal to 0 specifies that all coded slices of the picture have the same value of slice_type. When ph_multiple_slice_types_in_pic_flag is equal to 0, ph_slice_type is further signalled to specifiy the value of slice_type for all slices of the picture and slice_type in the slice header is not coded and is inferred to be equal to the value of ph_slice_type. In Method 1, the possible presence of inter/intra slices is derived from related syntax elements signalled in the PH. In Method 2, the possible presence of each specific slice type is derived from related syntax elements signalled in the PH. The PH syntax elements only applied to the B slice type are skipped when B slices are not allowed in the picture.

Method 1: Signalling the possible presence of inter/intra slices

In Method 1, when ph_multiple_slice_types_in_pic_flag is equal to 1, the syntax element ph_intra_slice_allowed_flag is further signalled to indicate whether the slice type I is allowed in the current picture. When ph_multiple_slice_types_in_pic_flag is equal to 0, ph_slice_type is further signalled to specifiy the value of slice_type for all slices of the picture. The values of the variables InterSliceAllowed and IntraSliceAllowed, respectively indicating whether the inter slice type (slice type B or P) and the intra slice type (slice type I) are allowed in the picture, can be derived from the values of ph_multiple_slice_types_in_pic_flag, ph_intra_slice_allowed_flag, and ph_slice_type. The PH syntax elements related to the coding tools that are only applied to the unused slice types in the picture are not coded. When ph_multiple_slice_types_in_pic_flag is equal to 0, slice_type in the slice header is inferred to be equal to the value of ph_slice_type in the PH. Otherwise, signalling slice_type is conditioned on the value of IntraSliceAllowed.

The proposed modifications to VVC Draft 8 are written in red and provided below:

- 7.3.2.7 Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| if( gdr_or_irap_pic_flag ) |  |
| gdr_pic_flag | u(1) |
| if( !no_pic_partition_flag && !( rect_slice_flag && num_slices_in_pic_minus1 = = 0 ) ) |  |
| ph_inter_slice_allowed_flag ph_multiple_slice_types_in_pic_flag | u(1) |
| if( ph_inter_slice_allowed_flag ph_multiple_slice_types_in_pic_flag ) |  |
| ph_intra_slice_allowed_flag | u(1) |
| else |  |
| ph_slice_type | ue(v) |
| non_reference_picture_flag | u(1) |
| ph_pic_parameter_set_id | ue(v) |
| ….. |  |
| if( partition_constraints_override_enabled_flag ) |  |
| partition_constraints_override_flag | u(1) |
| if( ph_intra_slice_allowed_flag IntraSliceAllowed ) { |  |
| if( partition_constraints_override_flag ) { |  |
| ph_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| ph_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( ph_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| ph_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| ph_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| ph_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| ph_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( ph_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| ph_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| ph_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| if( cu_qp_delta_enabled_flag ) |  |
| ph_cu_qp_delta_subdiv_intra_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| ph_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| } |  |
| if( ph_inter_slice_allowed_flag InterSliceAllowed ) { |  |
| if( partition_constraints_override_flag ) { |  |
| ph_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| ph_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( ph_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| ph_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| ph_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| } |  |
| if( cu_qp_delta_enabled_flag ) |  |
| ph_cu_qp_delta_subdiv_inter_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| ph_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| if( sps_temporal_mvp_enabled_flag ) { |  |
| ph_temporal_mvp_enabled_flag | u(1) |
| if( ph_temporal_mvp_enabled_flag && rpl_info_in_ph_flag ) { |  |
| ph_collocated_from_l0_flag | u(1) |
| if( ( ph_collocated_from_l0_flag && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( !ph_collocated_from_l0_flag && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) |  |
| ph_collocated_ref_idx | ue(v) |
| } |  |
| } |  |
| mvd_l1_zero_flag | u(1) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| ph_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_pic_present_flag ) |  |
| ph_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag ) |  |
| ph_disable_dmvr_flag | u(1) |
| if( sps_prof_pic_present_flag ) |  |
| ph_disable_prof_flag | u(1) |
| if( ( pps_weighted_pred_flag ¦ ¦ pps_weighted_bipred_flag ) && wp_info_in_ph_flag ) |  |
| pred_weight_table( ) |  |
| } |  |
| ….. |  |
| } |  |

- 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| if( picture_header_in_slice_header_flag ) |  |
| picture_header_structure( ) |  |
| if( subpic_info_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( ( rect_slice_flag && NumSlicesInSubpic[ CurrSubpicIdx ] > 1 ) ¦ ¦ ( !rect_slice_flag && NumTilesInPic > 1 ) ) |  |
| slice_address | u(v) |
| for( i = 0; i < NumExtraShBits; i++ ) |  |
| sh_extra_bit[ i ] | u(1) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| if( ph_inter_slice_allowed_flag ph_multiple_slice_types_in_pic_flag ) { |  |
| if( IntraSliceAllowed ) |  |
| slice_type | ue(v) |
| else |  |
| inter_slice_type | u(1) |
| } |  |
| ….. |  |
| } |  |

- 7.4.3.7 Picture header structure semantics

ph_multiple_slice_types_in_pic_flag equal to 1 specifies that coded slices of the picture may have different values of slice_type. ph_multiple_slice_types_in_pic_flag equal to 0 specifies that all coded slices of the picture have the same value of slice_type. When ph_multiple_slice_types_in_pic_flag is not present, it is inferred to be equal to 0.

ph_inter_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have slice_type equal to 2. ph_inter_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have slice_type equal to 0 or 1. [Ed. (YK): Double check the need/correctness of the inference rules for those syntax elements conditioned out by this flag equal to 0.]

ph_intra_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have slice_type equal to 0 or 1. ph_intra_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have slice_type equal to 2.When not present, the value of ph_intra_slice_allowed_flag is inferred to be equal to 1. [Ed. (YK): Double check the need/correctness of the inference rules for those syntax elements conditioned out by this flag equal to 1.]

ph_slice_type, when present, specifies the value of slice_type for all slice headers in the picture.

The variables InterSliceAllowed and IntraSliceAllowed are derived as follows:

- If ph_multiple_slice_types_in_pic_flag is equal to 1, InterSliceAllowed is set equal to 1 and IntraSliceAllowed is set equal to ph_intra_slice_allowed_flag

- Otherwise, InterSliceAllowed is set equal to ( ph_slice_type  !=  I ) and IntraSliceAllowed is set equal to ( ph_slice_type  = =  I )

NOTE 2 – For bitstreams that are suppposed to work subpicure based bitstream merging without the need of changing PH NAL units, the encoder is expected to set the values of both ph_inter_slice_allowed_flag InterSliceAllowed and ph_intra_slice_allowed_flag IntraSliceAllowed equal to 1.

#### 7.4.8.1 General slice header semantics

slice_type specifies the coding type of the slice according to Table 9.

Table 9 – Name association to slice_type

| slice_type | Name of slice_type |
| --- | --- |
| 0 | B (B slice) |
| 1 | P (P slice) |
| 2 | I (I slice) |

When not present, the value of slice_types is inferred to be equal to 2 ph_slice_type if ph_multiple_slice_types_in_pic_flag is equal to 0 and inferred to be equal to inter_slice_type otherwise.

inter_slice_type specifies the value of slice_type.

When ph_intra_slice_allowed_flag is equal to 0, the value of slice_type shall be equal to 0 or 1. When nal_unit_type is in the range of IDR_W_RADL to CRA_NUT, inclusive, and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, slice_type shall be equal to 2.

Method 2: Signalling the possible presence of each slice type

In Method 2, when ph_multiple_slice_types_in_pic_flag is equal to 1, a syntax element ph_allowed_slice_types_idc is further signalled for derivation of the allowed slice types for slices of the picture. When ph_multiple_slice_types_in_pic_flag is equal to 0, ph_slice_type is further signalled to specifiy the value of slice_type for all slices of the picture. The values of the variables BSliceAllowed, PSliceAllowed, and ISliceAllowed, respectively indicating whether slice types B, P, and I are allowed in the picture, can be derived from the values of ph_multiple_slice_types_in_pic_flag, ph_allowed_slice_types_idc, and ph_slice_type. The PH syntax elements related to the coding tools that are only applied to the unused slice types in the picture are not coded. When ph_multiple_slice_types_in_pic_flag is equal to 0, slice_type in the slice header is inferred to be equal to the value of ph_slice_type in the PH. Otherwise, signalling slice_type is conditioned on the value of ph_allowed_slice_types_idc.

The proposed modifications to VVC Draft 8 are written in red and provided below:

- 7.3.2.7 Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| if( gdr_or_irap_pic_flag ) |  |
| gdr_pic_flag | u(1) |
| if( !no_pic_partition_flag && !( rect_slice_flag && num_slices_in_pic_minus1 = = 0 ) ) |  |
| ph_inter_slice_allowed_flag ph_multiple_slice_types_in_pic_flag | u(1) |
| if( ph_inter_slice_allowed_flag ph_multiple_slice_types_in_pic_flag ) |  |
| ph_intra_slice_allowed_flag ph_allowed_slice_types_idc | u(2) |
| else |  |
| ph_slice_type | ue(v) |
| non_reference_picture_flag | u(1) |
| ph_pic_parameter_set_id | ue(v) |
| ….. |  |
| if( partition_constraints_override_enabled_flag ) |  |
| partition_constraints_override_flag | u(1) |
| if( ph_intra_slice_allowed_flag ISliceAllowed ) { |  |
| if( partition_constraints_override_flag ) { |  |
| ph_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| ph_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( ph_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| ph_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| ph_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| ph_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| ph_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( ph_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| ph_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| ph_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| if( cu_qp_delta_enabled_flag ) |  |
| ph_cu_qp_delta_subdiv_intra_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| ph_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| } |  |
| if( ph_inter_slice_allowed_flag BSliceAllowed ¦ ¦ PSliceAllowed ) { |  |
| if( partition_constraints_override_flag ) { |  |
| ph_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| ph_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( ph_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| ph_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| ph_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| } |  |
| if( cu_qp_delta_enabled_flag ) |  |
| ph_cu_qp_delta_subdiv_inter_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| ph_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| if( sps_temporal_mvp_enabled_flag ) { |  |
| ph_temporal_mvp_enabled_flag | u(1) |
| if( ph_temporal_mvp_enabled_flag && rpl_info_in_ph_flag ) { |  |
| if( BSliceAllowed ) |  |
| ph_collocated_from_l0_flag | u(1) |
| if( ( ph_collocated_from_l0_flag && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( !ph_collocated_from_l0_flag && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) |  |
| ph_collocated_ref_idx | ue(v) |
| } |  |
| } |  |
| if( BSliceAllowed ) |  |
| mvd_l1_zero_flag | u(1) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| ph_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_pic_present_flag && BSliceAllowed ) |  |
| ph_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag && BSliceAllowed ) |  |
| ph_disable_dmvr_flag | u(1) |
| if( sps_prof_pic_present_flag ) |  |
| ph_disable_prof_flag | u(1) |
| if( (( pps_weighted_pred_flag && PSliceAllowed ) ¦ ¦ ( pps_weighted_bipred_flag && BSliceAllowed )) && wp_info_in_ph_flag ) |  |
| pred_weight_table( ) |  |
| } |  |
| ….. |  |
| } |  |

- 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| if( picture_header_in_slice_header_flag ) |  |
| picture_header_structure( ) |  |
| if( subpic_info_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( ( rect_slice_flag && NumSlicesInSubpic[ CurrSubpicIdx ] > 1 ) ¦ ¦ ( !rect_slice_flag && NumTilesInPic > 1 ) ) |  |
| slice_address | u(v) |
| for( i = 0; i < NumExtraShBits; i++ ) |  |
| sh_extra_bit[ i ] | u(1) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| if( ph_inter_slice_allowed_flag ph_multiple_slice_types_in_pic_flag ) |  |
| if( ph_allowed_slice_types_idc = = 3 ) |  |
| slice_type | ue(v) |
| else |  |
| slice_type_modified | u(1) |
| ….. |  |
| } |  |

- 7.3.7.2 Weighted prediction parameters syntax

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( ChromaArrayType != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| if( wp_info_in_ph_flag ) |  |
| num_l0_weights | ue(v) |
| for( i = 0; i < NumWeightsL0; i++ ) |  |
| luma_weight_l0_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i < NumWeightsL0; i++ ) |  |
| chroma_weight_l0_flag[ i ] | u(1) |
| for( i = 0; i < NumWeightsL0; i++ ) { |  |
| if( luma_weight_l0_flag[ i ] ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_weight_l0_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| if( pps_weighted_bipred_flag && ( wp_info_in_ph_flag && BSliceAllowed ) ) |  |
| num_l1_weights | ue(v) |
| for( i = 0; i < NumWeightsL1; i++ ) |  |
| luma_weight_l1_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i < NumWeightsL1; i++ ) |  |
| chroma_weight_l1_flag[ i ] | u(1) |
| for( i = 0; i < NumWeightsL1; i++ ) { |  |
| if( luma_weight_l1_flag[ i ] ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_weight_l1_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |

- 7.4.3.7 Picture header structure semantics

ph_multiple_slice_types_in_pic_flag equal to 1 specifies that coded slices of the picture may have different values of slice_type. ph_multiple_slice_types_in_pic_flag equal to 0 specifies that all coded slices of the picture have the same value of slice_type. When ph_multiple_slice_types_in_pic_flag is not present, it is inferred to be equal to 0.

ph_allowed_slice_types_idc specifies the allowed values of slice_type for coded slices in the picture according to Table X when ph_multiple_slice_types_in_pic_flag is equal to 1.

Table X – The allowed values of slice_type specified by ph_allowed_slice_types_idc

| ph_allowed_slice_types_idc | allowed values of slice types |
| --- | --- |
| 0 | 1, 2 (P, I) |
| 1 | 0, 2 (B, I) |
| 2 | 0, 1 (B, P) |
| 3 | 0, 1, 2 (B, P, I) |

ph_slice_type specifies the value of slice_type for all slice headers in the picture when ph_muliple_slice_types_in_pic_flag is equal to 0.

The variables BSliceAllowed, PSliceAllowed and ISliceAllowed are derived as follows:

- If ph_multiple_slice_types_in_pic_flag is equal to 0, BSliceAllowed is set equal to ( ph_slice_type  = =  B ), PSliceAllowed is set equal to ( ph_slice_type  = =  P ), and ISliceAllowed is set equal to ( ph_slice_type  = =  I ).

- Otherwise, BSliceAllowed is set equal to ( ph_allowed_slice_types_idc  !=  0 ), PSliceAllowed is set equal to ( ph_allowed_slice_types_idc  !=  1 ), and ISliceAllowed is set equal to ( ph_allowed_slice_types_idc  !=  2 ).

ph_inter_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have slice_type equal to 2. ph_inter_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have slice_type equal to 0 or 1. [Ed. (YK): Double check the need/correctness of the inference rules for those syntax elements conditioned out by this flag equal to 0.]

ph_intra_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have slice_type equal to 0 or 1. ph_intra_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have slice_type equal to 2.When not present, the value of ph_intra_slice_allowed_flag is inferred to be equal to 1. [Ed. (YK): Double check the need/correctness of the inference rules for those syntax elements conditioned out by this flag equal to 1.]

NOTE 2 – For bitstreams that are suppposed to work subpicure based bitstream merging without the need of changing PH NAL units, the encoder is expected to set the values of both ph_inter_slice_allowed_flag and ph_intra_slice_allowed_flag BSliceAllowed, PSliceAllowed, and ISliceAllowed equal to 1.

#### 7.4.8.1 General slice header semantics

slice_type specifies the coding type of the slice according to Table 9.

Table 9 – Name association to slice_type

| slice_type | Name of slice_type |
| --- | --- |
| 0 | B (B slice) |
| 1 | P (P slice) |
| 2 | I (I slice) |

When not present, the value of slice_types is inferred to be equal to 2 derived as follows:

- If ph_multiple_slice_types_in_pic_flag is equal to 1, the value of slice_type is set equal to (slice_type_modified >= ph_allowed_slice_types_idc ? slice_type_modified + 1 : slice_type_modified ).

- Otherwise, the value of slice_type is set equal to the value of ph_slice_type.

slice_type_modified is used for deriving the value of slice_type when ph_multiple_slice_types_in_pic_flag is equal to 1 and the value of ph_allowed_slice_types_idc is not equal to 3.

When ph_intra_slice_allowed_flag is equal to 0, the value of slice_type shall be equal to 0 or 1. When nal_unit_type is in the range of IDR_W_RADL to CRA_NUT, inclusive, and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, slice_type shall be equal to 2.

- 7.4.8.2 Weighted prediction parameters semantics

num_l1_weights specifies the number of weights signalled for entries in reference picture list 1 when pps_weighted_bipred_flag and wp_info_in_ph_flag are both equal to 1. The value of num_l1_weights shall be in the range of 0 to Min( 15, num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] ), inclusive.

The variable NumWeightsL1 is derived as follows:

if( !pps_weighted_bipred_flag )
	NumWeightsL1 = 0
else if( wp_info_in_ph_flag )		(148)
	NumWeightsL1 = BSliceAllowed ? num_l1_weights : 0
else
	NumWeightsL1 = NumRefIdxActive[ 1 ]

Signalling pps_multiple_slice_types_in_pic_flag

A new syntax element pps_multiple_slice_types_in_pic_flag can be further signalled in the PPS. When pps_multiple_slice_types_in_pic_flag is eqaul to 0, ph_multiple_slice_types_in_pic_flag is inferred to be equal to 0 for all PHs referring to the PPS.

The related modifications to VVC Draft 8 are written in red and highlighted in yellow and are provided below:

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| … |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| …. |  |
| if( !(rect_slice_flag && num_slices_in_pic_minus1 = = 0) ) |  |
| pps_multiple_slice_types_in_pic_flag | u(1) |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

PH of Method 1

- 7.3.2.7 Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| if( gdr_or_irap_pic_flag ) |  |
| gdr_pic_flag | u(1) |
| if( !no_pic_partition_flag && pps_multiple_slice_types_in_pic_flag && !( rect_slice_flag && num_slices_in_pic_minus1 = = 0 ) ) |  |
| ph_inter_slice_allowed_flag ph_multiple_slice_types_in_pic_flag | u(1) |
| if( ph_inter_slice_allowed_flag ph_multiple_slice_types_in_pic_flag ) |  |
| ph_intra_slice_allowed_flag | u(1) |
| else |  |
| ph_slice_type | ue(v) |
| non_reference_picture_flag | u(1) |
| ph_pic_parameter_set_id | ue(v) |
| ….. |  |
| } |  |

PH of Method 2

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| if( gdr_or_irap_pic_flag ) |  |
| gdr_pic_flag | u(1) |
| if( !no_pic_partition_flag && pps_multiple_slice_types_in_pic_flag && !( rect_slice_flag && num_slices_in_pic_minus1 = = 0 ) ) |  |
| ph_inter_slice_allowed_flag ph_multiple_slice_types_in_pic_flag | u(1) |
| if( ph_inter_slice_allowed_flag ph_multiple_slice_types_in_pic_flag ) |  |
| ph_intra_slice_allowed_flag ph_allowed_slice_types_idc | u(2) |
| else |  |
| ph_slice_type | ue(v) |
| non_reference_picture_flag | u(1) |
| ph_pic_parameter_set_id | ue(v) |
| ….. |  |
| } |  |

- 7.4.3.4 Picture parameter set RBSP semantics

pps_multiple_slice_types_in_pic_flag equal to 1 specifies that coded slices of the picture may have different values of slice_type for all pictures referring to the PPS. pps_multiple_slice_types_in_pic_flag equal to 0 specifies that all coded slices of the picture have the same value of slice_type for all pictures referrring to the PPS. When pps_multiple_slice_types_in_pic_flag is not present, the value of pps_multiple_slice_types_in_pic_flag is inferred to be equal to 0.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-Q2001, 17th Meeting: Brussels, BE, 7–17 January 2020.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).