| sJoint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Alpbach, AT, 15–24 April 2020 | Document: JVET-R0054-v2 |
| --- | --- |

| Title: | AHG12: On combination of wavefront parallel processing and tile partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VVC Draft 8, parallel processing can be achieved by using wavefront parallel processing (WPP) or tile partitioning, and they are allowed to be simultaneously used within the same picture. However, allowing the combination of WPP and tile partitioning not only increases the effort of decoder verification but also introduces functionality redundancy between WPP and horizontal tile partitioning. In order to reduce the verification effort and remove the functionality redundancy, two methods are proposed in this contribution. In Method 1, if WPP is used in the current picture, the number of tile rows in the current picture shall be equal to 1. As a result, the functionality redundancy between WPP and horizontal tile partitioning is removed, and the behavior of CABAC context variable inheritance is also simplified. In Method 2, same as in HEVC, WPP and tile partitioning are disallowed to be simultaneously used within the same picture. It is claimed that Method 2 is simpler than Method 1 and simplifies the decoder verification significantly.

# Introduction

In VVC Draft 8 [1], WPP and tile partitioning are supported for parallel processing. When WPP is used in the current picture, each coding tree unit (CTU) row is processed in parallel relative to its preceding CTU row with one CTU delay, and the CABAC context variables are propagated from the end of the first CTU of the preceding CTU row to the start of the first CTU of the current CTU row. When tile partitioning is used in the current picture, each tile in one picture can be processed in parallel. The tile partitioning can be in horizontal, vertical, or both directions. The vertical tile partitioning not only can achieve parallel processing but also can reduce line buffer size. The horizontal tile partitioning is similar to WPP in the sense that they both can achieve parallel processing between different CTU rows.

However, it could have the following issues when allowing WPP and tile partitioning to be simultaneously used within the same picture:

- The decoder verification effort is increased when allowing the combination of WPP and tile partitioning. As for the decoder verification, we need to test lots of combinations of WPP and tile partitioning, e.g., one slice containing multiple tiles with and without applying WPP in each tile, one tile containing multiple slices with and without applying WPP in one slice, and so on.

- The parallel processing functionality between WPP and horizontal tile partitioning is redundant. The coding performance would also be decreased when WPP and horizontal tile partitioning are simultaneously used within the same picture.

- The initialization or inheritance of CABAC context variables is also different between WPP and tile partitioning. As the illustrated example in Fig.1, if both tools are enabled, for each non-first CTU row of WPP decoding thread, the inheritance of CABAC context variables from the upper CTU row will depend on whether this upper CTU row belongs to the same tile. This also complicates the decoder design.

In order to reduce the complexity of combining WPP and tile partitioning, two methods are proposed in the following section.

Fig. 1.  An example for enabling both WPP and tile partitioning. A picture with 10 by 8 luma CTUs that is partitioned into 9 tiles and 1 slice. Each CTU row is separately decoded by a WPP decoding thread.

# Proposed methods

Two methods are proposed to reduce the complexity of combining WPP and tile partitioning.

## Method 1

WPP and tile partitioning are allowed to be simultaneously used in the same picture, but when WPP is used in the current picture, the number of tile rows in the current picture shall be equal to 1. That is, the functionality redundancy between WPP and horizontal tile partitioning is removed. For example, WPP is allowed in Fig. 2 (a). By contrast, WPP is disallowed in Fig. 1 (b) and (c).

| (a) | (b) | (c) |
| --- | --- | --- |

Fig. 2.  Examples for allowing or disallowing WPP. A picture with 10 by 8 luma CTUs that is partitioned into (a) 3 vertical tiles and 1 slice, (b) 3 horizontal tiles and 1 slice, and (c) 9 tiles and 1 slice. Each CTU row is separately decoded by a WPP decoding thread.

The related syntax semantic modifications are highlighted in yellow as follows:

num_exp_tile_rows_minus1 plus 1 specifies the number of explicitly provided tile row heights. The value of num_exp_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive.  When no_pic_partition_flag is equal to 1, the value of num_tile_rows_minus1 is inferred to be equal to 0.

It is a requirement of bitstream conformance that when the value of sps_entropy_coding_sync_enabled_flag is equal to 1, the value of num_exp_tile_rows_minus1 shall be equal to 0.

tile_row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of CTBs for i in the range of 0 to num_exp_tile_rows_minus1 − 1, inclusive. tile_row_height_minus1[ num_exp_tile_rows_minus1 ] is used to derive the height of the tile rows with index greater than or equal to num_exp_tile_rows_minus1 as specified in clause 6.5.1. The value of tile_row_height_minus1[ i ] shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present, the value of tile_row_height_minus1[ 0 ] is inferred to be equal to PicHeightInCtbsY − 1.

It is a requirement of bitstream conformance that when the value of sps_entropy_coding_sync_enabled_flag is equal to 1, the value of tile_row_height_minus1[ 0 ] shall be equal to PicHeightInCtbsY − 1.

## Method 2

Same as in HEVC, WPP and tile partitioning are disallowed to be simultaneously used within the same picture. The related profile constraint is highlighted in yellow as follows:

    - Main 10 profile

Bitstreams conforming to the Main 10 profile shall obey the following constraints:

- Referenced SPSs shall have chroma_format_idc equal to 0 or 1.

- Referenced SPSs shall have bit_depth_minus8 in the range of 0 to 2, inclusive.

- Referenced SPSs shall have sps_palette_enabled_flag equal to 0.

- When sps_entropy_coding_sync_enabled_flag is equal to 1, an active PPS shall have NumTilesInPic equal to 1.

…

    - Main 4:4:4 10 profile

Bitstreams conforming to the Main 4:4:4 10 profile shall obey the following constraints:

- Referenced SPSs shall have chroma_format_idc in the range of 0 to 3, inclusive.

- Referenced SPSs shall have bit_depth_minus8 in the range of 0 to 2, inclusive.

- When sps_entropy_coding_sync_enabled_flag is equal to 1, an active PPS shall have NumTilesInPic equal to 1.

…

# References

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team, JVET-Q2001.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).