| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0057-v1 |
| --- | --- |

| Title: | LFNST redundant syntax removal |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Man-Shu Chiang, Chih-Wei Hsu, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, man-shu.chiang, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VVC Draft 8, the low frequency non-separable transform (LFNST) is applied to luma only and not applied to chroma when the current coding unit (CU) is coded in single tree structure. The LFNST index can be signalled when there is at least one nonzero AC coefficient in the current CU or the intra subblock partition (ISP) mode is used and when there is no nonzero coefficient in the LFNST zero-out region. At the same time, in case of single tree, chroma coefficients are also used to check both existence of nonzero AC coefficient in the current CU and existence of nonzero coefficient in the LFNST zero-out region. Therefore, a redundant LFNST index may be signalled in two following cases: case-1, when luma has nonzero DC and all-zero AC coefficients, and at least one of the chroma components has at least one nonzero AC coefficient in the non-zero-out region of a single tree coded CU; or in case-2, when luma has all-zero coefficients, and at least one of the chroma components has at least one non-zero AC coefficient in the non-zero-out region of a single tree coded CU. This contribution proposes two methods to reduce the syntax redundancy. In method-1, only luma coefficients are used for checking existence of nonzero AC coefficient in single tree, and it is asserted the syntax redundancy in both cases is removed. In method-2, the LFNST index is only signalled when luma has at least one nonzero coefficient in single tree, and it is asserted the syntax redundancy in case-2 is removed. Results reportedly show -0.01% and -0.01% luma BD-rates under RA for method-1 and method-2, respectively.

# Introduction

In VVC Draft 8 [1], the LFNST is only applied to luma when the current CU is coded in a single tree structure [2]. For LFNST index signalling, two control flag variables, LfnstDcOnly and LfnstZeroOutSigCoeffFlag, are derived. LfnstDcOnly indicates whether all TBs in the current CU do not have any nonzero AC coefficient. LfnstZeroOutSigCoeffFlag indicates whether all TBs in the current CU do not have any nonzero coefficient in the LFNST zero-out regions (i.e., scan position greater than 7 for 4x4 and 8x8 TBs, or scan position greater than 15 for other TBs). The LFNST index is only signalled when LfnstDcOnly is false or ISP is used and when LfnstZeroOutSigCoeffFlag is true. Meanwhile, LfnstDcOnly and LfnstZeroOutSigCoeffFlag both consider chroma coefficients in single tree coded CU even though LFNST is only applied to luma. This results in redundant LFNST index signalling in the following two cases.

- Case-1: Luma component has nonzero DC coefficient and all-zero AC coefficients. At least one of the chroma components has at least one nonzero AC coefficient, and all the nonzero chroma coefficients are not in the zero-out regions.

- Case-2: Luma component has all-zero coefficients. At least one of the chroma components has at least one nonzero AC coefficient, and all the nonzero chroma coefficients are not in the zero-out regions.

In these two cases, both LfnstDcOnly and LfnstZeroOutSigCoeffFlag satisify the condition of signalling LFNST index. In the original LFNST design, if the CU does not have any nonzero AC coefficient, the LFNST index is inferred as 0. However, for case-1, the LFNST is still signalled when the luma component does not have any nonzero AC coefficient in single tree coded CU. In case-2, the LFNST index is signalled but will not be applied at all because the luma component does not have any nonzero coefficient. The signalled LFNST index in these two cases is redundant. In this contribution, it is proposed to remove the redundant LFNST index signalling.

# Proposed methods

## Only check luma component for LfnstDcOnly derivation in single tree

In method-1, it is proposed to skip checking chroma components for LfnstDcOnly in single tree coded CU. In VVC Draft 8, the changes for the proposed method are highlighted as follows.

if( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && lastScanPos > 0 && (treeType!= SINGLE_TREE ¦ ¦ cIdx = = 0) )

LfnstDcOnly = 0

By this proposed method, redundant LFNST index signalling in both case-1 and case-2 is removed.

## Signal LFNST index when luma has at least one nonzero coefficient in single tree

In method-2, it is proposed to check whether the luma component has at least one nonzero coefficient for LFNST index signaling in single tree. The tu_cbf_luma information is used. Note that only tu_cbf_luma[ x0 ][ y0] is checked for the non-ISP coded CU, where (x0, y0) is the top left luma sample position of the current CU. For ISP coded CU, since it is guaranteed that at least one luma TB has one or more nonzero coefficients, the check of tu_cbf_luma[ x0 ][ y0] is not required. In VVC Draft 8, the changes for the proposed method are highlighted as follows.

| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && lfnstNotTsFlag = = 1 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| --- | --- |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ ( LfnstDcOnly = = 0 && (treeType != SINGLE_TREE ¦ ¦ tu_y_coded_flag [ x0 ][ y0 ] ) ) ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |

By this proposed method, redundant LFNST index signalling in case-2 is removed, while issue for the case-1 remains. This method is less strict than method-1.

# Experiment results

The proposed methods have been implemented on VTM8.0 [3]. Tables 1 and 2 show the results of the proposed method-1 and method-2 under the common test condition (CTC) [4], respectively.

Table 1.  Results of the proposed method-1

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.01% | 100% | 103% |
| Class A2 | -0.01% | -0.04% | 0.11% | 101% | 100% |
| Class B | 0.00% | 0.04% | 0.06% | 100% | 98% |
| Class C | -0.01% | -0.04% | -0.09% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.01% | 100% | 100% |
| Class D | 0.00% | 0.06% | -0.17% | 100% | 98% |
| Class F | 0.01% | 0.01% | -0.01% | 101% | 97% |

Table 2.  Results of the proposed method-2

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.14% | -0.15% | 101% | 99% |
| Class A2 | -0.01% | 0.06% | 0.06% | 101% | 101% |
| Class B | 0.00% | 0.00% | -0.03% | 101% | 99% |
| Class C | -0.02% | 0.02% | 0.04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.02% | 101% | 99% |
| Class D | -0.01% | -0.02% | -0.05% | 101% | 100% |
| Class F | 0.01% | 0.01% | 0.00% | 100% | 100% |

# Conclusions

This contribution presents two methods to remove (method-1) or reduce (method-2) the LFNST index signalling redundancy. In the proposed method-1, only luma component coefficients are used for LfnstDcOnly derivation in single tree. In method-2, the LFNST index is only signalled when luma has at least one nonzero coefficient in single tree. Results reportedly show minor BD-rate savings.

# References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team, JVET-Q2001.

- H. E. Egilmez, A. Nalci, A. K. Ramasubramonian, M. Coban, V. Seregin, M. Karczewicz, T. Hashimoto, T. Chujoh, E. Sasaki, T. Ikai, “Combination of JVET-Q0106, JVET-0686 and JVET-Q0133 for LFNST signalling, latency reduction, and scaling process,” Document of Joint Video Experts Team, JVET-Q0784.

- VTM8.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).