| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0059-v1 |
| --- | --- |

| Title: | AHG9: Cleanups on RPL and related signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Zhipin Deng2, Li Zhang1 Kai Zhang1, Jizheng Xu1 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com zhipin.deng@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes on RPL and related signalling:

- When the RPL information is signalled in the PH instead of the SH, derive the numbers of active entries to be equal to the default numbers of active entries signalled in the PPS, and update the SH syntax accordingly. Consequently, the following changes are also proposed:

  - In the syntax condition for signalling of the collocated picture in the PH, replace "num_ref_entries[ 0 ][ RplsIdx[ 0 ] ]" and " num_ref_entries[ 1 ][ RplsIdx[ 1 ] ]" with "NumRefIdxActive[ 0 ]" and " NumRefIdxActive[ 1 ]" respectively.

  - In the weighted prediction (WP) syntax pred_weight_table( ), remove the syntax elements num_l0_weights and num_l1_weights.

- In the reference picture list structure syntax, skip the signalling of the ltrp_in_header_flag[ listIdx ][ rplsIdx ] syntax element when the syntax strucure is directly included in the PH or SH instead of in the SPS.

- It is required that, when ph_temporal_mvp_enabled_flag and rpl_info_in_ph_flag are both equal to 1 and there is at least one P slice in the picture, the value of ph_collocated_from_l0_flag shall be equal to 1.

- Replace the existing constraint on the value of ph_temporal_mvp_enabled_flag with a NOTE, with the addition of the scaling window offsets to be also the same.

- Simplify the existing constraint in the slice header semantics on the collocated picture by just keeping the 0-valued-RprConstraintsActive[ ][ ] aspect, which is asserted to cover all aspects needed.

# Introduction

We observed the following issues on signalling of RPL and the related signalling of the collocated picture and weighted prediction, mostly resulted from the introduction of the RPL information being either in the PH or the SH:

- Currently, when the RPL information is signalled in the PH, the numbers of active entries are still signalled in the SH. This makes the syntax conditions for RPL signalling in the SH very complicated, as highlighted below:

| if( !rpl_info_in_ph_flag && ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) |  |
| --- | --- |
| ref_pic_lists( ) |  |
| if( ( rpl_info_in_ph_flag ¦ ¦ ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) && ( ( slice_type != I && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( slice_type = = B && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( slice_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |

Such complicated syntax condition is not only difficult to understand but also prone to errors. If the signalling of the numbers of active entries goes together with the RPL information itself, the syntax condition can be much simplified. Furthermore, we believe that in most cases, when the encoder uses the same RPL for all slices of a picture, it would also use the same numbers of active RPL entries for all the slices. In addition, if the signalling of the numbers of active entries goes together with the RPL information itself, the signalling of the collocated picture and weighted parameters can be improved and simplified by utilizing the known numbers of active entries.

- The signalling of the ltrp_in_header_flag[ listIdx ][ rplsIdx ] syntax element in the reference picture list structure syntax is meaningless when the syntax strucure is directly included in the PH or SH instead of in the SPS, as in this case the POC LSBs for the LTRP entries are included in the picture/slice header instead of in the SPS anyway.

- The signalling of the collocated picture includes whether it is from list 0 or list 1, and the reference index. Currently, the signalling of the collocated picture going together with the RPL information, either in the PH or in the SH (but never both).

When ph_temporal_mvp_enabled_flag and rpl_info_in_ph_flag are both equal to 1, the value of slice_collocated_from_l0_flag would be inferred to be equal to ph_collocated_from_l0_flag, as follows:

When slice_type is equal to B or P, ph_temporal_mvp_enabled_flag is equal to 1, and slice_collocated_from_l0_flag is not present, the following applies:

- If rpl_info_in_ph_flag is equal to 1, slice_collocated_from_l0_flag is inferred to be equal to ph_collocated_from_l0_flag.

- Otherwise (rpl_info_in_ph_flag is equal to 0 and slice_type is equal to P), the value of slice_collocated_from_l0_flag is inferred to be equal to 1.

Consequently, when ph_temporal_mvp_enabled_flag and rpl_info_in_ph_flag are both equal to 1, and ph_collocated_from_l0_flag is equal to 0, the value of slice_collocated_from_l0_flag is inferred to be equal to 0, including for P slices in the picture. However, a P slice shall only refer to entries in list 0.

- The existing constraint on the value of ph_temporal_mvp_enabled_flag, copied below, is not really needed, as what is prohibited is later disallowed by the SH constraint (also copied below) on the collocated picture to have the same resolution and the same scaling window offsets.

When no reference picture in the DPB has the same spatial resolution as the current picture, the value of ph_temporal_mvp_enabled_flag shall be equal to 0.

It is a requirement of bitstream conformance that the values of pic_width_in_luma_samples and pic_height_in_luma_samples of the reference picture referred to by slice_collocated_ref_idx shall be equal to the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of the current picture, and RprConstraintsActive[ slice_collocated_from_l0_flag ? 0 : 1 ][ slice_collocated_ref_idx ] shall be equal to 0.

- The SH constraint (copied above) on the collocated picture to have the same resolution and the same scaling window offsets has some redundance, as the 0-valued RprConstraintsActive[ ][ ] aspect covers both the same resolution aspect and the same scaling window offsets aspect. Therefore, the same resolution aspect can be removed.

# Proposal

## Summary of the proposed changes

The following changes are proposed to address the above issues:

- To address the 1st issue, when the RPL information is signalled in the PH instead of the SH, derive the numbers of active entries to be equal to the default numbers of active entries signalled in the PPS, and update the SH syntax accordingly. Consequently, the following two changes are also proposed:

  - In the syntax condition for signalling of the collocated picture in the PH, replace "num_ref_entries[ 0 ][ RplsIdx[ 0 ] ]" and " num_ref_entries[ 1 ][ RplsIdx[ 1 ] ]" with "NumRefIdxActive[ 0 ]" and " NumRefIdxActive[ 1 ]" respectively.

  - In the weighted prediction (WP) syntax pred_weight_table( ), remove the syntax elements num_l0_weights and num_l1_weights.

- To address the 2nd issue, in the reference picture list structure syntax, skip the signalling of the ltrp_in_header_flag[ listIdx ][ rplsIdx ] syntax element when the syntax strucure is directly included in the PH or SH instead of in the SPS.

- To address the 3rd issue, it is required that, when ph_temporal_mvp_enabled_flag and rpl_info_in_ph_flag are both equal to 1 and there is at least one P slice in the picture, the value of ph_collocated_from_l0_flag shall be equal to 1.

The actually proposed text is to add the following constraint, which we believe is better editorially, as it avoids specifying a constraint on the value of a syntax element based on the value of a lower-level syntax element: When ph_collocated_from_l0_flag is equal to 0, the value of slice_type shall be not be equal to 1.

There are certainly other approaches to resolve this issue. One approach is to disallow mixing of B and P slice types within a picture, and when the picture contains P slices only, the value of ph_collocated_from_l0_flag is required to be equal to 1. Another approach is to allow the collocated picture to be from list 1 (and possibly not present in list 0) regardless of whether the picture contains P slices or not. However, we think the proposed restriction is the safest fix at this stage, as the other possible approaches mentioned would likely require quite some changes and checking of other places in the spec and there is a good chance to have something wrong somewhere even after careful checking.

- To address the 4th issue, unless it is planned to systematically specify many informative notes in normative spec language style, it is proposed to replace the constraint on the value of ph_temporal_mvp_enabled_flag with the following NOTE:

NOTE – The value of ph_temporal_mvp_enabled_flag has to be equal to 0 when no reference picture in the DPB has the same spatial resolution and scaling window offsets as the current picture.

If it is planned to systematically specify many informative notes in normative spec language style, then it is proposed to just add the same-scaling-window-offsets aspect into the existing constraint.

- To address the 5th issue, in the SH constraint on the collocated picture to have the same resolution and the same scaling window offsets, remove the same-resolution aspect, just keep the 0-valued-RprConstraintsActive[ ][ ] aspect, which covers all needed aspects. On the other hand, for readability and easy understadning, add a NOTE to express that this means that the collocated picture has the same same spatial resolution and scaling window offsets as the current picture.

## Proposed text changes

#### 7.3.2.7	Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( sps_temporal_mvp_enabled_flag ) { |  |
| ph_temporal_mvp_enabled_flag | u(1) |
| if( ph_temporal_mvp_enabled_flag && rpl_info_in_ph_flag ) { |  |
| ph_collocated_from_l0_flag | u(1) |
| if( ( ph_collocated_from_l0_flag && NumRefIdxActive[ 0 ] > 1 num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( !ph_collocated_from_l0_flag && NumRefIdxActive[ 1 ] > 1 num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) |  |
| ph_collocated_ref_idx | ue(v) |
| } |  |
| } |  |
| ... |  |
| } |  |

#### 7.4.3.7	Picture header structure semantics

...

pic_output_flag affects the decoded picture output and removal processes as specified in Annex C. When pic_output_flag is not present, it is inferred to be equal to 1.

When rpl_info_in_ph_flag is equal to 1, for i in the range of 0 to 1, inclusive, the value of NumRefIdxActive[ i ] is derived to be equal to num_ref_idx_default_active_minus1[ i ] + 1.

partition_constraints_override_flag equal to 1 specifies that partition constraint parameters are present in the PH. partition_constraints_override_flag equal to 0 specifies that partition constraint parameters are not present in the PH. When not present, the value of partition_constraints_override_flag is inferred to be equal to 0.

ph_temporal_mvp_enabled_flag specifies whether temporal motion vector predictors can be used for inter prediction for slices associated with the PH. If ph_temporal_mvp_enabled_flag is equal to 0, the syntax elements of the slices associated with the PH shall be constrained such that no temporal motion vector predictor is used in decoding of the slices. Otherwise (ph_temporal_mvp_enabled_flag is equal to 1), temporal motion vector predictors may be used in decoding of the slices associated with the PH. When not present, the value of ph_temporal_mvp_enabled_flag is inferred to be equal to 0. When no reference picture in the DPB has the same spatial resolution as the current picture, the value of ph_temporal_mvp_enabled_flag shall be equal to 0.

NOTE – The value of ph_temporal_mvp_enabled_flag has to be equal to 0 when no reference picture in the DPB has the same spatial resolution and scaling window offsets as the current picture.

...

ph_collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. ph_collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1.

ph_collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction.

When ph_collocated_from_l0_flag is equal to 1, ph_collocated_ref_idx refers to an entry in reference picture list 0, and the value of ph_collocated_ref_idx shall be in the range of 0 to num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] NumRefIdxActive[ 0 ] − 1, inclusive.

When ph_collocated_from_l0_flag is equal to 0, ph_collocated_ref_idx refers to an entry in reference picture list 1, and the value of ph_collocated_ref_idx shall be in the range of 0 to num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] NumRefIdxActive[ 1 ] − 1, inclusive.

When not present, the value of ph_collocated_ref_idx is inferred to be equal to 0.

...

#### 7.3.7.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( ph_inter_slice_allowed_flag ) |  |
| slice_type | ue(v) |
| ... |  |
| if( !rpl_info_in_ph_flag && ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) { |  |
| ref_pic_lists( ) |  |
| if( ( rpl_info_in_ph_flag ¦ ¦ ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) && ( ( slice_type != I && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( slice_type = = B && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( slice_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| ... |  |
| } |  |

#### 7.4.8.1	General slice header semantics

...

slice_type specifies the coding type of the slice according to Table 9.

Table 9 – Name association to slice_type

| slice_type | Name of slice_type |
| --- | --- |
| 0 | B (B slice) |
| 1 | P (P slice) |
| 2 | I (I slice) |

When not present, the value of slice_type is inferred to be equal to 2.

When ph_intra_slice_allowed_flag is equal to 0, the value of slice_type shall be equal to 0 or 1. When nal_unit_type is in the range of IDR_W_RADL to CRA_NUT, inclusive, and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, slice_type shall be equal to 2.

When ph_collocated_from_l0_flag is equal to 0, the value of slice_type shall not be equal to 1.

num_ref_idx_active_override_flag equal to 1 specifies that the syntax element num_ref_idx_active_minus1[ 0 ] is present for P and B slices and the syntax element num_ref_idx_active_minus1[ 1 ] is present for B slices. num_ref_idx_active_override_flag equal to 0 specifies that the syntax elements num_ref_idx_active_minus1[ 0 ] and num_ref_idx_active_minus1[ 1 ] are not present. When rpl_info_in_ph_flag is equal to 0 and num_ref_idx_active_override_flag is not present, it is the value of num_ref_idx_active_override_flag is inferred to be equal to 1.

num_ref_idx_active_minus1[ i ] is used for the derivation of the variable NumRefIdxActive[ i ] as specified by Equation 142 when rpl_info_in_ph_flag is equal to 0. The value of num_ref_idx_active_minus1[ i ] shall be in the range of 0 to 14, inclusive.

For i equal to 0 or 1, when the current slice is a B slice, num_ref_idx_active_override_flag is equal to 1, and num_ref_idx_active_minus1[ i ] is not present, num_ref_idx_active_minus1[ i ] is inferred to be equal to 0.

When the current slice is a P slice, num_ref_idx_active_override_flag is equal to 1, and num_ref_idx_active_minus1[ 0 ] is not present, num_ref_idx_active_minus1[ 0 ] is inferred to be equal to 0.

The variable NumRefIdxActive[ i ] is derived as follows:

for( i = 0; i < 2; i++ ) {
	if( slice_type  = =  B  | |  ( slice_type  = =  P  &&  i  = =  0 ) ) {
		if( num_ref_idx_active_override_flag )
			NumRefIdxActive[ i ] = num_ref_idx_active_minus1[ i ] + 1	(142)
		else {
			if( num_ref_entries[ i ][ RplsIdx[ i ] ]  >=  num_ref_idx_default_active_minus1[ i ] + 1 )
				NumRefIdxActive[ i ] = num_ref_idx_default_active_minus1[ i ] + 1
			else
				NumRefIdxActive[ i ] = num_ref_entries[ i ][ RplsIdx[ i ] ]
		}
	} else /* slice_type  = =  I  | |  ( slice_type  = =  P  &&  i  = =  1 ) */
		NumRefIdxActive[ i ] = 0
}

The value of NumRefIdxActive[ i ] − 1 specifies the maximum reference index for reference picture list i that may be used to decode the slice. When the value of NumRefIdxActive[ i ] is equal to 0, no reference index for reference picture list i may be used to decode the slice.

When the current slice is a P slice, the value of NumRefIdxActive[ 0 ] shall be greater than 0.

When the current slice is a B slice, both NumRefIdxActive[ 0 ] and NumRefIdxActive[ 1 ] shall be greater than 0.

cabac_init_flag specifies the method for determining the initialization table used in the initialization process for context variables. When cabac_init_flag is not present, it is inferred to be equal to 0.

slice_collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. slice_collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1.

When slice_type is equal to B or P, ph_temporal_mvp_enabled_flag is equal to 1, and slice_collocated_from_l0_flag is not present, the following applies:

- If rpl_info_in_ph_flag is equal to 1, slice_collocated_from_l0_flag is inferred to be equal to ph_collocated_from_l0_flag.

- Otherwise (rpl_info_in_ph_flag is equal to 0 and slice_type is equal to P), the value of slice_collocated_from_l0_flag is inferred to be equal to 1.

slice_collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction.

When slice_type is equal to P or when slice_type is equal to B and slice_collocated_from_l0_flag is equal to 1, slice_collocated_ref_idx refers to an entry in reference picture list 0, and the value of slice_collocated_ref_idx shall be in the range of 0 to NumRefIdxActive[ 0 ] − 1, inclusive.

When slice_type is equal to B and slice_collocated_from_l0_flag is equal to 0, slice_collocated_ref_idx refers to an entry in reference picture list 1, and the value of slice_collocated_ref_idx shall be in the range of 0 to NumRefIdxActive[ 1 ] − 1, inclusive.

When slice_collocated_ref_idx is not present, the following applies:

- If rpl_info_in_ph_flag is equal to 1, the value of slice_collocated_ref_idx is inferred to be equal to ph_collocated_ref_idx.

- Otherwise (rpl_info_in_ph_flag is equal to 0), the value of slice_collocated_ref_idx is inferred to be equal to 0.

It is a requirement of bitstream conformance that the picture referred to by slice_collocated_ref_idx shall be the same for all slices of a coded picture.

It is a requirement of bitstream conformance that the values of pic_width_in_luma_samples and pic_height_in_luma_samples of the reference picture referred to by slice_collocated_ref_idx shall be equal to the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of the current picture, and RprConstraintsActive[ slice_collocated_from_l0_flag ? 0 : 1 ][ slice_collocated_ref_idx ] shall be equal to 0.

NOTE – The above constraint requires the collocated picture to have the same spatial resolution and scaling window offsets as the current picture.

slice_qp_delta specifies the initial value of QpY to be used for the coding blocks in the slice until modified by the value of CuQpDeltaVal in the coding unit layer.

When qp_delta_info_in_ph_flag is equal to 0, the initial value of the QpY quantization parameter for the slice, SliceQpY, is derived as follows:

SliceQpY = 26 + init_qp_minus26 + slice_qp_delta	(143)

The value of SliceQpY shall be in the range of −QpBdOffset to +63, inclusive.

When either of the following conditions is true, the value of NumRefIdxActive[ 0 ] shall be less than or equal to the value of NumWeightsL0:

- The value of wp_info_in_ph_flag is equal to 1, pps_weighted_pred_flag is equal to 1, and slice_type is equal to P.

- The value of wp_info_in_ph_flag is equal to 1, pps_weighted_bipred_flag is equal to 1, and slice_type is equal to B.

When wp_info_in_ph_flag is equal to 1, pps_weighted_bipred_flag is equal to 1, and slice_type is equal to B, the value of NumRefIdxActive[ 1 ] shall be less than or equal to the value of NumWeightsL1.

When either of the following conditions is true, for each value of i in the range of 0 to NumRefIdxActive[ 0 ] − 1, inclusive, the values of luma_weight_l0_flag[ i ] and chroma_weight_l0_flag[ i ] are both inferred to be equal to 0:

- The value of wp_info_in_ph_flag is equal to 1, pps_weighted_pred_flag is equal to 0, and slice_type is equal to P.

- The value of wp_info_in_ph_flag is equal to 1, pps_weighted_bipred_flag is equal to 0, and slice_type is equal to B.

...

#### 7.3.7.2	Weighted prediction parameters syntax

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( ChromaArrayType != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| if( wp_info_in_ph_flag ) |  |
| num_l0_weights | ue(v) |
| for( i = 0; i < NumRefIdxActive[ 0 ]NumWeightsL0; i++ ) |  |
| luma_weight_l0_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i < NumRefIdxActive[ 0 ]NumWeightsL0; i++ ) |  |
| chroma_weight_l0_flag[ i ] | u(1) |
| for( i = 0; i < NumRefIdxActive[ 0 ]NumWeightsL0; i++ ) { |  |
| if( luma_weight_l0_flag[ i ] ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_weight_l0_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| if( pps_weighted_bipred_flag && wp_info_in_ph_flag ) |  |
| num_l1_weights | ue(v) |
| numWeightsL1 = pps_weighted_bipred_flag ? NumRefIdxActive[ 1 ] : 0 |  |
| for( i = 0; i < NnumWeightsL1; i++ ) |  |
| luma_weight_l1_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i < NnumWeightsL1; i++ ) |  |
| chroma_weight_l1_flag[ i ] | u(1) |
| for( i = 0; i < NnumWeightsL1; i++ ) { |  |
| if( luma_weight_l1_flag[ i ] ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_weight_l1_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |

#### 7.4.8.2	Weighted prediction parameters semantics

...

num_l0_weights specifies the number of weights signalled for entries in reference picture list 0 when wp_info_in_ph_flag is equal to 1. The value of num_l0_weights shall be in the range of 0 to Min( 15, num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] ), inclusive.

If wp_info_in_ph_flag is equal to 1, the variable NumWeightsL0 is set equal to num_l0_weights. Otherwise (wp_info_in_ph_flag is equal to 0), NumWeightsL0 is set equal to NumRefIdxActive[ 0 ].

luma_weight_l0_flag[ i ] equal to 1 specifies that weighting factors for the luma component of list 0 prediction using RefPicList[ 0 ][ i ] are present. luma_weight_l0_flag[ i ] equal to 0 specifies that these weighting factors are not present.

...

num_l1_weights specifies the number of weights signalled for entries in reference picture list 1 when pps_weighted_bipred_flag and wp_info_in_ph_flag are both equal to 1. The value of num_l1_weights shall be in the range of 0 to Min( 15, num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] ), inclusive.

The variable NumWeightsL1 is derived as follows:

if( !pps_weighted_bipred_flag )
	NumWeightsL1 = 0
else if( wp_info_in_ph_flag )		(148)
	NumWeightsL1 = num_l1_weights
else
	NumWeightsL1 = NumRefIdxActive[ 1 ]

luma_weight_l1_flag[ i ] equal to 1 specifies that weighting factors for the luma component of list 1 prediction using RefPicList[ 1 ][ i ] are present. luma_weight_l1_flag[ i ] equal to 0 specifies that these weighting factors are not present. When not present, the value of luma_weight_l1_flag[ i ] is inferred to be equal to 0.

chroma_weight_l1_flag[ i ], delta_luma_weight_l1[ i ], luma_offset_l1[ i ], delta_chroma_weight_l1[ i ][ j ], and delta_chroma_offset_l1[ i ][ j ] have the same semantics as chroma_weight_l0_flag[ i ], delta_luma_weight_l0[ i ], luma_offset_l0[ i ], delta_chroma_weight_l0[ i ][ j ] and delta_chroma_offset_l0[ i ][ j ], respectively, with l0, L0, list 0 and List0 replaced by l1, L1, list 1 and List1, respectively.

The variable sumWeightL0Flags is derived to be equal to the sum of luma_weight_l0_flag[ i ] + 2 * chroma_weight_l0_flag[ i ], for i = 0..NumRefIdxActive[ 0 ] − 1.

When NumRefIdxActive[ 0 ] and NumRefIdxActive[ 1 ] are both greater than 0 slice_type is equal to B, the variable sumWeightL1Flags is derived to be equal to the sum of luma_weight_l1_flag[ i ] + 2 * chroma_weight_l1_flag[ i ], for i = 0..NumRefIdxActive[ 1 ] − 1.

It is a requirement of bitstream conformance that, when slice_type is equal to P, sumWeightL0Flags shall be less than or equal to 24 and, when NumRefIdxActive[ 0 ] and NumRefIdxActive[ 1 ] are both greater than 0 slice_type is equal to B, the sum of sumWeightL0Flags and sumWeightL1Flags shall be less than or equal to 24.

#### 7.3.9	Reference picture list structure syntax

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( long_term_ref_pics_flag && rplsIdx < num_ref_pic_lists_in_sps[ listIdx ] ) |  |
| ltrp_in_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( inter_layer_ref_pics_present_flag ) |  |
| inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( !inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| if( long_term_ref_pics_flag ) |  |
| st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| if( AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] > 0 ) |  |
| strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } else if( !ltrp_in_header_flag[ listIdx ][ rplsIdx ] ) |  |
| rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ j++ ] | u(v) |
| } else |  |
| ilrp_idx[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| } |  |
| } |  |

#### 7.4.10	Reference picture list structure semantics

...

ltrp_in_header_flag[ listIdx ][ rplsIdx ] equal to 0 specifies that the POC LSBs of the LTRP entries indicated in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure are present in the same syntax structure. ltrp_in_header_flag[ listIdx ][ rplsIdx ] equal to 1 specifies that the POC LSBs of the LTRP entries indicated in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure are not present in the same syntax structure. When long_term_ref_pics_flag is equal to 1 and rplsIdx is equal to num_ref_pic_lists_in_sps[ listIdx ], the vlaue of ltrp_in_header_flag[ listIdx ][ rplsIdx ] is inferred to be equal to 1.

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).