| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0063-v1 |
| --- | --- |

| Title: | AHG9: Signalling of LMCS control |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang, Ye-Kui Wang, Kai Zhang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | lizhang.idm@bytedance.com yekui.wang@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to replace the PH flag ph_lmcs_enabled_flag with a 2-bit ph_lmcs_mode_idc, with 3 modes specified: disabled (mode 0), used for all slices (mode 1), and enabled (mode 2). In mode 1, the LMCS is used for all slices of a picture, and no signalling of LMCS control flag is needed in the SH. The semantics of slice_lmcs_enabled_flag is revised accordingly. In addition, fixes of the semantics of ph_chroma_residual_scale_flag are proposed to reflect the intention of enabling/disabling chroma residual scaling for a picture or slice.

# Introduction

In current VVC specification, the LMCS control could be signalled in PH and SH. The related syntax elements and semantics are as follows:

7.3.2.7	Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_lmcs_enabled_flag ) { |  |
| ph_lmcs_enabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) { |  |
| ph_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| ph_chroma_residual_scale_flag | u(1) |
| } |  |
| … |  |
| } |  |
| … |  |

7.3.7.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| … |  |

ph_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is enabled for all slices associated with the PH. ph_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling may be disabled for one, or more, or all slices associated with the PH. When not present, the value of ph_lmcs_enabled_flag is inferred to be equal to 0.

ph_chroma_residual_scale_flag equal to 1 specifies that chroma residual  scaling is enabled for the all slices associated with the PH. ph_chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling may be disabled for one, or more, or all slices associated with the PH. When ph_chroma_residual_scale_flag is not present, it is inferred to be equal to 0.

slice_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is enabled for the current slice. slice_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is not enabled for the current slice. When slice_lmcs_enabled_flag is not present, it is inferred to be equal to 0.

Three issues in the current VVC specification were observed:

- The semantics of ph_chroma_residual_scale_flag is not accurate since all slices are referring to this flag, no additional control flag is signalled in slice headers.

- The semantics of ph_lmcs_enabled_flag is conflicting with the motivation of signalling slice level LMCS flag when ph_lmcs_enabled_flag is true. In the current VVC text, when ph_lmcs_enabled_flag is true, all slices shall enable LMCS. Therefore, there is no need to further signal LMCS enabling flags in slice header.

- In addition, when the picture header tells LMCS is enabled, typically, for all slices, LMCS are all enabled. The controlling of LCMS in slice header is mainly for handling corner cases. Therefore, if the PH LMCS flag is true and SH LMCS flag is always signalled, which may result in unnecessary bits signalled for common user cases.

# Proposal

## Summary of the proposal

To resolve all the above issues, it is proposed to replace the PH flag ph_lmcs_enabled_flag with a 2-bit ph_lmcs_mode_idc, with 3 modes specified: disabled (mode 0), used for all slices (mode 1), and enabled (mode 2). In mode 1, the LMCS is used for all slices of a picture, and no signalling of LMCS control flag is needed in the SH. The semantics of the SH LMCS control flag is revised accordingly. In addition, fixes of the semantics of ph_chroma_residual_scale_flag are proposed to reflect the intention of enabling/disabling chroma residual scaling for a picture or slice.

## Proposed text changes

7.3.2.7	Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_lmcs_enabled_flag ) { |  |
| ph_lmcs_enabled_flag ph_lmcs_mode_idc | u(1) u(2) |
| if( ph_lmcs_enabled_flag ph_lmcs_mode_idc > 0 ) { |  |
| ph_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| ph_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

7.3.7.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if(ph_lmcs_enabled_flag ph_lmcs_mode_idc = = 2 ) |  |
| slice_lmcs_used_flagenabled_flag | u(1) |
| … |  |
| } |  |

...

ph_lmcs_mode_idcenabled_flag equal to 1 specifies that luma mapping with chroma scaling is applied for all slices associated with the PH. ph_lmcs_mode_idc equal to 2 specifies that luma mapping with chroma scaling is enabled for the current picture and whether it is applied for each slice is further controlled by the slice_lmcs_used_flag signalled in the slice header. ph_lmcs_mode_idcenabled_flag equal to 0 specifies that luma mapping with chroma scaling may be disabled for one, or more, or is disabled for all slices associated with the PH. The value of ph_lmcs_mode_idc shall be less than 3. When not present, the value of ph_lmcs_mode_idcenabled_flag is inferred to be equal to 0.

ph_chroma_residual_scale_flag equal to 1 specifies that chroma residual scaling is enabled for all slices associated with the PH and whether it is applied for each slice is further controlled by the slice_lmcs_used_flag signalled in the slice header. ph_chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling may be disabled for one, or more, or is disabled for all slices associated with the PH. When ph_chroma_residual_scale_flag is not present, it is inferred to be equal to 0.

...

slice_lmcs_used_flagenabled_flag equal to 1 specifies that luma mapping with chroma scaling is applied for the current slice, and, when ph_chroma_residual_scale_flag is equal to 1, chroma residual scaling is also applied for the current slice. slice_lmcs_used_flagenabled_flag equal to 0 specifies that luma mapping with chroma scaling as well as chroma residual scaling is not applied for the current slice. When slice_lmcs_used_flagenabled_flag is not present, it is inferred to be equal to 0 ( ph_lmcs_mode_idc  = =1 ) ? 1 : 0.

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).