| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0066-v1 |
| --- | --- |

| Title: | AHG8/AHG9: On DPB memory allocation and derivation of NoOutputOfPriorPicsFlag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes related to DPB memory allocation and the derivation of the variable NoOutputOfPriorPicsFlag:

- The maximum values of chroma_format_idc and bit_depth_minus8 for all pictures of all layers are signalled in the VPS.

- The setting of the value of the variable NoOutputOfPriorPicsFlag is updated as follows:

  - To invole the maximum picture width and height values for all pictures of all layers signalled in the VPS instead of the values for a single layer.

  - To involve the maximum values of chroma_format_idc and bit_depth_minus8 for all pictures of all layers signalled in the VPS instead of the values for a single layer.

  - To not involve the value of the separate_colour_plane_flag.

- Both the semantics of no_output_of_prior_pics_flag and the use of this flag in the setting of NoOutputOfPriorPicsFlag are specified in an AU-specific manner (instead of in a PU-specific manner), and the value of no_output_of_prior_pics_flag, when present, is required to be the same for all pictures in an AU.

# Introduction

The following issues were observed in the existing scalability design in the latest VVC text (in JVET-Q2001-vE/v15):

- Currently, the maximum values of picture width and height for all pictures of all layers are signalled in the VPS, to enable the decoder to properly allocate the memory for the DPB. Like the picture width and height, the chroma format and the bit depth, currently specified by the SPS syntax elements chroma_format_idc and bit_depth_minus8, respectively, also affect the size for a picture storage buffer in the DPB. However, the maximum values of the chroma_format_idc and bit_depth_minus8 for all pictures of all layers are not signalled.

- Currently, the setting of the value of the variable NoOutputOfPriorPicsFlag has the following issues:

  - It involves the change of the value of pic_width_max_in_luma_samples or pic_height_max_in_luma_samples. However, the maximum values of picture width and height for all pictures of all layers should be used instead.

  - It involves the change of the value of chroma_format_idc or bit_depth_minus8. However, the maximum values of chroma format and bit depth for all pictures of all layers should be used instead.

  - It involves the change of the value of separate_colour_plane_flag. However, the separate_colour_plane_flag is only present and used when chroma_format_idc is equal to 3, which specifies the 4:4:4 chroma format, while for the 4:4:4 chroma format, the value of separate_colour_plane_flag being equal to 0 or 1 does not affect the buffer size needed for storing a decoded picture. Therefore, the setting of NoOutputOfPriorPicsFlag should not involve the change of the value of separate_colour_plane_flag.

- Currently, the no_output_of_prior_pics_flag is signalled in the PH for IRAP and GDR pictures, and both the semantics of this flag and the its use in the process for setting the value of NoOutputOfPriorPicsFlag are specified in a manner that no_output_of_prior_pics_flag is layer specific or PU specific. However, since the DPB operation is OLS specific or AU specific, both the semantics of no_output_of_prior_pics_flag and the use of this flag in the setting of NoOutputOfPriorPicsFlag should be specified in an AU-specific manner.

This contribution tries to address the above issues.

# Proposal

## Summary of the proposed changes

The following changes are proposed to address the above issues:

- The maximum values of chroma_format_idc and bit_depth_minus8 for all pictures of all layers are signalled in the VPS.

- The setting of the value of the variable NoOutputOfPriorPicsFlag is updated as follows:

  - To invole the maximum picture width and height values for all pictures of all layers signalled in the VPS instead of the values for a single layer.

  - To involve the maximum values of chroma_format_idc and bit_depth_minus8 for all pictures of all layers signalled in the VPS instead of the values for a single layer.

  - To not involve the value of the separate_colour_plane_flag.

- Both the semantics of no_output_of_prior_pics_flag and the use of this flag in the setting of NoOutputOfPriorPicsFlag are specified in an AU-specific manner (instead of in a PU-specific manner), and the value of no_output_of_prior_pics_flag, when present, is required to be the same for all pictures in an AU.

## Proposed text changes

The changed texts are based on the latest VVC text in JVET-Q2001-vE/v15. Most relevant parts that have been added or modified are highlighted in turquoise.

#### 7.3.2.2 Video parameter set syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| for( i = 0; i < TotalNumOlss; i++ ) { |  |
| if( NumLayersInOls[ i ] > 1 ) { |  |
| ols_dpb_pic_width[ i ] | ue(v) |
| ols_dpb_pic_height[ i ] | ue(v) |
| ols_dpb_chroma_format[ i ] | u(2) |
| ols_dpb_bitdepth_minus8[ i ] | ue(v) |
| if( vps_num_dpb_params > 1 ) |  |
| ols_dpb_params_idx[ i ] | ue(v) |
| } |  |
| } |  |
| ... |  |
| } |  |

...

#### 7.4.3.2 Video parameter set RBSP semantics

...

ols_dpb_pic_width[ i ] specifies the width, in units of luma samples, of each picture storage buffer for the i-th OLS.

ols_dpb_pic_height[ i ] specifies the height, in units of luma samples, of each picture storage buffer for the i-th OLS.

ols_dpb_chroma_format[ i ] specifies the greatest allowed value of chroma_format_idc for all SPSs that are referred to by CLVSs in the CVS for the i-th OLS.

ols_dpb_bitdepth_minus8[ i ] specifies the greatest allowed value of bit_depth_minus8 for all SPSs that are referred to by CLVSs in the CVS for the i-th OLS.

NOTE 2 – For decoding an OLS containing more than one layer and having OLS index i, the deoder can safely allocate memory for the DPB according to the values of the sntax elements ols_dpb_pic_width[ i ], ols_dpb_pic_height[ i ], ols_dpb_chroma_format[ i ], and ols_dpb_bitdepth_minus8[ i ].

ols_dpb_params_idx[ i ] specifies the index, to the list of dpb_parameters( ) syntax structures in the VPS, of the dpb_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. When present, the value of ols_dpb_params_idx[ i ] shall be in the range of 0 to vps_num_dpb_params − 1, inclusive. When ols_dpb_params_idx[ i ] is not present, the value of ols_dpb_params_idx[ i ] is inferred to be equal to 0.

When NumLayersInOls[ i ] is equal to 1, the dpb_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

...

#### 7.4.3.3 Sequence parameter set RBSP semantics

...

gdr_enabled_flag equal to 1 specifies that GDR pictures may be present in CLVSs referring to the SPS. gdr_enabled_flag equal to 0 specifies that GDR pictures are not present in CLVSs referring to the SPS.

chroma_format_idc specifies the chroma sampling relative to the luma sampling as specified in clause 6.2.

When sps_video_parameter_set_id is greater than 0, it is a requirement of bitstream conformance that, for any OLS with OLS index i that contains one or more layers that refers to the SPS, the value of chroma_format_idc shall be less than or equal to the value of ols_dpb_chroma_format[ i ].

...

bit_depth_minus8 specifies the bit depth of the samples of the luma and chroma arrays, BitDepth, and the value of the luma and chroma quantization parameter range offset, QpBdOffset, as follows:

BitDepth      = 8 + bit_depth_minus8		(45)

QpBdOffset = 6 * bit_depth_minus8		(46)

bit_depth_minus8 shall be in the range of 0 to 8, inclusive.

When sps_video_parameter_set_id is greater than 0, it is a requirement of bitstream conformance that, for any OLS with OLS index i that contains one or more layers that refers to the SPS, the value of bit_depth_minus8 shall be less than or equal to the value of ols_dpb_bitdepth_minus8[ i ].

...

#### 7.4.3.7 Picture header structure semantics

...

no_output_of_prior_pics_flag affects the output of previously-decoded pictures in the DPB after the decoding of a picture in a CVSS AU that is not the first AU in the bitstream as specified in Annex C.

It is a requirement of bitstream conformance that, when present, the value of no_output_of_prior_pics_flag shall be the same for all pictures of an AU.

When no_output_of_prior_pics_flag is present in the PHs of the pictures of an AU, the value of no_output_of_prior_pics_flag of the AU is the value of no_output_of_prior_pics_flag of the pictures of the AU.

...

#### C.1 General

...

For each bitstream conformance test, the CPB size (number of bits) is CpbSize[ Htid ][ ScIdx ] as specified in clause 7.4.6.3, where ScIdx and the HRD parameters are specified above in this clause, and the DPB parameters max_dec_pic_buffering_minus1[ Htid ], max_num_reorder_pics[ Htid ], and MaxLatencyPictures[ Htid ] are found in or derived from the dpb_parameters( ) syntax structure that applies to the target OLS as follows:

–	If NumLayersInOls[ TargetOlsIdx ] is equal to 1, the dpb_parameters( ) syntax structure is found in the SPS referred to be the layer in the target OLS, and the variables PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, MaxChromaFormat, and MaxBitDepthMinus8 are set equal to pic_width_max_in_luma_samples, pic_height_max_in_luma_samples, chroma_format_idc, and bit_depth_minus8, respectively, found in the SPS referred to by the layer in the target OLS.

–	Otherwise (the target OLS contains more than one layer), the dpb_parameters( ) is identified by ols_dpb_params_idx[ TargetOlsIdx ] found in the VPS, and the variables PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, MaxChromaFormat, and MaxBitDepthMinus8 are set equal to ols_dpb_pic_width[ TargetOlsIdx ], ols_dpb_pic_height[ TargetOlsIdx ], ols_dpb_chroma_format[ TargetOlsIdx ], and ols_dpb_bitdepth_minus8[ TargetOlsIdx ], respectively, found in the VPS.

...

#### C.3.2 Removal of pictures from the DPB before decoding of the current picture

The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously at the CPB removal time of the first DU of AU n (containing the current picture) and proceeds as follows:

–	The decoding process for reference picture list construction as specified in clause 8.3.2 is invoked and the decoding process for reference picture marking as specified in clause 8.3.3 is invoked.

–	When the current AU is a CVSS AU that is not AU 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the value of PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, MaxChromaFormat, MaxBitDepthMinus8, or max_dec_pic_buffering_minus1[ Htid ] derived for the current AU is different from the value of PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, MaxChromaFormat, MaxBitDepthMinus8, or max_dec_pic_buffering_minus1[ Htid ], respectively, derived for the preceding AU in decoding order, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag of the current AU.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag of the current AU is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag of the current AU.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD, such that when the value of NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain, and the DPB fullness is set equal to 0.

–	When both of the following conditions are true for any pictures k in the DPB, all such pictures k in the DPB are removed from the DPB:

–	picture k is marked as "unused for reference".

–	picture k has PictureOutputFlag equal to 0 or its DPB output time is less than or equal to the CPB removal time of the first DU (denoted as DU m) of the current picture n; i.e., DpbOutputTime[ k ] is less than or equal to DuCpbRemovalTime[ m ].

–	For each picture that is removed from the DPB, the DPB fullness is decremented by one.

#### C.5.2.2 Output and removal of pictures from the DPB

The output and removal of pictures from the DPB before the decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously when the first DU of the AU containing the current picture is removed from the CPB and proceeds as follows:

–	The decoding process for reference picture list construction as specified in clause 8.3.2 and decoding process for reference picture marking as specified in clause 8.3.3 are invoked.

–	If the current AU is a CVSS AU that is not AU 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the value of PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, MaxChromaFormat, MaxBitDepthMinus8, or max_dec_pic_buffering_minus1[ Htid ] derived for the current AU is different from the value of PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, MaxChromaFormat, MaxBitDepthMinus8, or max_dec_pic_buffering_minus1[ Htid ], respectively, derived for the preceding AU in decoding order, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag of the current AU.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag of the current AU is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag of the current AU.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD as follows:

–	If NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB are emptied without output of the pictures they contain and the DPB fullness is set equal to 0.

–	Otherwise (NoOutputOfPriorPicsFlag is equal to 0), all picture storage buffers containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output) and all non-empty picture storage buffers in the DPB are emptied by repeatedly invoking the "bumping" process specified in clause C.5.2.4 and the DPB fullness is set equal to 0.

–	Otherwise (the current picture is not a CLVSS picture or the CLVSS picture is picture 0), all picture storage buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). For each picture storage buffer that is emptied, the DPB fullness is decremented by one. When one or more of the following conditions are true, the "bumping" process specified in clause C.5.2.4 is invoked repeatedly while further decrementing the DPB fullness by one for each additional picture storage buffer that is emptied, until none of the following conditions are true:

- The number of pictures in the DPB that are marked as "needed for output" is greater than max_num_reorder_pics[ Htid ].

- max_latency_increase_plus1[ Htid ] is not equal to 0 and there is at least one picture in the DPB that is marked as "needed for output" for which the associated variable PicLatencyCount is greater than or equal to MaxLatencyPictures[ Htid ].

- The number of pictures in the DPB is greater than or equal to max_dec_pic_buffering_minus1[ Htid ] + 1.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).