| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0068-v1 |
| --- | --- |

| Title: | AHG8/AHG9/AHG12: Miscellaneous HLS topics |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Li Zhang1, Zhipin Deng2 Jizheng Xu1, Kai Zhang1, Kui Fan1 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com lizhang.idm@bytedance.com zhipin.deng@bytedance.com xujizheng@bytedance.com zhangkai.video@bytedance.com kui.fan@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes:

- Add the following two conditions to the constraint on when the value of slice_type shall be equal to 2: i) when intra_only_constraint_flag is equal to 1; and ii) when the NAL unit type is an IRAP NAL unit type and the current picture is the first picture in the current AU.

- Signal the level information for the IRAP-only representation, which consists of only the IRAP pictures.

- Update the semantics of sps_affine_amvr_enabled_flag equal to 1 to use the wording of "may be used" instead of "is used".

- Update the semantics of ph_alf_enabled_flag equal to 0 to use the wording of "is disabled for all slices" instead of "may be disabled for one, or more, or all slices".

- Use the subpicture index instead of the subpicture ID in the subpicture sub-bitstream extraction process.

- Require the value of pps_conformance_window_flag to be equal to 0 when the picture width and height are the maximum picture width and height, and infer the values of the PPS conformance window syntax elements to be the same as those signalled in the SPS if the picture width and height are the maximum picture width and height and to be equal to 0 otherwise.

- Change the syntax name of init_qp_minus26 to init_qp_minus32 and update the semantics accordingly.

- Condition slice_ts_residual_coding_disabled_flag based on sps_transform_skip_enabled_flag and infer slice_ts_residual_coding_disabled_flag to be equal 1 when not present.

- Signal five_minus_max_num_affine_merge_cand instead of five_minus_max_num_subblock_merge_cand when sps_affine_enabled_flag is equal to 1, and MaxNumSubblockMergeCand is derived as a sum of the maximum allowed number of sbTMVP candidates and the maximum allowed number of affine candidates.

# On slice_type constraint

The following constraint:

When nal_unit_type is in the range of IDR_W_RADL to CRA_NUT, inclusive, and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, slice_type shall be equal to 2.

is proposed to be changed as follows (because in the added conditions the value of slice_type also has to be equal to 2):

When intra_only_constraint_flag is equal to 1 or both of the following conditions are true, the value of slice_type shall be equal to 2:

- The value of nal_unit_type is in the range of IDR_W_RADL to CRA_NUT, inclusive.

- The value of vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1 or the current picture is the first picture in the current AU.

# On level information for the IRAP-only representation

In the intra-only trick mode (for fast forward, for example) used by some applications, the decoder only decodes the IRAP-only representation, which consists of only the IRAP pictures. It is therefore proposed to signal the level information for the IRAP-only representation.

The proposed changes for this purpose are provided below.

#### PTL syntax and semantics

| profile_tier_level( profileTierPresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( profileTierPresentFlag ) { |  |
| general_profile_idc | u(7) |
| general_tier_flag | u(1) |
| general_constraint_info( ) |  |
| } |  |
| general_level_idc | u(8) |
| if( profileTierPresentFlag ) { |  |
| num_sub_profiles | u(8) |
| for( i = 0; i < num_sub_profiles; i++ ) |  |
| general_sub_profile_idc[ i ] | u(32) |
| } |  |
| for( i = 0; i <= maxNumSubLayersMinus1; i++ ) |  |
| sublayer_level_present_flag[ i ] | u(1) |
| while( !byte_aligned( ) ) |  |
| ptl_alignment_zero_bit | f(1) |
| for( i = 0; i <= maxNumSubLayersMinus1; i++ ) |  |
| if( sublayer_level_present_flag[ i ] ) |  |
| sublayer_level_idc[ i ] | u(8) |
| } |  |

...

sublayer_level_present_flag[ i ] equal to 1, when i is greater than 0, specifies that level information is present in the profile_tier_level( ) syntax structure for the sublayer representation with TemporalId equal to i − 1, and, when i is equal to 0, specifies that level information is present in the profile_tier_level( ) syntax structure for the IRAP-only representation that consists of only the IRAP pictures and the associated non-VCL NAL units. sublayer_level_present_flag[ i ] equal to 0, when i is greater than 0, specifies that level information is not present in the profile_tier_level( ) syntax structure for the sublayer representation with TemporalId equal to i − 1, and, when i is equal to 0, specifies that level information is not present in the profile_tier_level( ) syntax structure for the IRAP-only representation.

ptl_alignment_zero_bits shall be equal to 0.

The semantics of the syntax element sublayer_level_idc[ i ] is, apart from the specification of the inference of not present values, the same as the syntax element general_level_idc, but apply to the sublayer representation with TemporalId equal to i − 1 when i is greater than 0, and apply to the IRAP-only representation when i is equal to 0.

When not present, the value of sublayer_level_idc[ i ] is inferred as follows:

–	sublayer_level_idc[ maxNumSubLayersMinus1 + 1 ] is inferred to be equal to general_level_idc of the same profile_tier_level( ) structure,

–	For i from maxNumSubLayersMinus1 − 1 to 0 (in decreasing order of values of i), inclusive, sublayer_level_idc[ i ] is inferred to be equal to sublayer_level_idc[ i + 1 ].

...

#### A.1	Overview of profiles, tiers and levels

...

For each operation point identified by TargetOlsIdx and Htid, the profile, tier, and level information is indicated through general_profile_idc and general_tier_flag in the VPS, and sublayer_level_idc[ Htid + 1 ] found in or derived from the VPS.

When no VPS is available, the profile and tier information is indicated through general_profile_idc and general_tier_flag in the SPS, and the level information is indicated as follows:

- If Htid is provided by external means indicating the highest TemporalId of any NAL unit in the bitstream, level information is indicated through sublayer_level_idc[ Htid + 1 ] found in or derived from the SPS.

- Otherwise (Htid is not provided by external means), level information is indicated through general_level_idc in the SPS.

...

# On the semantics of sps_affine_amvr_enabled_flag

The semantics of sps_affine_amvr_enabled_flag is proposed to be updated as follows:

sps_affine_amvr_enabled_flag equal to 1 specifies that adaptive motion vector difference resolution is may be used in motion vector coding of affine inter mode. sps_affine_amvr_enabled_flag equal to 0 specifies that adaptive motion vector difference resolution is not used in motion vector coding of affine inter mode. When not present, the value of sps_affine_amvr_enabled_flag is inferred to be equal to 0.

# On the semantics of ph_alf_enabled_flag

The semantics of ph_alf_enabled_flag is proposed to be updated as follows:

ph_alf_enabled_flag equal to 0 specifies that adaptive loop filter is may be disabled for one, or more, or all slices associated with the PH.

# On the subpicture sub-bitstream extraction process

Currently, the subpicture sub-bitstream extraction process takes as an input an array of target subpicture ID values for each layer. This would work when the subpicture ID mapping is signalled in the SPS and thus is static within a CLVS. However, when the subpicture ID mapping is signalled in the PPS instead and can be dynamic within a CLVS, there would be a problem as the sub-bitstream extraction is supposed to extract a sequence of collocated subpictures.

Therefore, it is proposed to use the subpicture index instead of the subpicture ID in the subpicture sub-bitstream extraction process.

The proposed text changes are shown below.

#### C.7 Subpicture sub-bitstream extraction process

Inputs to this process are a bitstream inBitstream, a target OLS index targetOlsIdx, a target highest TemporalId value tIdTarget, and an array of target subpicture ID index values for each layer subpicIdxTarget[ ].

Output of this process is a sub-bitstream outBitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that satisfies all of the following conditions shall be a conforming bitstream:

–	The output sub-bitstream is the output of the process specified in this clause with the bitstream, targetOlsIdx equal to an index to the list of OLSs specified by the VPS, and subpicIdxTarget[ ] equal to subpicture IDs present in the OLS, as inputs.

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in LayerIdInOls[ targetOlsIdx ].

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

NOTE – A conforming bitstream contains one or more coded slice NAL units with TemporalId equal to 0, but does not have to contain coded slice NAL units with nuh_layer_id equal to 0.

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to LayerIdInOls[ targetOlsIdx ][ i ] and with slice_subpic_id equal to the value in subpicIdxTarget[ i ] for each i in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1.

The output sub-bitstream outBitstream is derived as follows:

–	The sub-bitstream extraction process, specified in Annex C.6, is invoked with inBitstream, targetOlsIdx, and tIdTarget as inputs and the output of the process is assigned to outBitstream.

–	If some external means not specified in this Specification is available to provide replacement parameter sets for the sub-bitstream outBitstream, replace all parameter sets with the replacement parameter sets.

–	Otherwise, when subpicture level information SEI messages are present in inBitstream, the following applies:

–	Rewrite the value of general_level_idc in the ols_ptl_idx[ targetOlsIdx ]-th entry in the list of profile_tier_level() syntax structures in all referred to VPS NAL units to be equal to SubpicSetLevelIdc derived in D.3.8 for the set of subpictures consisting oof the subpictures with subpicture ID equal to all entries in subpicIdxTarget[ ].

–	Rewrite the values of cpb_size_value_minus1[ tIdTarget ][ j ] and bit_rate_value_minus1[ tIdTarget ][ j ] of the j-th CPB in the ols_hrd_idx[ targetOlsIdx ]-th entry in the list of ols_hrd_parameters() syntax structures in all referred to VPS NAL units to correspond to SubpicSetCpbSizeVcl[ 0 ], SubpicSetCpbSizeNal[ 0 ], SubpicSetBitrateVcl[ 0 ], and SubpicSetBitrateNal[ 0 ] as derived in D.3.8 for the set of subpictures consisting of the subpictures with subpicture ID equal to all entries in subpicIdxTarget[ ] and j in the range of 0 to hrd_cpb_cnt_minus1.

For the i-th layer with i in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1, the following applies.

–	subpicIdx is set equal to the value of subpicIdxTarget[ i ].

–	Rewrite the value of general_level_idc in the profile_tier_level() syntax structure in all referred to SPS NAL units with sps_ptl_dpb_hrd_params_present_flag equal to 1 to be equal to SubpicSetLevelIdc derived in D.3.8 for the set of subpictures consisting of the subpicture with subpicture ID index equal to subpicIdx.

–	Rewrite the values of cpb_size_value_minus1[ tIdTarget ][ j ] and bit_rate_value_minus1[ tIdTarget ][ j ] of the j-th CPB in the ols_hrd_idx[ targetOlsIdx ]-th entry in the list of ols_hrd_parameters() syntax structures in all referred to SPS NAL units to correspond to SubpicCpbSizeVcl[ 0 ][ SubpicIdxList[ subPicIdx ] ], SubpicCpbSizeNal[ 0 ][ SubpicIdxList[ subPicIdx ] ], SubpicBitrateVcl[ 0 ][ SubpicIdxList[ subPicIdx ] ], and SubpicBitrateNal[ 0 ][ SubpicIdxList[ subPicIdx ] ] as derived in D.3.8 for the subpicture with subpicture ID index equal to subpicIdx and j in the range of 0 to hrd_cpb_cnt_minus1.

–	Rewrite the value of the pic_width_max_in_luma_samples and pic_height_max_in_luma_samples in all referred to SPS NAL units and the value of pic_width_in_luma_samples and pic_height_in_luma_samples in all referred to PPS NAL units to be equal to subpic_width_minus1[ SubpicIdx [ subPicIdx ] ] and subpic_height_minus1[ SubpicIdxList[ subPicIdx ] ].

–	Rewrite the value of sps_num_subpics_minus1 in all referred to SPS NAL units and pps_num_subpics_minus1 in all referred to PPS NAL units to 1.

–	Rewrite the syntax elements subpic_ctu_top_left_x[ SubpicIdxList[ subPicIdx ] ], subpic_ctu_top_left_y[ SubpicIdx [ subPicIdx ] ], when present, in all referred to SPS NAL units to 0.

–	Remove the syntax elements subpic_ctu_top_left_x[ j ], subpic_ctu_top_left_y[ j ], subpic_width_minus1[ j ], subpic_height_minus1[ j ], subpic_treated_as_pic_flag[ j ], loop_filter_across_subpic_enabled_flag[ j ], and sps_subpic_id[ j ] in all referred to SPS NAL units and for each j for which SubpicIdx [ j ] is not equal to subPicIdx.

–	Rewrite the tiling and slice structure related syntax elements in all referred to PPS to remove all tiles rows, tile columns and slices that are not associated to the subpicture with a subpicture ID equal to subPicIdx.

–	Remove from outBitstream all VCL NAL units with nuh_layer_id equal to nuh_layer_id of the i-th layer and with slice_subpic_id not equal to subPicIdx.

–	When sli_cbr_constraint_flag is equal to 1, remove all NAL units with nal_unit_type equal to FD_NUT and filler payload SEI messages that are not associated with the VCL NAL units of a subpicture in subpicIdxTarget[ ] and set cbr_flag[ tIdTarget ][ j ] equal to 1 of the j-th CPB in the ols_hrd_idx[ targetOlsIdx ]-th entry in the list of ols_hrd_parameters() syntax structures in all referred to VPS NAL units and SPS NAL units and j in the range of 0 to hrd_cpb_cnt_minus1. Otherwise, (sli_cbr_constraint_flag is equal to 0), remove all NAL units with nal_unit_type equal to FD_NUT and filler payload SEI messages and set cbr_flag[ tIdTarget ][ j ] equal to 0.

–	When outBitstream contains SEI NAL units that contain a scalable nesting SEI message with nesting_ols_flag equal to 1 and nesting_subpic_flag equal to 1 that are applicable to outBitstream, extract appropriate non-scalable-nested SEI message with payloadType equal to 1 (picture timing), or 130 (decoding unit information) from the scalable nesting SEI message and place the extracted SEI messages into outBitstream.

# On signalling of conformance window parameters

Currently, the conformance window parameters are always signalled in the PPS, including when the picture width and height are identical to the maximum picture width and height signalled in the SPS referenced by the PPS. On the other hand, the conformance window parameters for pictures with the maximum picture width and height are also signalled in the SPS. The signalling of the conformance window parameters for pictures with the maximum picture width and height in the PPS is redundant.

It is therefore proposed to require the value of pps_conformance_window_flag to be equal to 0 when the picture width and height are the maximum picture width and height, and infer the values of the PPS conformance window syntax elements to be the same as those signalled in the SPS if the picture width and height are the maximum picture width and height and to be equal to 0 otherwise.

The proposed text changes are provided below.

#### 7.4.3.4	Picture parameter set RBSP semantics

...

pps_conformance_window_flag equal to 1 specifies indicates that the conformance cropping window offset parameters follow next in the PPS. pps_conformance_window_flag equal to 0 specifies indicates that the conformance cropping window offset parameters are not present in the PPS. When pic_width_in_luma_samples is equal to pic_width_max_in_luma_samples and pic_height_in_luma_samples is equal to pic_height_max_in_luma_samples, the value of pps_conformance_window_flag shall be equal to 0.

pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset specify the samples of the pictures in the CLVS that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output.

When pps_conformance_window_flag is equal to 0, the following applies:

–	If pic_width_in_luma_samples is equal to pic_width_max_in_luma_samples and pic_height_in_luma_samples is equal to pic_height_max_in_luma_samples, the values of pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset are inferred to be equal to sps_conf_win_left_offset, sps_conf_win_right_offset, sps_conf_win_top_offset, and sps_conf_win_bottom_offset, respectively.

–	Otherwise, the values of pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset are inferred to be equal to 0.

The conformance cropping window contains the luma samples with horizontal picture coordinates from SubWidthC * pps_conf_win_left_offset to pic_width_in_luma_samples − ( SubWidthC * pps_conf_win_right_offset + 1 ) and vertical picture coordinates from SubHeightC * pps_conf_win_top_offset to pic_height_in_luma_samples − ( SubHeightC * pps_conf_win_bottom_offset + 1 ), inclusive.

The value of SubWidthC * ( pps_conf_win_left_offset + pps_conf_win_right_offset ) shall be less than pic_width_in_luma_samples, and the value of SubHeightC * ( pps_conf_win_top_offset + pps_conf_win_bottom_offset ) shall be less than pic_height_in_luma_samples.

When ChromaArrayType is not equal to 0, the corresponding specified samples of the two chroma arrays are the samples having picture coordinates ( x / SubWidthC, y / SubHeightC ), where ( x, y ) are the picture coordinates of the specified luma samples.

NOTE 2 – The conformance cropping window offset parameters are only applied at the output. All internal decoding processes are applied to the uncropped picture size.

Let ppsA and ppsB be any two PPSs referring to the same SPS. It is a requirement of bitstream conformance that, when ppsA and ppsB have the same the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, ppsA and ppsB shall have the same values of pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset, respectively.

When pic_width_in_luma_samples is equal to pic_width_max_in_luma_samples and pic_height_in_luma_samples is equal to pic_height_max_in_luma_samples, it is a requirement of bitstream conformance that pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset, are equal to sps_conf_win_left_offset, sps_conf_win_right_offset, sps_conf_win_top_offset, and sps_conf_win_bottom_offset, respectively.

...

# On initial base QP in PPS

Currently, a syntax element named init_qp_minus26 is used to represent the initial base QP in the PPS. As the name indicates, the base QP will be ( 26 + init_qp_minus26 ). Considering that the base QP range has been extended to [0, 63], it is proposed to update the syntax to be in the middle of the range.

The proposed text changes are as follows:

#### 7.3.2.4	Picture parameter set syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| init_qp_minus2632 | se(v) |
| …… |  |

#### 7.4.3.4	Picture parameter set RBSP semantics

…

init_qp_minus2632 plus 2632 specifies the initial value of SliceQpY for each slice referring to the PPS. The initial value of SliceQpY is modified at the picture level when a non-zero value of ph_qp_delta is decoded or at the slice level when a non-zero value of slice_qp_delta is decoded. The value of init_qp_minus2632 shall be in the range of −( 2632 + QpBdOffset ) to +37, inclusive.

…

#### 7.4.3.7 Picture header structure semantics

…

When qp_delta_info_in_ph_flag is equal to 1, the initial value of the QpY quantization parameter for all slices of the picture, SliceQpY, is derived as follows:

SliceQpY = 2632 + init_qp_minus2632 + ph_qp_delta	(88)

The value of SliceQpY shall be in the range of −QpBdOffset to +63, inclusive.

…

#### 7.4.8.1 General slice header semantics

…

When qp_delta_info_in_ph_flag is equal to 0, the initial value of the QpY quantization parameter for the slice, SliceQpY, is derived as follows:

SliceQpY = 2632 + init_qp_minus2632 + slice_qp_delta	(143)

The value of SliceQpY shall be in the range of −QpBdOffset to +63, inclusive.

…

# On the signalling of slice_ts_residual_coding_disabled_flag

We observed the following two issues about the signalling of slice_ts_residual_coding_disabled_flag:

- Currently, for a transform unit, whether to use regular residual coding or not is conditioned on if( !transform_skip_flag[ ][ ][ ] | | slice_ts_residual_coding_disabled_flag ). However, when the transform skip is disabled, transform_skip_flag is equal to 0 and the regular residual coding will be used, regardless of the value of slice_ts_residual_coding_disabled_flag. Consequencely, when sps_transform_skip_enabled_flag is equal to 0, the signalling of  slice_ts_residual_coding_disabled_flag in the SH is redundent.

- Currently, slice_ts_residual_coding_disabled_flag is signalled in the SH without being conditioned. However, the semantics are worded as follows: When slice_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 0. It is confusing since the inference is not needed when always signalled.

It is therefore proposed to signal slice_ts_residual_coding_disabled_flag conditioned on the value of sps_transform_skip_enabled_flag, and infer the value of slice_ts_residual_coding_disabled_flag to be equal to 1 when not present.

The proposed text changes are provided below.

#### 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !sps_transform_skip_enabled_flag ) |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| … |  |

#### 7.4.8.1General slice header semantics

…

slice_ts_residual_coding_disabled_flag equal to 1 specifies that the residual_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. slice_ts_residual_coding_disabled_flag equal to 0 specifies that the residual_ts_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. When slice_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 01.

…

# On the derivation of MaxNumSubblockMergeCand

In the current VVC text, the maximum number of subblock-based merging MVP candidates, MaxNumSubblockMergeCand, is derived conditioned by whether sps_affine_enabled_flag is true or not, as follows:

if( sps_affine_enabled_flag )
	MaxNumSubblockMergeCand = 5 − five_minus_max_num_subblock_merge_cand	(87)
else
	MaxNumSubblockMergeCand = sps_sbtmvp_enabled_flag  &&  ph_temporal_mvp_enabled_flag

The value of MaxNumSubblockMergeCand shall be in the range of 0 to 5, inclusive.

The syntax element five_minus_max_num_subblock_merge_cand is signalled in SPS, conditioned by whether sps_affine_enabled_flag is true or not, as follows:

#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| … |  |

In VVC, there are two kinds of subblock-based merging candidates: sbTMVP merging candidate and affine merging candidate. It is not reasoanble in logic to signal five_minus_max_num_subblock_merge_cand only when affine prediction is enabled.

As a cleanup, it is proposed to signal five_minus_max_num_affine_merge_cand instead of five_minus_max_num_subblock_merge_cand when sps_affine_enabled_flag is equal to 1, and MaxNumSubblockMergeCand is derived as a sum of the maximum allowed number of sbTMVP candidates and the maximum allowed number of affine candidates.

The proposed text changes are as follows:

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| five_minus_max_num_subblock_merge_cand five_minus_max_num_affine_merge_cand | ue(v) |
| … |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

five_minus_max_num_subblock_merge_cand specifies the maximum number of subblock-based merging motion vector prediction candidates supported in the SPS subtracted from 5. The value of five_minus_max_num_subblock_merge_cand shall be in the range of 0 to 5, inclusive.

five_minus_max_num_affine_merge_cand specifies the maximum number of affine merging motion vector prediction candidates supported in the SPS subtracted from 5. The value of five_minus_max_num_affine_merge_cand shall be in the range of 0 to 5, inclusive. When not presented, five_minus_max_num_affine_merge_cand is set equal to 5.

#### 7.4.3.7 Picture header structure semantics

ph_temporal_mvp_enabled_flag specifies whether temporal motion vector predictors can be used for inter prediction for slices associated with the PH. If ph_temporal_mvp_enabled_flag is equal to 0, the syntax elements of the slices associated with the PH shall be constrained such that no temporal motion vector predictor is used in decoding of the slices. Otherwise (ph_temporal_mvp_enabled_flag is equal to 1), temporal motion vector predictors may be used in decoding of the slices associated with the PH. When not present, the value of ph_temporal_mvp_enabled_flag is inferred to be equal to 0. When no reference picture in the DPB has the same spatial resolution as the current picture, the value of ph_temporal_mvp_enabled_flag shall be equal to 0.

The maximum number of subblock-based merging MVP candidates, MaxNumSubblockMergeCand, is derived as follows:

if( sps_affine_enabled_flag )
	MaxNumSubblockMergeCand = 5 − five_minus_max_num_subblock_merge_cand	(87)
else
	MaxNumSubblockMergeCand = sps_sbtmvp_enabled_flag  &&  ph_temporal_mvp_enabled_flag

The value of MaxNumSubblockMergeCand shall be in the range of 0 to 5, inclusive.

MaxNumSubblockMergeCand = Min( 5, ( sps_sbtmvp_enabled_flag  &&	(87)
		ph_temporal_mvp_enable_flag ) + 5 − five_minus_max_num_affine_merge_cand )

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).