| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0069-v1 |
| --- | --- |

| Title: | AHG12: Control of loop filtering across subpicture/tile/slice boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang, Ye-Kui Wang, Kai Zhang (Bytedance) 8910 University Center Lane San Diego, CA 92122, USA Hendry, Naeri Park, Hyeongmun Jang, Junghak Nam, Seung Hwan Kim, Jaehyun Lim (LG Electronics) | Tel: Email: | lizhang.idm@bytedance.com yekui.wang@bytedance.com zhangkai.video@bytedance.com dr.hendry@lge.com naeri.park@lge.com |
| Source: | Bytedance Inc. and LG Electronics |  |  |

_____________________________

# Abstract

The meeting minutes of the 17th JVET meeting includes the following notes:

- JVET-Q0120: Remove loop_filter_across_slices_enabled_flag of the PPS. It was agreed that we plan to do this unless some reasonable justification to retain it is provided by the next meeting.

- In the discussion of JVET-Q0120, there was a suggestion to move loop_filter_across_tiles_enabled_flag from PPS to SPS rather than having control per subpicture. It was agreed that we plan to do this unless some reasonable justification to retain it is provided by the next meeting.

And if so, it was suggested that this flag might only control filtering within subpictures. This question was left open for determination at the next meeting.

This contribution proposes to confirm the two conditionally agreed items, i.e., the following:

- Remove the PPS flag loop_filter_across_slices_enabled_flag.

- Move loop_filter_across_tiles_enabled_flag from PPS to SPS. Note that this flag is not signalled for each subpicture.

This contribution also proposes to close the topic that was left open for determination at this meeting. It is proposed that the loop_filter_across_tiles_enabled_flag moved from PPS to SPS is renamed to be loop_filter_across_boundaries_within_subpics_enabled_flag, and it is used to control whether loop filters are applied across tile and slice boundaries inside subpictures (not to control filtering across tile and slice boundaries that are also subpicture boundaries).

- Introduction

In the latest VVC draft text, in-loop filtering across subpicture, tile, and slice boundaries is controlled by the following syntax elements:

- loop_filter_across_subpic_enabled_flag[ i ]: for controlling of deblocking, SAO, and ALF across subpicture boundaries, signalled in the SPS, one for each subpicture.

- loop_filter_across_tiles_enabled_flag: for controlling of deblocking, SAO, and ALF across tile boundaries, signalled in the PPS, just one (thus applicable to all tiles in all pictures referring to the PPS).

- loop_filter_across_slices_enabled_flag: for controlling of deblocking, SAO, and ALF across slice boundaries, signalled in the PPS, just one (thus applicable to all slices in all pictures referring to the PPS).

They work as follows: for any particular boundary, whenever one of the above flags indicates that filtering across the boundary is turned off, regardless of the values of other flags, filtering across the boundary is turned off.

The following issues regarding in-loop filtering across subpicture, tile, and slice boundaries were overserved:

- The different flags for controlling of in-loop filtering across subpicture, tile, and slice boundaries can be somewhat conflicting with each other. For example, it is possible that the encoder chooses to turn on filtering across the boundaries of some independently coded subpictures as it found that otherwise those subpictures being presented together has very annoying artifacts. However, in this case, if the loop_filter_across_tiles_enabled_flag or loop_filter_across_slices_enabled_flag is equal to 0, then filtering across the subpicture boundaries that are also tile or slice boundaries would be turned off anyway.

- The main (if not only) purpose of turning off filtering across tile boundaries is for reducing data exchange between different processing cores in parallel encoding. Such a purpose usually won't change across pictures, thus while it makes sense to signal tile layout in the PPS for load balancing purpose, signalling of the loop_filter_across_tiles_enabled_flag in the SPS makes more sense.

- The subpicture feature introduced to VVC was mainly for support of region-of-interest and other extraction-based use cases. For AVC, such use cases were supported by using slices or slices groups. For HEVC, these use cases were supported by slices and MCTSs. Therefore, the ability to turn off filtering across slice boundaries was needed in AVC and HEVC. However, in VVC, such need is gone due to the support of subpictures. Therefore, there is no need to have a separate flag to control filtering across slice boundaries apart from tiles.

- Proposal

To resolve the above issues, this contribution proposes the following summarized changes:

- The PPS flag loop_filter_across_slices_enabled_flag is removed.

- The PPS flag loop_filter_across_tiles_enabled_flag is moved to the SPS and renamed as loop_filter_across_boundaries_within_subpics_enabled_flag. This flag is used to control whether filtering across tile and slice boundaries is on or off for boundaries that are within subpictures but not for boundaries that are also subpicture boundaries.

- Proposed spec text changes

The proposed text changes on top of the latest VVC draft text are provided below. The most relevant parts that have been added or modified are highlighted in turquoise, and the most relevant removed parts are highlighted in yellow strikethrough in red fonts. There are some other changes that are editorial in nature and thus not highlighted.

7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| loop_filter_across_boundaries_within_subpics_enabled_flag | u(1) |
| sps_entropy_coding_sync_enabled_flag | u(1) |
| if( sps_entropy_coding_sync_enabled_flag ) |  |
| sps_wpp_entry_point_offsets_present_flag | u(1) |
| ... |  |
| } |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

...

loop_filter_across_boundaries_within_subpics_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across tile and slice boundaries within subpictures in each coded picture in the CLVS. Herein tile boundaries and slice boundaries within subpictures do not include tile boundaries and slice boundaries that are also subpicture boundaries. loop_filter_across_boundaries_within_subpics_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across tile boundaries and slice boundaries within subpictures in each coded picture in the CLVS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When there are no tile boundaries and no slice boundaries within subpictures, the value of loop_filter_across_boundaries_within_subpics_enabled_flag is ignored.

...

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| ... | u(1) |
| } |  |

#### 7.4.3.4 Picture parameter set RBSP semantics

...

loop_filter_across_tiles_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across tile boundaries in pictures referring to the PPS. loop_filter_across_tiles_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across tile boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of loop_filter_across_tiles_enabled_flag is inferred to be equal to 1.

loop_filter_across_slices_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across slice boundaries in pictures referring to the PPS. loop_filter_across_slice_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across slice boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of loop_filter_across_slices_enabled_flag is inferred to be equal to 0.

...

### 8.8.3 Deblocking filter process

#### 8.8.3.1 General

...

The deblocking filter process is applied to all coding subblock edges and transform block edges of a picture, except the following types of edges:

- Edges that are at the boundary of the picture,

- Edges that coincide with the boundaries of a subpicture with subpicture index subpicIdx and loop_filter_across_subpic_enabled_flag[ subpicIdx ] is equal to 0,

- Edges that coincide with the virtual boundaries of the picture when VirtualBoundariesPresentFlag is equal to 1,

- Edges that coincide with tile boundaries or slice boundaries within subpictures when loop_filter_across_tiles_enabled_flag loop_filter_across_boundaries_within_subpics_enabled_flag is equal to 0,

- Edges that coincide with slice boundaries when loop_filter_across_slices_enabled_flag is equal to 0,

- Edges that coincide with upper or left boundaries of slices with slice_deblocking_filter_disabled_flag equal to 1,

- Edges within slices with slice_deblocking_filter_disabled_flag equal to 1,

- Edges that do not correspond to 4×4 sample grid boundaries of the luma component,

- Edges that do not correspond to 8×8 sample grid boundaries of the chroma component,

- Edges within the luma component for which both sides of the edge have intra_bdpcm_luma_flag equal to 1,

- Edges within the chroma components for which both sides of the edge have intra_bdpcm_chroma_flag equal to 1,

- Edges of chroma subblocks that are not edges of the associated transform unit.

...

#### 8.8.3.2 Deblocking filter process for one direction

...

- The variable filterEdgeFlag is derived as follows:

- If edgeType is equal to EDGE_VER and one or more of the following conditions are true, filterEdgeFlag is set equal to 0:

- The left boundary of the current coding block is the left boundary of the picture.

- The left boundary of the current coding block coincides with the left boundary of the current subpicture and loop_filter_across_subpic_enabled_flag[ CurrSubpicIdx ] or loop_filter_across_subpic_enabled_flag[ subpicIdx ] is equal to 0, where subpicIdx is the subpicture index of the subpicture for which the left boundary of the current coding block coincides with the right subpicture boundary of that subpicture.

- The left boundary of the current coding block is the left boundary of the tile or the slice but is not a subpicure boundary and loop_filter_across_tiles_enabled_flag loop_filter_across_boundaries_within_subpics_enabled_flag is equal to 0.

- The left boundary of the current coding block is the left boundary of the slice and loop_filter_across_slices_enabled_flag is equal to 0.

- The left boundary of the current coding block is one of the vertical virtual boundaries of the picture and VirtualBoundariesPresentFlag is equal to 1.

- Otherwise, if edgeType is equal to EDGE_HOR and one or more of the following conditions are true, the variable filterEdgeFlag is set equal to 0:

- The top boundary of the current luma coding block is the top boundary of the picture.

- The top boundary of the current coding block coincides with the top boundary of the current subpicture and loop_filter_across_subpic_enabled_flag[ CurrSubpicIdx ] or loop_filter_across_subpic_enabled_flag[ subpicIdx ] is equal to 0, where subpicIdx is the subpicture index of the subpicture for which the top boundary of the current coding block coincides with the bottom subpicture boundary of that subpicture.

- The top boundary of the current coding block is the top boundary of the tile or the slice but is not a subpicure boundary and loop_filter_across_tiles_enabled_flag loop_filter_across_boundaries_within_subpics_enabled_flag is equal to 0.

- The top boundary of the current coding block is the top boundary of the slice and loop_filter_across_slices_enabled_flag is equal to 0.

- The top boundary of the current coding block is one of the horizontal virtual boundaries of the picture and VirtualBoundariesPresentFlag is equal to 1.

- Otherwise, filterEdgeFlag is set equal to 1.

...

### 8.8.4 Sample adaptive offset process

#### 8.8.4.2 CTB modification process

...

–	Otherwise, if SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2, the following ordered steps apply:

- The values of hPos[ k ] and vPos[ k ] for k = 0..1 are specified in Table 44 based on SaoEoClass[ cIdx ][ rx ][ ry ].

- The variable edgeIdx is derived as follows:

- The modified sample locations ( xSik′, ySjk′ ) and ( xYik′, yYjk′ ) are derived as follows:

( xSik′, ySjk′ ) = ( xSi + hPos[ k ], ySj + vPos[ k ] )	(1446)

( xYik′, yYjk′ ) = ( cIdx  = =  0 ) ? ( xSik′, ySjk′ ) : ( xSik′ * SubWidthC, ySjk′ * SubHeightC )	(1447)

- If one or more of the following conditions for all sample locations ( xSik′, ySjk′ ) and ( xYik′, yYjk′ ) with k = 0..1 are true, edgeIdx is set equal to 0:

- The sample at location ( xSik′, ySjk′ ) is outside the picture boundaries.

- The sample at location ( xSik′, ySjk′ ) belongs to a different subpicture and loop_filter_across_subpic_enabled_flag[ CurrSubpicIdx ] for the subpicture to which the sample recPicture[ xSi ][ ySj ] belongs to is equal to 0.

- loop_filter_across_slices_enabled_flag is equal to 0 and the sample at location ( xSik′, ySjk′ ) belongs to a different slice.

- loop_filter_across_tiles_enabled_flag is equal to 0 and the sample at location ( xSik′, ySjk′ ) belongs to a different tile or a different slice within the same subpicture.

- Otherwise, edgeIdx is derived as follows:

- The following applies:

edgeIdx = 2 + Sign( recPicture[ xSi ][ ySj ] − recPicture[ xSi + hPos[ 0 ] ][ ySj + vPos[ 0 ] ] ) +
		Sign( recPicture[ xSi ][ ySj ] − recPicture[ xSi + hPos[ 1 ] ][ ySj + vPos[ 1 ] ] )	(1448)

- When edgeIdx is equal to 0, 1, or 2, edgeIdx is modified as follows:

edgeIdx = ( edgeIdx  = =  2 ) ? 0 : ( edgeIdx + 1 )	(1449)

...

### 8.8.5 Adaptive loop filter process

#### 8.8.5.5 ALF boundary position derivation process

...

The variable clipTopPos is modified as follows:

  - If y − ( CtbSizeY − 4 ) is greater than or equal to 0, the variable clipTopPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if VirtualBoundariesPresentFlag is equal to 1, and yCtb + y − VirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 3 for any n = 0..NumHorVirtualBoundaries − 1, the following applies:

clipTopPos = VirtualBoundariesPosY[ n ]		(1510)

  - Otherwise, if y is less than 3 and one or more of the following conditions are true, the variable clipTopPos is set equal to yCtb:

    - The top boundary of the current coding tree block is the top boundary of the tile or the slice but is not a subpicture boundary, and loop_filter_across_tiles_enabled_flag loop_filter_across_boundaries_within_subpics_enabled_flag is equal to 0.

    - The top boundary of the current coding tree block is the top boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - The top boundary of the current coding tree block is the top boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ CurrSubpicIdx ] is equal to 0.

The variable clipBottomPos is modified as follows:

  - If VirtualBoundariesPresentFlag is equal to 1, VirtualBoundariesPosY[ n ] is not equal to pic_height_in_luma_samples − 1 or 0, and VirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 5 for any n = 0..NumHorVirtualBoundaries − 1, the following applies:

clipBottomPos = VirtualBoundariesPosY[ n ]		(1511)

  - Otherwise, if CtbSizeY − 4 − y is greater than 0 and is less than 5, the variable clipBottomPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if CtbSizeY − y is less than 5, and one or more of the following conditions are true, the variable clipBottomPos is set equal to yCtb + CtbSizeY:

    - The bottom boundary of the current coding tree block is the bottom boundary of the tile or the slice but is not a subpicture boundary, and loop_filter_across_tiles_enabled_flag loop_filter_across_boundaries_within_subpics_enabled_flag is equal to 0.

    - The bottom boundary of the current coding tree block is the bottom boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - The bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ CurrSubpicIdx ] is equal to 0.

The variable clipLeftPos is modified as follows:

  - If VirtualBoundariesPresentFlag is equal to 1, and xCtb + x − VirtualBoundariesPosX[ n ] is greater than or equal to 0 and less than 3 for any n = 0..NumVerVirtualBoundaries − 1, the following applies:

clipLeftPos = VirtualBoundariesPosX[ n ]		(1512)

  - Otherwise, if x is less than 3, and one or more of the following conditions are true, the variable clipLeftPos is set equal to xCtb:

    - The left boundary of the current coding tree block is the left boundary of the tile or the slice but is not a subpicture boundary, and loop_filter_across_tiles_enabled_flag loop_filter_across_boundaries_within_subpics_enabled_flag is equal to 0.

    - The left boundary of the current coding tree block is the left boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - The left boundary of the current coding tree block is the left boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ CurrSubpicIdx ] is equal to 0.

The variable clipRightPos is modified as follows:

  - If VirtualBoundariesPresentFlag is equal to 1, and VirtualBoundariesPosX[ n ]− xCtb − x is greater than 0 and less than 5 for any n = 0..NumVerVirtualBoundaries − 1, the following applies:

clipRightPos = VirtualBoundariesPosX[ n ]		(1513)

  - Otherwise, if CtbSizeY − x is less than 5, and one or more of the following conditions are true, the variable clipRightPos is set equal to xCtb + CtbSizeY:

    - The right boundary of the current coding tree block is the right boundary of the tile or the slice but is not a subpicture boundary, and loop_filter_across_tiless_enabled_flag loop_filter_across_boundaries_within_subpics_enabled_flag is equal to 0.

    - The right boundary of the current coding tree block is the right boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - The right boundary of the current coding tree block is the right boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ CurrSubpicIdx ] is equal to 0.

The variable clipTopLeftFlag and clipBotRightFlag are modified as following:

  - When the coding tree block covering the luma position ( xCtb, yCtb ) and the coding tree block covering the luma position ( xCtb − CtbSizeY, yCtb − CtbSizeY) belong to different slices of the same subpicture, and loop_filter_across_slices_enabled_flag loop_filter_across_boundaries_within_subpics_enabled_flag is equal to 0, clipTopLeftFlag is set equal to 1.

  - When the coding tree block covering the luma position ( xCtb, yCtb ) and the coding tree block covering the luma position ( xCtb + CtbSizeY, yCtb + CtbSizeY) belong to different slices of the same subpicture, and loop_filter_across_slices_enabled_flag loop_filter_across_boundaries_within_subpics_enabled_flag is equal to 0, clipBotRightFlag is set equal to 1.

...

- Patent rights declaration(s)

Bytedance Inc. and LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).