| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0074-v3 |
| --- | --- |

| Title: | AHG9: Removal of APS semantics dependencies on SPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Li Zhang2, Ye-Kui Wang2, Kai Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Ln, San Diego, CA 92012, USA | Tel: Email: | zhipin.deng@bytedance.com lizhang.idm@bytedance.com yekui.wang@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the latest VVC text, it is asserted that there are some APS semantics dependencies on SPS. It is further asserted that APS semantics dependencies on SPS should not occur, because there is no PPS ID or SPS ID in the APS syntax and consequently an APS may be applied to pictures (or slices of pictures) that refer to different SPSs.

This contribution proposes the following changes for removing of APS semantics dependencies on SPS:

- It is required that, when ph_scaling_list_aps_id is present, the value of scaling_list_chroma_present_flag of the APS NAL unit having aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_scaling_list_aps_id shall be equal to 0 if ChromaArrayType is equal to 0 and shall be equal to 1 otherwise.

- It is required that, when ph_alf_aps_id_luma[ i ] is present and ChromaArrayType is equal to 0, the values of alf_chroma_filter_signal_flag, alf_cc_cb_filter_signal_flag, and alf_cc_cr_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_alf_aps_id_luma[ i ] shall all be equal to 0.

  - Moreover, it is required that, when slice_alf_aps_id_luma[ i ] is present and ChromaArrayType is equal to 0, the values of alf_chroma_filter_signal_flag, alf_cc_cb_filter_signal_flag, and alf_cc_cr_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall all be equal to 0.

- It is required that, when ph_lmcs_aps_id is present, the value of lmcs_delta_abs_crs of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to ph_lmcs_aps_id shall be equal to 0 if ChromaArrayType is equal to 0 .

- Remove the existing constraints in the semantics of scaling_list_chroma_present_flag, alf_chroma_filter_signal_flag, alf_cc_cb_filter_signal_flag, and alf_cc_cr_filter_signal_flag.

# Introduction

The following issues on the semantics of APS syntax elements were observed in the current draft text:

- The value of the APS syntax element scaling_list_chroma_present_flag is constrained based on ChromaArrayType derived from SPS syntax elements chroma_format_idc and separate_colour_plane_flag, phrased as follows: scaling_list_chroma_present_flag shall be equal to 0 when ChromaArrayType is equal to 0, and shall be equal to 1 when ChromaArrayType is not equal to 0.

Such constraints in the semantics of the APS syntax element introduce semantics dependencies of APS on SPS, which should not occur, because there is no PPS ID or SPS ID in the APS syntax and consequently an APS may be applied to pictures (or slices of pictures) that refer to different SPSs, which may be associated with different values of ChromaArrayType.

- Additionally, similar APS-SPS semantics dependencies also exist in the semantics of some ALF/CC-ALF APS syntax elements, phrased as follows: When ChromaArrayType is equal to 0, alf_chroma_filter_signal_flag, alf_cc_cb_filter_signal_flag, and alf_cc_cr_filter_signal_flag shall to be equal to 0.

- When an LMCS APS is signalled, the chroma residual scaling related syntax elements are always signalled in the LMCS APS syntax structure, regardless of whether ChromaArrayType is equal to 0 (i.e., there is no chroma component in the CLVS). This results in unnecessary signalling of chroma-related syntax elements.

# Proposal

## Summary of the proposal

The following changes are proposed to address the above issues:

- To address the 1st issue, it is required that, when ph_scaling_list_aps_id is present, the value of scaling_list_chroma_present_flag of the APS NAL unit having aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_scaling_list_aps_id shall be equal to 0 if ChromaArrayType is equal to 0 and shall be equal to 1 otherwise.

- To address the 2nd issue, it is required that, when ph_alf_aps_id_luma[ i ] is present and ChromaArrayType is equal to 0, the values of alf_chroma_filter_signal_flag, alf_cc_cb_filter_signal_flag, and alf_cc_cr_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_alf_aps_id_luma[ i ] shall all be equal to 0.

  - Moreover, it is required that, when slice_alf_aps_id_luma[ i ] is present and ChromaArrayType is equal to 0, the values of alf_chroma_filter_signal_flag, alf_cc_cb_filter_signal_flag, and alf_cc_cr_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall all be equal to 0.

- To address the 3rd issue, it is required that, when ph_lmcs_aps_id is present, the value of lmcs_delta_abs_crs of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to ph_lmcs_aps_id shall be equal to 0 if ChromaArrayType is equal to 0.

- Accordingly, remove the existing constraints in the semantics of scaling_list_chroma_present_flag, alf_chroma_filter_signal_flag, alf_cc_cb_filter_signal_flag, and alf_cc_cr_filter_signal_flag.

## Proposed text changes

### Picture header structure semantics

…

ph_alf_enabled_flag equal to 1 specifies that adaptive loop filter is enabled for all slices associated with the PH and may be applied to Y, Cb, or Cr colour component in the  slices. ph_alf_enabled_flag equal to 0 specifies that adaptive loop filter may be disabled for one, or more, or all slices associated with the PH. When not present, ph_alf_enabled_flag is inferred to be equal to 0.

ph_num_alf_aps_ids_luma specifies the number of ALF APSs that the slices associated with the PH refers to.

ph_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slices associated with the PH refers to.

When ph_alf_aps_id_luma[ i ] is present, the following applies:

- The value of alf_luma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_alf_aps_id_luma[ i ] shall be equal to 1.

- The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_alf_aps_id_luma[ i ] shall be less than or equal to the TemporalId of the picture associated with the PH.

- When ChromaArrayType is equal to 0, the values of alf_chroma_filter_signal_flag, alf_cc_cb_filter_signal_flag, and alf_cc_cr_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_alf_aps_id_luma[ i ] shall all be equal to 0.

ph_alf_chroma_idc equal to 0 specifies that the adaptive loop filter is not applied to Cb and Cr colour components. ph_alf_chroma_idc equal to 1 indicates that the adaptive loop filter is applied to the Cb colour component. ph_alf_chroma_idc equal to 2 indicates that the adaptive loop filter is applied to the Cr colour component. ph_alf_chroma_idc equal to 3 indicates that the adaptive loop filter is applied to Cb and Cr colour components. When ph_alf_chroma_idc is not present, it is inferred to be equal to 0.

ph_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the ALF APS that the chroma component of the slices associated with the PH refers to.

When ph_alf_aps_id_chroma is present, the following applies:

- The value of alf_chroma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_alf_aps_id_chroma shall be equal to 1.

- The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_alf_aps_id_chroma shall be less than or equal to the TemporalId of the picture associated with the PH.

…

ph_cc_alf_cb_aps_id specifies the adaptation_parameter_set_id of the ALF APS that the Cb colour component of the slices associated with the PH refers to.

When ph_cc_alf_cb_aps_id is present, the following applies:

- The value of alf_cc_cb_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_cc_alf_cb_aps_id shall be equal to 1.

- The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_cc_alf_cb_aps_id shall be less than or equal to the TemporalId of the picture associated with the PH.

…

ph_cc_alf_cr_aps_id specifies the adaptation_parameter_set_id of the ALF APS that the Cr colour component of the slices associated with the PH refers to.

When ph_cc_alf_cr_aps_id is present, the following applies:

- The value of alf_cc_cr_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_cc_alf_cr_aps_id shall be equal to 1.

- The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_cc_alf_cr_aps_id shall be less than or equal to the TemporalId of the picture associated with the PH.

…

ph_lmcs_aps_id specifies the adaptation_parameter_set_id of the LMCS APS that the slices associated with the PH refers to.

When ph_lmcs_aps_id is present, the following applies:

- The TemporalId of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to ph_lmcs_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

- The value of lmcs_delta_abs_crs of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to ph_lmcs_aps_id shall be equal to 0 if ChromaArrayType is equal to 0.

…

ph_scaling_list_aps_id specifies the adaptation_parameter_set_id of the scaling list APS.

When ph_scaling_list_aps_id is present, the following applies:

- The TemporalId of the APS NAL unit having aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_scaling_list_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

- The value of scaling_list_chroma_present_flag of the APS NAL unit having aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_scaling_list_aps_id shall be equal to ChromaArrayType  = =  0 ? 0 : 1.

…

### General slice header semantics

…

slice_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slice refers to. When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_luma[ i ] is not present, the value of slice_alf_aps_id_luma[ i ] is inferred to be equal to the value of ph_alf_aps_id_luma[ i ].

When slice_alf_aps_id_luma[ i ] is present, the following applies:

- The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be less than or equal to the TemporalId of the coded slice NAL unit.

- The value of alf_luma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be equal to 1.

- When ChromaArrayType is equal to 0, the values of alf_chroma_filter_signal_flag, alf_cc_cb_filter_signal_flag, and alf_cc_cr_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall all be equal to 0.

…

slice_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the ALF APS that the chroma component of the slice refers to. When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_chroma is not present, the value of slice_alf_aps_id_chroma is inferred to be equal to the value of ph_alf_aps_id_chroma.

When slice_alf_aps_id_chroma is present, the following applies:

- The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be less than or equal to the TemporalId of the coded slice NAL unit.

- The value of alf_chroma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be equal to 1.

…

slice_cc_alf_cb_aps_id specifies the adaptation_parameter_set_id that the Cb colour component of the slice refers to. When slice_cc_alf_cb_enabled_flag is equal to 1 and slice_cc_alf_cb_aps_id is not present, the value of slice_cc_alf_cb_aps_id is inferred to be equal to the value of ph_cc_alf_cb_aps_id.

When slice_cc_alf_cb_aps_id is present, the following applies:

- The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_cc_alf_cb_aps_id shall be less than or equal to the TemporalId of the coded slice NAL unit.

- The value of alf_cc_cb_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_cc_alf_cb_aps_id shall be equal to 1.

…

slice_cc_alf_cr_aps_id specifies the adaptation_parameter_set_id that the Cr colour component of the slice refers to. When slice_cc_alf_cr_enabled_flag is equal to 1 and slice_cc_alf_cr_aps_id is not present, the value of slice_cc_alf_cr_aps_id is inferred to be equal to the value of ph_cc_alf_cr_aps_id.

When slice_cc_alf_cr_aps_id is present, the following applies:

- The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_cc_alf_cr_aps_id shall be less than or equal to the TemporalId of the coded slice NAL unit.

- The value of alf_cc_cr_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_cc_alf_cr_aps_id shall be equal to 1.

…

### Adaptive loop filter data semantics

…

alf_chroma_filter_signal_flag equal to 1 specifies that a chroma filter is signalled. alf_chroma_filter_signal_flag equal to 0 specifies that a chroma filter is not signalled. When ChromaArrayType is equal to 0, alf_chroma_filter_signal_flag shall be equal to 0.

…

alf_cc_cb_filter_signal_flag equal to 1 specifies that cross-component filters for the Cb colour component are signalled. alf_cc_cb_filter_signal_flag equal to 0 specifies that cross-component filters for Cb colour component are not signalled. When ChromaArrayType is equal to 0, alf_cc_cb_filter_signal_flag shall be equal to 0.

alf_cc_cr_filter_signal_flag equal to 1 specifies that cross-component filters for the Cr colour component are signalled. alf_cc_cr_filter_signal_flag equal to 0 specifies that cross-component filters for the Cr colour component are not signalled. When ChromaArrayType is equal to 0, alf_cc_cr_filter_signal_flag shall be equal to 0.

…

### Scaling list data semantics

…

scaling_list_chroma_present_flag equal to 1 specifies that chroma scaling lists are present in scaling_list_data( ). scaling_list_chroma_present_flag equal to 0 specifies that chroma scaling lists are not present in scaling_list_data( ). It is a requirement of bitstream conformance that scaling_list_chroma_present_flag shall be equal to 0 when ChromaArrayType is equal to 0, and shall be equal to 1 when ChromaArrayType is not equal to 0.

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).