| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0086-v1 |
| --- | --- |

| Title: | AHG9: Modification of general constraints flags |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Woong Lim, Gun Bang | Email: | woong.lim@etri.re.kr, gbang@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

- Abstract

This contribution proposes to modify the syntax structure in general constraints flags that are proposed in JVET-Q0114 [1] documents. There are three flags, one_tile_per_pic_constraint_flag, one_slice_per_pic_constraint_flag and one_subpic_per_pic_constraint_flag, adopted during the last meeting. Those flags are sequentially signalled in general constraints information syntax, but when only one subpicture exists in the current picture, i.e. one_subpic_per_pic_constraint_flag is equal to 1, one_slice_per_pic_constraint_flag can be omitted. This document therefore proposes a simple change of the syntax structure.

Introduction

In the last meeting, JVET-Q0114 proposal has been adopted and some flags have been added into general constraint information syntax. There are three new flags which are one_tile_per_pic_constraint_flag, one_slice_per_pic_constraint_flag and one_subpic_per_pic_constraint_flag, and sequentially signalled. Syntax and semantics of those flags are shown below.

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| … |  |
| one_tile_per_pic_constraint_flag | u(1) |
| one_slice_per_pic_constraint_flag | u(1) |
| one_subpic_per_pic_constraint_flag | u(1) |
| … |  |
| } |  |

one_tile_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one tile. one_tile_per_pic_constraint_flag equal to 0 does not impose such a constraint.

one_slice_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one slice. one_slice_per_pic_constraint_flag equal to 0 does not impose such a constraint.

one_subpic_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one subpicture. one_subpic_per_pic_constraint_flag equal to 0 does not impose such a constraint. When one_slice_per_pic_constraint_flag is equal to 1, the value of one_subpic_per_pic_constraint_flag shall be equal to 1.

In the current VVC specification [2], the definition of a subpicture and partitioning related to subpicture are described as shown below.

3.140 subpicture: An rectangular region of one or more slices within a picture.

- 6.3.1 Partitioning of pictures into subpictures, slices, and tiles

…

A subpicture contains one or more slices that collectively cover a rectangular region of a picture. Consequently, each subpicture boundary is also always a slice boundary, and each vertical subpicture boundary is always also a vertical tile boundary.

…

According to the definition and a part of subclause 6.3.1(Partitioning of pictures, subpictures, slices, and tiles) shown above, a subpicture contains one or more slices, therefore, when a picture contains two or more subpictures, the picture also contains two or more slices.

Proposal

The proposed simple change (yellow highlighted) in syntax structure is shown below.

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| … |  |
| one_subpic_per_pic_constraint_flag | u(1) |
| if(one_subpic_per_pic_constraint_flag){ |  |
| one_slice_per_pic_constraint_flag | u(1) |
| } |  |
| one_tile_per_pic_constraint_flag | u(1) |
| … |  |
| } |  |

one_subpic_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one subpicture. one_subpic_per_pic_constraint_flag equal to 0 does not impose such a constraint. When one_slice_per_pic_constraint_flag is equal to 1, the value of one_subpic_per_pic_constraint_flag shall be equal to 1.

one_tile_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one tile. one_tile_per_pic_constraint_flag equal to 0 does not impose such a constraint.

one_slice_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one slice. one_slice_per_pic_constraint_flag equal to 0 does not impose such a constraint. When one_slice_per_pic_constraint_flag is not present, it is inferred to be equal to 0.

Reference

[1] Y. Wang, “AHG9: A few more general constraints flags”, JVET-Q0114, 17th meeting, Brussels, BE, January 2020.

[2] B. Bross, J. Chen, S. Liu, Y. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001, 17th meeting, Brussels, BE, January 2020.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).