| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0088-v1 |
| --- | --- |

| Title: | AHG12: Modification of subpicture information in PPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Woong Lim, Gun Bang | Email: | woong.lim@etri.re.kr, gbang@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

- Abstract

This contribution proposes to modify syntax elements related to subpicture and slice in PPS. The number of subpictures and slices are dependent according to the current VVC specification. In PPS, those syntax elements are signalled regardless of each other. The following two proposals are described in this document.

Proposal 1) Subpicture-related syntax elements are signalled in PPS when a picture partitioned which refers to the PPS.

Proposal 2) Signaling difference between the number of slices and the number of subpictures instead of the number of slices.

Introduction

In the current VVC specification [1], the definition of a subpicture and partitioning related to subpicture are described as shown below.

3.140 subpicture: An rectangular region of one or more slices within a picture. … - 6.3.1 Partitioning of pictures into subpictures, slices, and tiles … A subpicture contains one or more slices that collectively cover a rectangular region of a picture. Consequently, each subpicture boundary is also always a slice boundary, and each vertical subpicture boundary is always also a vertical tile boundary. …

Proposal

The proposed specification change (yellow highlighted for the proposal 1 and green highlighted for the proposal 2) is shown below.

Proposal 1)

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| output_flag_present_flag | u(1) |
| subpic_id_mapping_in_pps_flag | u(1) |
| if( subpic_id_mapping_in_pps_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| subpic_id_mapping_in_pps_flag | u(1) |
| if( subpic_id_mapping_in_pps_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| if( NumTilesInPic > 1 ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| if( num_slices_in_pic_minus1 > 0 ) |  |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| if( NumTileColumns > 1 ) |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| if( NumTileRows > 1 && ( tile_idx_delta_present_flag ¦ ¦ SliceTopLeftTileIdx[ i ] % NumTileColumns = = 0 ) ) |  |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 && RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] > 1 ) { |  |
| num_exp_slices_in_tile[ i ] | ue(v) |
| for( j = 0; j < num_exp_slices_in_tile[ i ]; j++ ) |  |
| exp_slice_height_in_ctus_minus1[ i ][ j ] | ue(v) |
| i += NumSlicesInTile[ i ] − 1 |  |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| cabac_init_present_flag | u(1) |
| … |  |
| } |  |

Proposal 2)

6.5 Scanning process

6.5.1 CTB raster scanning, tile scanning, and subpicture scanning process

…

When rect_slice_flag is equal to 1, the variable NumSlicesInPic, specifying the number of slices, is set equal to num_slices_in_pic_minus_num_subpics_in_pic plus pps_num_subpics_minus1(which is equal to sps_num_subpics_minus1), the list NumCtusInSlice[ i ] for i ranging from 0 to num_slices_in_pic_minus1NumSlicesInPic - 1, inclusive, specifying the number of CTUs in the i-th slice, the list SliceTopLeftTileIdx[ i ] for i ranging from 0 to num_slices_in_pic_minus1NumSlicesInPic - 1, inclusive, specifying the tile index of the tile containing the first CTU in the slice, and the matrix CtbAddrInSlice[ i ][ j ] for i ranging from 0 to num_slices_in_pic_minus1NumSlicesInPic - 1, inclusive, and j ranging from 0 to NumCtusInSlice[ i ] − 1, inclusive, specifying the picture raster scan address of the j-th CTB within the i-th slice, and the variable NumSlicesInTile[ i ], specifying the number of slices in the tile containing the i-th slice, are derived as follows:

if( single_slice_per_subpic_flag ) {
	for( i = 0; i  <=   sps_num_subpics_minus1; i++ ) {
		NumCtusInSlice[ i ] = 0 
		if( subpicHeightLessThanOneTileFlag[ i ] ) /* The slice consists of a number of CTU rows in a tile. */
			AddCtbsToSlice( i, subpic_ctu_top_left_x[ i ],
				subpic_ctu_top_left_x[ i ] + subpic_width_minus1[ i ] + 1, subpic_ctu_top_left_y[ i ],
				subpic_ctu_top_left_y[ i ] + subpic_height_minus1[ i ] + 1 )
		else { /* The slice consists of a number of complete tiles covering a rectangular region. */
			tileX = CtbToTileColBd[ subpic_ctu_top_left_x[ i ] ]
			tileY = CtbToTileRowBd[ subpic_ctu_top_left_y[ i ] ]
			for( j = 0; j < SubpicHeightInTiles[ i ]; j++ )
				for( k = 0; k < SubpicWidthInTiles[ i ]; k++ )
					AddCtbsToSlice( i, tileColBd[ tileX + k ], tileColBd[ tileX + k + 1 ], tileRowBd[ tileY + j ],
							tileRowBd[ tileY + j + 1 ] )
		}
	}
} else {
	tileIdx = 0
	for( i = 0; i  <=  num_slices_in_pic_minus1NumSlicesInPic; i++ )
		NumCtusInSlice[ i ] = 0
	for( i = 0; i  <=  num_slices_in_pic_minus1NumSlicesInPic; i++ ) {
		SliceTopLeftTileIdx[ i ] = tileIdx
		tileX = tileIdx % NumTileColumns
		tileY = tileIdx / NumTileColumns
		if( i < num_slices_in_pic_minus1NumSlicesInPic - 1) {
			sliceWidthInTiles[ i ] = slice_width_in_tiles_minus1[ i ] + 1
			sliceHeightInTiles[ i ] = slice_height_in_tiles_minus1[ i ] + 1
		} else {
			sliceWidthInTiles[ i ] = NumTileColumns − tileX
			sliceHeightInTiles[ i ] = NumTileRows − tileY
			NumSlicesInTile[ i ] = 1
		}
		if( slicWidthInTiles[ i ]  = =  1  &&  sliceHeightInTiles[ i ]  = =  1 ) {	(30)
			if( num_exp_slices_in_tile[ i ]  = = 0 ) {
				NumSlicesInTile[ i ] = 1
				sliceHeightInCtus[ i ] = RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ]
			} else {
				remainingHeightInCtbsY = RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ]
				for( j = 0; j < num_exp_slices_in_tile[ i ] − 1; j++ ) {
					sliceHeightInCtus[ i + j ] = exp_slice_height_in_ctus_minus1[ i ][ j ] + 1
					remainingHeightInCtbsY   −=  sliceHeightInCtus[ i + j ]
				}
				uniformSliceHeight = exp_slice_height_in_ctus_minus1[ i ][ j ] + 1
				while( remainingHeightInCtbsY  >=  uniformSliceHeight ) {
					sliceHeightInCtus[ i + j ] = uniformSliceHeight
					remainingHeightInCtbsY  −=  uniformSliceHeight
					j++
				}
				if( remainingHeightInCtbsY > 0 ) {
					sliceHeightInCtus[ i + j ] = remainingHeightInCtbsY
					j++
				}
				NumSlicesInTile[ i ] = j
			}
			ctbY = tileRowBd[ tileY ]
			for( j = 0; j < NumSlicesInTile[ i ]; j++ ) {
				AddCtbsToSlice( i + j, tileColBd[ tileX ], tileColBd[ tileX + 1 ],
						ctbY, ctbY + sliceHeightInCtus[ i + j ] )
				ctbY  +=  sliceHeightInCtus[ i + j ]
			}
			i  +=  NumSlicesInTile[ i ] − 1
		} else
			for( j = 0; j < sliceHeightInTiles[ i ]; j++ )
				for( k = 0; k < sliceWidthInTiles[ i ]; k++ )
					AddCtbsToSlice( i, tileColBd[ tileX + k ], tileColBd[ tileX + k + 1 ],
						tileRowBd[ tileY + j ], tileRowBd[ tileY + j + 1 ] )
		if( i < num_slices_in_pic_minus1NumSlicesInPic – 1 ) {
			if( tile_idx_delta_present_flag )
				tileIdx  +=  tile_idx_delta[ i ]
			else {
				tileIdx  +=  sliceWidthInTiles[ i ]
				if( tileIdx % NumTileColumns  = =  0 )
					tileIdx  +=  ( sliceHeightInTiles[ i ] − 1 ) * NumTileColumns
			}
		}
	}
}

Where the function AddCtbsToSlice( sliceIdx, startX, stopX, startY, stopY) is specified as follows: [Ed. (YK): Consider defining this function in a style consistent with other functions defined in the spec.]

for( ctbY = startY; ctbY < stopY; ctbY++ )
	for( ctbX = startX; ctbX < stopX; ctbX++ ) {
		CtbAddrInSlice[ sliceIdx ][ NumCtusInSlice[ sliceIdx ] ] = ctbY * PicWidthInCtbsY + ctbX	(31)
		NumCtusInSlice[ sliceIdx ]++
	}

It is a requirement of bitstream conformance that the values of NumCtusInSlice[ i ] for i ranging from 0 to num_slices_in_pic_minus1NumSlicesInPic - 1, inclusive, shall be greater than 0. Additionally, it is a requirement of bitstream conformance that the matrix CtbAddrInSlice[ i ][ j ] for i ranging from 0 to num_slices_in_pic_minus1NumSlicesInPic - 1, inclusive, and j ranging from 0 to NumCtusInSlice[ i ] − 1, inclusive, shall include each of all CTB addresses in the range of 0 to PicSizeInCtbsY − 1, inclusive, once and only once.

The list NumSlicesInSubpic[ i ], specifying the number of rectangular slices in the i-th subpicture, is derived as follows:

for( i = 0; i  <=  sps_num_subpics_minus1; i++ ) {
	NumSlicesInSubpic[ i ] = 0
	for( j = 0; j  <=num_slices_in_pic_minus1NumSlicesInPic; j++ ) {
		posX = CtbAddrInSlice[ j ][ 0 ] % PicWidthInCtbsY
		posY = CtbAddrInSlice[ j ][ 0 ] / PicWidthInCtbsY
		if( ( posX  >=  subpic_ctu_top_left_x[ i ] )  &&	(32)
				( posX < subpic_ctu_top_left_x[ i ] + subpic_width_minus1[ i ] + 1 )  &&
				( posY  >=  subpic_ctu_top_left_y[ i ] )  &&
				( posY < subpic_ctu_top_left_y[ i ] + subpic_height_minus1[ i ] + 1 ) )
			NumSlicesInSubpic[ i ]++
}

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| … |  |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1num_slices_in_pic_minus_num_subpics_in_pic | ue(v) |
| if(num_slices_in_pic_minus1 > 0 NumSlicesInPic > 1 ) |  |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1 NumSlicesInPic - 1; i++ ) { |  |
| … |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 NumSlicesInPic - 1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |

no_pic_partition_flag equal to 1 specifies that no picture partitioning is applied to each picture referring to the PPS. no_pic_partition_flag equal to 0 specifies each picture referring to the PPS may be partitioned into more than one tile or slice.

It is a requirement of bitstream conformance that the value of no_pic_partition_flag shall be the same for all PPSs that are referred to by coded pictures within a CLVS.

It is a requirement of bitstream conformance that the value of no_pic_partition_flag shall not be equal to 1 when the value of sps_num_subpics_minus1 + 1 is greater than 1.

num_slices_in_pic_minus1 plus 1 specifies the number of rectangular slices in each picture referring to the PPS. The value of num_slices_in_pic_minus1 shall be in the range of 0 to MaxSlicesPerPicture − 1, inclusive, where MaxSlicesPerPicture is specified in Annex A. When no_pic_partition_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to 0. When single_slice_per_subpic_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to sps_num_subpics_minus1.

num_slices_in_pic_minus1num_slices_in_pic_minus_num_subpics_in_pic plus 1pps_num_subpics_minus1 + 1 specifies the number of rectangular slices in each picture referring to the PPS. The value of num_slices_in_pic_minus1num_slices_in_pic_minus_num_subpics_in_pic shall be in the range of 0 to MaxSlicesPerPicture − 1pps_num_subpics_minus1, inclusive, where MaxSlicesPerPicture is specified in Annex A.

When no_pic_partition_flag is equal to 1, the value of NumSlicesInPic is inferred to be equal to 01. When single_slice_per_subpic_flag is equal to 1, the value of num_slices_in_pic_minus1NumSlicesInPic is inferred to be equal to sps_num_subpics_minus1 + 1.

Reference

[1] B. Bross, J. Chen, S. Liu, Y. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001, 17th meeting, Brussels, BE, January 2020.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).