| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0089-v1 |
| --- | --- |

| Title: | AHG9: On slice level control of LMCS and explicit scaling list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaehong Jung Dongcheol Kim Geonjung Ko Ju-Hyung (John) Son Jin Sam Kwak | Tel: Email: | +82-31-712-0523 jason.jung@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

This contribution proposes two aspects regarding slice level control of LMCS and explicit scaling list:

- Proposal 1: If the current picture consists of only one slice, slice level control flags of LMCS and explicit scaling list (slice_lmcs_enabled_flag and slice_explicit_scaling_list_used_flag) are not signalled and each flag is inferred to be equal to the picture level flag.

- Proposal 2: Add slice_chroma_residual_scale_flag to slice header to indicate whether chroma residual scaling is enabled for the current slice.

Problem statement

In the current VVC [1], LMCS and explicit scaling list can be turned on and off at sequence, picture and slice levels. The following texts are syntax and semantics related to the control of LMCS and explicit scaling list.

#### 7.3.2.7	Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| ⋯ |  |
| if( sps_lmcs_enabled_flag ) { |  |
| ph_lmcs_enabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) { |  |
| ph_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| ph_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| if( sps_explicit_scaling_list_enabled_flag ) { |  |
| ph_explicit_scaling_list_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| ph_scaling_list_aps_id | u(3) |
| } |  |
| ⋯ |  |
| } |  |

- 7.4.3.7	Picture header structure semantics

ph_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is enabled for all slices associated with the PH. ph_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling may be disabled for one, or more, or all slices associated with the PH. When not present, the value of ph_lmcs_enabled_flag is inferred to be equal to 0.

ph_chroma_residual_scale_flag equal to 1 specifies that chroma residual  scaling is enabled for the all slices associated with the PH. ph_chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling may be disabled for one, or more, or all slices associated with the PH. When ph_chroma_residual_scale_flag is not present, it is inferred to be equal to 0.

ph_explicit_scaling_list_enabled_flag equal to 1 specifies that the use of the explicit scaling list, which is signalled in the referenced scaling list APS (i.e., the APS with aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_explicit_scaling_list_aps_id), in the scaling process for transform coefficients when decoding a slice is enabled for the picture. ph_explicit_scaling_list_enabled_flag equal to 0 specifies that the use of an explicit scaling list in the scaling process for transform coefficients when decoding a slice is disabled for the picture. When not present, the value of ph_explicit_scaling_list_enabled_flag is inferred to be equal to 0.

#### 7.3.7.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| if( picture_header_in_slice_header_flag ) |  |
| picture_header_structure( ) |  |
| ⋯ |  |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| ⋯ |  |
| } |  |

#### 7.4.8.1	General slice header semantics

slice_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is enabled for the current slice. slice_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is not enabled for the current slice. When slice_lmcs_enabled_flag is not present, it is inferred to be equal to 0.

slice_explicit_scaling_list_used_flag equal to 1 specifies that the explicit scaling list, which is signalled in the referenced scaling list APS with aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_scaling_list_aps_id, is used in the scaling process for transform coefficients when decoding of the current slice. slice_explicit_scaling_list_used_flag equal to 0 specifies that no explicit scaling list is not used in the scaling process for transform coefficients when decoding of the current slice. When not present, the value of slice_explicit_scaling_list_used_flag is inferred to be equal to 0.

We report that the current VVC has the following issues related to the control of LMCS and explicit scaling list:

- Redundant signalling in slice header: When a coded picture consists of a single coded slice, picture header NAL unit is not present in the PU, while it is present in slice header. In the case where ph_lmcs_enabled_flag is equal to 1, slice level LMCS on/off flag is signalled in slice header. Since a coded picture contains only one coded slice, signalling of slice level LMCS on/off flag is redundant. There is similar redundancy in signalling of explicit scaling list. When a coded picture consists of a single coded slice and ph_explicit_scaling_list_enabled_flag is equal to 1, still the slice level explicit scaling list on/off flag is signalled in slice header which is also redundant.

- Slice level control of chroma residual scaling: LMCS consists of two parts: in-loop mapping of luma component and luma-dependent chroma residual scaling. At a picture level, luma mapping and chroma residual scaling are separately controlled using ph_lmcs_enabled_flag and ph_chroma_residual_scale_flag. However, at a slice level, slice_lmcs_enabled_flag controls both luma mapping and chroma residual scaling. That means the current draft cannot support the following configuration:

- slice 0 in the current picture: luma mapping on / chroma residual scaling off

- slice 1 in the current picture: luma mapping on / chroma residual scaling on

When LMCS was adopted, chroma residual scaling could be turned on and off at a slice level if luma mapping is enabled. Since it is beneficial to have slice level on/off control of LMCS, slice level on/off control of chroma residual scaling is desirable as in earlier version of VVC.

Proposal

- Proposal 1: Slice level control flags of LMCS and explicit scaling list (slice_lmcs_enabled_flag and slice_explicit_scaling_list_used_flag) are not signalled if the current picture consists of only a single slice. Each flag is inferred to be equal to the picture level flag. (Spec text changes are highlighted in yellow and strikethrough.)

- Proposal 2: Add slice_chroma_residual_scale_flag to slice header to indicate whether chroma residual scaling is enabled for the current slice. slice_chroma_residual_scale_flag is signalled if the current picture consists of multiple slices and chroma residual scaling is enabled in the current picture and LMCS is enabled in the current slice. When the current picture consists of only one slice, slice_chroma_residual_scale_flag is inferred to be equal to ph_chroma_residual_scale_flag. When the current picture consists of multiple slices and slice_chroma_residual_scale_flag is not present, it is inferred to be equal to 0. (Spec text changes are highlighted in green and strikethrough.)

7.3.7.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| picture_header_in_slice_header_flag | u(1) |
| if( picture_header_in_slice_header_flag ) |  |
| picture_header_structure( ) |  |
| ⋯ |  |
| if( ph_lmcs_enabled_flag && !picture_header_in_slice_header_flag ) { |  |
| slice_lmcs_enabled_flag | u(1) |
| if( slice_lmcs_enabled_flag && ph_chroma_residual_scale_flag ) |  |
| slice_chroma_residual_scale_flag | u(1) |
| } |  |
| if( ph_explicit_scaling_list_enabled_flag && !picture_header_in_slice_header_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| ⋯ |  |
| byte_alignment( ) |  |
| } |  |

7.4.8.1	General slice header semantics

slice_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is enabled for the current slice. slice_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is not enabled for the current slice. When slice_lmcs_enabled_flag is not present, it is inferred to be equal to 0ph_lmcs_enabled_flag.

slice_chroma_residual_scale_flag equal to 1 specifies that chroma residual scaling is enabled for the current slice. slice_chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling is not enabled for the current slice. When picture_header_in_slice_header_flag is equal to 1, slice_chroma_residual_scale_flag is not present and it is inferrred to be equal to ph_chroma_residual_scale_flag. When slice_chroma_residual_scale_flag is not present and picture_header_in_slice_header_flag is equal to 0, it is inferred to be equal to 0.

slice_explicit_scaling_list_used_flag equal to 1 specifies that the explicit scaling list, which is signalled in the referenced scaling list APS with aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_scaling_list_aps_id, is used in the scaling process for transform coefficients when decoding of the current slice. slice_explicit_scaling_list_used_flag equal to 0 specifies that no explicit scaling list is not used in the scaling process for transform coefficients when decoding of the current slice. When not present, the value of slice_explicit_scaling_list_used_flag is inferred to be equal to 0ph_explicit_scaling_list_enabled_flag.

#### 8.7.5.3	Picture reconstruction with luma dependent chroma residual scaling process for chroma samples

Inputs to this process are:

- a chroma location ( xCurr, yCurr ) of the top-left chroma sample of the current chroma transform block relative to the top-left chroma sample of the current picture,

- a variable nCurrSw specifying the chroma transform block width,

- a variable nCurrSh specifying the chroma transform block height,

- a variable tuCbfChroma specifying the coded block flag of the current chroma transform block,

- an (nCurrSw)x(nCurrSh) array predSamples specifying the chroma prediction samples of the current block,

- an (nCurrSw)x(nCurrSh) array resSamples specifying the chroma residual samples of the current block,

Output of this process is a reconstructed chroma picture sample array recSamples.

The variable sizeY is set equal to Min( CtbSizeY, 64 ).

The reconstructed chroma picture sample recSamples is derived as follows for i = 0..nCurrSw − 1, j = 0..nCurrSh − 1:

- If one or more of the following conditions are true, recSamples[ xCurr + i ][ yCurr + j ] is set equal to Clip1( predSamples[ i ][ j ] + resSamples[ i ][ j ] ):

- ph_chroma_residual_scale_flag is equal to 0.

- slice_lmcs_enabled_flagslice_chroma_residual_scale_flag is equal to 0.

- nCurrSw * nCurrSh is less than or euqal to 4.

- tu_cb_coded_flag [ xCurr ][ yCurr ] is equal to 0 and tu_cr_coded_flag [ xCurr ][ yCurr ] is equal to 0.

Reference

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8),” document JVET-Q2001, 17th meeting: Brussels, BE, 7–17 January 2020

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).