| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0091 |
| --- | --- |

| Title: | AHG9: Issue with slice indexing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon | Tel: Email: | +49-173-6192228 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

There are currently two different indexing orders for slices: the order in which slices are signalled within the PPS and the order in which slices are coded within the bitstream. Unfortunately, the current draft specification may fail when these two orders differ from each other, which may easily happen when subpictures are used.

This proposal suggests two alternative solutions to solve this issue:

- Introduce constraints to ensure that the slice signalling order in the PPS and the slice coding order within the bitstream are the same.

- Introduce a mapping between the two indexing orders.

# Problem Statement

Slices are currently signalled within the PPS in terms of tiles. The relevant syntax elements for the signalling of the size and position of slices within the picture are slice_width_in_tiles_minus1, slice_height_in_tiles_minus1, and in case the slice contains one tile or less, num_exp_slices_in_tile and exp_slice_height_in_ctus_minus1. All these syntax elements are used within the CTB raster scanning, tile scanning, and subpicture scanning processes to create the following arrays and matrices describing the slices of the picture:

- The list NumCtusInSlice[ i ] specifies the number of CTU in the i-th slice;

- The list SliceTopLeftTileIdx[ i ] specifies the index of the top-left tile of the i-th slice; and

- The matrix CtbAddrInSlice[ i ][ j ] specifies the picture raster scan address of the j-th CTB within the i-th slice.

In all three of these arrays and matrices, the index i represents the index of the slice in the order in which the slices are signalled within the PPS, i.e. the index i follows the same order as the syntax elements slice_width_in_tiles_minus1, slice_height_in_tiles_minus1, num_exp_slices_in_tile and exp_slice_height_in_ctus_minus1 from the PPS.

Unfortunately, this slice indexing order is not necessarily the same as the coding order of the slice NAL units within the bitstream. In particular, when subpictures are used, the slice coding order is constrained by the following:

The order of the VCL NAL units within a coded picture is constrained as follows:

–	For any two coded slice NAL units A and B of a coded picture, let subpicIdxA and subpicIdxB be their subpicture level index values, and sliceAddrA and sliceddrB be their slice_address values.

–	When either of the following conditions is true, coded slice NAL unit A shall precede coded slice NAL unit B:

–	subpicIdxA is less than subpicIdxB.

–	subpicIdxA is equal to subpicIdxB and sliceAddrA is less than sliceAddrB.

At the last meeting, it was decided to have the internal picture slice index follow the subpicture slice address signalled in the slice header for clarity and easiness of implementation (since the VTM encoder encodes slices in the order in which they are indexed internally):

picLevelSliceIdx = slice_address
for( j = 0; j < CurrSubpicIdx; j++ )
	picLevelSliceIdx  +=  NumSlicesInSubpic[ j ]
NumCtusInCurrSlice = NumCtusInSlice[picLevelSliceIdx ]
for( i = 0; i < NumCtusInCurrSlice; i++ )
	CtbAddrInCurrSlice[ i ] = CtbAddrInSlice[picLevelSliceIdx ][ i ]

Since it was also decided at the last meeting that the subpicture slice index should follow the slice coding order (see constraint above), it derives that the internal picture slice index also follows the slice coding order (which is a good thing since it simplifies coding of the slices at the encoder).

Unfortunately, these decisions from last meeting had assumed that the signalling slice order would also follow the coding slice order. But it was noticed during implementation of the decisions from last meeting that this is not necessarily the case. It means that the arrays NumCtusInSlice and CtbAddrInSlice are accessed using a wrong index and therefore give the wrong information about the size and position of the current slice.

In particular, the previous version of the configuration file subpicture_4Slice2VerSubPic.cfg failed because slice 1 is signalled before slice 2 but encoded after slice 2 (see Figure 1).

Figure 1: Slice signalling order in original version of the configuration file subpicture_4Slice2VerSubPic.cfg

It has however been confirmed that a slightly different version of this configuration file whereby slice 1 and slice 2 are signalled in the same order as their coding order works without issue.

Figure 2: Slice signalling order in revised version of the configuration file

The present document therefore suggests addressing this bug by proposing two alternative solutions for the group to discuss. The two proposed solutions are the following:

- Introduce constraints to ensure that the slice signalling order in the PPS and the slice coding order within the bitstream are the same.

- Introduce a mapping between the two indexing orders.

The author of the present document has a preference for the first solution, since it is believed to simplify implementation and remove corner use cases that have the potential to cause headaches and bugs in subpictures-based implementations.

# Solution #1: Constraint slice signalling order and slice coding order to be the same

The first solution proposes to introduce two constraints to ensure that the slice signalling order in the PPS and the slice coding order within the bitstream are always the same. This solution avoids mapping between different indexing orders and simplifies implementations because the internal picture level slice index can be used for all slice related information.

The first constraint intends to constraint the signalling order of slices that cover full tiles:

The signalling order of slices in the PPS shall follow the decoding order of slice NAL units. Let slice A be signalled by the syntax elements slice_width_in_tiles_minus1[ sA ], slice_height_in_tiles_minus1[ sA ], num_exp_slices_in_tile[ sA ] and exp_slice_height_in_ctus_minus1[ sA ] and let slice B be signalled by the syntax elements slice_width_in_tiles_minus1[ sB ], slice_height_in_tiles_minus1[ sB ], num_exp_slices_in_tile[ sB ] and exp_slice_height_in_ctus_minus1[ sB ]. If coded slice NAL unit A precedes coded slice NAL unit B in the bitstream, then sA shall be less than sB.

However, there is still an issue for the case of subpictures containing sub-tiles of different tiles. Indeed, in such cases, it is not possible at all for the signalling order of slices to be the same as the coding order of slices. This is due to the current syntax, whereby slices that belong to the same tile must be signalled together and their index will automatically follow each other. In the example of Figure 3, slice 3 cannot be signalled before slice 2 with the current syntax, although rules on slice coding order mean that slice 3 has to be coded before slice 2.

Figure 3: Subpictures containing sub-tiles of different tiles

While the utility of the use case of subpicture containing sub-tiles of different tiles has never been clearly demonstrated, allowing such a configuration makes all subpictures based implementations (not only indexing mentioned in the present document, but also subpicture extraction) complicated and easily prone to bugs. Furthermore, the author of this document believes that a configuration such as the one illustrated in Figure 3 can be easily enabled without the need for subpictures containing sub-tiles of different tiles (adding one more row of tiles would enable to create the same subpictures configuration).

The following constraint is therefore proposed:

One or both of the following conditions shall be fulfilled for each subpicture and tile:

–	All CTUs in a subpicture belong to the same tile

–	All CTUs in a tile belong to the same subpicture

# Solution #2: Mapping between slice signalling and slice coding order

Should the group wish to keep the flexibility of having a slice signalling order that may differ from the slice coding order, a mapping between the two indexing orders is required as follows:

The array SliceCodingToSignallingOrder[ i ], specifying the slice index as signalled in the PPS of the i-th slice in coding order, is derived as follows:

for( j = 0; j  <=  sps_num_subpics_minus1; j++ )
	sliceAddrInSubpic[ j ] = 0
for( i = 0; i  <=  num_slices_in_pic_minus1; i++ ) {
	picLevelSliceIdx = sliceAddrInSubpic[ SliceSubpicIdx[ i ] ]
	for( j = 0; j  <  SliceSubpicIdx[ i ]; j++ )
		picLevelSliceIdx += NumSlicesInSubpic[ j ]
	sliceAddrInSubpic[ SliceSubpicIdx[ i ] ]++
	SliceCodingToSignallingOrder[ picLevelSliceIdx ] = i
}

This new mapping array is then used as follows to access the correct information for each slice:

picLevelSliceIdx = slice_address
for( j = 0; j < CurrSubpicIdx; j++ )
	picLevelSliceIdx  +=  NumSlicesInSubpic[ j ]
NumCtusInCurrSlice = NumCtusInSlice[ SliceCodingToSignallingOrder[ picLevelSliceIdx ] ]
for( i = 0; i < NumCtusInCurrSlice; i++ )
	CtbAddrInCurrSlice[ i ] = 
		CtbAddrInSlice[ SliceCodingToSignallingOrder[ picLevelSliceIdx ] ][ i ]

# Conclusion

A bug currently existing in the current VVC draft specification and VTM software is documented. The bug is related to the use of two different slice indexing orders. Two solutions are proposed to solve the issue:

- Introduce constraints to ensure that the slice signalling order in the PPS and the slice coding order within the bitstream are the same.

- Introduce a mapping between the two indexing orders.

The author prefers the first solution, since it is believed to simplify implementations.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).