| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Alpbach, AT, 15–24 April 2020 | Document: JVET-R0093 |
| --- | --- |

| Title: | AHG12: Subpictures and conformance cropping window |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon | Tel: Email: | +49-173-6192228 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

The current VVC draft is silent on how to deal with a bitstream that uses both a conformance cropping window and subpictures. In order to ensure that extraction of all subpictures of a picture is possible, a constraint is proposed such that no subpicture can be located completely outside of the conformance cropping window. It is also proposed to extend the subpicture sub-bitstream extraction process to clarify how to rewrite the conformance cropping window offsets when extracting a subpicture.

# Problem Statement

VVC Draft 8 is currently silent on how to deal with a bitstream that has non-zero conformance cropping window offsets and subpictures. One issue is that if subpictures are defined completely outside of the conformance cropping window, it is unclear if these subpictures can be extracted, given that none of the samples within these subpictures are intended to be output by the decoder. Figure 1 illustrates such a problematic configuration, whereby it is unclear if and how subpictures E, F, K, J and L can be extracted.

Figure 1: Example of problematic configuration of conformance cropping window and subpictures

The present document therefore proposes to introduce a constraint whereby subpictures must be defined in such a way that at least one part of every subpicture within the picture is located within the conformance cropping window of the full picture.

In addition, a process to rewrite the conformance cropping window is proposed to be added to the subpicture extraction process.

# Constraint on conformance cropping window and subpictures

It is proposed to add a constraint such that subpictures cannot be located completely outside of the conformance cropping window. The constraint below is proposed to be added to the SPS semantics:

For all subpictures i in the range of 0 to sps_num_subpics_minus1, it is a requirement of bitstream conformance that all of the following conditions are true:

–	(subpic_ctu_top_left_x[ i ] * CtbSizeY) shall be less than (pic_width_max_in_luma_samples – 1 – sps_conf_win_right_offset * SubWidthC).

–	((subpic_ctu_top_left_x[ i ] + subpic_width_minus1[ i ] + 1) * CtbSizeY – 1) shall be greater than (sps_conf_win_left_offset * SubWidthC).

–	(subpic_ctu_top_left_y[ i ] * CtbSizeY) shall be less than (pic_height_max_in_luma_samples – 1 – sps_conf_win_bottom_offset * SubHeightC).

–	((subpic_ctu_top_left_y[ i ] + subpic_height_minus1[ i ] + 1) * CtbSizeY – 1) shall be greater than (sps_conf_win_top_offset * SubHeightC).

# Subpicture extraction with a conformance cropping window

It is also proposed to add a description to the subpicture sub-bitstream extraction process in clause C.7 to clarify how to rewrite the conformance cropping window offsets in the SPS and PPS. In particular, the conformance cropping window offsets of the full picture are kept or not depending on where the subpicture is located within the full picture. If the subpicture is located in the middle of the picture, the conformance cropping window offsets for the subpicture are set to zero.

The following process is suggested:

–	The variable subpicIdx is derived to be such that SubpicIdVal[ subpicIdx ] is equal to subpicId.

–	The variables subpicConfWinLeftOffset, subpicConfWinRightOffset, subpicConfWinTopOffset and subpicConfWinBottomOffset are derived as follows:

if ( subpic_ctu_top_left_x[ subpicIdx ]  ==  0 )
	subpicConfWinLeftOffset = sps_conf_win_left_offset
else
	subpicConfWinLeftOffset = 0
if ( (subpic_ctu_top_left_x[ subpicIdx ] + subpic_width_minus1[ subpicIdx ] + 1) * CtbSizeY  >=  
		pic_width_max_in_luma_samples )
	subpicConfWinRightOffset = sps_conf_win_right_offset
else
	subpicConfWinRightOffset = 0
if ( subpic_ctu_top_left_y[ subpicIdx ]  ==  0 )
	subpicConfWinTopOffset = sps_conf_win_top_offset
else
	subpicConfWinTopOffset = 0
if ( (subpic_ctu_top_left_y[ subpicIdx ] + subpic_height_minus1[ subpicIdx ] + 1) * CtbSizeY  >=  
		pic_height_max_in_luma_samples )
	subpicConfWinBottomOffset = sps_conf_win_bottom_offset
else
	subpicConfWinBottomOffset = 0

–	Rewrite the values of sps_conf_win_left_offset, sps_conf_win_right_offset, sps_conf_win_top_offset and sps_conf_win_bottom_offset in all referred to SPS NAL units and the values of pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset and pps_conf_win_bottom_offset in all referred to PPS NAL units to be equal to subpicConfWinLeftOffset, subpicConfWinRightOffset, subpicConfWinTopOffset and subpicConfWinBottomOffset.

# Conclusion

A constraint on the usage of conformance cropping window and subpictures is proposed, together with an extension to the subpicture sub-bitstream extraction process to clarify how to rewrite the conformance cropping window offsets in the SPS and PPS.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).