| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0096 |
| --- | --- |

| Title: | AHG9: On signaling of chroma residual scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer Yan Ye, Jiancong Luo, Jie Chen 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | + 1 408-858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

In current VVC draft, if a picture contains multiple slices, the chroma residual scaling cannot be independently controlled for individual slices. In order to allow slice level control of chroma residual scaling, this contribution proposes a slice level chroma residual scaling flag slice_chroma_residual_scale_flag. Two methods are proposed. In the first method, picture level ph_chroma_residual_scale_flag is removed and replaced by slice level slice_chroma_residual_scale_flag. In the second method, slice level slice_chroma_residual_scale_flag is conditionally signalled based on the value of picture level ph_chroma_residual_scale_flag.

# Problem Statement

In VVC draft 8[1], LMCS can be controlled in both picture and slice level. The LMCS syntax of picture and slice header are shown in Table 1 and Table 2.

Table 1: picture header syntax of VVC draft 8

| if( sps_lmcs_enabled_flag ) { |  |
| --- | --- |
| ph_lmcs_enabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) { |  |
| ph_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| ph_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |

Table 2: slice header syntax of VVC draft 8

| if( ph_lmcs_enabled_flag ) |  |
| --- | --- |
| slice_lmcs_enabled_flag | u(1) |

In VVC draft 8, the chroma residual scaling of a slice is enabled if BOTH of the following conditions are satisfied:

- slice_lmcs_enabled_flag is equal to 1 AND

- ph_chroma_residual_scale_flag is equal to 1

For a given picture, if some (or all) of the slices in the picture have slice_lmcs_enabled_flag equal to 1, then for these slices, whether chroma residual scaling is used must be the same for all these slices, i.e., they must be all on or all off based on the value of ph_chroma_residual_scale_flag. Therefore, if a picture contains multiple slices, the chroma residual scaling cannot be independently controlled for individual slices in some cases. Consider the following simple example where there are two slices in the picture, both of them have slice level LMCS (slice_lmcs_enabled_flag == 1) enabled. In this simple example, the current VVC draft does NOT allow following combination:

- Slice 1: Luma mapping ON and Chroma residual scaling OFF

- Slice 2: Luma mapping ON and Chroma residual scaling ON

# Proposed methods

## Method 1

In method 1, signaling of chroma residual scaling is moved from the picture header to slice header.  The proposed method allows to enable/disable chroma residual scaling method in individual slice for which slice_lmcs_enabled_flag is enabled. The semantic of the slice level chroma residual scaling flag is defined as follows.

slice_chroma_residual_scale_enabled_flag equal to 1 specifies that chroma residual scaling is enabled for the slice. slice_chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling is disabled for  the slice. When slice_chroma_residual_scale_flag is not present, it is inferred to be equal to 0.

Table 3 and Table 4 show the part of picture header and slice header syntax of the proposed method. The changes as compared to VVC draft 8 are highlighted.

Table 3: picture header syntax of the method 1

| if( sps_lmcs_enabled_flag ) { |  |
| --- | --- |
| ph_lmcs_enabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) { |  |
| ph_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| ph_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |

Table 4: slice header syntax of the method 1

| if( ph_lmcs_enabled_flag ){ |  |
| --- | --- |
| slice_lmcs_enabled_flag | u(1) |
| if(slice_lmcs_enabled_flag && ChromaArrayType != 0 ) |  |
| slice_chroma_residual_scale_enabled_flag | u(1) |
| } |  |

## Method 2

In method 2, chroma residual scaling can be controlled both in picture level and in slice level.  If picture ph_chroma_residual_scale_flag is equal to 0, the slice level chroma residual scaling flag is not signalled. Table 5 shows the slice header syntax table of the proposed method. The picture header syntax table is same as VVC draft 8 ( as shown in Table 1).  The semantics of the slice_chroma_residual_scale_enabled_flag is the same as in method 1.

Table 5: Slice header of method 2

| if( ph_lmcs_enabled_flag ){ |  |
| --- | --- |
| slice_lmcs_enabled_flag | u(1) |
| if(slice_lmcs_enabled_flag && ChromaArrayType != 0 && ph_chroma_residual_scale_flag) |  |
| slice_chroma_residual_scale_enabled_flag | u(1) |
| } |  |

# Conclusion

In this contribution, it is proposed to allow slice level control of chroma residual scaling process. In the first method, it is proposed to move the chroma residual scaling flag from the picture header to the slice header. In the second method, control of chroma residual scaling can be in both picture and slice level. The slice level flag slice_chroma_residual_scale_enabled_flag is not signalled if the picture level flag ph_chroma_residual_scale_flag is 0.

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Jan. 2020.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).