| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0098-v1 |
| --- | --- |

| Title: | AHG9: On Slice Header Signalling of LMCS and Scaling Lists Information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson Andrew Segall Philip Cowan | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes modifications to slice header signalling related to LMCS and scaling list. Following is proposed:

- Proposal 1: Not signalling and inferring slice_lmcs_enabled_flag when picture header is included in slice header.

- Proposal 2: Not signalling and inferring slice_explicit_scaling_list_used_flag when picture header is included in slice header.

# Introduction

It is asserted that in VVC WD8 [1] when picture header is included in slice header the flags slice_lmcs_enabled_flag and slice_explicit_scaling_list_used_flag do not need to be signalled, since the picture header flags ph_lmcs_enabled_flag and ph_explicit_scaling_list_enabled_flag  are sufficient to control turning LMCS and scaling list On/ Off. Thus following is proposed:

- Proposal 1: Not signalling and inferring slice_lmcs_enabled_flag when picture header is included in slice header.

- Proposal 2: Not signalling and inferring slice_explicit_scaling_list_used_flag when picture header is included in slice header.

# Proposal

## Proposal 1

It is proposed to not signal slice_lmcs_enabled_flag when picture_header_in_slice_header_flag is equal to 1 and instead infer its value in this case.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ph_lmcs_enabled_flag && !picture_header_in_slice_header_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| if( entry_point_offsets_present_flag && NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

slice_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is enabled for the current slice. slice_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is not enabled for the current slice. When slice_lmcs_enabled_flag is not present and picture_header_in_slice_header_flag is equal to 1, slice_lmcs_enabled_flag is inferred to be equal to ph_lmcs_enabled_flag. Otherwise Wwhen slice_lmcs_enabled_flag is not present it is infered to be equal to 0.

## Proposal 2

It is proposed to not signal slice_explicit_scaling_list_used_flag when picture_header_in_slice_header_flag is equal to 1 and instead infer its value in this case.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ph_lmcs_enabled_flag) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag && !picture_header_in_slice_header_flag) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| if( entry_point_offsets_present_flag && NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

slice_explicit_scaling_list_used_flag equal to 1 specifies that the explicit scaling list, which is signalled in the referenced scaling list APS with aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_scaling_list_aps_id, is used in the scaling process for transform coefficients when decoding of the current slice. slice_explicit_scaling_list_used_flag equal to 0 specifies that no explicit scaling list is not used in the scaling process for transform coefficients when decoding of the current slice. When slice_explicit_scaling_list_used_flag is not present and picture_header_in_slice_header_flag is equal to 1, slice_explicit_scaling_list_used_flag is inferred to be equal to ph_explicit_scaling_list_enabled_flag. Otherwise Wwhen not present, the value of slice_explicit_scaling_list_used_flag is inferred to be equal to 0.

# Conclusion

This document proposes modifications to slice header signalling related to LMCS and scaling list. It is proposed to adopt the proposals in the next version of VVC draft.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-Q2001-vE, Versatile Video Coding (Draft 8), Brussels, January 2020