| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0115 |
| --- | --- |

| Title: | AHG9: On signalling of cropping windows and scaling windows |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Andrew Segall | Email: | samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution includes a proposal of a bitstream constraint for the cropping window offsets and scaling window offsets, requiring that at least one of the offsets is different than its default value when the flag that controls their presence is equal to 1.

# Introduction

In the default configuration of VTM (including the common test conditions), the flags sps_conformance_window_flag and pps_conformance_window_flag are both signalled equal to 1 even when all of the offsets are equal to 0. It is suggested to change VTM so that it never signals these flags equal to 1 when there is no offset.

Additionally, it is proposed to introduce bitstream constraints to prohibit that sps_conformance_window_flag and pps_conformance_window_flag are signalled equal to 1 when all offsets are equal to 0. It is asserted that there is no benefit or reason to allow setting the flags equal to 1 when all the corresponding offsets are equal to 0. On the contrary, it seems confusing to allow that to be signaled and it is asserted that it would be beneficial for decoders and bitstream analyzers to be able to just look at the value of a single flag in order to determine if cropping will be applied to the output pictures or not.

The following is proposed:

When sps_conformance_window_flag is equal to 1, it is a requirement of bitstream conformance that at least one of sps_conf_win_left_offset, sps_conf_win_right_offset, sps_conf_win_top_offset, and sps_conf_win_bottom_offset is not equal to 0.

When pps_conformance_window_flag is equal to 1, it is a requirement of bitstream conformance that at least one of pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset is not equal to 0.

A corresponding bitstream requirement is proposed for scaling_window_flag, but in this case expressed in relation to the values of the cropping window since these are the values the offsets will be inferred to when scaling_window_flag is equal to 0:

When scaling_window_flag is equal to 1, it is a requirement of bitstream conformance that at least one of scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset is not equal to pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset, respectively.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).