| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0118-v2 |
| --- | --- |

| Title: | AHG9/AHG12: On signaling of subpicture partitioning in SPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes the following HLS clean-ups in SPS:

- Proposal-1: Signaling sps_picture_header_in_slice_header_flag

- Proposal-2: Alignment on subpic_treated_as_pic_flag value across layers

# Proposal-1: Signaling sps_picture_header_in_slice_header_flag

When the picture header is contained in a slice header, it is assumed that the subpicture information is never used and only one slice is present per picture in the CLVS referring to the SPS. This assumption was hypothetically agreed, but not clearly described. To make HLS clean-ups, we propose the flag sps_picture_header_in_slice_header_flag. sps_picture_header_in_slice_header_flag equal to 1 specifies that the PH syntax structure is present in the slice header and only one slice is present per picture in the CLVS. In addition, no subpicture information is present when sps_picture_header_in_slice_header_flag equal to 1.

Proposed Text of Proposal-1:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_picture_header_in_slice_header_flag | u(1) |
| if( !sps_picture_header_in_slice_header_flag ) |  |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| … |  |
| } |  |
| … |  |
| } |  |

sps_picture_header_in_slice_header_flag equal to 1 specifies that the PH syntax structure is present in the slice header and only one slice is present per picture in the CLVS. sps_picture_header_in_slice_header_flag equal to 0 specifies that the PH syntax structure is not present in the slice header in the CLVS.

subpic_info_present_flag equal to 1 specifies that subpicture information is present for the CLVS and there may be one or more than one subpicture in each picture of the CLVS. subpic_info_present_flag equal to 0 specifies that subpicture information is not present for the CLVS and there is only one subpicture in each picture of the CLVS. When not present, the value of subpic_info_present_flag is inferred to be equal to 0.

When res_change_in_clvs_allowed_flag is equal to 1, the value of subpic_info_present_flag shall be equal to 0.

picture_header_in_slice_header_flag equal to 1 specifies that the PH syntax structure is present in the slice header. picture_header_in_slice_header_flag equal to 0 specifies that the PH syntax structure is not present in the slice header.

It is a requirement of bitstream conformance that the value of picture_header_in_slice_header_flag shall be the same in all coded slices in a CLVS.

The value of picture_header_in_slice_header_flag shall be equal to sps_picture_header_in_slice_header_flag.

When picture_header_in_slice_header_flag is equal to 1 for a coded slice, it is a requirement of bitstream conformance that no VCL NAL unit with nal_unit_type equal to PH_NUT shall be present in the CLVS.

When picture_header_in_slice_header_flag is equal to 0, all coded slices in the current picture shall have picture_header_in_slice_header_flag is equal to 0, and the current PU shall have a PH NAL unit.

# Proposal-2: Alignment on subpic_treated_as_pic_flag value across layers

For subpicture extraction from a multilayer bitstream, when subpic_treated_as_pic_flag[ i ] is equal to 1,  for each output layer and its reference layers in an OLS, all pictures in the output layer and its reference layers shall have the value of subpic_treated_as_pic_flag[ j ] equal to 1 for each value of j in the range of 0 to sps_num_subpics_minus1. Therefore, we propose to add the following constraint.

Proposed Text of Proposal-2:

subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to sps_independent_subpics_flag.

When subpic_treated_as_pic_flag[ i ] is equal to 1, it is a requirement of bitstream conformance that all of the following conditions are true for each output layer and its reference layers in an OLS that includes the layer containing the i-th subpicture as an output layer:

–	All pictures in the output layer and its reference layers shall have the same value of pic_width_in_luma_samples and the same value of pic_height_in_luma_samples.

–	All the SPSs referred to by the output layer and its reference layers shall have the same value of sps_num_subpics_minus1 and shall have the same values of subpic_ctu_top_left_x[ j ], subpic_ctu_top_left_y[ j ], subpic_width_minus1[ j ], subpic_height_minus1[ j ], and loop_filter_across_subpic_enabled_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	All pictures in each access unit in the output layer and its reference layers shall have the same value of SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	All pictures in the output layer and its reference layers shall have the value of subpic_treated_as_pic_flag[ j ] equal to 1 for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).