| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0121 |
| --- | --- |

| Title: | AHG9/AHG12: Virtual boundary signaling with subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes to relocate the virtual boundary syntax elements from PH to PPS, so that the picture-level updates of virtual boundary information are possible, when a picture is partitioned into multiple subpictures and the subpicture ID mapping information is updated by PPS within a coded video sequence.

# Problem Statement

In the latest VVC spec. draft, virtual boundary signaling may be present in SPS or Picture Header (PH). If a picture-level adaptation of virtual boundary information, the syntax elements would be signaled in PH. However, when the subpicture partitioning is used, it is disallowed to signal the virtual boundary information in PH, but allowed to be present in SPS only. Thus, when the picture is partitioned into subpictures, we may lose the functionality to change the virtual boundaries in picture-level.

The issue is that both subpicture and virtual boundary have the same main use cases, VR360. As shown in Fig. 1, for example, the original picture having virtual boundaries (four vertical lines) can be partitioned into three subpictures. At the beginning of the coded sequence, only left and center subpictures are contained in the extracted subpicture. After remapping subpicture ID values in PPS and the contents of subpictures are relocated, only center and right subpictures can be contained in the extracted subpicture. In that case, the virtual boundary information needs to be updated after subpicture ID remapping. However, the current spec. prohibits the picture-level updates of virtual boundary.

# Proposal

To address the above issue, we propose to move the virtual boundary syntax elements from PH to PPS, so that the virtual boundary information can be updated with subpicture ID remapping by PPS. As long as the syntax elements are present in PH, the picture-level update of virtual boundary information is not possible because modifying PH for bitstream extraction or merge is not desired. In the proposed spec. text, the virtual boundary information may be present in SPS or PPS, optionally.

Fig. 1 Example of subpictures with virtual boundary

Proposed Text

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| mixed_nalu_types_in_pic_flag | u(1) |
| … |  |
| pps_virtual_boundaries_present_flag | u(1) |
| if( pps_virtual_boundaries_present_flag ) { |  |
| pps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_ver_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_x[ i ] | u(13) |
| pps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_hor_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_y[ i ] | u(13) |
| } |  |
| … |  |
| } |  |

pps_virtual_boundaries_present_flag equal to 1 specifies that information of virtual boundaries is signalled in the PPS. pps_virtual_boundaries_present_flag equal to 0 specifies that information of virtual boundaries is not signalled in the PPS. When there is one or more than one virtual boundaries signalled in the PPS, the in-loop filtering operations are disabled across the virtual boundaries in the picture. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of pps_virtual_boundaries_present_flag is inferred to be equal to 0.

When sps_virtual_boundaries_present_flag is equal to 1, pps_virtual_boundaries_present_flag shall be equal to 0.

It is a requirement of bitstream conformance that, when subpic_info_present_flag is equal to 1, the value of pps_virtual_boundaries_present_flag shall be equal to 0.

The variable VirtualBoundariesPresentFlag is derived as follows:

VirtualBoundariesPresentFlag = 0
if( sps_virtual_boundaries_enabled_flag )
	VirtualBoundariesPresentFlag = sps_virtual_boundaries_present_flag  | |
			pps_virtual_boundaries_present_flag

pps_num_ver_virtual_boundaries specifies the number of pps_virtual_boundaries_pos_x[ i ] syntax elements that are present in the PH. When pps_num_ver_virtual_boundaries is not present, it is inferred to be equal to 0.

The variable NumVerVirtualBoundaries is derived as follows:

NumVerVirtualBoundaries = 0
if( sps_virtual_boundaries_enabled_flag )
	NumVerVirtualBoundaries = sps_virtual_boundaries_present_flag ?
			sps_num_ver_virtual_boundaries : pps_num_ver_virtual_boundaries

pps_virtual_boundaries_pos_x[ i ] specifies the location of the i-th vertical virtual boundary in units of luma samples divided by 8. The value of pps_virtual_boundaries_pos_x[ i ] shall be in the range of 1 to Ceil( pic_width_in_luma_samples ÷ 8 ) − 1, inclusive.

The list VirtualBoundariesPosX[ i ] for i ranging from 0 to NumVerVirtualBoundaries − 1, inclusive, in units of luma samples, specifying the locations of the vertical virtual boundaries, is derived as follows:

for( i = 0; i < NumVerVirtualBoundaries; i++)
	VirtualBoundariesPosX[ i ] = ( sps_virtual_boundaries_present_flag ?
			sps_virtual_boundaries_pos_x[ i ] : pps_virtual_boundaries_pos_x[ i ] ) * 8

The distance between any two vertical virtual boundaries shall be greater than or equal to CtbSizeY luma sample.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).