| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0137-v3 |
| --- | --- |

| Title: | On mvd_l1_zero_flag and NoBackwadPredFlag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomohiro Ikai | Tel: Email: | +81-50-5213-6334 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

In this contribution, some solutions for two problems of current VVC Draft 8 have been proposed. One problem is that there is no specification of the variables ColPic and NoBackwardPredFlag and the other problem is that mvd_l1_zero_flag is specified in only picture header even if reference picture list structure can be changed on slice header. Two solutions for the first problem have been shown Option 1 is that the variable ColPic as the almost same as that of HEVC is defined and a new variable IdenticalDirectionalFlag which is replaced to previous NoBackwadPredFlag is specified by using the decoding process for symmetric motion vector difference reference indices. Option 2 is that the variables ColPic and NoBackwadPredFlag as the almost same as that of HEVC are defined. Also, two solutions for the second problem have been shown. Option 1 is that the change of enabling condition of symmetric motion vector difference and mvd_l1_zero_flag by the variable IdenticalDirectionalFlag and option 2 is that mvd_l1_zero_flag is specified in the picture header or in the slice header by the syntax element rpl_info_in_ph_flag exclusively. Neither proposal changes the results of the CTC.

Problem Statement

Problem 1

One problem of current VVC Draft 8 [1] is that there is no definition of the variables ColPic and NoBackwardPredFlag. Those variables have been already used for temporal motion vector prediction and BCW.

In VVD Draft 8, there is a following note.

[Ed. (BB): Define ColPic NoBackwardPredFlag.]

Problem 2

The other problem is that mvd_l1_zero_flag is only specified in the picture header. When there are multiple slices in a picture and different reference picture list structs can be selected for each slice, mvd_l1_zero_flag may not be useful.

On CTC, there is one slice in one picture and mvd_l1_zero_flag is switched picture by picture. In the case of the RA structure, the first B slice of GOP uses mvd_l1_zero_flag set to 1, but the other slices do not use it. In the case of the LB structure, all B slices use it. mvd_l1_zero_flag has some coding gains if the two directions of prediction are identical, however, it may not have any coding gain, if the two directions of prediction are different. As an experimental result of the RA condition on a modified CTC shown in Table 1, there was an average of 1.24% loss if mvd_l1_zero_flag is set to 1 on all B slices. Therefore, the identical direction seems to be a necessary condition for mvd_l1_zero_flag.

In cases of multiple slices in a picture, there can be some slices with identical direction while other slices without identical direction, the above mvd_l1_zero_flag condition should be changed accordingly. Otherwise, there may be some loss. Therefore, it is proposed that mvd_l1_zero_flag is to be specified depending on the reference picture list structure.

Table 1	An experimental result of mvd_l1_zero_flag set to 1 on all B slices

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.10% | 1.25% | 1.26% | 97% | 101% |
| Class A2 | 1.91% | 2.10% | 2.00% | 97% | 101% |
| Class B | 1.21% | 1.41% | 1.45% | 97% | 101% |
| Class C | 0.88% | 1.07% | 1.09% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 1.24% | 1.43% | 1.42% | 98% | 101% |
| Class D | 0.82% | 0.87% | 0.64% | 96% | 100% |
| Class F | 0.80% | 0.95% | 0.90% | 98% | 100% |

Proposal

Solutions for problem 1

In this section, two solutions for problem 1 have been proposed. Regarding the variable ColPic, both methods are identical. Regarding the variable NoBackwadPredFlag, one is that it is replaced with a new variable IdenticalDirectionFlag and the other is that it is defined as almost the same as that of HEVC. Both options can solve problem 1.

Option 1.1

The decoding process for the variable ColPic is defined almost the same as that of HEVC. The variable NoBackwadPredFlag is replaced with a new variable IdenticalDirectionFlag which specifies that there is no backward prediction for each active short-term reference picture in RefPicList0 or RefPicList1 of the current slice or there is no forward prediction in RefPicList0 or RefPicList1 of the current slice. This can be defined by symmetric motion vector difference reference indices simply.

The variables RefIdxSymL1 and RefIdxSymL1 specify the list 0 and list 1 reference picture indices for symmetric motion vector differences. (RefIdxSymL0 > −1 && RefIdxSymL1 > −1) means that there is at least one set with different directions in picture reference lists. !(RefIdxSymL0 > −1 && RefIdxSymL1 > −1) means that the directions of all sets in picture reference lists are identical.

- 8.3.5 Decoding process for symmetric motion vector difference reference indices and IdenticalDirectionFlag

Output of this process are RefIdxSymL0 and RefIdxSymL1 specifying the list 0 and list 1 reference picture indices for symmetric motion vector differences, i.e., when sym_mvd_flag is equal to 1 for a coding unit.

The variable RefIdxSymLX with X being 0 and 1 is derived as follows:

- The variable currPic specifies the current picture.

- RefIdxSymL0 is set equal to −1.

- For each index i with i = 0..NumRefIdxActive[ 0 ] − 1, the following applies:

- When all of the following conditions are true, RefIdxSymL0 is set to i:

- RefPicList[ 0 ][ i ] is a short-term-reference picture,

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) > 0,

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) < DiffPicOrderCnt( currPic, RefPicList[ 0 ][ RefIdxSymL0 ] ) or RefIdxSymL0 is equal to −1.

- RefIdxSymL1 is set equal to −1.

- For each index i with i = 0..NumRefIdxActive[ 1 ]  − 1, the following applies:

- When all of the following conditions are true, RefIdxSymL1 is set to i:

- RefPicList[ 1 ][ i ] is a short-term-reference picture,

- DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) < 0,

- DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) > DiffPicOrderCnt( currPic, RefPicList[ 1 ][ RefIdxSymL1 ] ) or RefIdxSymL1 is equal to −1.

- When RefIdxSymL0 is equal to −1 or RefIdxSymL1 is equal to −1, the following applies:

- RefIdxSymL0 is set equal to −1 and RefIdxSymL1 is set equal to −1.

- For each index i with i = 0..NumRefIdxActive[ 0 ]  − 1, the following applies:

- When all of the following conditions are true, RefIdxSymL0 is set to i:

- RefPicList[ 0 ][ i ] is a short-term-reference picture,

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) < 0,

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) > DiffPicOrderCnt( currPic, RefPicList[ 0 ][ RefIdxSymL0 ] ) or RefIdxSymL0 is equal to −1.

- For each index i with i = 0..NumRefIdxActive[ 1 ]  − 1, the following applies:

- When all of the following conditions are true, RefIdxSymL1 is set to i:

- RefPicList[ 1 ][ i ] is a short-term-reference picture,

- DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) > 0,

- DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) < DiffPicOrderCnt( currPic, RefPicList[ 1 ][ RefIdxSymL1 ] ) or RefIdxSymL1 is equal to −1.

The variable IdenticalDirectionFlag is derived as follows:

IdenticalDirectionFlag = ( RefIdxSymL0 > −1 && RefIdxSymL1 > −1 ) ? 0 : 1

### 8.3.6 Decoding process for collocated picture

This process is invoked at the beginning of the decoding process for each P or B slice, after decoding of the slice header as well as the invocation of the decoding process for reference picture list construction for the slice as specified in clause 8.3.2, but prior to the decoding of any coding unit.

When ph_temporal_mvp_enabled_flag is equal to 1, the variable ColPic is derived as follows:

- If slice_type is equal to B and slice_collocated_from_l0_flag is equal to 0, ColPic is set equal to RefPicList1[ slice_collocated_ref_idx ].

- Otherwise (slice_type is equal to B and slice_collocated_from_l0_flag is equal to 1, or slice_type is equal to P), ColPic is set equal to RefPicList0[ slice_collocated_ref_idx ].

Option 1.2

The decoding process for the variables ColPic and NoBackwadPredFlag are defined almost the same as that of HEVC. This specification is almost the same as VTM software; however, current VTM software does not check long-term reference pictures in the reference picture lists when the variable NoBackwadPredFlag is derived.

### 8.3.6 Decoding process for collocated picture and no backward prediction flag

This process is invoked at the beginning of the decoding process for each P or B slice, after decoding of the slice header as well as the invocation of the decoding process for reference picture list construction for the slice as specified in clause 8.3.2, but prior to the decoding of any coding unit.

When ph_temporal_mvp_enabled_flag is equal to 1, the variable ColPic is derived as follows:

- If slice_type is equal to B and slice_collocated_from_l0_flag is equal to 0, ColPic is set equal to RefPicList1[ slice_collocated_ref_idx ].

- Otherwise (slice_type is equal to B and slice_collocated_from_l0_flag is equal to 1, or slice_type is equal to P), ColPic is set equal to RefPicList0[ slice_collocated_ref_idx ].

The variable NoBackwardPredFlag is derived as follows:

- If DiffPicOrderCnt( aPic, CurrPic ) is less than 0 for each active short-term reference picture aPic in RefPicList0 or RefPicList1 of the current slice, NoBackwardPredFlag is set equal to 1.

- Otherwise, NoBackwardPredFlag is set equal to 0.

Solutions for problem 2

In this section, two different solutions for problem 2 have been proposed. One is the change of enabling condition for symmetric motion vector difference and mvd_l1_zero_flag, and the other is the change of syntax for mvd_l1_zero_flag. Both options can solve problem 2.

Option 2.1 for problem 2

By using the variable IdenticalDirectionFlag which specifies whether two directions of reference pictures are identical, the two modes of symmetric motion vector difference and mvd_l1_zero_flag can be encoded or decoded exclusively.

#### 7.3.10.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_smvd_enabled_flag && !mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && !inter_affine_flag[ x0 ][ y0 ] && !IdenticalDirectionFlag RefIdxSymL0 > −1 && RefIdxSymL1 > −1 ) |  |
| sym_mvd_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( NumRefIdxActive[ 0 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( NumRefIdxActive[ 1 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && IdenticalDirectionFlag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 1 ] = 0 |  |
| } else { |  |
| if( sym_mvd_flag[ x0 ][ y0 ] ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = −MvdL0[ x0 ][ y0 ][ 0 ] |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = −MvdL0[ x0 ][ y0 ][ 1 ] |  |
| } else |  |
| mvd_coding( x0, y0, 1, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 1, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 1, 2 ) |  |
| } |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| … |  |
| } |  |
| } |  |
| } |  |

If the variable IdenticalDirectionFlag is not defined, the following description is suggested. The specification is equivalent to the above.

#### 7.3.10.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_smvd_enabled_flag && !mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && !inter_affine_flag[ x0 ][ y0 ] && RefIdxSymL0 > −1 && RefIdxSymL1 > −1 ) |  |
| sym_mvd_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( NumRefIdxActive[ 0 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( NumRefIdxActive[ 1 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && !( RefIdxSymL0 > −1 && RefIdxSymL1 > −1 ) && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 1 ] = 0 |  |
| } else { |  |
| if( sym_mvd_flag[ x0 ][ y0 ] ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = −MvdL0[ x0 ][ y0 ][ 0 ] |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = −MvdL0[ x0 ][ y0 ][ 1 ] |  |
| } else |  |
| mvd_coding( x0, y0, 1, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 1, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 1, 2 ) |  |
| } |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| … |  |
| } |  |
| } |  |
| } |  |

Option 2.2 for problem 2

By using the syntax element rpl_info_in_ph_flag which specifies whether reference picture list information is present in the PH, mvd_l1_zero_flag is specified in the picture header or in the slice header exclusively. As a result, the encoder can switch mvd_l1_zero_flag by the reference picture list structure.

#### 7.3.2.7 Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( rpl_info_in_ph_flag ) |  |
| ref_pic_lists( ) |  |
| … |  |
| if( ph_inter_slice_allowed_flag ) { |  |
| … |  |
| if( rpl_info_in_ph_flag ) |  |
| mvd_l1_zero_flag | u(1) |
| … |  |
| } |  |
| } |  |

#### 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !rpl_info_in_ph_flag && ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) |  |
| ref_pic_lists( ) |  |
| … |  |
| if( !rpl_info_in_ph_flag && slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| … |  |
| } |  |

Reference

[1]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 8), Joint Video Experts Team Document, JVET-Q2001-vD, Brussels, March 2020.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).