| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0141_r2 |
| --- | --- |

| Title: | Disabling Dependent Quantization and Sign Data Hiding in Transform Skip blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomonori Hashimoto Eiichi Sasaki Tomoko Aono Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-50-5213-6337 tomonori.hashimoto@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to disable dependent quantization (DQ) and sign data hiding (SDH) in transform skip blocks normatively. In the current specification, DQ and SDH can be enabled in transform skip blocks when slice_ts_residual_coding_disabled_flag is enabled. It is asserted that decoder and encoder design become complicated due to the combination of DQ and TS or SDH and TS. This cleanup contribution proposes not to use DQ and SDH in TS blocks with syntax level restriction (i.e. exclusive usage of DQ and TS / SDH and TS in syntax level).

In r2, simulation results were updated.

Introduction

In the previous meeting, a syntax element of slice_ts_residual_coding_disabled_flag [1] was adopted into the VVC specification [2]. When this flag is enabled, regular residual coding (RRC) is used instead of transform skip residual coding (TSRC), even if transform skip is applied. In the current specification, dependent quantization (DQ) and sign data hiding (SDH) are applied in RRC. It means DQ and SDH can be enabled in transform skip case which hadn’t occurred before.

The authors think it would be questionable whether allowing these combinations is intentional. Basically, increasing number of combinations in transform coefficients stage incurs verification costs and is not desirable.

Furthermore, the VTM doesn’t support the above combination properly (encoder / decoder mismatch occurs when slice_ts_residual_coding_disabled_flag, BDPCM and SDH are enabled) where BDPCM encoder doesn’t support SDH as reported in ticket #981.

This contribution proposes to disable dependent quantization and sign data hiding in transform skip normatively to resolve these problems as a cleanup

Proposed syntax change

We propose the following syntax changes to ensure DQ and TS / SDH and TS exclusive usage. The added parts are highlighted in yellow.

- 7.3.10.11	Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| if( sps_mts_enabled_flag && cu_sbt_flag && cIdx = = 0 && log2TbWidth = = 5 && log2TbHeight < 6 ) |  |  |
| … |  |  |
| QState = 0 |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| startQStateSb = QState |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( i < lastSubBlock && i > 0 ) { |  |
| sb_coded_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| if( sb_coded_flag[ xS ][ yS ] && ( xS > 3 ¦ ¦ yS > 3 ) && cIdx = = 0 ) |  |
| MtsZeroOutSigCoeffFlag = 0 |  |
| firstSigScanPosSb = numSbCoeff |  |
| lastSigScanPosSb = −1 |  |
| firstPosMode0 = ( i = = lastSubBlock ? lastScanPos : numSbCoeff − 1 ) |  |
| firstPosMode1 = firstPosMode0 |  |
| for( n = firstPosMode0; n >= 0 && remBinsPass1 >= 4; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( sb_coded_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) && ( xC != LastSignificantCoeffX ¦ ¦ yC != Last SignificantCoeffY ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| remBinsPass1− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |  |
| abs_level_gtx_flag[ n ][ 0 ] | ae(v) |
| remBinsPass1− − |  |
| if( abs_level_gtx_flag[ n ][ 0 ] ) { |  |
| par_level_flag[ n ] | ae(v) |
| remBinsPass1− − |  |
| abs_level_gtx_flag[ n ][ 1 ] | ae(v) |
| remBinsPass1− − |  |
| } |  |
| if( lastSigScanPosSb = = −1 ) |  |
| lastSigScanPosSb = n |  |
| firstSigScanPosSb = n |  |
| } |  |
| AbsLevelPass1[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + par_level_flag[ n ] + abs_level_gtx_flag[ n ][ 0 ] + 2 * abs_level_gtx_flag[ n ][ 1 ] |  |
| if( ph_dep_quant_enabled_flag && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] ) |  |
| QState = QStateTransTable[ QState ][ AbsLevelPass1[ xC ][ yC ] & 1 ] |  |
| firstPosMode1 = n − 1 |  |
| } |  |
| for( n = firstPosMode0; n > firstPosMode1; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( abs_level_gtx_flag[ n ][ 1 ] ) |  |
| abs_remainder[ n ] | ae(v) |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] +2 * abs_remainder[ n ] |  |
| } |  |
| for( n = firstPosMode1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( sb_coded_flag[ xS ][ yS ] ) |  |
| dec_abs_level[ n ] | ae(v) |
| if( AbsLevel[ xC ][ yC ] > 0 ) { |  |
| if( lastSigScanPosSb = = −1 ) |  |
| lastSigScanPosSb = n |  |
| firstSigScanPosSb = n |  |
| } |  |
| if( ph_dep_quant_enabled_flag && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] ) |  |
| QState = QStateTransTable[ QState ][ AbsLevel[ xC ][ yC ] & 1 ] |  |
| } |  |
| if( ph_dep_quant_enabled_flag ¦ ¦ !pic_sign_data_hiding_enabled_flag ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ cIdx ] ) |  |
| signHidden = 0 |  |
| else |  |
| signHidden = ( lastSigScanPosSb − firstSigScanPosSb > 3 ? 1 : 0 ) |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( ( AbsLevel[ xC ][ yC ] > 0 ) && ( !signHidden ¦ ¦ ( n != firstSigScanPosSb ) ) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| if( ph_dep_quant_enabled_flag && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] ) { |  |
| QState = startQStateSb |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( AbsLevel[ xC ][ yC ] > 0 ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 2 * AbsLevel[ xC ][ yC ] − ( QState > 1 ? 1 : 0 ) ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| QState = QStateTransTable[ QState ][ AbsLevel[ xC ][ yC ] & 1 ] |  |
| } else { |  |
| sumAbsLevel = 0 |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( AbsLevel[ xC ][ yC ] > 0 ) { |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = AbsLevel[ xC ][ yC ] * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( signHidden ) { |  |
| sumAbsLevel += AbsLevel[ xC ][ yC ] |  |
| if( ( n = = firstSigScanPosSb ) && ( sumAbsLevel % 2 ) = = 1 ) ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = −TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |  |

Simulation

For the sanity check purpose, we conducted the simulation and confirmed that disabling combination doesn’t change / harm coding efficiency.

Note: In current VTM, slice_ts_residual_coding_disabled_flag is disabled in CTC.

Following software is used for the comparison tests of these performance:

VTM-8.0 + MergeRequest!1475 + MergeRequest!1480

MergeRequest!1475 and MergeRequest!1480 are not included in current VTM, however, another encoder / decoder mismatch related to SDH occurs without these fixes. Therefore, these fixes are applied to check the performance the combination of SDH and TS in this contribution.

Performance of the combination of TS and DQ

Anchor:  CTC (slice_ts_residual_coding_disabled_flag = 0)

VVC8:   DQ enabled for TS block (slice_ts_residual_coding_disabled_flag = 1)

Proposed: DQ disabled for TS block (slice_ts_residual_coding_disabled_flag = 1)

Following tables show summary of the performance of the combination of TS and DQ based on current VVC spec and proposal in IO, RA and LDB.

IO

|  | DQ | SDH | Y | U | V | Y(classF) | U(classF) | V(classF) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VVC8 | Y (Y in TS) | N | 0.29 | 0.18 | 0.10 | 7.93 | 6.23 | 6.32 |
| Proposed | Y (N in TS) | N | 0.28 | 0.18 | 0.10 | 7.17 | 5.47 | 5.71 |

RA

|  | DQ | SDH | Y | U | V | Y(classF) | U(classF) | V(classF) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VVC8 | Y (Y in TS) | N | 0.15 | 0.03 | 0.02 | 6.64 | 5.25 | 5.37 |
| Proposed | Y (N in TS) | N | 0.15 | 0.05 | 0.07 | 6.09 | 4.60 | 4.87 |

LDB

|  | DQ | SDH | Y | U | V | Y(classF) | U(classF) | V(classF) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VVC8 | Y (Y in TS) | N | 0.11 | -0.15 | -0.17 | 5.92 | 4.44 | 4.07 |
| Proposed | Y (N in TS) | N | 0.10 | -0.07 | 0.08 | 5.19 | 4.28 | 3.42 |

Conclusion

This contribution proposes to disable dependent quantization and sign data hiding in transform skip (i.e. exclusive usage of DQ and TS / SDH and TS in syntax level). It is recommended to adopt this fix in the next VVC specification.

Reference

[1]	A. Nalci, H. Wang, M. Coban, M. Karczewicz, T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, T.-C. Ma, H.-J. Jhu, X. Xiu, Y.-W. Chen, X. Wang, “CE3-2.4: Luma and Chroma BDPCM for Lossless Coding with Regular Residual Coding,” Joint Video Expert Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q0089, 17th Meeting: Brussels, BE, 7–17 January 2020.

[2]	B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8),” Joint Video Expert Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q2001, 17th Meeting: Brussels, BE, 7–17 January 2020.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).