| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0147-v1 |
| --- | --- |

| Title: | AHG9: On picture order count and output order |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is asserted that when a spliced bitstream is formed by concatenating a first bitstream and a second bitstream where the latter starts with a picture having ph_poc_msb_present_flag equal to 1, the "bumping" process could output pictures in an incorrect order. This contribution proposes to overcome the asserted issue similarly to what is specified in HEVC, including both of the following proposal pieces:

- Add poc_decr_flag in the picture header, gated by gdr_or_irap_pic_flag. poc_decr_flag equal to 1 causes PicOrderCntVal values of the pictures in the DPB to be decremented. poc_decr_flag shall not be to 1 unless the picture is in a CVSS AU.

- When poc_decr_flag is equal to 1, derive a value to be decremented from the PicOrderCntVal values of the pictures in the DPB. The derivation is asserted to be equivalent to one of the POC resetting options of HEVC. The resulting PicOrderCntVal values of the pictures in the DPB are smaller than the PicOrderCntVal of the current picture.

# Introduction

Figure 1 presents an example that is used to explain the asserted problem. Originally, two separate bitstreams have been created. The last picture in bitstream 1 has PicOrderCntVal equal to M. The first picture in bitstream 2 has ph_poc_msb_present_flag equal to 1 and PicOrderCntVal equal to N. The two bitstreams are spliced by inserting bitstream 2 after bitstream 1.

Figure 1. Example of concatenating two bitstreams into a spliced bitstream.

Clause C.5.2.4 specifies the "bumping" process for output order DPB, including the following step:

- The picture or pictures that are first for output are selected as the one having the smallest value of PicOrderCntVal of all pictures in the DPB marked as "needed for output".

When N < M, the "bumping" process may therefore output some pictures of bitstream 2 before some pictures of bitstream 1. The picture output order would differ from the output order produced by an output time conforming decoder. Moreover, since the picture content of bitstreams 1 and 2 is unrelated, such "interleaved" output would be undesirable.

It is asserted that VVC Draft 8 does not offer good means for bumping all pictures of bitstream 1 before any pictures of bitstream 2. The following means were considered but found unsuitable:

- Setting poc_msb_val of the first picture in bitstream 2 so that N becomes greater than M. However, this would require the splicer to parse all subsequent picture headers and modify their poc_msb_val values (if any) with the same delta value as was used to modify the poc_msb_val of the first picture in bitstream 2. This process is needed to keep the delta POC values between pictures of bitstream 2 unchanged so that the reference picture list syntax structures apply without changes.

- Setting no_output_of_prior_pics_flag equal to 1 in the first picture of bitstream 2. However, this would cause some pictures of bitstream 1 not to be output at all.

HEVC solves the asserted issue by indicating POC resetting in the slice header. When POC resetting is indicated for a picture, the POC values of pictures that are present in the DPB are decremented so that they become smaller than the POC of the current picture. This contribution proposes to introduce an approach with a POC resetting indication and POC decrementing similarly to what is specified in HEVC.

# Proposal

## Summary

This contribution proposes the following:

- Add poc_decr_flag in the picture header, gated by gdr_or_irap_pic_flag. poc_decr_flag equal to 1 causes PicOrderCntVal values of the pictures in the DPB to be decremented. poc_decr_flag shall not be to 1 unless the picture is in a CVSS AU.

- When poc_decr_flag is equal to 1, derive a value to be decremented from the PicOrderCntVal values of the pictures in the DPB. The derivation is asserted to be equivalent to one of the POC resetting options of HEVC. As a result, the PicOrderCntVal values of the pictures in the DPB are smaller than the PicOrderCntVal of the current picture.

## Proposed picture header changes

| … |  |
| --- | --- |
| if( gdr_or_irap_pic_flag ) { |  |
| no_output_of_prior_pics_flag | u(1) |
| poc_decr_flag | u(1) |
| } |  |
| … |  |

poc_decr_flag equal to 1 causes PicOrderCntVal values of the pictures in the DPB to be decremented. poc_decr_flag equal to 0 causes no impact on PicOrderCntVal values of the pictures in the DPB. When not present, the value of poc_decr_flag is inferred to be equal to 0. poc_decr_flag shall have the same value in all picture headers of an AU. When an AU is not a CVSS AU, poc_decr_flag in each picture header of the AU shall be equal to 0.

## Proposed changes in clause 8.3.1 (decoding process for picture order count)

…

When ph_poc_msb_present_flag is equal to 0 and the current picture is not a CLVSS picture,When the current AU is not the first AU of the bitstream, the variables prevPicOrderCntLsb and prevPicOrderCntMsb are derived as follows:

- Let prevTid0Pic be the previous picture in decoding order that has nuh_layer_id equal to the nuh_layer_id of the current picture, has TemporalId equal to 0, and is not a RASL or RADL picture.

- The variable prevPicOrderCntLsb is set equal to ph_pic_order_cnt_lsb of prevTid0Pic.

- The variable prevPicOrderCntMsb is set equal to PicOrderCntMsb of prevTid0Pic.

The variable PicOrderCntMsb of the current picture is derived as follows:

- If ph_poc_msb_present_flag is equal to 1, PicOrderCntMsb is set equal to poc_msb_val * MaxPicOrderCntLsb.

- Otherwise (ph_poc_msb_present_flag is equal to 0), if the current picture is a CLVSS picture, PicOrderCntMsb is set equal to 0.

- Otherwise, PicOrderCntMsb is derived as follows:

if( ( ph_pic_order_cnt_lsb < prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − ph_pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(199)
else if( ( ph_pic_order_cnt_lsb > prevPicOrderCntLsb )  &&
		( ( ph_pic_order_cnt_lsb − prevPicOrderCntLsb ) > ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

PicOrderCntVal is derived as follows:

PicOrderCntVal = PicOrderCntMsb + ph_pic_order_cnt_lsb	(200)

NOTE 1 – All CLVSS pictures for which poc_msb_val is not present will have PicOrderCntVal equal to ph_pic_order_cnt_lsb since for those pictures PicOrderCntMsb is set equal to 0.

The value of PicOrderCntVal shall be in the range of −231 to 231 − 1, inclusive.

In one CVS, the PicOrderCntVal values for any two coded pictures with the same value of nuh_layer_id shall not be the same.

All pictures in any particular AU shall have the same value of PicOrderCntVal.

When the current picture is the first picture in an AU, the current picture is not the first picture in the bitstream, and poc_decr_flag is equal to 1, the following applies:

- The variable decrPicOrderCntMsb is derived as follows:

if( ( ph_pic_order_cnt_lsb < prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − ph_pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	decrPicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(3)
else if( ( ph_pic_order_cnt_lsb > prevPicOrderCntLsb )  &&
		( ( ph_pic_order_cnt_lsb − prevPicOrderCntLsb ) > ( MaxPicOrderCntLsb / 2 ) ) )
	decrPicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	decrPicOrderCntMsb = prevPicOrderCntMsb

- PicOrderCntVal of each picture in the DPB is decremented by ( decrPicOrderCntMsb + ph_pic_order_cnt_lsb ).

# Examples

In the examples, a spliced bitstream is formed as presented Figure 1. The splicer includes an EOS NAL unit before the first picture of bitstream 2 to indicate that it is a CVSS AU and sets its poc_decr_flag value equal to 1.

In the first example, let us assume that MaxPicOrderCntLsb is equal to 256, M is equal 2000, and N is equal to 0. When decoding the picture that was originally the first picture of bitstream 2:

- prevPicOrderCntMsb is derived to be equal to 1792.

- prevPicOrderCntLsb is derived to be equal to 208.

- decrPicOrderCntMsb is derived to be equal to 1792 + 256 = 2048.

- PicOrderCntVal of each picture in the DPB is decremented by 2048.

In the second example, let us assume that MaxPicOrderCntLsb is equal to 256, M is equal 2000, and N is equal to 1000. Consequently, the picture that was originally the first picture of bitstream 2 has ph_pic_order_cnt_lsb equal to 232, and its decoding causes the following:

- prevPicOrderCntMsb is derived to be equal to 1792.

- prevPicOrderCntLsb is derived to be equal to 208.

- decrPicOrderCntMsb is derived to be equal to 1792.

- PicOrderCntVal of each picture in the DPB is decremented by ( 1792 + 232 ) = 2024.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).