| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0151-v3 |
| --- | --- |

| Title: | AHG6/AHG12: Uncoded subpictures and potential applications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Johannes Sauer | Tel: Email: | +49241-80-27678 sauer@ient.rwth-aachen.de |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

In this document it is shown how VVC can be extended with a mechanism that enables uncoded subpictures. Uncoded subpictures can be used for efficient coding when subpictures do not completely fill up a picture, by providing completely unused regions. Examples are shown for OMAF use cases, as well as for 360° video coding of 4x3 cubemaps. Further, uncoded subpictures can be used to reserve space which is not filled with coded data, but with content generated from already coded content. Here, the example of high-level, efficient geometry padding for 360° video is shown.

# Introduction

Uncoded subpictures can be enabled very easily. Apart from some conformance statements only the sequence parameter set is changed as follows (changes in yellow):

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag | u(1) |
| sps_allow_uncoded_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| if( sps_allow_uncoded_subpics_flag) { |  |
| subpic_is_uncoded_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_len_minus1 | ue(v) |
| subpic_id_mapping_explicitly_signalled_flag | u(1) |
| if( subpic_id_mapping_explicitly_signalled_flag ) { |  |
| subpic_id_mapping_in_sps_flag | u(1) |
| if( subpic_id_mapping_in_sps_flag ) |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |

# Use cases of uncoded subpictures

## Flexible packing of bitstreams of different resolutions

Extractor tracks generated in OMAF typically cover a whole picture (Figure 1). Unused regions would have a penalty on the coding performance.

Figure 1 – Example from OMAF working draft: Example of extractor track combining MCTSs from bitstreams of different resolution

Allowing uncoded subpictures enables more flexibility when choosing the resolutions of the different bitstreams).

Figure 2: Extractor track with arbitrary ratio of high and low resolutions for the component bitstreams. Orange: Uncoded subpicture.

## Coding of occupancy in V-PCC

## Video based point cloud compression (V-PCC) expresses a point cloud using several pictures for geometry, texture/attributes and occupancy (Figure 3, Figure 5 and Figure 7). The occupancy map has non-rectangular shapes. However, if there are large regions which are not occupied, they could be approximated using uncoded subpictures (Figure 8). The geometry and attribute pictures can make use of uncoded subpictures likewise (Figure 4, Figure 6). 
The large, not occupied regions can occur due to the packing of the different regions into frames. Especially, the size of the pictures is defined by the largest picture in the sequence. It can be significantly larger than the packing required for a given frame.
However, for this use case signaling of the uncoded subpictures alone in the SPS is likely not sufficient, as the unused regions change during the sequence. A suitable subpicture partitioning could be chosen and configured in the SPS, signaling of which subpictures are coded could then occur in PPS as follows (changes in yellow):

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpic_id_mapping_in_pps_flag | u(1) |
| if(sps_allow_uncoded_subpics_flag) { |  |
| subpic_is_uncoded_in_pps_flag | u(1) |
| } |  |
| if( subpic_id_mapping_in_pps_flag ¦¦ subpic_is_uncoded_in_pps_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| if( subpic_id_mapping_in_pps_flag ) { |  |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| if( subpic_is_uncoded_in_pps_flag ) { |  |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_is_uncoded_flag[ i ] | u(v) |
| } |  |
| … |  |

## Further the packing (optimal arrangement) of the different parts of the point cloud into a rectangular picture of minimum size is a NP-hard problem. It might be possible to simplify this problem by loosening the minimum size constraint, i.e. using uncoded subpictures to increase the size of the coded pictures, without affecting coding efficiency.

## A short experiment was performed to estimate the amount of unused regions in V-PCC. A segment of 32 frames of the following sequences was encoded using the RA configuration and the number of uncoded CTUs determined from an analysis of the occupancy maps:

| Sequence | # Used CTUs | # Unused CTUs | Percentage Unused CTUs |
| --- | --- | --- | --- |
| basketball_player_vox11 | 7481 | 10439 | 58,25 |
| dancer_vox11 | 7947 | 10613 | 57,18 |
| longdress_vox10 | 2301 | 1539 | 40,08 |
| loot_vox10 | 2446 | 754 | 23,56 |
| queen_vox10 | 2276 | 1244 | 35,34 |
| redandblack_vox10 | 2258 | 1262 | 35,85 |
| soldier_vox10 | 2806 | 394 | 12,31 |
| Average |  |  | 37,51 |

## The high average value of 37% of unused CTUs makes it likely that there will be sufficiently large regions that can be approximated using uncoded subpictures.

| Figure 3: Geometry | uncoded Figure 4: Geometry using uncoded subpictures |
| --- | --- |
| Figure 5: Texture / Attributes | uncoded Figure 6: Texture / Attributes using uncoded subpictures |
| Figure 7: Occupancy | uncoded Figure 8: Occupancy using uncoded subpictures |

uncoded

Coding of 360° video in 4x3 face layout

Cubemap based 360° video is typically coded using two face rows (3x2 layout). While the 4x3 layout has a higher number of continuous boundaries it also has unused regions which have negative impact on the coding efficiency. This can also be addressed by using uncoded subpictures. Further, this scheme allows the usage of the reference sample wrap-around for cubemap based 360° video coding (Figure 9). JVET-P0316 uses uncoded areas, which are not subpicture based. It shows that this can achieve coding gains compared to the face row (3x2) layout.

0

4

3

5

1

2

uncoded

uncoded

Figure 9: Non-compact cube face layout / 4x3 layout. Reference sample wraparound can be used between face 1 and 4.

## Alternative approach to layered coding

By using subpictures all representations (different resolutions, texture and depthmap, multiple views) for a given POC can be tied into one unit (the picture). If this causes unused regions they can be handled by using uncoded subpictures (Figure 10). This could also require mechanisms for predicting subpictures from other subpictures, potentially from the same POC, which is not further discussed in this document.

Base layer

Upsampled base layer
(empty subpicture)

Enhancement layer

Unused

Figure 10: Example for using subpictures and uncoded subpictures for scalable coding.

## Providing geometry padding using uncoded subpictures

From the already performed CE13 experiments it could be observed that geometry padding can improve the objective coding performance and subjective quality (seam artifacts) significantly (JVET-L0647).

For geometry padding samples need to be projected and interpolated from other cube faces. This can be done on the fly. However this would require changes of the design at CTU level. Further, it requires arbitrary buffer access, as the required samples can be located at up to three (cube corner) locations in the picture.
Alternatively, a different reference picture can be used for each cube face, which only contains the cube face plus a padding area. This approach has the problem of increased memory usage and generation of unused samples at the decoder side.

Here we describe how the problems of the buffer level approach can be avoided by using subpictures and uncoded subpictures. A cube-mapping based 360° video is arranged into a picture such that each face belongs to a different subpicture. Between the faces additional uncoded subpictures are inserted (Figure 11). These subpictures do not have to be encoded, but can be generated by reprojection from the coded cube faces.

Figure 11: A picture consisting of regular and uncoded subpictures. 
Green: Regular subpictures: the coded cube faces.
White: Uncoded subpictures, which will be filled by the geometry padding generation process.

### Loop filter configuration

Loop filters should be turned off between face boundaries, especially between discontinuous face boundaries. This can be done by appropriate configuration of the subpictures.

### Signalling of the 360° geometry

Application of geometry padding requires knowledge of the 360° geometry. Here we describe a simple signaling of the 360° geometry which is heavily based on the region-wise packing SEI message of HEVC.
The coded cube faces can have different sizes and do not need to have any specific relative arrangement (continuous face boundaries). The changed sequence parameters set is:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_geometry_padding_enabled_flag | u(1) |
| if( sps_geometry_padding_enabled_flag ) |  |
| cube_packing( ) |  |
| … |  |
| } |  |

| cube_packing( ) { | Descriptor |
| --- | --- |
| 360_video_geometry_type | u(3) |
| if( 360_video_geometry_type == FORMAT_360_CMP ¦¦ |  |
| 360_video_geometry_type == FORMAT_360_EAC ¦¦ |  |
| 360_video_geometry_type == FORMAT_360_HEC) { |  |
| max_geometry_padding_size | ue(v) |
| for( i = 0; i < num_cube_face_equal_to_6; i++ ) { |  |
| coded_face_id[ i ] | ue(v) |
| coded_face_rot[ i ] | ue(v) |
| coded_face_width[ i ] | ue(v) |
| coded_face_height[ i ] | ue(v) |
| coded_face_top[ i ] | ue(v) |
| coded_face_left[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |

### Geometry padding APS

Geometry padding is applied at picture level. Generation of the complete padding regions may be unnecessary. Thus a high level signaling of the required geometry padding amount was added, we use an adaptation parameter set (APS) for this purpose.
The decoder has to track the generated amount of geometry padding for each reference picture. Since many pictures serve as reference pictures several times, additionally required padding is added incrementally when required.

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| else if( aps_params_type = = GP_APS ) |  |
| gp_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| gp_data () { | Descriptor |
| --- | --- |
| incremental_padding_flag | u(1) |
| for ( list = 0; list < 2; list++ ) { |  |
| num_ref_idx_minus1[ list ] | u(2) |
| for ( idx = 0; idx < NumRefIdx[ list ]; idx++ ) { |  |
| pad_picture_flag[ list ][ idx ] | u(1) |
| if ( pad_picture_flag[ list ][ idx ] ) { |  |
| for ( face = 0; face < 6; face++ ) { |  |
| pad_top_boundary_flag[ list ][ idx ][ face] | u(1) |
| pad_bottom_boundary_flag[ list ][ idx ][ face ] | u(1) |
| pad_left_boundary_flag[ list ][ idx ][ face ] | u(1) |
| pad_right_boundary_flag[ list ][ idx ][ face ] | u(1) |
| } |  |
| for ( face = 0; face < 6; face++ ) { |  |
| if ( pad_top_boundary_flag[ list ][ idx ][ face ] ) { |  |
| pad_top_boundary_width[ list ][ idx ][ face ] | ue(v) |
| } |  |
| if ( pad_bottom_boundary_flag[ list ][ idx ][ face ] ) { |  |
| pad_bottom_boundary_width[ list ][ idx ][ face ] | ue(v) |
| } |  |
| if ( pad_left_boundary_flag[ list ][ idx ][ face ] ) { |  |
| pad_left_boundary_width[ list ][ idx ][ face ] | ue(v) |
| } |  |
| if ( pad_right_boundary_flag[ list ][ idx ][ face ] ) { |  |
| pad_right_boundary_width[ list ][ idx ][ face ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

### Processes for buffer based / high level 360° geometry padding

Here we give an overview of the processes which operate at buffer level for enabling 360° coding tools. A detailed processes can be found in the spec text provided along with this document.
The padded regions in the buffer are processed at buffer/level. Inputs and outputs are the reference pictures. At CTU level only the actual samples from the neighborhood are used for prediction.
The geometry padding is only described for the luma component so far.

#### Overview of generation of samples for geometry padding

The process for doing geometry padding is illustrated as follows (compare Figure 12):

- An integer sample location in the padded region is selected

- The location is mapped to a unit cube packing location

- The unit cube packing location is projected to a 3D cube location

- The 3D cube location is back-projected to a source unit cube packing location. This automatically moves the sample from the padding region inside another face.

- The source unit cube packing is mapped to a fractional sample location in the coded face arrangement.

- Nearest neighbor interpolation is used to obtain an integer sample location.

- The sample of step 1 is filled with the found source sample location.

Figure 12: Derivation of source sample location for filling of padding region

### Implementation status

VTM 8.0 has been modified to provide uncoded subpictures and add the picture level geometry padding, as well as the signaling of geometry padding usage. Notable difference to the description above are:

- Padding usage is signaled as SEI instead of APS.

### Results

Compared to the results reported in JVET-M0033, particularly CE13.5.1.a and CE13.5.2.a the gain of geometry padding is reduced (Table 1). Partly this is likely due to the resetting of the entropy coding process at subpicture boundaries. However, there is a clear loss for chroma, which is still being investigated.
The decoder complexity of the picture level approach is largely reduced by signaling the required padding amount.

Note: The encoder always generates complete padding of each face, while the decoder only produces the required padding. In order to check for match on encoder and decoder the padding region is cleared before the check is performed. Since this clearing is not actually required it was disabled for the timing measurements. Timing measurements are likely unreliable. Although run on the same machine significant differences have been observed in the runtime of the decoder for the same sequence, when run again. A mismatch occurred in both original and modified decoder for the BranCastle sequence (QP22). It only occurs when decoding the concatenated bitstream (using parcatStatic).

#### Implementation in VTM 8.0

|  | End-to-end WS-PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class S1 | -0,84% | 1,95% | 3,05% |  | 101% |
| Class S2 | -2,00% | -0,56% | -1,06% |  | 103% |
| Overall | -1,31% | 0,95% | 1,41% |  | 102% |

Table 1: Geometry padding (1 subpicture per face) against original

#### Bugfix

The derivation of the JointCbCr Sign Flag did not consider (ignore) uncoded areas. This led to a decrease of coding efficiency. The bug has been fixed.

|  | End-to-end WS-PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class S1 | -0,91% | -2,48% | -2,95% |  | 102% |
| Class S2 |  | -2,03% |  | -3,12% |  | -4,37% |  |  |  | 104% |  |
| Overall |  | -1,36% |  | -2,74% |  | -3,52% |  |  |  | 103% |  |

Table 2: Geometry padding (1 subpicture per face) against original. Bug fixed for derivation of JointCbCr Sign Flag

# Patent rights declaration(s)

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, would be prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form), or make them available free of charge (per box 1 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form). We are however aware that also other entities may have current or pending patent rights relating to the technology described in this contribution.