| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0155_v1 |
| --- | --- |

| Title: | AHG9/14 : On lossless coding granularity |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Jungah Choi Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seunghwan Kim 10225 Willow Creek Rd San Diego, CA 92131 United States | Tel: Email: | +82-2-6912-6561 sunmi.yoo@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In this contribution, two granularities for the lossless coding is provided by high level syntax changes. Currently, the lossy and lossless mixture can be allowed at a picture level since the dependent quantization (DQ) and the sign data hiding (SDH) switches are defined in the picture header. Therefore, the different granularities for the lossless coding is proposed.

- Method 1: Slice level lossless coding

- Method 2: Picture level lossless coding

Introduction

At the Brussels meeting, slice_ts_residual_coding_disabled_flag is introduced to determine which residual coding method is used when transform_skip_flag is equal to 1[1]. This flag is intentionally defined to support the efficient lossless coding, as the residual information to be coded using the regular residual coding syntax. Allowing the residual coding switch in the slice header implies that the lossless coding can be used at a slice level. However, the lossless coding can be determined at a picture level since the DQ and SDH switches are defined in the picture header.

Proposed method

Method 1: Allowing the lossless coding at a slice level

If the DQ and SDH flags are defined in the slice level, the minimal unit for the lossless coding can be at the slice level, since both of these tools shall be 0 for the lossless coding. The proposed spec text is described using strikethrough for the removal and highlights for the addition.

- 7.3.2.7 Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_dep_quant_enabled_flag ) |  |
| ph_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !ph_dep_quant_enabled_flag ) |  |
| pic_sign_data_hiding_enabled_flag | u(1) |
| … |  |
| } |  |

- 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( sps_dep_quant_enabled_flag ) |  |
| slice_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !slice_dep_quant_enabled_flag ) |  |
| slice_sign_data_hiding_enabled_flag | u(1) |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| if( NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

- 7.4.3.7 Picture header structure semantics

…

ph_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. ph_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When ph_dep_quant_enabled_flag is not present, it is inferred to be equal to 0.

pic_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. pic_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When pic_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

…

- 7.4.8.1 General slice header semantics

…

slice_ts_residual_coding_disabled_flag equal to 1 specifies that the residual_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. slice_ts_residual_coding_disabled_flag equal to 0 specifies that the residual_ts_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. When slice_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 0.

slice_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. slice_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When slice_dep_quant_enabled_flag is not present, it is inferred to be equal to 0.

slice_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. slice_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When slice_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

…

Method 1 + JVET-R0153

If the DQ and SDH flags are defined in the slice level, the minimal unit for the lossless coding can be at the slice level, since both of these tools shall be 0 for the lossless coding. Moreover, the proposed method in JVET-R0153[2] can be combined with this method. The proposed spec text is described using strikethrough for the removal and highlights for the addition.

- 7.3.2.7 Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_dep_quant_enabled_flag ) |  |
| ph_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !ph_dep_quant_enabled_flag ) |  |
| pic_sign_data_hiding_enabled_flag | u(1) |
| … |  |
| } |  |

- 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_dep_quant_enabled_flag ) |  |
| slice_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !slice_dep_quant_enabled_flag ) |  |
| slice_sign_data_hiding_enabled_flag | u(1) |
| if( !slice_dep_quant_enabled_flag && !slice_sign_data_hiding_enabled_flag) |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| if( NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

- 7.4.3.7 Picture header structure semantics

…

ph_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. ph_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When ph_dep_quant_enabled_flag is not present, it is inferred to be equal to 0.

pic_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. pic_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When pic_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

…

- 7.4.8.1 General slice header semantics

…

slice_ts_residual_coding_disabled_flag equal to 1 specifies that the residual_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. slice_ts_residual_coding_disabled_flag equal to 0 specifies that the residual_ts_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. When slice_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 0.

slice_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. slice_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When slice_dep_quant_enabled_flag is not present, it is inferred to be equal to 0.

slice_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. slice_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When slice_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

…

Method 2: Picture level lossless coding

If there are more than one slice in a picture, signalling slice_ts_residual_coding_disabled_flag in the every slice header does not seem to be desirable. Therefore, it is proposed to signal ph_ts_residual_coding_disabled_flag in picture header. The proposed spec text is described using strikethrough for the removal and highlights for the addition.

- 7.3.2.7 Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_dep_quant_enabled_flag ) |  |
| ph_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !ph_dep_quant_enabled_flag ) |  |
| pic_sign_data_hiding_enabled_flag | u(1) |
| ph_ts_residual_coding_disabled_flag | u(1) |
| … |  |
| } |  |

- 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| if( NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| … |  |
| } |  |

- 7.4.3.7 Picture header structure semantics

…

ph_ts_residual_coding_disabled_flag equal to 1 specifies that the residual_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. ph_ts_residual_coding_disabled_flag equal to 0 specifies that the residual_ts_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. When ph_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 0.

ph_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. ph_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When ph_dep_quant_enabled_flag is not present, it is inferred to be equal to 0.

pic_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. pic_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When pic_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

…

- 7.4.8.1 General slice header semantics

…

slice_ts_residual_coding_disabled_flag equal to 1 specifies that the residual_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. slice_ts_residual_coding_disabled_flag equal to 0 specifies that the residual_ts_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. When slice_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 0.

…

Method 2 + JVET-R0153

If there are more than one slice in a picture, signalling slice_ts_residual_coding_disabled_flag in the every slice header does not seem to be desirable. Therefore, it is proposed to signal ph_ts_residual_coding_disabled_flag in picture header. Moreover, the proposed method in JVET-R0153 can be combined with this method. The proposed spec text is described using strikethrough for the removal and highlights for the addition.

- 7.3.2.7 Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_dep_quant_enabled_flag ) |  |
| ph_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !ph_dep_quant_enabled_flag ) |  |
| pic_sign_data_hiding_enabled_flag | u(1) |
| if( !ph_dep_quant_enabled_flag && !pic_sign_data_hiding_enabled_flag) |  |
| ph_ts_residual_coding_disabled_flag | u(1) |
| … |  |
| } |  |

- 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_ts_residual_coding_disabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| slice_explicit_scaling_list_used_flag | u(1) |
| if( NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

- 7.4.3.7 Picture header structure semantics

…

ph_ts_residual_coding_disabled_flag equal to 1 specifies that the residual_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. ph_ts_residual_coding_disabled_flag equal to 0 specifies that the residual_ts_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. When ph_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 0.

ph_dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled for the current picture. ph_dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled for the current picture. When ph_dep_quant_enabled_flag is not present, it is inferred to be equal to 0.

pic_sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled for the current picture. pic_sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled for the current picture. When pic_sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

…

- 7.4.8.1 General slice header semantics

…

slice_ts_residual_coding_disabled_flag equal to 1 specifies that the residual_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. slice_ts_residual_coding_disabled_flag equal to 0 specifies that the residual_ts_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. When slice_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 0.

…

Reference(s)

- B. Bross, J. Chen, S. Liu, and Y. Wang, “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-Q2001, 17th Meeting: Brussels, BE, 7–17 Jan. 2020.

- J. Choi, S. Yoo, J. Heo, J. Choi, J. Lim, S. Kim, “AHG9/AHG16: On slice_ts_residual_coding_disabled_flag,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-R0153, 17th Meeting: by teleconference, 15–24 April 2020.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).