| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0156-v1 |
| --- | --- |

| Title: | Signaling cleanup on SPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Biao Wang, Semih Esenlik, Han Gao, Anand Meher Kotra, Elena Alshina | Tel: Email: | biao.wang@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes following cleanups for SPS:

- When sps_video_parameter_set_id is equal to 0, sps_ptl_dpb_hrd_params_present_flag is not signaled but inferred to be equal to 1.

- When sps_ptl_dpb_hrd_params_present_flag is equal to 1, inter_layer_ref_pics_present_flag is not signaled but inferred to be equal to 0.

- The signaling of sps_sublayer_dpb_params_flag is based on not only sps_max_sublayers_minus1, but also sps_ptl_dpb_hrd_params_present_flag.

- The signaling of sps_independent_subpics_flag is enabled only when there are at least two subpictures, and add a constraint to the semantics of subpic_info_present_flag

# Proposed Cleanups

For the latest working draft of VVC [1], four aspects of cleaning up of SPS signaling are proposed.

## Signaling of sps_ptl_dpb_hrd_params_present_flag

The VVC Specification constrains the value of sps_ptl_dpb_hrd_params_present_flag shall be equal to vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ], as shown below:

“sps_ptl_dpb_hrd_params_present_flag equal to 1 specifies that a profile_tier_level( ) syntax structure and a dpb_parameters( ) syntax structure are present in the SPS, and a general_hrd_parameters( ) syntax structure and an ols_hrd_parameters( ) syntax structure may also be present in the SPS. sps_ptl_dpb_hrd_params_present_flag equal to 0 specifies that none of these four syntax structures is present in the SPS. The value of sps_ptl_dpb_hrd_params_present_flag shall be equal to vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ]”.

Meanwhile, the following constraint is applied to the bitstream when sps_video_parameter_set_id is equal to 0.

‘‘‘

when sps_video_parameter_set_id is equal to 0, the following applies:

–	The SPS does not refer to a VPS.

–	No VPS is referred to when decoding each CLVS referring to the SPS.

–	The value of vps_max_layers_minus1 is inferred to be equal to 0.

–	The CVS shall contain only one layer (i.e., all VCL NAL unit in the CVS shall have the same value of nuh_layer_id).

–	The value of GeneralLayerIdx[ nuh_layer_id ] is inferred to be equal to 0.

–	The value of vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is inferred to be equal to 1.

’’’

Therefore, when sps_video_parameter_set_id is equal to 0, it is not necessary to signal the value of sps_ptl_dpb_hrd_params_present_flag because it shall be equal to 1.

The following modification is proposed to clean up the redundancy of these two syntax elements, with additions are highlighted (also applies to later sections)

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| if (sps_video_parameter_set_id !=0) |  |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |
| … |  |

Meanwhile, the semantics sps_ptl_dpb_hrd_params_present_flag is changed as follows:

“sps_ptl_dpb_hrd_params_present_flag equal to 1 specifies that a profile_tier_level( ) syntax structure and a dpb_parameters( ) syntax structure are present in the SPS, and a general_hrd_parameters( ) syntax structure and an ols_hrd_parameters( ) syntax structure may also be present in the SPS. sps_ptl_dpb_hrd_params_present_flag equal to 0 specifies that none of these four syntax structures is present in the SPS. The value of sps_ptl_dpb_hrd_params_present_flag shall be equal to vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ]. When not present, the value of sps_ptl_dpb_hrd_params_present_flag is inferred to be equal to 1.”

## Signaling of inter_layer_ref_pics_present_flag

The inter_layer_ref_pics_present_flag has following semantics:

“inter_layer_ref_pics_present_flag equal to 0 specifies that no ILRP is used for inter prediction of any coded picture in the CLVS. inter_layer_ref_pics_present_flag equal to 1 specifies that ILRPs may be used for inter prediction of one or more coded pictures in the CLVS. When sps_video_parameter_set_id is equal to 0, the value of inter_layer_ref_pics_present_flag is inferred to be equal to 0. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of inter_layer_ref_pics_present_flag shall be equal to 0.”

When sps_ptl_dpb_hrd_params_present_flag is equal to 1, the value of vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] shall be equal to 1, and hence the value of inter_layer_ref_pics_present_flag shall be equal to 0. Therefore, it is not necessary to signal the value of inter_layer_ref_pics_present_flag in this case.

It is suggested to modify the signaling of inter_layer_ref_pics_present_flag as follows:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| long_term_ref_pics_flag | u(1) |
| if (!sps_ptl_dpb_hrd_params_present_flag) |  |
| inter_layer_ref_pics_present_flag | u(1) |
| … |  |

Meanwhile, the semantics is modified as follows, with deletion striked through (also applies to later sections):

“inter_layer_ref_pics_present_flag equal to 0 specifies that no ILRP is used for inter prediction of any coded picture in the CLVS. inter_layer_ref_pics_flag equal to 1 specifies that ILRPs may be used for inter prediction of one or more coded pictures in the CLVS. When sps_video_parameter_set_id is equal to 0, the value of inter_layer_ref_pics_present_flag is inferred to be equal to 0. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of inter_layer_ref_pics_present_flag shall be equal to 0 When not present, the value of inter_layer_ref_pics_present_flag is inferred to be equal to 0.”

## Signaling of sps_sublayer_dpb_params_flag

The signaling of sps_sublayer_dpb_params_flag is only based on sps_max_sublayers_minus1 in current VVC, as shown below.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_max_sublayers_minus1 > 0 ) |  |
| sps_sublayer_dpb_params_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| dpb_parameters( sps_max_sublayers_minus1, sps_sublayer_dpb_params_flag ) |  |
| … |  |

The sps_sublayer_dpb_params_flag is used as an input parameter of the syntax element structure dpb_parameters and is not used anywhere else. However, when sps_ptl_dpb_hrd_params_present_flag is equal to 0, the sps_sublayer_dpb_params_flag would never be used. Therefore, in this case, the signaling of sps_sublayer_dpb_params_flag shall be skipped to avoid signaling redundancy.

Therefore, the signaling of sps_sublayer_dpb_params_flag shall be considered together with syntax structure dpb_parameters, and the following signaling is proposed for them:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if(sps_ptl_dpb_hrd_params_present_flag){ |  |
| if (sps_max_sublayers_minus1 > 0) |  |
| sps_sublayer_dpb_params_flag | u(1) |
| dpb_parameters( sps_max_sublayers_minus1, sps_sublayer_dpb_params_flag ) |  |
| } |  |
| … |  |

## Signaling of sps_independent_subpics_flag

The sps_independent_subpics_flag is signaled always when subpic_info_present_flag is equal to 1 in current signaling in SPS, as shown below:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| … |  |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag | u(1) |
| for ( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |
| … |  |

However, when sps_num_subpics_minus1 is equal to 0, i.e. there is only one subpicture in pictures referring to the current SPS, the for loop condition “sps_num_subpics_minus1 > 0” will prevent the usage of “if( !sps_independent_subpics_flag)” in the loop content. In other words, it doesn’t matter which value sps_independent_subpics_flag is signaled when sps_num_subpics_minus1 is equal to 0 as it will never be used.

To remove signaling redundancy in this case, the signaling of sps_independent_subpics_flag is proposed to be modified as follows:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| … |  |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| if (sps_num_subpics_minus1>0) |  |
| sps_independent_subpics_flag | u(1) |
| for ( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |
| … |  |

Meanwhile, the semantics of sps_independent_subpics_flag is changed as follows:

“sps_independent_subpics_flag equal to 1 specifies that all subpicture boundaries in the CLVS are treated as picture boundaries and there is no loop filtering across the subpicture boundaries. sps_independent_subpics_flag equal to 0 does not impose such a constraint. When not present, the value of sps_independent_subpics_flag is inferred to be equal to 10.”

It is also suggested adding the following constraint for the semantics of subpic_info_present_flag:

“subpic_info_present_flag equal to 1 specifies that subpicture information is present for the CLVS and there may be one or more than one subpicture in each picture of the CLVS. subpic_info_present_flag equal to 0 specifies that subpicture information is not present for the CLVS and there is only one subpicture in each picture of the CLVS. It is a requirement of bitstream conformance that when the value of subpic_info_present_flag is 1, for the value of pic_width_max_in_luma_samples and the value of pic_height_max_in_luma_samples, at least one of them shall be larger than the CTB size CtbSizeY.”

Because when both pic_width_max_in_luma_samples and pic_height_max_in_luma_samples are not larger than CTB size, dividing a picture into multiple subpictures is impossible since subpicture information is counted in units of CTB size.

# Conclusions

This contribution proposes 4 aspects to clean up the redundancy of signaling in SPS. The source code based on VTM-8.0 is attached for the proposed aspects. It is suggested to adopt the proposed cleanups to the next version of VVC specification.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “Versatile Video Coding (Draft 8)”, JVET-Q2001-vE, B. Bross, J. Chen, S. Liu, Y.-K. Wang