| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 202 | Document: JVET-R0160-v1 |
| --- | --- |

| Title: | AHG9: High level syntax cleanup for LMCS, ALF and SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Jiancong Luo, Mohammed Golam Sarwer, Yan Ye, Ru-Ling Liao Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution includes two aspects.

The first aspect of this contribution is to align the high level control scheme of Adaptive Loop Filter (ALF) and Sample Adaptive Offset (SAO) with that of Luma Mapping with Chroma Scaling (LMCS) by controlling ALF and SAO at SPS, PH and SH level and removing the slice level ALF parameter adaptation.

The second aspect of this contribution is to fix the semantics of ALF, SAO and LMCS high level control related syntax elements according to the current control scheme.

Proposal 1: ALF/SAO and LMCS high level control scheme alignment

Problem statement

In VVC draft 8 [1], ALF and SAO share the same high level control scheme. Both of these two coding tools can be controlled at sequence level and one of picture level or slice level (but not at both picture and slice level). First, a SPS enabled flag is signaled to control ALF/SAO at CLVS level. At PPS level, a PPS flag is signaled to indicate whether ALF/SAO is further controlled in picture level or slice level. If the PPS flag indicates ALF/SAO is further controlled in picture level, a PH ALF/SAO enabled flag is signaled followed by ALF parameters (if it is enabled); if the PPS flag indicates ALF/SAO is further controlled in slice level, a SH ALF/SAO enabled flag is signaled followed by ALF parameters (if it is enabled). The high level ALF syntax are shown in Table 1 to Table 4 and the high level SAO syntax are shown in Table 5 to Table 8.

However, LMCS has a different high level control scheme from ALF and SAO. LMCS can be controlled at sequence level, picture level and slice level. First, an SPS enabled flag controls LMCS at CLVS level. If the SPS enabled flag is equal to 1, a PH enabled flag is further signaled to control LMCS at picture level, and if it is enabled in picture level, the LMCS parameter information is signaled in PH as well. If PH enabled flag is equal to 1, a SH enabled flag is further signaled to control LMCS at slice level but LMCS parameter information cannot be signaled in SH even if it is enabled in slice level. The LMCS control syntax is shown in Table 9 to Table 11.

Table 1 ALF syntax in SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_alf_enabled_flag | u(1) |
| if( sps_alf_enabled_flag && ChromaArrayType != 0 ) |  |
| sps_ccalf_enabled_flag | u(1) |

Table 2 ALF syntax in PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| alf_info_in_ph_flag | u(1) |

Table 3: ALF syntax in the picture header

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag && alf_info_in_ph_flag ) { |  |
| ph_alf_enabled_flag | u(1) |
| if( ph_alf_enabled_flag ) { |  |
| ph_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < ph_num_alf_aps_ids_luma; i++ ) |  |
| ph_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| ph_alf_chroma_idc | u(2) |
| if( ph_alf_chroma_idc ) |  |
| ph_alf_aps_id_chroma | u(3) |
| if( sps_ccalf_enabled_flag ) { |  |
| ph_cc_alf_cb_enabled_flag | u(1) |
| if( ph_cc_alf_cb_enabled_flag ) |  |
| ph_cc_alf_cb_aps_id | u(3) |
| ph_cc_alf_cr_enabled_flag | u(1) |
| if( ph_cc_alf_cr_enabled_flag ) |  |
| ph_cc_alf_cr_aps_id | u(3) |
| } |  |
| } |  |
| } |  |

Table 4:ALF syntax in the slice header

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag && !alf_info_in_ph_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| if( sps_ccalf_enabled_flag ) { |  |
| slice_cc_alf_cb_enabled_flag | u(1) |
| if( slice_cc_alf_cb_enabled_flag ) |  |
| slice_cc_alf_cb_aps_id | u(3) |
| slice_cc_alf_cr_enabled_flag | u(1) |
| if( slice_cc_alf_cr_enabled_flag ) |  |
| slice_cc_alf_cr_aps_id | u(3) |
| } |  |
| } |  |
| } |  |

Table 5 SAO syntax in SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_sao_enabled_flag | u(1) |

Table 6: SAO syntax in PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sao_info_in_ph_flag | u(1) |

Table 7: SAO syntax in the picture header

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_sao_enabled_flag && sao_info_in_ph_flag ) { |  |
| ph_sao_luma_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| ph_sao_chroma_enabled_flag | u(1) |
| } |  |

Table 8: SAO syntax in the slice header

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_sao_enabled_flag && !sao_info_in_ph_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |

Table 9 LMCS syntax in SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_lmcs_enabled_flag | u(1) |

Table 10: LMCS syntax in the picture header

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_lmcs_enabled_flag ) { |  |
| ph_lmcs_enabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) { |  |
| ph_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| ph_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |

Table 11: LMCS syntax in the slice header

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ph_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |

From the above description, there are two inconsistencies between high level control scheme of LMCS and ALF/SAO.

First, LMCS can be controlled in three levels, SPS, PH and SH. When it is enabled in a higher level, the lower level can disable it. But for ALF and SAO, they can only be controlled at two levels, SPS and one of PH or SH. When ALF and SAO is enabled in SPS level, they can be further controlled in PH or SH, the decision is indicated by a flag signaled in PPS. So the control mechanism is different between LMCS and ALF/SAO

Second, when ALF is controlled in PH and enabled, the parameter information of ALF is signaled in PH. When ALF is controlled in SH and enabled, the parameter information of ALF is signaled in SH. So for ALF, when the control is at the slice level, different slices may have different ALF parameters. In contrast, for LMCS, SH can only enable or disable it but cannot signal the parameter information when enabled. That is to say, LMCS parameters must be the same for all the slices (that enable LMCS) within the same picture. It is another inconsistency between ALF and LMCS. In VTM-8.0, the encoder derives the ALF or LMCS parameters at the picture level; in our knowledge, there has been no evidence presented that shows slice-level adaptation of ALF parameters has coding efficiency benefit.

Proposed method

To align the high level control of LMCS and ALF/SAO, it is proposed to align the ALF /SAO controlling scheme with that of LMCS. That is, ALF/SAO are controlled at three levels: sequence, picture and slice level. When ALF/SAO is enabled in a higher level, the lower level can disable it. When it is disabled in a higher level, a lower level cannot enable it. Besides, slice level ALF parameter adaption is removed. So ALF can only be enabled or disabled at SH, the parameters of ALF cannot be changed at slice level. It is asserted such alignment simplifies the signaling of ALF-related syntax elements in the SH.

The syntax changes of ALF are shown in Table 12 to Table 14 and the semantics changes are given below the table. Note that CCALF is considered a sub-case of ALF so corresponding changes/simplifications are made to CCALF as well.

Table 12. ALF PPS syntax changes in proposal 1

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| alf_info_in_ph_flag | u(1) |

Table 13: ALF picture header syntax changes in proposal 1

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag && alf_info_in_ph_flag ) { |  |
| ph_alf_enabled_flag | u(1) |
| if( ph_alf_enabled_flag ) { |  |
| ph_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < ph_num_alf_aps_ids_luma; i++ ) |  |
| ph_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| ph_alf_chroma_idc | u(2) |
| if( ph_alf_chroma_idc ) |  |
| ph_alf_aps_id_chroma | u(3) |
| if( sps_ccalf_enabled_flag ) { |  |
| ph_cc_alf_cb_enabled_flag | u(1) |
| if( ph_cc_alf_cb_enabled_flag ) |  |
| ph_cc_alf_cb_aps_id | u(3) |
| ph_cc_alf_cr_enabled_flag | u(1) |
| if( ph_cc_alf_cr_enabled_flag ) |  |
| ph_cc_alf_cr_aps_id | u(3) |
| } |  |
| } |  |
| } |  |

Table 14: ALF Slice header syntax changes in proposal 1

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag && !alf_info_in_ph_flag ph_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| if( sps_ccalf_enabled_flag) { |  |
| if(ph_cc_alf_cb_enabled_flag) |  |
| slice_cc_alf_cb_enabled_flag | u(1) |
| if( slice_cc_alf_cb_enabled_flag ) |  |
| slice_cc_alf_cb_aps_id | u(3) |
| if(ph_cc_alf_cr_enabled_flag) |  |
| slice_cc_alf_cr_enabled_flag | u(1) |
| if( slice_cc_alf_cr_enabled_flag ) |  |
| slice_cc_alf_cr_aps_id | u(3) |
| } |  |
| } |  |
| } |  |

Table 15: ALF CTU syntax changes in proposal 1

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = CtbAddrX << CtbLog2SizeY |  |
| yCtb = CtbAddrY << CtbLog2SizeY |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( CtbAddrX, CtbAddrY ) |  |
| if( slice_alf_enabled_flag ){ |  |
| alf_ctb_flag[ 0 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( alf_ctb_flag[ 0 ][ CtbAddrX ][ CtbAddrY ] ) { |  |
| if( sliceph_num_alf_aps_ids_luma > 0 ) |  |
| alf_use_aps_flag | ae(v) |
| if( alf_use_aps_flag ) { |  |
| if(sliceph_num_alf_aps_ids_luma > 1 ) |  |
| alf_luma_prev_filter_idx | ae(v) |
| } else |  |
| alf_luma_fixed_filter_idx | ae(v) |
| } |  |
| if(sliceph _alf_chroma_idc = = 1 ¦ ¦ sliceph _alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 1 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( alf_ctb_flag[ 1 ][ CtbAddrX ][ CtbAddrY ] && alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 0 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| } |  |
| if(sliceph _alf_chroma_idc = = 2 ¦ ¦ sliceph _alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 2 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( alf_ctb_flag[ 2 ][ CtbAddrX ][ CtbAddrY ] && alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 1 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| } |  |
| } |  |
| if( slice_cc_alf_cb_enabled_flag ) |  |
| alf_ctb_cc_cb_idc[ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( slice_cc_alf_cr_enabled_flag ) |  |
| alf_ctb_cc_cr_idc[ CtbAddrX ][ CtbAddrY ] | ae(v) |

sps_alf_enabled_flag equal to 0 specifies that the adaptive loop filter is disabled. sps_alf_enabled_flag equal to 1 specifies that the adaptive loop filter is may be enabled.

sps_ccalf_enabled_flag equal to 0 specifies that the cross-component adaptive loop filter is disabled. sps_ccalf_enabled_flag equal to 1 specifies that the cross-component adaptive loop filter may be enabled.

alf_info_in_ph_flag equal to 1 specifies that ALF information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. alf_info_in_ph_flag equal to 0 specifies that ALF information is not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure.

ph_alf_enabled_flag equal to 1 specifies that adaptive loop filter is may be enabled for one, or more, or all slices associated with the PH and may be applied to Y, Cb, or Cr colour component in the  slices. ph_alf_enabled_flag equal to 0 specifies that adaptive loop filter may be is disabled for one, or more, or all slices associated with the PH. When not present, ph_alf_enabled_flag is inferred to be equal to 0.

ph_cc_alf_cb_enabled_flag equal to 1 specifies that cross-component filter for Cb colour component is may be enabled for one, or more, or all slices associated with the PH and may be applied to Cb colour component in the slices. ph_cc_alf_cb_enabled_flag equal to 0 specifies that cross-component filter for Cb colour component may be is disabled for one, or more, or all slices associated with the PH. When not present, ph_cc_alf_cb_enabled_flag is inferred to be equal to 0.

ph_cc_alf_cr_enabled_flag equal to 1 specifies that cross-compoent filter for Cr colour component is may be enabled for one, or more, or all slices associated with the PH and may be applied to Cr colour component in the  slices. ph_cc_alf_cr_enabled_flag equal to 0 specifies that cross-component filter for Cr colour component may be is disabled for one, or more, or all slices associated with the PH. When not present, ph_cc_alf_cr_enabled_flag is inferred to be equal to 0.

slice_alf_enabled_flag equal to 1 specifies that adaptive loop filter is enabled and may be applied to Y, Cb, or Cr colour component in a slice. slice_alf_enabled_flag equal to 0 specifies that adaptive loop filter is disabled for all colour components in a slice. When not present, the value of slice_alf_enabled_flag is inferred to be equal to ph_alf_enabled_flag 0.

slice_num_alf_aps_ids_luma specifies the number of ALF APSs that the slice refers to. When slice_alf_enabled_flag is equal to 1 and slice_num_alf_aps_ids_luma is not present, the value of slice_num_alf_aps_ids_luma is inferred to be equal to the value of ph_num_alf_aps_ids_luma.

slice_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slice refers to. The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_luma[ i ] is not present, the value of slice_alf_aps_id_luma[ i ] is inferred to be equal to the value of ph_alf_aps_id_luma[ i ].

The value of alf_luma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be equal to 1.

slice_alf_chroma_idc equal to 0 specifies that the adaptive loop filter is not applied to Cb and Cr colour components. slice_alf_chroma_idc equal to 1 indicates that the adaptive loop filter is applied to the Cb colour component. slice_alf_chroma_idc equal to 2 indicates that the adaptive loop filter is applied to the Cr colour component. slice_alf_chroma_idc equal to 3 indicates that the adaptive loop filter is applied to Cb and Cr colour components. When slice_alf_chroma_idc is not present, it is inferred to be equal to ph_alf_chroma_idc.

slice_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the ALF APS that the chroma component of the slice refers to. The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_chroma is not present, the value of slice_alf_aps_id_chroma is inferred to be equal to the value of ph_alf_aps_id_chroma.

The value of alf_chroma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be equal to 1.

slice_cc_alf_cb_enabled_flag equal to 0 specifies that the cross-component filter is not applied to the Cb colour component. slice_cc_alf_cb_enabled_flag equal to 1 indicates that the cross-component filter is enabled and may be applied to the Cb colour component. When slice_cc_alf_cb_enabled_flag is not present, it is inferred to be equal to ph_cc_alf_cb_enabled_flag 0.

slice_cc_alf_cb_aps_id specifies the adaptation_parameter_set_id that the Cb colour component of the slice refers to.

The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_cc_alf_cb_aps_id shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_cc_alf_cb_enabled_flag is equal to 1 and slice_cc_alf_cb_aps_id is not present, the value of slice_cc_alf_cb_aps_id is inferred to be equal to the value of ph_cc_alf_cb_aps_id.

The value of alf_cc_cb_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_cc_alf_cb_aps_id shall be equal to 1.

slice_cc_alf_cr_enabled_flag equal to 0 specifies that the cross-component filter is not applied to the Cr colour component. slice_cc_alf_cb_enabled_flag equal to 1 indicates that the cross-component adaptive loop filter is enabled and may be applied to the Cr colour component. When slice_cc_alf_cr_enabled_flag is not present, it is inferred to be equal to ph_cc_alf_cr_enabled_flag 0.

slice_cc_alf_cr_aps_id specifies the adaptation_parameter_set_id that the Cr colour component of the slice refers to. The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_cc_alf_cr_aps_id shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_cc_alf_cr_enabled_flag is equal to 1 and slice_cc_alf_cr_aps_id is not present, the value of slice_cc_alf_cr_aps_id is inferred to be equal to the value of ph_cc_alf_cr_aps_id.

The value of alf_cc_cr_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_cc_alf_cr_aps_id shall be equal to 1.

The syntax changes of SAO are shown in Table 16 to Table 18 and the semantics are given below the table.

Table 16: SAO PPS syntax changes in proposal 1

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sao_info_in_ph_flag | u(1) |

Table 17: SAO picture header syntax changes in proposal 1

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_sao_enabled_flag && sao_info_in_ph_flag ) { |  |
| ph_sao_luma_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| ph_sao_chroma_enabled_flag | u(1) |
| } |  |

Table 18: SAO slice header syntax changes in proposal 1

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_sao_enabled_flag && !sao_info_in_ph_flag) { |  |
| if( ph_sao_luma_enabled_flag) |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 && ph_sao_chroma_enabled_flag) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |

sps_sao_enabled_flag equal to 1 specifies that the sample adaptive offset process is may be applied to the reconstructed picture after the deblocking filter process. sps_sao_enabled_flag equal to 0 specifies that the sample adaptive offset process is not applied to the reconstructed picture after the deblocking filter process.

sao_info_in_ph_flag equal to 1 specifies that SAO filter information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. sao_info_in_ph_flag equal to 0 specifies that SAO filter information is not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure.

ph_sao_luma_enabled_flag equal to 1 specifies that SAO is may be enabled for the luma component in one, or more, or all slices associated with the PH; ph_sao_luma_enabled_flag equal to 0 specifies that SAO for the luma component may be is disabled for one, or more, or all slices associated with the PH. When ph_sao_luma_enabled_flag is not present, it is inferred to be equal to 0.

ph_sao_chroma_enabled_flag equal to 1 specifies that SAO is may be enabled for the chroma component in one, or more, or all slices associated with the PH; ph_sao_chroma_enabled_flag equal to 0 specifies that SAO for chroma component may be is disabled for one, or more, or all slices associated with the PH. When ph_sao_chroma_enabled_flag is not present, it is inferred to be equal to 0.

slice_sao_luma_flag equal to 1 specifies that SAO is enabled for the luma component in the current slice; slice_sao_luma_flag equal to 0 specifies that SAO is disabled for the luma component in the current slice. When slice_sao_luma_flag is not present, it is inferred to be equal to ph_sao_luma_enabled_flag 0.

slice_sao_chroma_flag equal to 1 specifies that SAO is enabled for the chroma component in the current slice; slice_sao_chroma_flag equal to 0 specifies that SAO is disabled for the chroma component in the current slice. When slice_sao_chroma_flag is not present, it is inferred to be equal to ph_sao_chroma_enabled_flag 0.

Proposal 2: LMCS and ALF semantics fix

As discussed before, for LMCS, ALF and SAO, when a higher level enable them, a lower level can disable them; when a higher level disable them, a lower level cannot enable them. According to this, the current semantics of the enabled flag of these three tools are not accurate. It is proposed to change these semantics as follows

sps_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is may be used in the CLVS. sps_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is not used in the CLVS.

ph_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is may be enabled for one, or more, or all slices associated with the PH. ph_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling may be is disabled for one, or more, or all slices associated with the PH. When not present, the value of ph_lmcs_enabled_flag is inferred to be equal to 0.

ph_chroma_residual_scale_flag equal to 1 specifies that chroma residual scaling is may be enabled for one, or more, or the all slices associated with the PH. ph_chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling may be is disabled for one, or more, or all slices associated with the PH. When ph_chroma_residual_scale_flag is not present, it is inferred to be equal to 0.

sps_alf_enabled_flag equal to 0 specifies that the adaptive loop filter is disabled. sps_alf_enabled_flag equal to 1 specifies that the adaptive loop filter is may be enabled.

sps_sao_enabled_flag equal to 1 specifies that the sample adaptive offset process is may be applied to the reconstructed picture after the deblocking filter process. sps_sao_enabled_flag equal to 0 specifies that the sample adaptive offset process is not applied to the reconstructed picture after the deblocking filter process.

Moveover, for LMCS, luma mapping and chroma scaling are separately controlled in PH but jointly controlled in SH. If luma mapping is enabled and chroma scaling is disabled in PH (ph_lmcs_enabled_flag=1 and ph_chroma_residual_scale_flag=0), SH can only keep luma mapping enabled but cannot enable chroma scaling by setting slice_lmcs_enabled_flag to 1. Thus, based on this scheme, the semantics of slice_lmcs_enabled_flag should be fixed as follows:

slice_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is enabled for the current slice and chroma scaling may be enabled for the current slice. slice_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is not enabled for the current slice. When slice_lmcs_enabled_flag is not present, it is inferred to be equal to 0.

Conclusion

Two aspects are included in this contribution. The first aspect proposes to align high level control scheme of ALF and SAO with that of LMCS. By doing this, the high level control syntax of ALF and SAO are much clearer and simpler. The second aspect is just to fix the semantics of ALF, SAO and LMCS high level related syntax element. It is suggested to adopt this contribution into next version of VVC draft.

Reference

B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001, October 2019.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).