| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0161-v1 |
| --- | --- |

| Title: | AHG8/AHG9: On VPS syntax signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Jiancong Luo, Yan Ye, Ru-Ling Liao Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution includes three aspects for VPS syntax cleanup as follows

In the first aspect, two options are provided. Option 1 proposes to change the coding method of ols_ptl_idx[ i ] from “u(8)” to “u(v)” to reduce bit overhead. Option 2 proposes to change the coding method of vps_num_ptls_minus1 and ols_ptl_idx[ i ] from “u(8)” to “ue(v)” to align with vps_num_dpb_params, ols_dpb_params_idx[ i ], num_ols_hrd_params_minus1 and ols_hrd_idx[ i ].

In the second aspect, it is proposed to skip the signaling of ols_ptl_idx[ i ] when vps_num_ptls_minus1+1  is equal to TotalNumOlss and skip the signaling of ols_dpb_params_idx[ i ] when vps_num_dpb_params  is equal to TotalNumOlss.

In the third aspect, it is proposed to change the coding method of num_output_layer_sets_minus1 from “u(8)” to “u(v)” to reduce bit overhead.

Current design

In VVC draft 8 [1], the output layer sets (OLSs), the Profile, Tier and Level (PTL) parameters, Decoded Picture Buffer (DPB) parameters and Hypothetical Reference Decoder (HRD) parameters are signaled in Video Parameter Set (VPS) within syntax structure profile_tier_level(), dpb_parameters() and ols_hrd_parameters(). The syntax is shown in Table 1.

To specify the syntax structure for each Output Layout Set (OLS), the number of syntax structures and the index of the syntax structure applied to each OLS are signaled.

Table 1 layers, PTL, DPB and HRD related sytnax in VPS

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| vps_max_layers_minus1 | u(6) |  |
| … |  |  |
| if( vps_max_layers_minus1 > 0 ) { |  |  |
| if( vps_all_independent_layers_flag ) |  |  |
| each_layer_is_an_ols_flag | u(1) |  |
| if( !each_layer_is_an_ols_flag ) { |  |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| ols_mode_idc | u(2) |  |
| if( ols_mode_idc = = 2 ) { |  |  |
| num_output_layer_sets_minus1 | u(8) |  |
| for( i = 1; i <= num_output_layer_sets_minus1; i ++) |  |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |  |
| ols_output_layer_flag[ i ][ j ] | u(1) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| vps_num_ptls_minus1 | u(8) |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) { |  |  |
| if( i > 0 ) |  |  |
| pt_present_flag[ i ] | u(1) |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| ptl_max_temporal_id[ i ] | u(3) |  |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| vps_ptl_alignment_zero_bit /* equal to 0 */ | f(1) |  |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) |  |  |
| profile_tier_level( pt_present_flag[ i ], ptl_max_temporal_id[ i ] ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| if(vps_num_ptls_minus1 > 0 ) |  |  |
| ols_ptl_idx[ i ] | u(8) |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| vps_num_dpb_params | ue(v) |
| if( vps_num_dpb_params > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_dpb_params_present_flag | u(1) |  |
| for( i = 0; i < vps_num_dpb_params; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| dpb_max_temporal_id[ i ] | u(3) |  |
| dpb_parameters( dpb_max_temporal_id[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| } |  |
| for( i = 0; i < TotalNumOlss; i++ ) { |  |  |
| if( NumLayersInOls[ i ] > 1 ) { |  |  |
| ols_dpb_pic_width[ i ] | ue(v) |  |
| ols_dpb_pic_height[ i ] | ue(v) |  |
| if( vps_num_dpb_params > 1 ) |  |  |
| ols_dpb_params_idx[ i ] | ue(v) |  |
| } |  |  |
| } |  |  |
| if( !each_layer_is_an_ols_flag ) |  |  |
| vps_general_hrd_params_present_flag | u(1) |  |
| if( vps_general_hrd_params_present_flag ) { |  |  |
| general_hrd_parameters( ) |  |  |
| if( vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_cpb_params_present_flag | u(1) |  |
| num_ols_hrd_params_minus1 | ue(v) |  |
| for( i = 0; i <= num_ols_hrd_params_minus1; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| hrd_max_tid[ i ] | u(3) |  |
| firstSubLayer = vps_sublayer_cpb_params_present_flag ? 0 : hrd_max_tid[ i ] |  |  |
| ols_hrd_parameters( firstSubLayer, hrd_max_tid[ i ] ) |  |  |
| } |  |  |
| if( num_ols_hrd_params_minus1 + 1 != TotalNumOlss && num_ols_hrd_params_minus1 > 0 ) |  |  |
| for( i = 1; i < TotalNumOlss; i++ ) |  |  |
| if( NumLayersInOls[ i ] > 1 ) |  |  |
| ols_hrd_idx[ i ] | ue(v) |  |
| } |  |  |
| … |  |  |
| } |  |  |

Proposal 1: coding method of vps_num_ptls_minus1 and ols_ptl_idx[ i ]

Option 1

As shown in Table 1, the number of PTL syntax stucture (vps_num_ptls_minus1) and the index of PTL syntax stucture (ols_ptl_idx[ i ]) are coded with “u(8)”, a fixed length coding method with bit length equal to 8. So regardless of the total number of syntax structure profile_tier_level(), ols_ptl_idx[ i ] always takes 8 bits. In the case that vps_num_ptls_minus1 is a small number, 8-bit coding of ols_ptl_idx[ i ] is not necessary.

Thus, the first option is to change coding method of ols_ptl_idx[ i ] from “u(8)” to “u(v)” to improve the signaling efficient. And the bit length of ols_ptl_idx[ i ] is dependent on vps_num_ptls_minus1.

The specification change is give below.

Table 2. syntax change of option 1 of proposal 1

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| vps_num_ptls_minus1 | u(8) |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) { |  |  |
| if( i > 0 ) |  |  |
| pt_present_flag[ i ] | u(1) |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| ptl_max_temporal_id[ i ] | u(3) |  |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| vps_ptl_alignment_zero_bit /* equal to 0 */ | f(1) |  |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) |  |  |
| profile_tier_level( pt_present_flag[ i ], ptl_max_temporal_id[ i ] ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| if(vps_num_ptls_minus1 > 0 ) |  |  |
| ols_ptl_idx[ i ] | u(8) u(v) |  |

ols_ptl_idx[ i ] specifies the index, to the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to the i-th OLS. The length of the syntax element is Ceil(Log2(vps_num_ptls_minus1+1)). When present, the value of ols_ptl_idx[ i ] shall be in the range of 0 to vps_num_ptls_minus1, inclusive. When vps_num_ptls_minus1 is equal to 0, the value of ols_ptl_idx[ i ] is inferred to be equal to 0.

Option 2

It is noticed that when signaling Decoded Picture Buffer (DPB) parameters and Hypothetical Reference Decoder (HRD) parameters in VPS, the number of dpb_parameters() (vps_num_dpb_params), the index of dpb_parameters() (ols_dpb_params_idx[ i ]), the number of ols_hrd_parameters() (num_ols_hrd_params_minus1) and the index of ols_hrd_parameters() (ols_hrd_idx[ i ]) are all coded with “ue(v)”.

So considering the consistency between syntax element coding method, in option 2, it is proposed to change the coding method of ols_ptl_idx[ i ] and vps_num_ptls_minus1 from “u(8)” to “ue(v)”.

The syntax change is given below. There is no semantics and decoding process change.

Table 3 syntax changes of option 2 of proposal 1

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| vps_num_ptls_minus1 | u(8)ue(v) |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) { |  |  |
| if( i > 0 ) |  |  |
| pt_present_flag[ i ] | u(1) |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| ptl_max_temporal_id[ i ] | u(3) |  |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| vps_ptl_alignment_zero_bit /* equal to 0 */ | f(1) |  |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) |  |  |
| profile_tier_level( pt_present_flag[ i ], ptl_max_temporal_id[ i ] ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| if(vps_num_ptls_minus1 > 0 ) |  |  |
| ols_ptl_idx[ i ] | u(8)ue(v) |  |

Proposal 2: skip signaling of ols_ptl_idx[ i ] and ols_dpb_params_idx[ i ]

As shown in Table 1, when signaling the index of HRD syntax structure, if the number of HRD syntax structure() is equal to the number of OLS (i.,e, num_ols_hrd_params_minus1 + 1  ==  TotalNumOlss), the index of HRD syntax structure, ols_ptl_idx[ i ], is not signaled but inferred to be i. It is a way to improve the signaling efficiency and thus was adopted in VVC draft 8.

However, this signaling skip mechansim is not applied on PTL and DPB syntax structure. Thus, it is proposed to skip ols_ptl_idx[ i ] and ols_dpb_params_idx[ i ] signalling when the syntax structure of profile_tier_level() and dpb_parameters() is equal to the number of OLS.

The specification change is given below

Table 4 syntax changes of proposal 2

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| vps_num_ptls_minus1 | u(8) |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) { |  |  |
| if( i > 0 ) |  |  |
| pt_present_flag[ i ] | u(1) |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| ptl_max_temporal_id[ i ] | u(3) |  |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| vps_ptl_alignment_zero_bit /* equal to 0 */ | f(1) |  |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) |  |  |
| profile_tier_level( pt_present_flag[ i ], ptl_max_temporal_id[ i ] ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| if(vps_num_ptls_minus1+1 != TotalNumOlss && vps_num_ptls_minus1 > 0 ) |  |  |
| ols_ptl_idx[ i ] | u(8) |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| vps_num_dpb_params | ue(v) |
| if( vps_num_dpb_params > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_dpb_params_present_flag | u(1) |  |
| for( i = 0; i < vps_num_dpb_params; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| dpb_max_temporal_id[ i ] | u(3) |  |
| dpb_parameters( dpb_max_temporal_id[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| } |  |
| for( i = 0; i < TotalNumOlss; i++ ) { |  |  |
| if( NumLayersInOls[ i ] > 1 ) { |  |  |
| ols_dpb_pic_width[ i ] | ue(v) |  |
| ols_dpb_pic_height[ i ] | ue(v) |  |
| if(vps_num_dpb_params != TotalNumOlss &&vps_num_dpb_params > 1 ) |  |  |
| ols_dpb_params_idx[ i ] | ue(v) |  |
| } |  |  |
| } |  |  |
| … |  |  |
| } |  |  |

ols_ptl_idx[ i ] specifies the index, to the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to the i-th OLS. When present, the value of ols_ptl_idx[ i ] shall be in the range of 0 to vps_num_ptls_minus1, inclusive. If the value of vps_num_ptls_minus1+1 is equal to TotalNumOlss, the value of ols_ptl_idx[i] is inferred to be queal to i, otherwise, when vps_num_ptls_minus1 is equal to 0, the value of ols_ptl_idx[ i ] is inferred to be equal to 0.

ols_dpb_params_idx[ i ] specifies the index, to the list of dpb_parameters( ) syntax structures in the VPS, of the dpb_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. When present, the value of ols_dpb_params_idx[ i ] shall be in the range of 0 to vps_num_dpb_params − 1, inclusive. When ols_dpb_params_idx[ i ] is not present, if the value of the vps_num_dpb_params  is equal to TotalNumOlss, the value of ols_dpb_params_idx[i] is inferred to be i, otherwise, value of ols_dpb_params_idx[ i ] is inferred to be equal to 0.

Proposal 3: coding method of num_output_layer_sets_minus1

In VVC draft 8, when ols_mode_idc is equal to 2, num_output_layer_sets_minus1 is signaled to indicate the total number of OLSs minus 1. Then each OLS is defined by signaling a flag (ols_output_layer_flag ) for each layer in CVS to indicate whether the layer is an output layer or not. The syntax element num_output_layer_sets_minus1 is fixed length coded with code length 8. However, total number of OLSs is not larger than the number of combinations of ols_output_layer_flag of all layers in the CVS, i.e.

TotalNumOlss <= 2^(vps_max_layers_minus1+1)

num_output_layer_sets_minus1 can be represented in (vps_max_layers_minus1 + 1) bits. When vps_max_layers_minus1 + 1 is smaller than 8, using 8-bit fixed length coding for num_output_layer_sets_minus1 is not necessary.

In method 3, it is proposed to use variable length coding for num_output_layer_sets_minus1, where the code length is min(8, vps_max_layers_minus1 + 1).

The syntax change is shown in the following:

Table 4 syntax changes of proposal 3

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_max_layers_minus1 | u(6) |
| … |  |
| if( vps_max_layers_minus1 > 0 ) { |  |
| if( vps_all_independent_layers_flag ) |  |
| each_layer_is_an_ols_flag | u(1) |
| if( !each_layer_is_an_ols_flag ) { |  |
| if( !vps_all_independent_layers_flag ) |  |
| ols_mode_idc | u(2) |
| if( ols_mode_idc = = 2 ) { |  |
| num_output_layer_sets_minus1 | u(8) u(v) |
| for( i = 1; i <= num_output_layer_sets_minus1; i ++) |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |
| ols_output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

num_output_layer_sets_minus1 plus 1 specifies the total number of OLSs specified by the VPS when ols_mode_idc is equal to 2. The length of num_output_layer_sets_minus1 is min( 8, vps_max_layers_minus1 + 1 ).

Conclusion

This contribution proposes three aspects to cleanup the syntax elements in VPS.

In the first aspect, the coding method of ols_ptl_idx[ i ] is changed from “u(8)” to “u(v)” to improve the signaling efficiency. Or the coding method of vps_num_ptls_minus1 and ols_ptl_idx[ i ] from “u(8)” to “ue(v)” to align with vps_num_dpb_params, ols_dpb_params_idx[ i ], num_ols_hrd_params_minus1 and ols_hrd_idx[ i ].

In the second aspect, signaling of ols_ptl_idx[ i ] or ols_dpb_params_idx[ i ] are skipped when vps_num_ptls_minus1+1 is equal to TotalNumOlss or vps_num_dpb_params is equal to TotalNumOlss to align with the signaling of ols_hrd_idx[ i ].

In the third aspect, the coding method of num_output_layer_sets_minus1 is changed from “u(8)” to “u(v)” to improve the signaling efficiency.

It is suggested to adopt this contribution into next version of VVC draft.

Reference

B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001, October 2019.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).