| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0175 |
| --- | --- |

| Title: | AhG9: An SPS Flag for IBC-AMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Michel Kerdranvat Tangi Poirier Antoine Robert | Tel: Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

AMVR can be used for regular blocks, affine block and IBC blocks. Unlike IBC blocks, VVC specification allows enabling/disabling AMVR. This contribution proposes to add an SPS flag to control IBC-AMVR to provide consistent HLS design and to offer further encoder flexibility.

# Introduction

AMVR is controlled by two SPS flag. The first one (sps_amvr_enabled_flag) is to control AMVR for regular CU (including IBC mode) and the second (sps_affine_amvr_enabled_flag) is for controlling AMVR for affine modes only. The precision of each of these three modes (regular, affine and IBC) is different. According to the specification:

Table 16 – Specification of AmvrShift

| amvr_flag | amvr_precision_idx | AmvrShift |  |  |
| --- | --- | --- | --- | --- |
|  |  | inter_affine_flag = =1 | CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) | inter_affine_flag = =0 && CuPredMode[ chType ][ x0 ][ y0 ] ! = MODE_IBC |
| 0 | - | 2 (1/4 luma sample) | - | 2 (1/4 luma sample) |
| 1 | 0 | 0 (1/16 luma sample) | 4 (1 luma sample) | 3 (1/2 luma sample) |
| 1 | 1 | 4 (1 luma sample) | 6 (4 luma samples) | 4 (1 luma sample) |
| 1 | 2 | - | - | 6 (4 luma samples) |

That is, AMVR precision index (amvr_precision_idx) can be either 0 or 1 for affine and IBC, while it can be 0, 1 or 2 for regular mode. Additionally, the semantics of the index change according to the mode. Therefore, the three AMVR are completely different and having 2 SPS flags does not provide the full flexibility. In the following proposal, a modification is made to better control AMVR.

# Proposal

Similar to sps_affine_amvr_enabled_flag, which controls affine AMVR combination, it is proposed to add another flag, called sps_ibc_amvr_enabled_flag to control IBC AMVR combination. This is done in the following way:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if(sps_ibc_enabled_flag && sps_amvr_enabled_flag) { |  |
| sps_ibc_amvr_enabled_flag | u(1) |
| } |  |
| … |  |

In the coding unit, the following is done:

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_PLT ) { |  |
| … |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER or MODE_IBC */ |  |
| … |  |
| if( general_merge_flag[ x0 ][ y0 ] ) |  |
| … |  |
| else if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) { |  |
| … | ae(v) |
| if( sps_amvr_enabled_flag sps_ibc_amvr_enabled_flag && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ) ) |  |
| amvr_precision_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| … |  |
| } |  |

In the semantics, the following is added:

7.4.3.3 Sequence parameter set RBSP semantics

...

sps_ibc_enabled_flag equal to 1 specifies that the IBC prediction mode may be used in decoding of pictures in the CLVS. sps_ibc_enabled_flag equal to 0 specifies that the IBC prediction mode is not used in the CLVS. When sps_ibc_enabled_flag is not present, it is inferred to be equal to 0.

sps_ibc_amvr_enabled_flag equal to 1 specifies that adaptive motion vector difference resolution is used in motion vector coding of intra-block copy mode. sps_ibc_amvr_enabled_flag equal to 0 specifies that adaptive motion vector difference resolution is not used in motion vector coding of intra-block copy mode. When not present, the value of sps_ibc_amvr_enabled_flag is inferred to be equal to 0

…

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).