| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0191 |
| --- | --- |

| Title: | AHG9: On miscellaneous updates for HLS signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seethal Paluri Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | dr.hendry@lge.com seethal.paluri@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics. |  |  |

____________________________

# Abstract

This contribution proposed the following changes to the HLS related signalling as follows:

- Replace the constraint that the value of sps_ptl_dpb_hrd_params_present_flag shall be equal to vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] with the following:

The value of sps_ptl_dpb_hrd_params_present_flag shall be equal to 1 when there is an OLS that contains only one layer and the layer id is equal to nuh_layer_id of the SPS.

- Constrain that for each independent layer (i.e., the value of vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1), there shall be an OLS that contains that layer only. It is remarked that this is not needed if the above proposal 1 is agreed.

- Constrain that each DPB parameter structure and HRD parameter structure that are signalled in VPS shall be associated with at least one OLS that contains more than one layer. Furthermore, update the range value for num_ols_hrd_params_minus1 and vps_num_dpb_params to be more precise.

Also, constrain that each PTL structure that is signalled in VPS shall be associated with at least one OLS.

- Constrain that when sps_virtual_boundaries_enabled_flag is equal to 1 and sps_virtual_boundaries_present_flag is equal to 0, there shall be at least one picture header in the CLVS with ph_virtual_boundaries_present_flag equal to 1.

- Specify a constraint flag on LMCS i.e., no_lmcs_constraint_flag.

# Proposal 1: On the constraint for sps_ptl_dpb_hrd_params_present_flag value

In the current VVC working draft, the value of sps_ptl_dpb_hrd_params_present_flag is constrained to be equal to the value of vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ]. This constraint is asserted to be not accurate since PTL, DPB, and HRD are not signalled on layer basis but on OLS basis and currently there is no requirement that there must be an OLS that contains each of the independent layer.

We propose that the constraint for the value of sps_ptl_dpb_hrd_params_present_flag be updated such that its value shall be equal to 1 when there is an OLS that contains only one layer and the layer Id is equal to nuh_layer_id of the SPS.

The spec text changes for this is as follows:

...

sps_ptl_dpb_hrd_params_present_flag equal to 1 specifies that a profile_tier_level( ) syntax structure and a dpb_parameters( ) syntax structure are present in the SPS, and a general_hrd_parameters( ) syntax structure and an ols_hrd_parameters( ) syntax structure may also be present in the SPS. sps_ptl_dpb_hrd_params_present_flag equal to 0 specifies that none of these four syntax structures is present in the SPS. The value of sps_ptl_dpb_hrd_params_present_flag shall be equal to vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ]. The value of sps_ptl_dpb_hrd_params_present_flag shall be equal to 1 when there is one OLS that contains only one layer and the layer id of the layer is equal to nuh_layer_id.

...

# Proposal 2: On the OLS for independent layer

If proposal 1 is not agreed, alternatively, we propose to specify a constraint such that for each independent layer (i.e., the value of vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1), there shall be an OLS that only contains the layer.

The spec text changes for this is as follows:

...

ols_mode_idc equal to 0 specifies that the total number of OLSs specified by the VPS is equal to vps_max_layers_minus1 + 1, the i-th OLS includes the layers with layer indices from 0 to i, inclusive, and for each OLS only the highest layer in the OLS is output.

ols_mode_idc equal to 1 specifies that the total number of OLSs specified by the VPS is equal to vps_max_layers_minus1 + 1, the i-th OLS includes the layers with layer indices from 0 to i, inclusive, and for each OLS all layers in the OLS are output.

ols_mode_idc equal to 2 specifies that the total number of OLSs specified by the VPS is explicitly signalled and for each OLS the output layers are explicitly signalled and other layers are the layers that are direct or indirect reference layers of the output layers of the OLS.

The value of ols_mode_idc shall be in the range of 0 to 2, inclusive. The value 3 of ols_mode_idc is reserved for future use by ITU-T | ISO/IEC.

When vps_all_independent_layers_flag is equal to 1 and each_layer_is_an_ols_flag is equal to 0, the value of ols_mode_idc is inferred to be equal to 2.

num_output_layer_sets_minus1 plus 1 specifies the total number of OLSs specified by the VPS when ols_mode_idc is equal to 2.

The variable TotalNumOlss, specifying the total number of OLSs specified by the VPS, is derived as follows:

if( vps_max_layers_minus1  = =  0 )
	TotalNumOlss = 1
else if( each_layer_is_an_ols_flag  | |  ols_mode_idc  = =  0  | |  ols_mode_idc  = =  1 )
	TotalNumOlss = vps_max_layers_minus1 + 1	(39)
else if( ols_mode_idc  = =  2 )
	TotalNumOlss = num_output_layer_sets_minus1 + 1

For each layer layerA with vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] equal to 1, there shall be an OLS that contains only layerA.

...

# Proposal 3: On unreferred PTL, HRD parameter, and DPB parameter structures

We assert that VPS should not contain information that is not used / referred to by other syntax element or process. Based on this understanding, we propose the following:

- Constrain that each DPB parameter structure and HRD parameter structure that are signalled in VPS shall be associated with at least one OLS that contains more than one layer. Furthermore, update the range value for num_ols_hrd_params_minus1 and vps_num_dpb_params to be more precise.

- Constrain that each PTL structure that is signalled in VPS shall be associated with at least one OLS.

The spec text changes for this is as follows:

...

The variable NumLayersInOls[ i ], specifying the number of layers in the i-th OLS, and the variable LayerIdInOls[ i ][ j ], specifying the nuh_layer_id value of the j-th layer in the i-th OLS, and NumMultiLayeredOlss, specifying the number of OLSs that contain more than one layer, are derived as follows:

NumLayersInOls[ 0 ] = 1
LayerIdInOls[ 0 ][ 0 ] = vps_layer_id[ 0 ]
NumMultiLayeredOlss = 0
for( i = 1; i < TotalNumOlss; i++ ) {
	if( each_layer_is_an_ols_flag ) {
		NumLayersInOls[ i ] = 1
		LayerIdInOls[ i ][ 0 ] = vps_layer_id[ i ]		(41)
	} else if( ols_mode_idc  = =  0  | |  ols_mode_idc  = =  1 ) {
		NumLayersInOls[ i ] = i + 1
		for( j = 0; j < NumLayersInOls[ i ]; j++ )
			LayerIdInOls[ i ][ j ] = vps_layer_id[ j ]
	} else if( ols_mode_idc  = =  2 ) {
		for( k = 0, j = 0; k  <=  vps_max_layers_minus1; k++ )
			if( layerIncludedInOlsFlag[ i ][ k ] )
				LayerIdInOls[ i ][ j++ ] = vps_layer_id[ k ]
		NumLayersInOls[ i ] = j
	}
	if( NumLayersInOls[ i ] > 1 )
		NumMultiLayeredOlss++
}

NOTE 1 – The 0-th OLS contains only the lowest layer (i.e., the layer with nuh_layer_id equal to vps_layer_id[ 0 ]) and for the 0-th OLS the only included layer is output.

...

ols_ptl_idx[ i ] specifies the index, to the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to the i-th OLS. When present, the value of ols_ptl_idx[ i ] shall be in the range of 0 to vps_num_ptls_minus1, inclusive. When vps_num_ptls_minus1 is equal to 0, the value of ols_ptl_idx[ i ] is inferred to be equal to 0.

When NumLayersInOls[ i ] is equal to 1, the profile_tier_level( ) syntax structure that applies to the i-th OLS is also present in the SPS referred to by the layer in the i-th OLS. It is a requirement of bitstream conformance that, when NumLayersInOls[ i ] is equal to 1, the profile_tier_level( ) syntax structures signalled in the VPS and in the SPS for the i-th OLS shall be identical.

For each profile_tier_level( ) that is present in the VPS, there shall be at least one OLS associated with it.

...

vps_num_dpb_params specifies the number of dpb_parameters( ) syntax strutcures in the VPS. The value of vps_num_dpb_params shall be in the range of 0 to NumMultiLayeredOlss16, inclusive. When not present, the value of vps_num_dpb_params is inferred to be equal to 0.

...

ols_dpb_params_idx[ i ] specifies the index, to the list of dpb_parameters( ) syntax structures in the VPS, of the dpb_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. When present, the value of ols_dpb_params_idx[ i ] shall be in the range of 0 to vps_num_dpb_params − 1, inclusive. When ols_dpb_params_idx[ i ] is not present, the value of ols_dpb_params_idx[ i ] is inferred to be equal to 0.

When NumLayersInOls[ i ] is equal to 1, the dpb_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

For each dpb_parameters( ) that is present in the VPS, there shall be at least one OLS associated with it.

...

num_ols_hrd_params_minus1 plus 1 specifies the number of ols_hrd_parameters( ) syntax structures present in the VPS when vps_general_hrd_params_present_flag is equal to 1. The value of num_ols_hrd_params_minus1 shall be in the range of 0 to NumMultiLayeredOlssTotalNumOlss – 1, inclusive.

...

ols_hrd_idx[ i ] specifies the index, to the list of ols_hrd_parameters( ) syntax structures in the VPS, of the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. The value of ols_hrd_idx[[ i ] shall be in the range of 0 to num_ols_hrd_params_minus1, inclusive.

When NumLayersInOls[ i ] is equal to 1, the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

If the value of num_ols_hrd_param_minus1 + 1 is equal to TotalNumOlss, the value of ols_hrd_idx[ i ] is inferred to be equal to i. Otherwise, when NumLayersInOls[ i ] is greater than 1 and num_ols_hrd_params_minus1 is equal to 0, the value of ols_hrd_idx[[ i ] is inferred to be equal to 0.

For each ols_hrd_parameters( ) that is present in the VPS, there shall be at least one OLS associated with it.

...

# Proposal 4: Constraint for virtual boundary signalling in picture header

Currently, the feature to disable loop filter across virtual boundaries that are used in the 360 and GDR applications are controlled using an sps_virtual_boundaries_enabled_flag in the SPS. In the existing design, it is possible that the sps_virtual_boundaries_enabled_flag is enabled but none of flags that specifying the location of the virtual boundary position (i.e., sps_virtual_boundaries_present_flag and ph_virtual_boundaries_present_flag) is true. It is asserted that it is redundant to allow the first flag to be equal to 1 without actually having any picture in the coded layer video sequence to contain any virtual boundary.

We propose to specify a constraint such that when sps_virtual_boundaries_enabled_flag is equal to 1 and sps_virtual_boundaries_present_flag is equal to 0, there shall be a least one picture with ph_virtual_boundaries_present_flag equal to 1.

The spec text changes for this is as follows:

...

ph_virtual_boundaries_present_flag equal to 1 specifies that information of virtual boundaries is signalled in the PH. ph_virtual_boundaries_present_flag equal to 0 specifies that information of virtual boundaries is not signalled in the PH. When there is one or more than one virtual boundaries signalled in the PH, the in-loop filtering operations are disabled across the virtual boundaries in the picture. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of ph_virtual_boundaries_present_flag is inferred to be equal to 0.

It is a requirement of bitstream conformance that, when subpic_info_present_flag is equal to 1, the value of ph_virtual_boundaries_present_flag shall be equal to 0.

It is a requirement of bitstream conformance that, when sps_virtual_boundaries_enabled_flag is equal to 1 and sps_virtual_boundaries_present_flag is equal to 0, there shall be at least one picture referring to the SPS with the value of ph_virtual_boundaries_present_flag is equal to 1.

...

# Proposal 5: On signalling of contraint flag for LMCS

It is asserted that among tools whose parameters that may be signalled in APS (i.e., ALF, Scaling list, and LMCS), only LMCS tool does not have constraint flag signalled in general_constraint_info( ). We propose that such flag is specified.

The spec text changes for this is as follows:

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| … |  |
| no_lmcs_constraint_flag | u(1) |
| … |  |
| } |  |

no_lmcs_constraint_flag equal to 1 specifies that sps_lmcs_enabled_flag shall be equal to 0. no_lmcs_constraint_flag equal to 0 does not impose such a constraint.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).