| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0193-r1 |
| --- | --- |

| Title: | AHG8/AHG9: On signalling of syntax element max_tid_il_ref_pics_plus1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry1 Seethal Paluri1 Seung Hwan Kim1 Yago Sanchez2 Robert Skupin2 | Email | dr.hendry@lge.com seethal.paluri@lge.com seunghwan3.kim@lge.com {firstname.lastname} @hhi.fraunhofer.de |
| Source: | LG Electronics.1 and Fraunhofer HHI2 |  |  |

_____________________________

# Abstract

It is asserted that the signalling of max_tid_il_ref_pics_plus1[ i ] does not provide optimal mechanism to support the intended feature for assisting removal of temporal sublayer in non-output layers in an OLS during bitstream extraction process. This is because the signalling only specifies on value of max temporal Id for that a layer uses for inter-layer prediction while the layer may actually have multiple reference layers.

This contribution proposed to signal max_tid_il_ref_pics_plus1 value for each direct reference layer of a layer.

In the first revision of this proposal, the following changes were made:

- The derivation for NumSubLayersInLayerInOLS[ i ][ j ] was updated. It is remarked that this change covers the bug fix reported in the JVET-R0296 aspect 2 (i.e., derivation of NumSubLayersInLayerInOLS[][] when ols_mode_idc is equal to 0 and also the editorial bug fix suggested by JVET-R0119 aspect 3.

- The referencing to the value of NumSubLayersInLayerInOLS[ ][ ] in bitstream extraction process was updated to fix the current existing bug.

# Introduction

The current signalling of related to syntax element max_tid_il_ref_pics_plus1[ i ] is asserted to be not optimal for supporting the intended functionality. The syntax element is designed to allow derivation of variable NumSubLayersInLayerInOLS[ i ][ j ] that specify the number of sublayers in the j-th layer in the i-th OLS. This variable can be used in bitstream extraction process to remove pictures in sublayer(s) within a layer that is not an output layer in the extracted output layer set. However, it is asserted that such mechanism is not optimal when a layer uses more than one reference layer for inter-layer prediction and the number of sublayers used from each of the reference layers are not the same.

Consider the following example. Assume that an OLS contains 3 layers (i.e., layer 0, layer 1, and layer 2). Layer 2 has direct dependency to layer 0 and layer 1. For inter-layer prediction, layer 2 uses the first 2 sublayers from layer 0 and the first 3 sublayers from layer 1. In this situation, according to the current signalling mechanism, the value of max_tid_il_ref_pics_plus1[ i ] associated with layer 2 is 3 (i.e., layer 2 uses sublayers 0, 1, and 3 from its reference layers for inter-layer prediction). Because of that, when the OLS is extracted from the bitstream, we cannot remove pictures with Tid equal to 2 in layer 0 even though none of those pictures is needed in the extracted OLS.

# Proposal

The following changes are proposed to address the above issues:

- For signalling of the number of maximum sublayers used for inter-layer prediction for a layer, instead of signalling one value for all reference layers, for each reference layer of the layer, the number of maximum sublayer used from that reference layer is signalled.

- The number of maximum sublayers used from a reference layer j for layer i, is present only when layer j is a direct reference layer of layer i.

# Proposed spec text changes

Texts that are newly added are highlighted and texts that are removed are marked as stricken-red-font text.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_max_layers_minus1 | u(6) |
| vps_max_sublayers_minus1 | u(3) |
| if( vps_max_layers_minus1 > 0 && vps_max_sublayers_minus1 > 0 ) |  |
| vps_all_layers_same_num_sublayers_flag | u(1) |
| if( vps_max_layers_minus1 > 0 ) |  |
| vps_all_independent_layers_flag | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_layer_id[ i ] | u(6) |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |
| vps_independent_layer_flag[ i ] | u(1) |
| if( !vps_independent_layer_flag[ i ] ) { |  |
| max_tid_ref_present_flag[ i ] | u(1) |
| for( j = 0; j < i; j++ ) { |  |
| vps_direct_ref_layer_flag[ i ][ j ] | u(1) |
| max_tid_ref_present_flag[ i ] | u(1) |
| if( max_tid_ref_present_flag[ i ] && vps_direct_ref_layer_flag[ i ][ j ] ) |  |
| max_tid_il_ref_pics_plus1[ i ][ j ] | u(3) |
| } |  |
| } |  |
| } |  |
| } |  |

...

vps_independent_layer_flag[ i ] equal to 1 specifies that the layer with index i does not use inter-layer prediction. vps_independent_layer_flag[ i ] equal to 0 specifies that the layer with index i may use inter-layer prediction and the syntax elements vps_direct_ref_layer_flag[ i ][ j ] for j in the range of 0 to i − 1, inclusive, are present in VPS. When not present, the value of vps_independent_layer_flag[ i ] is inferred to be equal to 1.

...

max_tid_ref_present_flag[ i ] equal to 1 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ][ j ] is present. max_tid_ref_present_flag[ i ] equal to 0 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ][ j ] is not present.

vps_direct_ref_layer_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. vps_direct_ref_layer_flag [ i ][ j ] equal to 1 specifies that the layer with index j is a direct reference layer for the layer with index i. When vps_direct_ref_layer_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, inclusive, it is inferred to be equal to 0. When vps_independent_layer_flag[ i ] is equal to 0, there shall be at least one value of j in the range of 0 to i − 1, inclusive, such that the value of vps_direct_ref_layer_flag[ i ][ j ] is equal to 1.

The variables NumDirectRefLayers[ i ], DirectRefLayerIdx[ i ][ d ], NumRefLayers[ i ], RefLayerIdx[ i ][ r ], and LayerUsedAsRefLayerFlag[ j ] are derived as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	for( j = 0; j  <=  vps_max_layers_minus1; j++ ) {
		dependencyFlag[ i ][ j ] = vps_direct_ref_layer_flag[ i ][ j ]
		for( k = 0; k < i; k++ )
			if( vps_direct_ref_layer_flag[ i ][ k ]  &&  dependencyFlag[ k ][ j ] )
				dependencyFlag[ i ][ j ] = 1
	}
	LayerUsedAsRefLayerFlag[ i ] = 0
}
for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	for( j = 0, d = 0, r = 0; j  <=  vps_max_layers_minus1; j++ ) {	(37)
		if( vps_direct_ref_layer_flag[ i ][ j ] ) {
			DirectRefLayerIdx[ i ][ d++ ] = j
			LayerUsedAsRefLayerFlag[ j ] = 1
		}
		if( dependencyFlag[ i ][ j ] )
			RefLayerIdx[ i ][ r++ ] = j
	}
	NumDirectRefLayers[ i ] = d
	NumRefLayers[ i ] = r
}

The variable GeneralLayerIdx[ i ], specifying the layer index of the layer with nuh_layer_id equal to vps_layer_id[ i ], is derived as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ )		(38)
	GeneralLayerIdx[ vps_layer_id[ i ] ] = i

For any two different values of i and j, both in the range of 0 to vps_max_layers_minus1, inclusive, when dependencyFlag[ i ][ j ] equal to 1, it is a requirement of bitstream conformance that the values of chroma_format_idc and bit_depth_minus8 that apply to the i-th layer shall be equal to the values of chroma_format_idc and bit_depth_minus8, respectively, that apply to the j-th layer.

max_tid_ref_present_flag[ i ] equal to 1 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ] is present. max_tid_ref_present_flag[ i ] equal to 0 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ] is not present.

max_tid_il_ref_pics_plus1[ i ][ j ] equal to 0 specifies that the j-th layer is not used as reference layer for inter-layer prediction by non-IRAP pictures of the i-th layer. max_tid_il_ref_pics_plus1[ i ][ j ] greater than 0 specifies that, for decoding pictures of the i-th layer, no picture from the j-th layer with TemporalId greater than max_tid_il_ref_pics_plus1[ i ][ j ] − 1 is used as ILRP. When not present, the value of max_tid_il_ref_pics_plus1[ i ][ j ] is inferred to be equal to 7.

…

ols_output_layer_flag[ i ][ j ] equal to 1 specifies that the layer with nuh_layer_id equal to vps_layer_id[ j ] is an output layer of the i-th OLS when ols_mode_idc is equal to 2. ols_output_layer_flag[ i ][ j ] equal to 0 specifies that the layer with nuh_layer_id equal to vps_layer_id[ j ] is not an output layer of the i-th OLS when ols_mode_idc is equal to 2.

The variable NumOutputLayersInOls[ i ], specifying the number of output layers in the i-th OLS, the variable NumSubLayersInLayerInOLS[ i ][ j ], specifying the number of sublayers in the j-th layer in the i-th OLS, the variable OutputLayerIdInOls[ i ][ j ], specifying the nuh_layer_id value of the j-th output layer in the i-th OLS, and the variable LayerUsedAsOutputLayerFlag[ k ], specifying whether the k-th layer is used as an output layer in at least one OLS, are derived as follows:

NumOutputLayersInOls[ 0 ] = 1
OutputLayerIdInOls[ 0 ][ 0 ] = vps_layer_id[ 0 ]
NumSubLayersInLayerInOLS[ 0 ][ 0 ] = vps_max_sub_layers_minus1 + 1
LayerUsedAsOutputLayerFlag[ 0 ] = 1
for( i = 1, i  <=  vps_max_layers_minus1; i++ ) {
	if( each_layer_is_an_ols_flag  | |  ols_mode_idc < 2 )
		LayerUsedAsOutputLayerFlag[ i ] = 1
	else /*( !each_layer_is_an_ols_flag  &&  ols_mode_idc  = =  2 ) */
		LayerUsedAsOutputLayerFlag[ i ] = 0
}
for( i = 1; i < TotalNumOlss; i++ ) {
	if( each_layer_is_an_ols_flag  | |  ols_mode_idc  = =  0 ) {
		NumOutputLayersInOls[ i ] = 1
		OutputLayerIdInOls[ i ][ 0 ] = vps_layer_id[ i ]
		for( j = 0; j < i  &&  ( ols_mode_idc  = =  0 ); j++ )
			NumSubLayersInLayerInOLS[ i ][ j ] = max_tid_il_ref_pics_plus1[ i ][ j ]
		NumSubLayersInLayerInOLS[ i ][ i ] = vps_max_sub_layers_minus1 + 1
		if( each_layer_is_an_ols_flag )
			NumSubLayersInLayerInOLS[ i ][ 0 ] = vps_max_sub_layers_minus1 + 1
		else {
			NumSubLayersInLayerInOLS[ i ][ i ] = vps_max_sub_layers_minus1 + 1
			for( k = i − 1, k  >=  0; k− − ) {
				NumSubLayersInLayerInOLS[ i ][ k ] = 0
				for( l = k + 1; l  <=  i; l++ ) {
					maxSublayerNeeded = min( NumSubLayersInLayerInOLS[ i ][ l ],
								max_tid_il_ref_pics_plus1[ l ][ k ] )
					if( vps_direct_ref_layer_flag[ l ][ k ]  &&
					NumSubLayersInLayerInOLS[ i ][ k ] < maxSublayerNeeded)
						NumSubLayersInLayerInOLS[ i ][ k ] = maxSublayerNeeded
				}
			}
		}
	} else if( ols_mode_idc  = =  1 ) {
		NumOutputLayersInOls[ i ] = i + 1
		for( j = 0; j < NumOutputLayersInOls[ i ]; j++ ) {
			OutputLayerIdInOls[ i ][ j ] = vps_layer_id[ j ]
			NumSubLayersInLayerInOLS[ i ][ j ] = vps_max_sub_layers_minus1 + 1
		}
	} else if( ols_mode_idc  = =  2 ) {
		for( j = 0; j  <=  vps_max_layers_minus1; j++ ) {
			layerIncludedInOlsFlag[ i ][ j ] = 0
			NumSubLayersInLayerInOLS[ i ][ j ] = 0
		}
		highestIncludedLayer = 0
		numLayerInOls = 0
		for( k = 0, j = 0; k  <=  vps_max_layers_minus1; k++ )	(40)
			if( ols_output_layer_flag[ i ][ k ] ) {
				layerIncludedInOlsFlag[ i ][ k ] = 1
				highestIncludedLayer = k
				numLayerInOls++
				LayerUsedAsOutputLayerFlag[ k ] = 1
				OutputLayerIdx[ i ][ j ] = k
				OutputLayerIdInOls[ i ][ j++ ] = vps_layer_id[ k ]
				NumSubLayersInLayerInOLS[ i ][ k ] = vps_max_sub_layers_minus1 + 1
			}

		NumOutputLayersInOls[ i ] = j
		for( j = 0; j < NumOutputLayersInOls[ i ]; j++ ) {
			idx = OutputLayerIdx[ i ][ j ]
			for( k = 0; k < NumRefLayers[ idx ]; k++ ) {
				if (!layerIncludedInOlsFlag[ i ][ RefLayerIdx[ idx ][ k ] ] )
					numLayerInOls++
				layerIncludedInOlsFlag[ i ][ RefLayerIdx[ idx ][ k ] ] = 1

				
					
				if( NumSubLayersInLayerInOLS[ i ][ RefLayerIdx[ idx ][ k ] ] <
							max_tid_il_ref_pics_plus1[ OutputLayerIdInOls[ i ][ j ] ] )
					NumSubLayersInLayerInOLS[ i ][ RefLayerIdx[ idx ][ k ] ] = 
							max_tid_il_ref_pics_plus1[ OutputLayerIdInOls[ i ][ j ] ]
			}
		}
		for( k = highestIncludedLayer − 1; k  >=  0; k− − )
			if( layerIncludedInOlsFlag[ i ][ k ] ) {
				if( !ols_output_layer_flag[ i ][ k ] )
					for( l = k + 1; l  <=  highestIncludedLayer; l++ ) {
						maxSublayerNeeded = min( NumSubLayersInLayerInOLS[ i ][ l ],
									max_tid_il_ref_pics_plus1[ l ][ k ] )
						if( vps_direct_ref_layer_flag[ l ][ k ]  &&  layerIncludedInOlsFlag[ i ][ l ]  &&
							NumSubLayersInLayerInOLS[ i ][ k ] < maxSublayerNeeded )
								NumSubLayersInLayerInOLS[ i ][ k ] = maxSublayerNeeded
					}
	}
}

...

Sub-bitstream extraction process

...

The output sub-bitstream OutBitstream is derived as follows:

–	The bitstream outBitstream is set to be identical to the bitstream inBitstream.

–	Remove from outBitstream all NAL units with TemporalId greater than tIdTarget.

–	Remove from outBitstream all NAL units with nal_unit_type not equal to any of VPS_NUT, DCI_NUT, and EOB_NUT and with nuh_layer_id not included in the list LayerIdInOls[ targetOlsIdx ].

–	Remove from outBitstream all NAL units for which all of the following conditions are true:

–	nal_unit_type is not equal to IDR_W_RADL, IDR_N_LP, or CRA_NUT.

–	nuh_layer_id is equal to LayerIdInOls[ targetOlsIdx ][ j ] for a value of j in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1 inclusive.

–	TemporalId is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ j GeneralLayerIdx[ nuh_layer_id ] ].

–	Remove from outBitstream all SEI NAL units that contain a scalable nesting SEI message that has nesting_ols_flag equal to 1 and there is no value of i in the range of 0 to nesting_num_olss_minus1, inclusive, such that NestingOlsIdx[ i ] is equal to targetOlsIdx.

# Patent rights declaration(s)

LG Electronics and Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).