| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0194 |
| --- | --- |

| Title: | AHG8/AHG9: On parameter set sharing in multi-layered bitstream |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution asserted two problems related to parameter sharing in multi-layered bitstream as follows:

- Currently it is allowed for a slice to refer to a parameter set even when the parameter set is in the layer that is not the direct / indirect reference layer of the layer where the slice belongs to, as long as there is at least one OLS that includes both layers. This would allow a slice in layerA refer to a parameter set in layerB even when layerA and layerB are not included in the current OLS being decoded as long as there is an OLS defined in VPS that contains both layerA and layerB.

- When reference picture resampling (RPR) is not allowed, currently it is constrained that the picture size that is signalled in PPS shall be the same as the maximum picture size in the SPS. This constraint would prevent SPS sharing in spatial scalability bitstreams.

To resolve the above asserted problems, the following is proposed:

- Update the constraint about parameter set sharing such that a slice in layerA can refer to a parameter set in layerB only when either layerA is equal to layerB or layerB is a direct or indirect reference layer of layerA.

- Update the current constraint regarding picture size in PPS and maximum picture size in SPS as follows:

  - When RPR is not allowed, the value of pic_width_in_luma_samples and pic_width_in_luma_samples in all PPS in the CLVS shall have the same values, respectively.

  - When RPR is not allowed and the sps_video_parameter_set_id is equal to 0, the value of pic_width_in_luma_samples and pic_height_in_luma_samples in all PPS in the CLVS shall be the same as pic_width_max_in_luma_samples and pic_height_max_in_luma_samples, respectively.

# Problem description

The following to issues are asserted preventing parameter set sharing in multi-layered bitstream for some situations. The first issue is regarding the constraint on the layer Id of the parameter set and the layer Id of the slice that refers to the parameter set. According to the current VVC working draft, it is allowed for a slice to refer to a parameter set even when the parameter set is in the layer that is not the direct / indirect reference layer of the layer where the slice belongs to, as long as there is at least one OLS that includes both layers. This would allow a slice in layerA refer to a parameter set in layerB even when layerA and layerB are not included in the current OLS being decoded as long as there is an OLS defined in VPS that contains both layerA and layerB. This is asserted to be undesirable.

The second asserted issue is regarding the picture size signalled in PPS and the maximum picture size signalled in SPS. According to the current VVC working draft, when reference picture resampling (RPR) is not allowed, it is constrained that the picture size signalled in PPS shall be the same as the maximum picture size in the SPS. It is asserted that this constraint would prevent SPS sharing in spatial scalability bitstreams.

# Proposal

To resolve the above asserted problems, we propose the following:

- Update the constraint about parameter set sharing such that a slice in layerA can refer to a parameter set in layerB only when either layerA is equal to layerB or layerB is a direct or indirect reference layer of layerA.

- Update the current constraint regarding picture size in PPS and maximum picture size in SPS as follows:

  - When RPR is not allowed, the value of pic_width_in_luma_samples and pic_width_in_luma_samples in all PPS in the CLVS shall have the same values, respectively.

  - When RPR is not allowed, the values of nuh_layer_id of PPS and its referenced SPS are the same, and the sps_video_parameter_set_id is equal to 0, the value of pic_width_in_luma_samples and pic_height_in_luma_samples in all PPS in the CLVS shall be the same as pic_width_max_in_luma_samples and pic_height_max_in_luma_samples, respectively.

# Proposed spec text changes

In subclause 7.4.3.2 (Video parameter set RBSP semantics)

…

The variables NumDirectRefLayers[ i ], DirectRefLayerIdx[ i ][ d ], NumRefLayers[ i ], RefLayerIdx[ i ][ r ], DependencyFlag[ i ][ j ], and LayerUsedAsRefLayerFlag[ j ] are derived as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	for( j = 0; j  <=  vps_max_layers_minus1; j++ ) {
		DependencyFlag[ i ][ j ] = vps_direct_ref_layer_flag[ i ][ j ]
		for( k = 0; k < i; k++ )
			if( vps_direct_ref_layer_flag[ i ][ k ]  &&  DependencyFlag[ k ][ j ] )
				DependencyFlag[ i ][ j ] = 1
	}
	LayerUsedAsRefLayerFlag[ i ] = 0
}
for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	for( j = 0, d = 0, r = 0; j  <=  vps_max_layers_minus1; j++ ) {	(37)
		if( vps_direct_ref_layer_flag[ i ][ j ] ) {
			DirectRefLayerIdx[ i ][ d++ ] = j
			LayerUsedAsRefLayerFlag[ j ] = 1
		}
		if( DependencyFlag[ i ][ j ] )
			RefLayerIdx[ i ][ r++ ] = j
	}
	NumDirectRefLayers[ i ] = d
	NumRefLayers[ i ] = r
}

…

In subclause 7.4.3.3 (Sequence parameter set RBSP semantics)

...

sps_seq_parameter_set_id provides an identifier for the SPS for reference by other syntax elements.

SPS NAL units, regardless of the nuh_layer_id values, share the same value space of sps_seq_parameter_set_id.

Let spsLayerId be the value of the nuh_layer_id of a particular SPS NAL unit, and vclLayerId be the value of the nuh_layer_id of a particular VCL NAL unit. The particular VCL NAL unit shall not refer to the particular SPS NAL unit unless spsLayerId is less than or equal to vclLayerId and the layer with nuh_layer_id equal to spsLayerId is included in at least one OLS that includes the layer with nuh_layer_id equal to vclLayerId.

Let spsLayerId be the value of the nuh_layer_id of a particular SPS NAL unit, and vclLayerId be the value of the nuh_layer_id of a particular VCL NAL unit. The particular VCL NAL unit shall not refer to the particular SPS NAL unit unless one of the following condition is satisfied:

- spsLayerId is equal to vclLayerId.

- DependencyFlag[ vclLayerId ][ spsLayerId ] is equal to 1.

…

In subclause 7.4.3.4 (Picture parameter set RBSP semantics)

...

pps_pic_parameter_set_id identifies the PPS for reference by other syntax elements. The value of pps_pic_parameter_set_id shall be in the range of 0 to 63, inclusive.

PPS NAL units, regardless of the nuh_layer_id values, share the same value space of pps_pic_parameter_set_id.

Let ppsLayerId be the value of the nuh_layer_id of a particular PPS NAL unit, and vclLayerId be the value of the nuh_layer_id of a particular VCL NAL unit. The particular VCL NAL unit shall not refer to the particular PPS NAL unit unless ppsLayerId is less than or equal to vclLayerId and the layer with nuh_layer_id equal to ppsLayerId is included in at least one OLS that includes the layer with nuh_layer_id equal to vclLayerId.

Let ppsLayerId be the value of the nuh_layer_id of a particular PPS NAL unit, and vclLayerId be the value of the nuh_layer_id of a particular VCL NAL unit. The particular VCL NAL unit shall not refer to the particular PPS NAL unit unless one of the following condition is satisfied:

- ppsLayerId is equal to vclLayerId.

- DependencyFlag[ vclLayerId ][ ppsLayerId ] is equal to 1.

pps_seq_parameter_set_id specifies the value of sps_seq_parameter_set_id for the SPS. The value of pps_seq_parameter_set_id shall be in the range of 0 to 15, inclusive. The value of pps_seq_parameter_set_id shall be the same in all PPSs that are referred to by coded pictures in a CLVS.

...

pic_width_in_luma_samples specifies the width of each decoded picture referring to the PPS in units of luma samples. pic_width_in_luma_samples shall not be equal to 0, shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_width_max_in_luma_samples.

When res_change_in_clvs_allowed_flag equal to 0, the value of pic_width_in_luma_samples shall be equal to pic_width_max_in_luma_samples.

When res_change_in_clvs_allowed_flag equal to 0, the value of pic_width_in_luma_samples shall be the same for all PPSs that are referred to by coded pictures within a CLVS.

When res_change_in_clvs_allowed_flag equal to 0 and sps_video_parameter_set_id is equal to 0, the value of pic_width_in_luma_samples shall be equal to pic_width_max_in_luma_samples.

When sps_ref_wraparound_enabled_flag is equal to 1, the value of ( CtbSizeY / MinCbSizeY + 1 ) shall be less than or equal to the value of ( pic_width_in_luma_samples / MinCbSizeY − 1 ).

pic_height_in_luma_samples specifies the height of each decoded picture referring to the PPS in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_height_max_in_luma_samples.

When res_change_in_clvs_allowed_flag equal to 0, the value of pic_height_in_luma_samples shall be equal to pic_height_max_in_luma_samples.

When res_change_in_clvs_allowed_flag equal to 0, the value of pic_height_in_luma_samples shall be the same for all PPSs that are referred to by coded pictures within a CLVS.

When res_change_in_clvs_allowed_flag equal to 0 and sps_video_parameter_set_id is equal to 0, the value of pic_height_in_luma_samples shall be equal to pic_height_max_in_luma_samples.

...

In subclause 7.4.3.5 (Adaptation parameter set RBSP semantics)

...

adaptation_parameter_set_id provides an identifier for the APS for reference by other syntax elements.

When aps_params_type is equal to ALF_APS or SCALING_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 7, inclusive.

When aps_params_type is equal to LMCS_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

Let apsLayerId be the value of the nuh_layer_id of a particular APS NAL unit, and vclLayerId be the value of the nuh_layer_id of a particular VCL NAL unit. The particular VCL NAL unit shall not refer to the particular APS NAL unit unless apsLayerId is less than or equal to vclLayerId and the layer with nuh_layer_id equal to apsLayerId is included in at least one OLS that includes the layer with nuh_layer_id equal to vclLayerId.

Let apsLayerId be the value of the nuh_layer_id of a particular APS NAL unit, and vclLayerId be the value of the nuh_layer_id of a particular VCL NAL unit. The particular VCL NAL unit shall not refer to the particular APS NAL unit unless one of the following condition is satisfied:

- apsLayerId is equal to vclLayerId.

- DependencyFlag[ vclLayerId ][ apsLayerId ] is equal to 1.

...

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).