| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0223 |
| --- | --- |

| Title: | AHG16: On DMVR and wraparound motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiancong Luo, Yan Ye 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | jiancong.luo@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

In VVC draft 8, when wrap around motion compensation is enabled, the bilinear interpolation in DMVR motion search process also uses wrap-around clipping operation. In this contribution, it is proposed to apply regular clipping operation during DMVR motion search, and apply wrap-around clipping operation only for regular interpolation during the final motion compensation process. It is asserted that this simplifies the motion search process in DMVR. Experiment results reportedly show that the BD rate difference is -0.01%, -0.01%, -0.02% for end-to-end WS-PSNR for Y, U and V respectively. Informal subjective viewing was conducted and no visible difference was observed.

# Introduction

In VVC draft 8 [1], wrap around motion compensation tool may be enabled to improve coding performance and visual quality of reconstructed 360-degree video in the equi-rectangular (ERP) projection format or padded equi-rectangular (PERP) projection format [2][3]. This can be achieved by setting a Picture Parameter Set (PPS) flag called pps_ref_wraparound_enabled_flag to 1. When coding the current coding unit, if all the DMVR conditions are satisfied, DMVR motion search is applied to the current block at the encoder and decoder to search for the refined motion vector. The intermediate prediction samples in DMVR motion search are generated with bilinear interpolation. In VVC draft 8, combination of wrap around motion compensation and DMVR motion search is allowed, which means that if wrap around is enabled, during DMVR motion search, a more complicated clipping process involving the wrap around offset PpsRefWraparoundOffset in [6] is applied. After obtaining the refined MV, final prediction samples are generated with the regular 8-tap interpolation for luma (and 4-tap for chroma). If wrap-around is enabled, the final motion compensation process applies the wrap-around offset in order to obtain the coding efficiency and subjective quality benefit.

It is asserted that applying the more complicated clipping operation during the DMVR motion search process can increase the decoder complexity with limited benefit, because the DMVR motion search is only applied as an intermediate process in order to obtain the refined motion vectors.

# Proposed method

To simplify the operation of DMVR when it is combined with wraparound motion compensation, it is proposed to not to perform wrap-around clipping operation for the bilinear interpolation in DMVR motion search process, and only apply wrap-around clipping operation during the final motion compensation process when regular 8-tap motion interpolation filter is used for luma (and 4-tap motion interpolation is used for chroma). The DMVR process is illustrated in the block diagram in Figure 1. The grey blocks are proposed to removed.

In Figure 1, the horizontal wraparound clipping (ClipH) and regular clipping (Clip3) are define as follows, where o is the wraparound offset derived based on PpsRefWraparoundOffset:

Clip3( x, y, z ) =  x;z < xy;z > yz;otherwise

ClipH( o, W, x ) =   x  + o;x  < 0x -o;x  > W-1x;otherwise

Figure 1. DMVR process block diagram

# Proposed text

It is noted that the proposed change also simplifies the specification text, as shown in the followings:

(the changes are highlighted in yellow).

      - 8.5.3.2 Fractional sample bilinear interpolation process

        - 8.5.3.2.1 General

Inputs to this process are:

- a luma location ( xSb, ySb ) specifying the top-left sample of the current subblock relative to the top-left luma sample of the current picture,

- a variable pbWidth specifying the width of the current prediction block in luma samples,

- a variable pbHeight specifying the height of the current prediction block in luma samples,

- a luma motion vector mvLX given in 1/16-luma-sample units,

- the selected reference picture sample array refPicLXL,

- a variable refPicIsScaled indicating whether the selected reference picture requires scaling,

- the refinement search range srRange.

Output of this process is:

- a ( pbWidth ) x ( pbHeight ) array predSamplesLXL of luma prediction sample values.

Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in 1/16-sample units. These variables are used only in this clause for specifying fractional-sample locations inside the reference sample array refPicLXL.

For each luma sample location ( xL = 0..pbWidth − 1, yL = 0..pbHeight − 1 ) inside the luma prediction sample array predSamplesLXL, the corresponding luma prediction sample value predSamplesLXL[ xL ][ yL ] is derived as follows:

- The variables xIntL, yIntL, xFracL and yFracL are derived as follows:

xIntL = xSb + ( mvLX[ 0 ]  >>  4 ) + xL − srRange		(629)

yIntL = ySb + ( mvLX[ 1 ]  >>  4 ) + yL − srRange		(630)

xFracL = mvLX[ 0 ] & 15		(631)

yFracL = mvLX[ 1 ] & 15		(632)

- The variable refWraparoundEnabledFlag is set equal to ( pps_ref_wraparound_enabled_flag  &&  !refPicIsScaled ).

- The luma prediction sample value predSamplesLXL[ xL ][ yL ] is derived by invoking the luma sample bilinear interpolation process specified in clause 8.5.3.2.2 with ( xIntL, yIntL ), ( xFracL, yFracL ), refPicLXL and refWraparoundEnabledFlag as inputs.

        - 8.5.3.2.2 Luma sample bilinear interpolation process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL,

- a variable refWraparoundEnabledFlag indicating whether horizontal wrap-around motion compensation is enabled.

Output of this process is a predicted luma sample value predSampleLXL.

The variables shift1, shift2, shift3, shift4, offset1, offset2 and offset3 are derived as follows:

shift1 = BitDepth − 6		(633)

offset1 = 1 << ( shift1 − 1 )		(634)

shift2 = 4					(635)

offset2 = 1 << ( shift2 − 1 )		(636)

shift3 = 10 − BitDepth		(637)

shift4 = BitDepth − 10		(638)

offset4 = 1 << ( shift4 − 1 )		(639)

The variable picW is set equal to pic_width_in_luma_samples of the reference picture refPicLX and the variable picH is set equal to pic_height_in_luma_samples of the reference picture refPicLX.

The luma interpolation filter coefficients fbL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 26.

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

–	If subpic_treated_as_pic_flag[ CurrSubpicIdx ] is equal to 1, the following applies:

xInti = Clip3( SubpicLeftBoundaryPos, SubpicRightBoundaryPos, xIntL + i )	(640)

yInti = Clip3( SubpicTopBoundaryPos, SubpicBotBoundaryPos, yIntL + i )	(641)

–	Otherwise (subpic_treated_as_pic_flag[ CurrSubpicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, refWraparoundEnabledFlag ?
		ClipH( ( PpsRefWraparoundOffset ) * MinCbSizeY, picW, ( xIntL + i ) ) : xIntL + i )	(642)

yInti = Clip3( 0, picH − 1, yIntL + i )		(643)

# Results

The results have been generated using 360Lib-10.1 and common test condition for 360° video [4]. The anchor is VTM 8.0. A summary of the test results for random access configuration are shown in the followings:

Table 1. BD rate comparison between proposed method and VTM 8.0

|  | End-to-end WS-PSNR |  |  | Codec WS-PSNR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | EncT | DecT |  |
| Class S1 | -0.01% | -0.02% | -0.04% | -0.01% | -0.02% | -0.04% | 100% | 101% |  |
| Class S2 | 0.00% | 0.00% | 0.02% | 0.00% | 0.00% | 0.02% | 100% | 101% |  |
| Overall | -0.01% | -0.01% | -0.02% | -0.01% | -0.01% | -0.02% | 100% | 101% |  |

Informal subjective viewing was conducted and no visible difference was observed.

# Conclusion

In order the simplify the DMVR process, this contribution proposes to only apply regular clipping operation for the bilinear interpolation in DMVR motion search process, regardless of whether wrap around is enabled or not. The test results show that the performance difference is negligible.

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Jan. 2020.

P. Hanhart, Y. He, Y. Yan, “AHG8: Horizontal geometry padding for PERP”, JVET-K0333, July 2018, Ljubljana, SI.

P. Hanhart, Y. He, Y. Yan, “CE13: PERP with horizontal geometry padding of reference pictures (Test 3.3)”, JVET-L0231, October 2018, Macau, CN.

P. Hanhart, J. Boyce, and K. Choi, “JVET common test conditions and evaluation procedures for 360° video,” document JVET-L1012, 12th JVET meeting, Jan. Marrakech, 2019.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).