| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0225-v1 |
| --- | --- |

| Title: | AHG9: On ALF/CC-ALF high level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng1, Xiaozhen Zheng2, Shanshe Wang1, Siwei Ma1 Peking University1, SZ DJI Technology Co., Ltd.2 | Tel: Email: | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution proposes the following two aspects,

Aspect 1: Use two separate syntax (ph_alf_cb_enabled_flag and ph_alf_cr_enabled_flag) to replace ph_alf_chroma_idc in picture header. And use slice_alf_cb_enabled_flag and slice_alf_cr_enabled_flag to replace slice_alf_chroma_idc in slice header. The separate design of Cb and Cr can make the ALF spec more clear.

Aspect 2: Use two separate syntax (alf_ctb_cc_cb_flag and alf_ctb_cc_cr_idx) to replace alf_ctb_cc_cb_idc in CTU level.

Introduction

In the latest VVC text, ph_alf_chroma_idc and slice_alf_chroma_idc are used for the on/off control of ALF chroma component in picture/slice level. ph_alf_chroma_idc equal to 0 specifies that the adaptive loop filter is not applied to Cb and Cr colour components. ph_alf_chroma_idc equal to 1 indicates that the adaptive loop filter is applied to the Cb colour component. ph_alf_chroma_idc equal to 2 indicates that the adaptive loop filter is applied to the Cr colour component. ph_alf_chroma_idc equal to 3 indicates that the adaptive loop filter is applied to Cb and Cr colour components.

alf_ctb_cc_cb_idc/alf_ctb_cc_cr_idc indicates the on/off and filter index of CCALF cb/cr component. alf_ctb_cc_cb_idc[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] equal to 0 specifies that the cross-component filter is not applied to the coding tree block of the Cb colour component at luma location ( xCtb, yCtb ). When alf_cc_cb_idc[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] not equal to 0, alf_cc_cb_idc[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] − 1 specifies the filter set index of the cross-component filter applied to the coding tree block of the Cb colour component at luma location ( xCtb, yCtb ).

This mixed syntax design makes the stream analysis process complicated and difficult to understand.

Proposed Method

In this contribution, we propose to use separate syntax for Cb/Cr Slice/Picture header on/off control and separate syntax for CTU level flag and filter index of CCALF. The sepc change is as follows.

Aspect 1: ALF picture/slice header

| if( sps_alf_enabled_flag && alf_info_in_ph_flag ) { |  |
| --- | --- |
| ph_alf_enabled_flag | u(1) |
| if( ph_alf_enabled_flag ) { |  |
| ph_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < ph_num_alf_aps_ids_luma; i++ ) |  |
| ph_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ){ |  |
| ph_alf_chroma_idc | u(2) |
| ph_alf_cb_enabled_flag | u(1) |
| ph_alf_cr_enabled_flag | u(1) |
| } |  |
| if( ph_alf_chroma_idc > 0 ) |  |
| if( ph_alf_cb_enabled_flag ¦¦ ph_alf_cr_enabled_flag ) |  |
| ph_alf_aps_id_chroma | u(3) |
| if( sps_ccalf_enabled_flag ) { |  |
| ph_cc_alf_cb_enabled_flag | u(1) |
| if( ph_cc_alf_cb_enabled_flag ) |  |
| ph_cc_alf_cb_aps_id | u(3) |
| ph_cc_alf_cr_enabled_flag | u(1) |
| if( ph_cc_alf_cr_enabled_flag ) |  |
| ph_cc_alf_cr_aps_id | u(3) |
| } |  |
| } |  |
| } |  |

ph_alf_chroma_idc equal to 0 specifies that the adaptive loop filter is not applied to Cb and Cr colour components. ph_alf_chroma_idc equal to 1 indicates that the adaptive loop filter is applied to the Cb colour component. ph_alf_chroma_idc equal to 2 indicates that the adaptive loop filter is applied to the Cr colour component. ph_alf_chroma_idc equal to 3 indicates that the adaptive loop filter is applied to Cb and Cr colour components. When ph_alf_chroma_idc is not present, it is inferred to be equal to 0.

ph_alf_cb_enabled_flag equal to 0 specifies that the adaptive loop filter is not applied to Cb colour component. ph_alf_cb_enabled_flag equal to 1 indicates that the adaptive loop filter is applied to the Cb colour component. When ph_alf_cb_enabled_flag is not present, it is inferred to be equal to 0.

ph_alf_cr_enabled_flag equal to 0 specifies that the adaptive loop filter is not applied to Cr colour component. ph_alf_cr_enabled_flag equal to 1 indicates that the adaptive loop filter is applied to the Cr colour component. When ph_alf_cr_enabled_flag is not present, it is inferred to be equal to 0.

| if( sps_alf_enabled_flag && !alf_info_in_ph_flag ) { |  |
| --- | --- |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ){ |  |
| slice_alf_chroma_idc | u(2) |
| slice_alf_cb_enabled_flag | u(1) |
| slice_alf_cr_enabled_flag | u(1) |
| } |  |
| if( slice_alf_chroma_idc ) |  |
| if( slice_alf_cb_enabled_flag ¦¦ slice_alf_cr_enabled_flag ) |  |
| slice_alf_aps_id_chroma | u(3) |
| if( sps_ccalf_enabled_flag ) { |  |
| slice_cc_alf_cb_enabled_flag | u(1) |
| if( slice_cc_alf_cb_enabled_flag ) |  |
| slice_cc_alf_cb_aps_id | u(3) |
| slice_cc_alf_cr_enabled_flag | u(1) |
| if( slice_cc_alf_cr_enabled_flag ) |  |
| slice_cc_alf_cr_aps_id | u(3) |
| } |  |
| } |  |
| } |  |

slice_alf_chroma_idc equal to 0 specifies that the adaptive loop filter is not applied to Cb and Cr colour components. slice_alf_chroma_idc equal to 1 indicates that the adaptive loop filter is applied to the Cb colour component. slice_alf_chroma_idc equal to 2 indicates that the adaptive loop filter is applied to the Cr colour component. slice_alf_chroma_idc equal to 3 indicates that the adaptive loop filter is applied to Cb and Cr colour components. When slice_alf_chroma_idc is not present, it is inferred to be equal to ph_alf_chroma_idc.

slice_alf_cb_enabled_flag equal to 0 specifies that the adaptive loop filter is not applied to Cb colour component. slice_alf_cb_enabled_flag equal to 1 indicates that the adaptive loop filter is applied to the Cb colour component. When slice_alf_cb_enabled_flag is not present, it is inferred to be equal to ph_alf_cb_enabled_flag.

slice_alf_cr_enabled_flag equal to 0 specifies that the adaptive loop filter is not applied to Cr colour component. slice_alf_cr_enabled_flag equal to 1 indicates that the adaptive loop filter is applied to the Cr colour component. When slice_alf_cr_enabled_flag is not present, it is inferred to be equal to ph_alf_cr_enabled_flag.

Aspect 2: CCALF CTU level

| if( slice_alf_enabled_flag ){ |  |
| --- | --- |
| alf_ctb_flag[ 0 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( alf_ctb_flag[ 0 ][ CtbAddrX ][ CtbAddrY ] ) { |  |
| if( slice_num_alf_aps_ids_luma > 0 ) |  |
| alf_use_aps_flag | ae(v) |
| if( alf_use_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 1 ) |  |
| alf_luma_prev_filter_idx | ae(v) |
| } else |  |
| alf_luma_fixed_filter_idx | ae(v) |
| } |  |
| if( slice_alf_chroma_idc = = 1 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| if( slice_alf_cb_enabled_flag ) { |  |
| alf_ctb_flag[ 1 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( alf_ctb_flag[ 1 ][ CtbAddrX ][ CtbAddrY ] && alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 0 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| } |  |
| if( slice_alf_chroma_idc = = 2 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| if( slice_alf_cr_enabled_flag ) { |  |
| alf_ctb_flag[ 2 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( alf_ctb_flag[ 2 ][ CtbAddrX ][ CtbAddrY ] && alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 1 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| } |  |
| } |  |
| if( slice_cc_alf_cb_enabled_flag ){ |  |
| alf_ctb_cc_cb_idc[ CtbAddrX ][ CtbAddrY ] | ae(v) |
| alf_ctb_cc_cb_flag[ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if (alf_ctb_cc_cb_flag[ CtbAddrX ][ CtbAddrY ] && alf_cc_cb_filters_signalled_minus1 > 0) |  |
| alf_ctb_cc_cb_idx[ CtbAddrX ][ CtbAddrY ] | ae(v) |
| } |  |
| if( slice_cc_alf_cr_enabled_flag ) { |  |
| alf_ctb_cc_cr_idc[ CtbAddrX ][ CtbAddrY ] | ae(v) |
| alf_ctb_cc_cr_flag[ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if (alf_ctb_cc_cr_flag[ CtbAddrX ][ CtbAddrY ] && alf_cc_cr_filters_signalled_minus1 > 0) |  |
| alf_ctb_cc_cr_idx[ CtbAddrX ][ CtbAddrY ] | ae(v) |
| } |  |

alf_ctb_cc_cb_idc[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] equal to 0 specifies that the cross-component filter is not applied to the coding tree block of the Cb colour component at luma location ( xCtb, yCtb ). When alf_cc_cb_idc[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] not equal to 0, alf_cc_cb_idc[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] − 1 specifies the filter set index of the cross-component filter applied to the coding tree block of the Cb colour component at luma location ( xCtb, yCtb ).

When alf_ctb_cc_cb_idc[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, it is inferred to be equal to 0.

alf_ctb_cc_cr_idc[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] equal to 0 specifies that the cross-component filter is not applied to the coding tree block of the Cr colour component at luma location ( xCtb, yCtb ). when alf_cc_cr_idc[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] not equal to 0, alf_cc_cb_idc[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] − 1 specifies the filter set index of the cross-component filter applied to the coding tree block of the Cr colour component at luma location ( xCtb, yCtb ).

When alf_ctb_cc_cr_idc[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, it is inferred to be equal to 0.

alf_ctb_cc_cb_flag[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] equal to 1 specifies that the cross-component filter is applied to the coding tree block of the Cb colour component at luma location ( xCtb, yCtb ). alf_ctb_cc_cb_flag[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] equal to 0 specifies that the cross-component filter is not applied to the coding tree block of the Cb colour component at luma location ( xCtb, yCtb ). When alf_ctb_cc_cb_flag [ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, it is inferred to be equal to 0.

alf_ctb_cc_cb_idx[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] specifies the filter set index of the cross-component filter applied to the coding tree block of the Cb colour component at luma location ( xCtb, yCtb ). When alf_ctb_cc_cb_idx [ chromaIdx ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, it is infered to be equal to zero.

alf_ctb_cc_cr_flag[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] equal to 1 specifies that the cross-component filter is applied to the coding tree block of the Cr colour component at luma location ( xCtb, yCtb ). alf_ctb_cc_cr_flag[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] equal to 0 specifies that the cross-component filter is not applied to the coding tree block of the Cr colour component at luma location ( xCtb, yCtb ). When alf_ctb_cc_cr_flag [ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, it is inferred to be equal to 0.

alf_ctb_cc_cr_idx[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] specifies the filter set index of the cross-component filter applied to the coding tree block of the Cr colour component at luma location ( xCtb, yCtb ). When alf_ctb_cc_cr_idx [ chromaIdx ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, it is infered to be equal to zero.

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q2001, Jan. 2020

- K. Misra, F. Bossen, A. Segall, et.al, “CC-ALF: Integrated Text for the Cross Component Adaptive Loop Filter,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q0795, Jan. 2020

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).