| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0234-v2 |
| --- | --- |

| Title: | Removal of redundant LFNST index signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Alican Nalci, Vadim Seregin, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA | Email: | hegilmez@qti.qualcomm.com analci@qti.qualcomm.com vseregin@qti.qualcomm.com wchien@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In VVC draft 8, an LFNST can only be applied to luma component and is disabled for chroma components in single-tree partition mode. In single-tree partition mode, there is a rare case that an LFNST index can be redundantly signalled. This document proposes to remove this case of redundant signaling of the LFNST index. The experimental results under common test conditions (CTC) show that the proposed change has negligible impact in terms of average BD-rates, since this case rarely occurs under CTC.

# Introduction

In the VVC specification working draft (JVET-Q2001-vE), the LFNST index (lfnst_idx) is signalled after residual coding of all color components based on the following set of conditions:

| LfnstDcOnly = 1 |  |
| --- | --- |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsDcOnly = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| … |  |
| lfnstNotTsFlag = ( treeType = = DUAL_TREE_CHROMA ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 ) && ( treeType = = DUAL_TREE_LUMA ¦ ¦ ( transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 ) ) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && lfnstNotTsFlag = = 1 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |

where the LfnstDcOnly variable is used to check if there exists a non-DC coefficient in a TU. If there is at least one non-DC coefficient (i.e., LfnstDcOnly is equal to 0), the LFNST index can be signalled.

The problem in the current version of VVC is that the value of LfnstDcOnly depends on chroma components even though LFNST is disabled for chroma in single-tree partitioning mode. This may lead to redundant signaling of LFNST index when there is no non-DC coefficient in luma component and there exists a non-DC coefficent in chroma components.

The following part of the specification text (JVET-Q2001-vE) shows the conditions where LfnstDcOnly flag is updated:

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |
| if( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && lastScanPos > 0 ) |  |
| LfnstDcOnly = 0 |  |
| if( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ ( lastScanPos > 7 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) ) |  |
| LfnstZeroOutSigCoeffFlag = 0 |  |
| if( ( lastSubBlock > 0 ¦ ¦ lastScanPos > 0 ) && cIdx = = 0 ) |  |
| MtsDcOnly = 0 |  |
| … |  |

# Proposed Method

The following condition highlighted in yellow is added on top of the VVC specification working draft (JVET-Q2001-vE) in order to skip checking DC-coefficients for chroma components in the single-tree partitioning mode, so that the DC-coefficient check is only done for luma component:

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |
| if( treeType != SINGLE_TREE ¦ ¦ cIdx = = 0 ) { |  |
| if( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && lastScanPos > 0 ) |  |
| LfnstDcOnly = 0 |  |
| } |  |
| if( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ ( lastScanPos > 7 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) ) |  |
| LfnstZeroOutSigCoeffFlag = 0 |  |
| if( ( lastSubBlock > 0 ¦ ¦ lastScanPos > 0 ) && cIdx = = 0 ) |  |
| MtsDcOnly = 0 |  |
| … |  |

This removes the case of redundant signaling of the LFNST index in the VVC design.

# Experimental Results

The proposed method is implemented on top of VTM-8.0 and evaluated under common test conditions (CTC). Note that low-delay B results are omitted, since LFNST is disabled in CTC for low-delay B.

Table 1.  Results of the proposed method over VTM-8.0 under CTC

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.01% | 101% | 100% |
| Class A2 | -0.01% | -0.04% | 0.11% | 100% | 100% |
| Class B | 0.00% | 0.04% | 0.06% | 102% | 101% |
| Class C | -0.01% | -0.04% | -0.09% | 98% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.01% | 100% | 99% |
| Class D | 0.00% | 0.06% | -0.17% | 100% | 104% |
| Class F | 0.01% | 0.01% | -0.01% | 101% | 100% |

# Conclusions

This document proposes to remove the case of redundant signaling of LFNST index when LFNST is not applied. Although the redundant signaling rarely occurs under CTC and the proposed fix has negligible impact in terms of average BD-rates, the redundant signaling is undesirable and it leads to unnecessary use of the CABAC engine. It is recommended to adopt the proposed method in VVC.

The file “JVET-Q2001-vE_JVET-R0234-spec.docx” attached to this submission includes the proposed changes over the VVC specification working draft (JVET-Q2001-vE).

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8),” JVET-Q2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 Jan. 2020.

- H.E. Egilmez, A. Nalci, M. Coban, V. Seregin, M. Karczewicz, “Chroma LFNST Simplification and Signaling,” JVET-Q0686, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 Jan. 2020.

- H.E. Egilmez, A. Nalci, A.K. Ramasubramonian, M. Coban, V. Seregin, M. Karczewicz, T. Hashimoto, T. Chujoh, E. Sasaki, T. Ikai, “Combination of JVET-Q0106, JVET-0686 and JVET-Q0133 for LFNST Signaling, Latency Reduction and Scaling Process,” JVET-Q0784, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 Jan. 2020.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).