| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0240-v2 |
| --- | --- |

| Title: | On maximum palette size and palette predictor size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yung-Hsuan Chao, Ted Hsieh, Wei-Jung Chien, Vadim Seregin, Marta Karczewicz | Tel: Email: | +1-858-8452163 yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to reduce maximum palette and palette predictor size. Three methods are proposed in this contribution:

- Reduce the maximum palette and palette predictor size to 16 and 32 in single tree slice and to 8 and 16 in dual tree slice

- Reduce the maximum palette and palette predictor size to 16 and 32 in single tree slice and to 8 and 16 in dual tree slice for YUV420 format, while the size for other color formats stays the same as in VVC draft 8.

- Allow the signaling of maximum palette size and maximum palette predictor size in SPS, same syntax as in HEVC SCC extension. The allowed maximum palette and palette predictor size are restricted to 32 and 64.

The results of method 1 on YUV4:4:4 screen content sequences versus non-420 CTC anchor (PLT=1) are shown as:

1.48% AI, 1.05% RA and x.xx% LDB

The results of method 1 and 2 on YUV4:2:0 class F andTGM versus VTM8 with PLT=1 are shown as:

Class F: 0.16% AI, 0.30% RA and 0.09% LDB

Class TGM: 0.71% AI, 0.59% RA, and 0.19% LDB

The results of method 3 with maximum palette and palette predictor size signaled to be 16 and 32 for YUV4:2:0 format versus VTM8 with PLT=1 are shown as:

Class F: 0.16% AI, 0.30% RA and 0.09% LDB

Class TGM: 0.72% AI, 0.59% RA, and 0.19% LDB

# Introduction

In VVC Draft 8, the maximum palette size is fixed as 31 for single tree slice and 15 for dual tree slice regardless of the color format. For palette predictor, the maximum size is fixed as 63 for single tree slice and 31 for dual tree slice. In this contribution, we propose to reduce the maximum size of palette table and palette predictor, which can significantly reduce hardware memory and speed up the parsing and table look up.

The contribution proposes three methods:

- Reduce the maximum palette and palette predictor size to 16 and 32 in single tree slice and to 8 and 16 in dual tree slice

- Reduce the maximum palette and palette predictor size to 16 and 32 in single tree slice and to 8 and 16 in dual tree slice for YUV420 format, since the memory requirement and the impact of palette size on throughput may be different based on colour format.

- Signal maximum palette size and maximum palette predictor size in SPS, same as in HEVC SCM. The allowed maximum palette and palette predictor size are restricted to 32 and 64. The feature allows encoder to reduce maximum palette and palette predictor size, which helps reducing the time in palette table look up.

# Syntax changes

## Method 1

# 7.3.10.6	 Palette coding syntax

| palette_coding( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| startComp = ( treeType = = DUAL_TREE_CHROMA ) ? 1 : 0 |  |
| numComps = ( treeType = = SINGLE_TREE ) ? ( ChromaArrayType = = 0 ? 1 : 3 ) : ( treeType = = DUAL_TREE_CHROMA ) ? 2 : 1 |  |
| maxNumPaletteEntries = ( treeType = = SINGLE_TREE ) ? 16 : 8 |  |

8.4.5.3	Decoding process for palette mode

Depending on the value of treeType, the variables startComp, numComps and maxNumPalettePredictorSize are derived as follows:

- If treeType is equal to SINGLE_TREE:

startComp = 0		(430)

numComps = ChromaArrayType = = 0 ? 1 : 3		(431)

maxNumPalettePredictorSize = 32		(432)

- Otherwise, treeType is equal to DUAL_TREE_LUMA:

startComp = 0		(433)

numComps = 1		(434)

maxNumPalettePredictorSize = 16	(435)

- Otherwise, treeType is equal to DUAL_TREE_CHROMA:

startComp = 1		(436)

numComps = 2		(437)

maxNumPalettePredictorSize = 16		(438)

When localDualTree is equal to 1, the following applies:

- The variables CurrentPaletteSize[ 0 ], startComp, numComps and maxNumPalettePredictorSize are derived as follows:

CurrentPaletteSize[ 0 ] = CurrentPaletteSize[ startComp ]	(453)

startComp = 0		(454)

numComps = 3		(455)

maxNumPalettePredictorSize = 32		(456)

## Method 2

# 7.3.10.6	 Palette coding syntax

| palette_coding( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| startComp = ( treeType = = DUAL_TREE_CHROMA ) ? 1 : 0 |  |
| numComps = ( treeType = = SINGLE_TREE ) ? ( ChromaArrayType = = 0 ? 1 : 3 ) : ( treeType = = DUAL_TREE_CHROMA ) ? 2 : 1 |  |
| maxNumPaletteEntries = ( treeType = = SINGLE_TREE ) ? 31 : 15 |  |
| if ( ChromaArrayType = = 1 ) |  |
| maxNumPaletteEntries = ( maxNumPaletteEntries + 1 ) >> 1 |  |

8.4.5.3	Decoding process for palette mode

Depending on the value of treeType, the variables startComp, numComps and maxNumPalettePredictorSize are derived as follows:

- If treeType is equal to SINGLE_TREE:

startComp = 0		(430)

numComps = ChromaArrayType = = 0 ? 1 : 3		(431)

maxNumPalettePredictorSize = ( ChromaArrayType = = 1 )? 32: 63		(432)

- Otherwise, treeType is equal to DUAL_TREE_LUMA:

startComp = 0		(433)

numComps = 1		(434)

maxNumPalettePredictorSize = ( ChromaArrayType = = 1 )? 16: 31	(435)

- Otherwise, treeType is equal to DUAL_TREE_CHROMA:

startComp = 1		(436)

numComps = 2		(437)

maxNumPalettePredictorSize = ( ChromaArrayType = = 1 )? 16: 31	(438)

When localDualTree is equal to 1, the following applies:

- The variables CurrentPaletteSize[ 0 ], startComp, numComps and maxNumPalettePredictorSize are derived as follows:

CurrentPaletteSize[ 0 ] = CurrentPaletteSize[ startComp ]	(453)

startComp = 0		(454)

numComps = 3		(455)

maxNumPalettePredictorSize = ( ChromaArrayType = = 1 )? 32: 63	(456)

## Method 3

| sps_palette_enabled_flag | u(1) |
| --- | --- |
| if( sps_palette_enabled_flag ) { |  |
| palette_max_size | ue(v) |
| delta_palette_max_predictor_size | ue(v) |
| } |  |
| if( ChromaArrayType = = 3 && !sps_max_luma_transform_size_64_flag ) |  |
| sps_act_enabled_flag | u(1) |

palette_max_size specifies the maximum allowed palette size. When not present, the value of palette_max_size is inferred to be 0.

It is a requirement of bitstream conformance that, palette_max_size shall be less than or equal to 32.

delta_palette_max_predictor_size specifies the difference between the maximum allowed palette predictor size and the maximum allowed palette size. When not present, the value of delta_palette_max_predictor_size is inferred to be 0. The variable PaletteMaxPredictorSize is derived as follows:

PaletteMaxPredictorSize = palette_max_size + delta_palette_max_predictor_size	(7-35)

It is a requirement of bitstream conformance that, when palette_max_size is equal to 0, the value of delta_palette_max_predictor_size shall be equal to 0, and PaletteMaxPredictorSize shall be less than or equal to 64.

# 7.3.10.6	 Palette coding syntax

| palette_coding( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| startComp = ( treeType = = DUAL_TREE_CHROMA ) ? 1 : 0 |  |
| numComps = ( treeType = = SINGLE_TREE ) ? ( ChromaArrayType = = 0 ? 1 : 3 ) : ( treeType = = DUAL_TREE_CHROMA ) ? 2 : 1 |  |
| maxNumPaletteEntries = ( treeType = = SINGLE_TREE ) ? palette_max_size: (palette_max_size+1) >> 1 |  |

8.4.5.3	Decoding process for palette mode

Depending on the value of treeType, the variables startComp, numComps and maxNumPalettePredictorSize are derived as follows:

- If treeType is equal to SINGLE_TREE:

startComp = 0		(430)

numComps = ChromaArrayType = = 0 ? 1 : 3		(431)

maxNumPalettePredictorSize = PaletteMaxPredictorSize	(432)

- Otherwise, treeType is equal to DUAL_TREE_LUMA:

startComp = 0		(433)

numComps = 1		(434)

maxNumPalettePredictorSize = ( PaletteMaxPredictorSize + 1 ) >> 1	(435)

- Otherwise, treeType is equal to DUAL_TREE_CHROMA:

startComp = 1		(436)

numComps = 2		(437)

maxNumPalettePredictorSize = ( PaletteMaxPredictorSize + 1 ) >> 1	(438)

When localDualTree is equal to 1, the following applies:

- The variables CurrentPaletteSize[ 0 ], startComp, numComps and maxNumPalettePredictorSize are derived as follows:

CurrentPaletteSize[ 0 ] = CurrentPaletteSize[ startComp ]	(453)

startComp = 0		(454)

numComps = 3		(455)

maxNumPalettePredictorSize = PaletteMaxPredictorSize	(456)

# Results

## The proposed methods are evaluated for YUV420 sequences according to the SDR common test condition [1] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. Besides YUV420, method 1 is also evaluated for YUV444 sequences according to non-420 common tests condition [2].

## Method 1

For test on YUV4:4:4 sequences, palette mode is enabled ( -- PLT=1 ) in VTM8.0 for anchor.

Table 1: Results of method 1 on YUV4:4:4 sequences

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 2.80% | 3.25% | 3.03% | 100% | 99% |
| TGM 720p | 0.88% | 1.02% | 1.20% | 98% | 95% |
| Animation | 0.38% | 0.58% | 0.58% | 97% | 92% |
| Mixed content | 1.61% | 1.81% | 1.53% | 99% | 98% |
| Overall | 1.48% | 1.73% | 1.66% | 99% | 96% |
|  | Random access Main10 |  |  |  |  |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 1.82% | 2.20% | 1.98% | 99% | 99% |
| TGM 720p | 0.88% | 1.09% | 1.43% | 97% | 97% |
| Animation | 0.23% | 0.33% | 0.37% | 96% | 95% |
| Mixed content | 1.08% | 1.51% | 1.33% | 99% | 100% |
| Overall | 1.05% | 1.33% | 1.34% | 98% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.54% | 0.87% | 0.67% | 100% | 104% |
| TGM 720p | 0.39% | 0.40% | 0.43% | 95% | 94% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Mixed content | 0.93% | 1.16% | 1.21% | 99% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

For test on YUV4:2:0 sequences, the method is evaluated against CTC anchor (--PLT=0) and VTM8.0 with palette mode enabled (--PLT=1).

Table 2: Results of method 1 on YUV4:2:0 screen content sequences

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM8.0 anchor (PLT=0) |  |  |  |  | Over VTM8.0 with PLT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | -1.09% | -0.38% | -0.47% | 103% | 91% | 0.16% | 0.06% | 0.00% | 100% | 100% |
| TGM | -5.90% | -4.92% | -4.71% | 100% | 93% | 0.71% | 0.28% | 0.26% | 100% | 100% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM8.0 anchor (PLT=0) |  |  |  |  | Over VTM8.0 with PLT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | c | V | EncT | DecT |
| Class F | -1.01% | -0.33% | -0.34% | 105% | 100% | 0.30% | 0.14% | 0.22% | 101% | 100% |
| TGM | -3.25% | -2.21% | -2.14% | 99% | 95% | 0.59% | 0.25% | 0.29% | 99% | 101% |
|  | Low delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM8.0 anchor (PLT=0) |  |  |  |  | Over VTM8.0 with PLT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | -0.47% | -0.32% | -1.34% | 107% | 99% | 0.09% | -0.08% | -0.10% | 100% | 100% |
| TGM | -1.03% | -0.46% | -0.61% | 98% | 96% | 0.19% | 0.13% | 0.05% | 99% | 99% |

## Method 2

Table 3: Results of method 2 on YUV4:2:0 screen content sequences

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM8.0 anchor (PLT=0) |  |  |  |  | Over VTM8.0 with PLT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | -1.09% | -0.38% | -0.47% | 103% | 91% | 0.16% | 0.06% | 0.00% | 100% | 100% |
| TGM | -5.90% | -4.92% | -4.71% | 100% | 93% | 0.71% | 0.28% | 0.26% | 100% | 100% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM8.0 anchor (PLT=0) |  |  |  |  | Over VTM8.0 with PLT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | c | V | EncT | DecT |
| Class F | -1.01% | -0.33% | -0.34% | 105% | 100% | 0.30% | 0.14% | 0.22% | 101% | 100% |
| TGM | -3.25% | -2.21% | -2.14% | 99% | 95% | 0.59% | 0.25% | 0.29% | 99% | 101% |
|  | Low delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM8.0 anchor (PLT=0) |  |  |  |  | Over VTM8.0 with PLT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | -0.47% | -0.32% | -1.34% | 107% | 99% | 0.09% | -0.08% | -0.10% | 100% | 100% |
| TGM | -1.03% | -0.46% | -0.61% | 98% | 96% | 0.19% | 0.13% | 0.05% | 99% | 99% |

## Method 3

Table 4: Results of method 3 on YUV4:2:0 screen content sequences

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM8.0 anchor (PLT=0) |  |  |  |  | Over VTM8.0 with PLT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | -1.09% | -0.37% | -0.46% | 102% | 91% | 0.16% | 0.07% | 0.01% | 100% | 100% |
| TGM | -5.90% | -4.92% | -4.71% | 100% | 94% | 0.72% | 0.28% | 0.26% | 100% | 100% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM8.0 anchor (PLT=0) |  |  |  |  | Over VTM8.0 with PLT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | c | V | EncT | DecT |
| Class F | -1.00% | -0.33% | -0.33% | 102% | 94% | 0.30% | 0.14% | 0.23% | 100% | 99% |
| TGM | -3.25% | -2.21% | -2.14% | 99% | 95% | 0.59% | 0.25% | 0.29% | 99% | 101% |
|  | Low delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM8.0 anchor (PLT=0) |  |  |  |  | Over VTM8.0 with PLT=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | -0.47% | -0.32% | -1.33% | 102% | 91% | 0.09% | -0.08% | -0.10% | 100% | 101% |
| TGM | -1.03% | -0.46% | -0.61% | 98% | 95% | 0.19% | 0.13% | 0.05% | 99% | 98% |

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010

[2] Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats”, JVET-Q2013

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).