| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, AT, 15–24 April 2020 | Document: JVET-R0247-v2 |
| --- | --- |

| Title: | AHG9: Signalling rectangular slice partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Chih-Wei Hsu, Olena Chubach, Lulin Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, cw.hsu, olena.chubach, lulin.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes two high-level syntax modifications related to signalling rectangular slice partitioning of the coded picture.

- For the rectangular slice mode, the signalling of tile_idx_delta_present_flag is further conditioned on whether the number of tile columns or the number of tile rows is greater than 1 in the coded picture.

- For the rectangular slice mode, loop_filter_across_slices_enabled_flag is not signalled for single slice per picture.

In version 2 of this proposal, a bug in the syntax table is fixed.

Proposed modifications

Proposal 1: Conditionally signalling tile_idx_delta_present_flag

In Versatile Video Coding (VVC) Draft 8 as specified in [1], when the rectangular slice mode is used (with rect_slice_flag = 1), a syntax element tile_idx_delta_present_flag may be signalled to specify whether the coded picture is partitioned into rectangular slice rows and rectangular slice columns in slice raster order. tile_idx_delta_present_flag equal to 0 specifies that tile_idx_delta[ i ] syntax elements are not present in the PPS and all pictures referring to the PPS are partitioned into rectangular slice rows and rectangular slice columns in slice raster order. tile_idx_delta_present_flag equal to 1 specifies that tile_idx_delta[ i ] syntax elements, indicating the difference between the tile index of the curent slice and the tile index of the next slice, may be present in the PPS. All rectangular slices in pictures referring to the PPS are specified in the order indicated by the values of the tile_idx_delta[ i ] in increasing values of i.

However, when the number of tile columns or the number of tile rows in the coded picture is equal to 1, the coded picture is always partitioned into rectangular slice rows and rectangular slice columns in slice raster order (either vertically or horizontally). In the proposed method, signalling tile_idx_delta_present_flag is further conditioned on whether the number of tile columns or the number of tile rows in the coded picture is greater than 1 in the coded picture.

Proposal 2: Conditionally signalling the syntax element loop_filter_across_slices_enabled_flag

The syntax element loop_filter_across_slices_enabled_flag specifying whether in-loop filtering operations shall be applied across slice boundaries is relevant only for the coded pictures containing more than one slice. In the proposed method, for the rectangular slice mode, loop_filter_across_slices_enabled_flag in the PPS is not signalled when the number of the slices in the coded picture is equal to 1.

Modifications to specification

The modifications to the VVC Draft 8 related to Proposal 1 are written in red. The modifications to the VVC Draft 8 related to Proposal 2 are written in blue. Both proposed modifications are provided below:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| …. |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| if( NumTilesInPic > 1 ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| if( num_slices_in_pic_minus1 > 0 && NumTileColumns > 1 && NumTileRows > 1 ) |  |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| if( NumTileColumns > 1 ) |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| if( NumTileRows > 1 && ( tile_idx_delta_present_flag ¦ ¦ SliceTopLeftTileIdx[ i ] % NumTileColumns = = 0 ) ) |  |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 && RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] > 1 ) { |  |
| num_exp_slices_in_tile[ i ] | ue(v) |
| for( j = 0; j < num_exp_slices_in_tile[ i ]; j++ ) |  |
| exp_slice_height_in_ctus_minus1[ i ][ j ] | ue(v) |
| i += NumSlicesInTile[ i ] − 1 |  |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag |  |
| if( !rect_slice_flag ¦ ¦ single_slice_per_subpic_flag ¦ ¦ num_slices_in_pic_minus1 > 0 ) |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| …. |  |
| } |  |

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-Q2001, 17th Meeting: Brussels, BE, 7–17 January 2020.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).