| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0259-v3 |
| --- | --- |

| Title: | AHG7: On CCALF filtering of chroma sample location type-2 content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer Yan Ye, Jiancong Luo 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | + 1 408-858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

It is asserted that the CCALF filter shape is not optimal for chroma sample location type-2 content. Accordingly, this contribution proposes three cross shaped CCALF filters. Following results are reported.

For chroma sample location type-2 content,

- 9 tap 5x5 cross shaped filter:

  - AI :  0.01% (Y), -0.77% (Cb), -0.81% (Cr)

  - RA:  0.07% (Y), -1.99% (Cb), -1.95% (Cr)

  - LB:  0.19% (Y), -5.85% (Cb), -9.09% (Cr)

- 13 tap 7x7 cross shaped filter:

  - AI :  0.05% (Y), -1.40% (Cb), -1.34% (Cr)

  - RA:  0.04% (Y), -2.85% (Cb), -3.22% (Cr)

  - LB:   0.10% (Y), -7.28% (Cb), -11.34% (Cr)

- 8 tap 5x4 cross shaped filter:

  - AI :  0.00% (Y), -0.35% (Cb), -0.39% (Cr)

  - RA:  0.05% (Y), -1.43% (Cb), -1.52% (Cr)

  - LB:   0.17% (Y), -4.10% (Cb), -7.06% (Cr)

When the proposed cross-shape filters are applied on chroma sample location type-0 content, it is reported that some coding gain can also be achieved:

- 9 tap 5x5 cross shaped filter:

  - AI :  0.02% (Y), -0.26% (Cb), -0.68% (Cr)

  - RA:  0.01% (Y), -0.39 % (Cb), -0.48 % (Cr)

  - LB:  -0.04 % (Y), -1.06 % (Cb), -1.19 % (Cr)

- 13 tap 7x7 cross shaped filter:

  - AI :  0.04% (Y), -0.82% (Cb), -1.16% (Cr)

  - RA:  0.00% (Y), -1.30% (Cb), -1.28% (Cr)

  - LB:   -0.02% (Y), -3.25% (Cb), -2.80% (Cr)

- 8 tap 5x4 cross shaped filter:

  - AI :  0.01% (Y), -0.09% (Cb), -0.55% (Cr)

  - RA:  -0.01% (Y), -0.19% (Cb), -0.31% (Cr)

  - LB:   -0.01% (Y), -0.55% (Cb), -0.91% (Cr)

# Introduction

In VVC draft 8, the Cross Component Adaptive Loop Filter (CCALF) was adopted [1]. The CCALF process uses a linear filter to filter luma sample values and generate a residual correction for the chroma samples.  In VVC, 8-tap hexagon shaped filter is used in CCALF process, which is illustrated in Figure 1.

Figure 1: CCALF filters of VVC draft 8

In VVC, the CCALF filter is designed based on chroma sample location type 0 (aka “type-0”), since it is the most commonly used case for SDR video. However, for HDR video, a different chroma sample location type, type-2 in Figure 1, is often used. As shown in Figure 1, for type-2 content, the current CCALF filter is not symmetric, which may not achieve optimal coding performance.

# Proposed filters

In the proposed method, three cross shaped filters are proposed.

- 9 tap 5x5 cross shaped filter as illustrated in Figure 2

- 13 tap 7x7 cross shaped filter as illustrated in Figure 3

- 8 tap 5x4 cross shaped filter as illustrated in Figure 4

Figure 2: proposed 9 tap 5x5 cross filter

Figure 3: proposed 13 tap 7x7 cross filter

Figure 4: proposed 8  tap 5x4  cross filter

# Simulation results

The proposed filters are implemented on top of VTM-8.0 reference software. Both chroma sample location type-0 and type-2 contents are tested.

## Results of chroma sample location type-2 content

In order to verify the performance of the proposed filters in chroma sample location type-2 content, HDR H2 sequences are tested under HDR CTC test conditions [2]. The performance of the proposed 9 tap, 13 tap and 8 tap filters for HDR H2 sequences are tabulated in Table 1, Table 2, and Table 3 respectively.

Table 1: performance of the proposed 9  tap 5x5  cross filter for type-2 sequence

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.29% | 0.01% | 0.01% | -0.61% | -0.63% | 0.01% | -0.77% | -0.81% | 100% | 99% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.62% | 0.08% | 0.07% | -1.94% | -2.40% | 0.07% | -1.99% | -1.95% | 99% | 99% |
|  | Low delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -3.05% | 0.17% | 0.19% | -6.12% | -9.67% | 0.19% | -5.85% | -9.09% | 99% | 100% |

Table 2: performance of the proposed 13 tap 7x7 cross filter for type-2 sequences

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.39% | 0.04% | 0.05% | -1.06% | -1.12% | 0.05% | -1.40% | -1.34% | 100% | 101% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -1.22% | 0.05% | 0.04% | -2.68% | -3.66% | 0.04% | -2.85% | -3.22% | 100% | 100% |
|  | Low delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -4.18% | 0.06% | 0.10% | -7.19% | -11.70% | 0.10% | -7.28% | -11.34% | 100% | 101% |

Table 3:  performance of the proposed 8 tap 5x4 cross filter for type-2 sequences

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.11% | 0.00% | 0.00% | -0.16% | -0.24% | 0.00% | -0.35% | -0.39% | 100% | 100% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.27% | 0.06% | 0.05% | -1.28% | -1.85% | 0.05% | -1.43% | -1.52% | 100% | 101% |
|  | Low delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -2.14% | 0.13% | 0.17% | -3.99% | -7.62% | 0.17% | -4.10% | -7.04% | 100% | 105% |

## Results of chroma sample location type-0 content

In order to verify the proposed filters in chroma sample location type-0 content, both SDR CTC test sequences and HDR H2 sequences are tested. HDR H2 sequences are tested under HDR CTC test conditions [2] and SDR sequences are tested under SDR test conditions. The performance of the proposed 9 tap, 13 tap and 8 tap filters for type-0 sequences are tabulated in Table 4, Table 5 and Table 6 respectively.

Table 4: performance of the proposed 9 tap 5x5  cross filter for type-0 sequence

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.25% | -0.89% | 100% | 100% |
| Class A2 | 0.00% | -0.47% | -0.04% | 100% | 100% |
| Class B | 0.01% | -0.35% | -0.71% | 100% | 99% |
| Class C | 0.02% | -0.34% | -0.38% | 100% | 96% |
| Class E | 0.04% | 0.20% | -1.46% | 100% | 97% |
| Overall | 0.02% | -0.26% | -0.68% | 100% | 98% |
| Class D | 0.02% | -0.09% | -0.65% | 100% | 94% |
| Class F | 0.02% | 0.22% | 0.44% | 100% | 97% |
| Class SCC | 0.01% | 0.87% | 0.81% | 100% | 99% |
| Class HDR H2 | 0.00% | 0.53% | 0.17% | 99% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.27% | -0.63% | 99% | 100% |
| Class A2 | -0.02% | -0.60% | 0.35% | 100% | 100% |
| Class B | 0.02% | -0.37% | -0.86% | 99% | 96% |
| Class C | 0.00% | -0.32% | -0.53% | 100% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.39% | -0.48% | 99% | 98% |
| Class D | 0.02% | -0.19% | -0.69% | 100% | 98% |
| Class F | 0.02% | 0.22% | 0.20% | 100% | 98% |
| Class SCC | -0.02% | 0.88% | 0.76% | 100% | 97% |
| Class HDR H2 | 0.00% | -0.02% | -0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -1.25% | -1.28% | 100% | 101% |
| Class C | -0.08% | -1.56% | -0.69% | 100% | 92% |
| Class E | 0.00% | -0.08% | -1.71% | 100% | 92% |
| Overall | -0.04% | -1.06% | -1.19% | 100% | 96% |
| Class D | 0.01% | -1.55% | -1.84% | 100% | 93% |
| Class F | -0.02% | 0.05% | -0.53% | 100% | 93% |
| Class SCC | 0.00% | 1.09% | 0.73% | 99% | 96% |
| Class HDR H2 | -0.01% | -1.91% | -0.64% | 100% | 101% |

Table 5: performance of the proposed 13 tap 7x7  filter for “type-0’ sequences

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.57% | -1.43% | 100% | 102% |
| Class A2 | 0.01% | -1.43% | -0.52% | 100% | 101% |
| Class B | 0.02% | -0.99% | -1.19% | 100% | 101% |
| Class C | 0.06% | -0.72% | -0.68% | 100% | 101% |
| Class E | 0.10% | -0.32% | -2.14% | 100% | 101% |
| Overall | 0.04% | -0.82% | -1.16% | 100% | 101% |
| Class D | 0.03% | -0.22% | -0.42% | 100% | 100% |
| Class F | 0.06% | -0.19% | 0.13% | 100% | 100% |
| Class SCC | 0.02% | 0.84% | 0.80% | 100% | 101% |
| Class HDR H2 | 0.01% | -0.23% | -0.23% | 99% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.63% | -1.58% | 101% | 101% |
| Class A2 | -0.04% | -1.87% | -1.30% | 101% | 101% |
| Class B | 0.02% | -1.61% | -1.48% | 100% | 101% |
| Class C | -0.01% | -1.00% | -0.77% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -1.30% | -1.28% | 101% | 101% |
| Class D | 0.00% | -0.42% | -0.80% | 101% | 101% |
| Class F | 0.03% | -0.04% | -0.07% | 101% | 102% |
| Class SCC | 0.00% | 0.77% | 0.65% | 101% | 101% |
| Class HDR H2 | 0.01% | -1.46% | -1.11% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -4.45% | -3.28% | 101% | 101% |
| Class C | -0.10% | -3.34% | -2.00% | 100% | 101% |
| Class E | 0.09% | -1.15% | -3.08% | 100% | 100% |
| Overall | -0.02% | -3.25% | -2.80% | 100% | 101% |
| Class D | 0.00% | -2.99% | -2.00% | 100% | 99% |
| Class F | -0.02% | -0.67% | -0.61% | 101% | 101% |
| Class SCC | 0.02% | 0.71% | 0.48% | 100% | 101% |
| Class HDR H2 | -0.03% | -4.95% | -4.05% | 100% | 100% |

Table 6: performance of proposed 8 tap 5x4 filter for type-0 sequences

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.12% | -0.76% | 100% | 101% |
| Class A2 | 0.00% | -0.19% | 0.08% | 100% | 100% |
| Class B | 0.00% | -0.23% | -0.52% | 100% | 100% |
| Class C | 0.01% | -0.18% | -0.26% | 100% | 101% |
| Class E | 0.03% | 0.38% | -1.43% | 100% | 101% |
| Overall | 0.01% | -0.09% | -0.55% | 100% | 101% |
| Class D | 0.01% | -0.10% | -0.67% | 100% | 100% |
| Class F | 0.01% | 0.35% | 0.54% | 100% | 100% |
| Class SCC | 0.00% | 0.88% | 0.82% | 100% | 101% |
| Class HDR H2 | 0.00% | 0.73% | 0.28% | 99% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.17% | -0.47% | 101% | 101% |
| Class A2 | -0.03% | -0.22% | 0.52% | 100% | 101% |
| Class B | 0.01% | -0.21% | -0.71% | 100% | 101% |
| Class C | -0.03% | -0.17% | -0.30% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.19% | -0.31% | 100% | 101% |
| Class D | 0.03% | -0.19% | -0.94% | 101% | 101% |
| Class F | 0.01% | 0.21% | 0.26% | 101% | 102% |
| Class SCC | -0.03% | 0.94% | 0.81% | 100% | 101% |
| Class HDR H2 | 0.00% | 0.58% | 0.09% | 0.00% | 0.58% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.77% | -0.72% | 101% | 100% |
| Class C | -0.06% | -1.36% | -0.59% | 100% | 103% |
| Class E | 0.07% | 0.89% | -1.64% | 100% | 101% |
| Overall | -0.01% | -0.55% | -0.91% | 100% | 101% |
| Class D | 0.01% | -1.50% | -1.38% | 100% | 100% |
| Class F | 0.12% | 0.76% | 0.17% | 100% | 100% |
| Class SCC | 0.04% | 1.16% | 0.88% | 100% | 100% |
| Class HDR H2 | 0.01% | -0.92% | 0.04% | 100% | 100% |

# Complexity analysis

In VVC CCALF process, in order to remove multiplication, filter coefficients are restricted to having only values in the form of power of 2, from the following set: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}. Therefore, as compared to VVC’s 8-tap filter, the proposed 9 tap filter requires additionally 1 shifting and 1 addition operation and the proposed 13 tap filter requires additionally 5 shifting and 5 addition operation. The complexity of the proposed 8 tap filter is same as VVC 8 tap filter.

# Conclusion

In this contribution, three filters are tested to improve the compression performance of chroma sample location type-2 content. It is reported that, the proposed 9-tap filter can achieve 0.07%(Y), -1.99%(U), -1.95%(V) BD-rate (based on PSNR metric) of HDR H1 sequences in RA conditions.  The proposed 13 tap filter can achieve 0.04%(Y), -2.85%(U), -3.22%(V) BD-rate (based on PSNR metric) and the proposed 8-tap filter can achieve 0.05%(Y), -1.43%(U), -1.52%(V) BD-rate (based on PSNR metric) of HDR H1 sequences in RA conditions.

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Jan. 2020.

A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “ JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-P2011

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).