| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0311_r1 |
| --- | --- |

| Title: | [AHG2] Fix cu_skip_flag signaling for IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Seunghwan Kim Jaehyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In VVC draft 8, when all of the below conditions are satisfied, cu_skip_flag is signalled to indicate whether IBC prediction mode with skip or not although IBC is not allowed in this case.

- ModeType is not equal to MODE_TYPE_INTRA

- CU size is larger than 64x64

- sps_ibc_enabled_flag is equal to 1

It is asserted that the problem is caused by the fact that modeType is not defined as MODE_TYPE_INTRA for a CU which is in I_SLICE despite disallowing inter prediction mode for CU in I-Slice. It is reported at #440 and several different solutions were suggested. But it has been not solved yet. Therefore this contribution suggests the three solutions to solve this problem.

- Update modeType as MODE_TYPE_INTRA for a CU which is in I-SLICE

- Update modeType as MODE_TYPE_INTRA for a CU which is larger than 64x64 and is in I-SLICE

- Add I_SLICE condition for checking modeType.

Problem statement

Currently, CU skip flag can be signalled as below:

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && ( ( !( cbWidth = = 4 && cbHeight = = 4 ) && modeType != MODE_TYPE_INTRA ) ¦ ¦ ( sps_ibc_enabled_flag && cbWidth <= 64 && cbHeight <= 64 ) ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |

The above syntax table part has mistmatch between SPEC and SW by signaling cu_skip_flag if the below conditions are all true.

- sps_ibc_enabled_flag == 1

- treeType == SINGLE_TREE

- slice_type == I_SLICE

- CU size is larger than 64x64

When a CU is larger than 64x64, IBC is not allowed and inter prediction mode is also not allowed for the CU which is in I_SLICE. Therefore, cu_skip_flag signalling is not needed. This problem is caused by defining the modeType as MODE_TYPE_ALL for this corner case despite fact that the CU is in I_SLICE do not allow inter prediction.

Proposal

Update modeType to MODE_TYPE_INTRA for a CU which is in I_SLICE

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| modeType = slice_type != I ? modeType : MODE_TYPE_INTRA |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && ( ( !( cbWidth = = 4 && cbHeight = = 4 ) && modeType != MODE_TYPE_INTRA ) ¦ ¦ ( sps_ibc_enabled_flag && cbWidth <= 64 && cbHeight <= 64 ) ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |

Update modeType to MODE_TYPE_INTRA for a CU which is larger than 64x64 and is in I_SLICE

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| modeType = (slice_type != I && max(cbWidth, cbHeight) <= 64 ? modeType : MODE_TYPE_INTRA |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && ( ( !( cbWidth = = 4 && cbHeight = = 4 ) && modeType != MODE_TYPE_INTRA ) ¦ ¦ ( sps_ibc_enabled_flag && cbWidth <= 64 && cbHeight <= 64 ) ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |

Check slice type

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && ( ( !( cbWidth = = 4 && cbHeight = = 4 ) && modeType != MODE_TYPE_INTRA && slice_type != I ) ¦ ¦ ( sps_ibc_enabled_flag && cbWidth <= 64 && cbHeight <= 64 ) ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)