| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 14–25 April 2020 | Document: JVET-R0313 |
| --- | --- |

| Title: | AHG2/AHG16: Cleanups of chroma ALF and CC-ALF on/off control |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Li Zhang2, Hongbin Liu1, Kai Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com lizhang.idm@bytedance.com liuhongbin.01@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In current VVC, chroma ALF and CC-ALF are disabled implicitly when luma ALF is disabled at SPS/PH/SH as it is unlikely that chroma ALF/CC-ALF would be used when luma ALF is disabled. Such a design could benefit power consumption. However, luma ALF, chroma ALF and CC-ALF are controlled independently at CTU level. Therefore, for a slice, it is still possible that chroma ALF/CC-ALF is enabled in some CTUs, even luma ALF is disabled for all CTUs, which conflicts with the original intention. In this contribution, it is proposed to disable chroma ALF/CC-ALF implicitly when luma ALF is disabled for a CTU to keep the design consist for all video processing units. Simulation results reportedly show that BD-rate changes are {0.00%, 0.02%, 0.04%}, {-0.01%, 0.18%, 0.14%}, and {-0.09%, 0.56%, 0.04%} with AI, RA, and LDB configurations under CTC, respectively.

# Introduction

In current VVC [1], chroma ALF and CC-ALF are disabled implicitly when luma ALF is disabled at SPS/PH/SH. However, such a restriction is not applied at CTU level. When luma ALF is disabled for a CTU, the chroma ALF and CC-ALF may still be applied for the CTU. Such a design is conflicting with the intention of controlling chroma ALF/CC-ALF based on luma ALF at higher levels.

# Proposed method

In this contribution, it is proposed that chroma ALF/CC-ALF on/off controlling flags are not signaled and inferred to be false when luma ALF is disabled for a CTU. In other words, chroma ALF/CC-ALF are disabled when luma ALF is disabled in a CTU.

# Simulation results

The proposed method has been implemented on top of VTM-8.0 [2]. Simulations are performed following the common test conditions [3]. The results for CTC are shown in Table 1. More detailed results could be found in the attached excel sheets.

Table 1: Coding performance

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.08% | 98% | 97% |
| Class A2 | 0.00% | 0.05% | 0.06% | 94% | 96% |
| Class B | 0.00% | 0.03% | 0.06% | 96% | 97% |
| Class C | 0.00% | 0.01% | 0.01% | 99% | 101% |
| Class E | 0.00% | 0.00% | 0.01% | 99% | 100% |
| Overall | 0.00% | 0.02% | 0.04% | 97% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Class F (optional) | 0.00% | 0.14% | 0.15% | 97% | 95% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.10% | 103% | 103% |
| Class A2 | -0.02% | 0.18% | 0.36% | 106% | 105% |
| Class B | -0.01% | 0.26% | 0.05% | 97% | 97% |
| Class C | 0.00% | 0.18% | 0.12% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.18% | 0.14% | 101% | 101% |
| Class D | -0.02% | 0.11% | 0.00% | 98% | 98% |
| Class F (optional) | 0.01% | 0.37% | 0.34% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | 0.71% | 0.02% | 100% | 99% |
| Class C | -0.09% | -0.03% | 0.12% | 101% | 101% |
| Class E | -0.17% | 1.11% | -0.03% | 103% | 99% |
| Overall | -0.09% | 0.56% | 0.04% | 101% | 100% |
| Class D | -0.06% | -0.33% | -0.01% | 102% | 102% |
| Class F (optional) | 0.00% | 0.69% | -0.24% | 101% | 98% |

# Conclusions

It is proposed to disable chroma ALF and CC-ALF when luma ALF is disabled in a CTU. It is recommended adopting the proposed method into the next version of VVC.

# Proposed changes to working draft

The suggested specification changes on top of JVET-Q2001-vE [1] is described as follows. Deleted text is marked with red strikethrough, and newly added text is highlighted in green.

7.3.10.2 Coding tree unit syntax

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = CtbAddrX << CtbLog2SizeY |  |
| yCtb = CtbAddrY << CtbLog2SizeY |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( CtbAddrX, CtbAddrY ) |  |
| if( slice_alf_enabled_flag ){ |  |
| alf_ctb_flag[ 0 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( alf_ctb_flag[ 0 ][ CtbAddrX ][ CtbAddrY ] ) { |  |
| if( slice_num_alf_aps_ids_luma > 0 ) |  |
| alf_use_aps_flag | ae(v) |
| if( alf_use_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 1 ) |  |
| alf_luma_prev_filter_idx | ae(v) |
| } else |  |
| alf_luma_fixed_filter_idx | ae(v) |
| } |  |
| if( slice_alf_chroma_idc = = 1 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 1 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( alf_ctb_flag[ 1 ][ CtbAddrX ][ CtbAddrY ] && alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 0 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| } |  |
| if( slice_alf_chroma_idc = = 2 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 2 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( alf_ctb_flag[ 2 ][ CtbAddrX ][ CtbAddrY ] && alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 1 ][ CtbAddrX ][ CtbAddrY ] | ae(v) |
| } |  |
| } |  |
| if( slice_cc_alf_cb_enabled_flag ) |  |
| alf_ctb_cc_cb_idc[ CtbAddrX ][ CtbAddrY ] | ae(v) |
| if( slice_cc_alf_cr_enabled_flag ) |  |
| alf_ctb_cc_cr_idc[ CtbAddrX ][ CtbAddrY ] | ae(v) |
| } |  |
| } |  |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag ) |  |
| dual_tree_implicit_qt_split( xCtb, yCtb, CtbSizeY, 0 ) |  |
| else |  |
| coding_tree( xCtb, yCtb, CtbSizeY, CtbSizeY, 1, 1, 0, 0, 0, 0, 0, SINGLE_TREE, MODE_TYPE_ALL ) |  |
| } |  |

# Reference

- B. Bross, J. Chen, S. Liu, and Y. Wang, “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team, JVET-Q2001-vE.

- VTM-8.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-8.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).