| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0316-v3 |
| --- | --- |

| Title: | AHG12: Normative constraints on BT and TT split under MER |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Kai Zhang2, Li Zhang2, Hongbin Liu1, Zhipin Deng1 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com liuhongbin.01@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

Merge estimation region (MER) was adopted for parallel encoding at the last JVET meeting. When MER is applied with a region smaller than a CTU, some non-normative constraints on BT and TT splits are applied. However, such non-normative constraints bring redundancy since the indications of BT or TT split still need to be signalled even when the constraints are satisfied. This contribution proposes to change the non-normative constraints on BT and TT splits under MER to be normative. It is asserted that the redundancy of the signalling when MER is used can be removed. Compared to VTM-8.0 with MER enabled, experimental results reportedly show luma BD-rate changes of -0.26%/-0.39%, -0.39%/-0.54%, and -0.15%/-0.17% with RA/LDB configurations on average for MER size equal to 88, 1616, and 3232, respectively.

In v3, supplementary results are provided.

Introduction

In the current VVC [1], merge estimation region (MER) can be used for parallel encoding. Independent derivation of the merge candidate list for each coding unit in the same MER is allowed. A neighbouring block that is in the same MER as the current coding unit cannot be used to construct the merge candidate list of the current coding unit. At encoder side, non-normative constraints on BT and TT splits are applied to support MER as below:

- If cbWidth > R and cbHeight <= R, then horizontal BT split is disabled for the current block.

- If cbWidth <= R and cbHeight > R, then vertical BT split is disabled for the current block.

- If (cbWidth > R || cbHeight > R) and cbHeight <= 2 * R, then horizontal TT split is disabled for the current block.

- If cbWidth <= 2 * R and (cbWidth > R || cbHeight > R), then vertical TT split is disabled for the current block.

wherein R is equal to 1 << Log2ParMrgLevel, representing the size of MER.

The non-normative constraints bring redundancy, since the flags indicating BT or TT split which is always equal to false required by the constraints still need to be signalled.

Proposed method

In this contribution, it is proposed to change the non-normative constrains on BT and TT splits to be normative. It is asserted that the redundancy of the signalling when MER is used can be removed. And only 4 lines texts need to be added to the current VVC text.

Simulation results

The proposed method has been implemented on top of VTM-8.0 [2]. Simulations are performed following the common test conditions [3] with MER enabled, i.e., “--Log2ParMrgLevel=3/4/5”. More detailed results could be found in the attached excel sheets.

The test results for different MER sizes are shown in Table 1, Table 2, and Table 3,respectively.

Table 1: Coding performance for MER size 88

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.10% | -0.17% | 99% | 99% |
| Class A2 | -0.27% | -0.25% | -0.28% | 97% | 97% |
| Class B | -0.26% | -0.27% | -0.19% | 102% | 102% |
| Class C | -0.36% | -0.48% | -0.50% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.26% | -0.29% | -0.29% | 100% | 100% |
| Class D | -0.39% | -0.32% | -0.41% | 101% | 100% |
| Class F (optional) | -0.23% | -0.18% | -0.19% | 101% | 100% |
| Class SCC 1080p | -0.67% | -0.63% | -0.61% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.33% | -0.20% | -0.50% | 100% | 98% |
| Class C | -0.46% | -0.32% | -0.38% | 100% | 97% |
| Class E | -0.40% | -1.25% | -0.98% | 101% | 102% |
| Overall | -0.39% | -0.50% | -0.58% | 100% | 99% |
| Class D | -0.49% | -0.44% | -0.44% | 100% | 99% |
| Class F (optional) | -0.41% | -0.36% | -0.26% | 102% | 102% |
| Class SCC 1080p | -0.88% | -0.94% | -0.95% | 101% | 101% |

Table 2: Coding performance for MER size 1616

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.37% | -0.33% | -0.44% | 104% | 102% |
| Class A2 | -0.42% | -0.29% | -0.36% | 103% | 99% |
| Class B | -0.39% | -0.36% | -0.43% | 101% | 98% |
| Class C | -0.37% | -0.35% | -0.36% | 102% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.39% | -0.34% | -0.40% | 102% | 99% |
| Class D | -0.34% | -0.23% | -0.24% | 101% | 98% |
| Class F (optional) | -0.26% | -0.25% | -0.22% | 102% | 101% |
| Class SCC 1080p | -0.33% | -0.33% | -0.29% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.39% | -0.50% | -0.58% | 100% | 99% |
| Class C | -0.46% | -0.35% | -0.54% | 102% | 97% |
| Class E | -0.69% | -0.98% | -0.49% | 101% | 96% |
| Overall | -0.54% | -0.62% | -0.42% | 102% | 98% |
| Class D | -0.38% | -0.52% | -0.88% | 101% | 99% |
| Class F (optional) | -0.30% | -0.65% | -1.24% | 102% | 101% |
| Class SCC 1080p | -0.53% | -0.55% | -0.47% | 99% | 97% |

Table 3: Coding performance for MER size 3232

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.30% | -0.27% | 104% | 100% |
| Class A2 | -0.19% | -0.19% | -0.21% | 103% | 101% |
| Class B | -0.12% | -0.03% | -0.09% | 102% | 100% |
| Class C | -0.05% | -0.02% | -0.18% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.15% | -0.11% | -0.17% | 102% | 100% |
| Class D | -0.05% | -0.08% | -0.10% | 101% | 99% |
| Class F (optional) | -0.08% | -0.11% | 0.00% | 100% | 99% |
| Class SCC 1080p | -0.09% | -0.08% | -0.08% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.19% | -0.36% | -0.19% | 102% | 100% |
| Class C | -0.04% | -0.16% | -0.14% | 100% | 98% |
| Class E | -0.32% | -0.43% | -0.37% | 102% | 102% |
| Overall | -0.17% | -0.31% | -0.22% | 101% | 100% |
| Class D | 0.01% | -0.16% | -0.31% | 101% | 103% |
| Class F (optional) | -0.20% | -0.54% | -0.61% | 100% | 99% |
| Class SCC 1080p | -0.17% | -0.20% | -0.23% | 101% | 100% |

In addition, compared to VTM-8.0, when MER is used, the average coding loss reduction of the proposed method under CTC QP and high QP are shown in Table 4 and Table 5.When MER is enabled, the average coding performance of VVC and the proposed method compared to HEVC are shown in Table 6.

Table 4: Coding loss reduction for CTC QP

| High QP {27, 32, 37, 42} | MER size | Configuration | VTM-8.0 MER on vs VTM-8.0 | Proposed vs VTM-8.0 | Loss reduction |
| --- | --- | --- | --- | --- | --- |
|  | 8×8 | RA | 0.76% | 0.50% | 34% |
|  |  | LDB | 0.96% | 0.53% | 45% |
|  | 16×16 | RA | 2.35% | 1.87% | 20% |
|  |  | LDB | 2.89% | 2.24% | 22% |
|  | 32×32 | RA | 2.58% | 2.33% | 10% |
|  |  | LDB | 3.08% | 2.79% | 9% |

Table 5: Coding loss reduction for high QP

| High QP {27, 32, 37, 42} | MER size | Configuration | VTM-8.0 MER on vs VTM-8.0 | Proposed vs VTM-8.0 | Loss reduction |
| --- | --- | --- | --- | --- | --- |
|  | 8×8 | RA | 0.76% | 0.50% | 34% |
|  |  | LDB | 0.96% | 0.53% | 45% |
|  | 16×16 | RA | 2.35% | 1.87% | 20% |
|  |  | LDB | 2.89% | 2.24% | 22% |
|  | 32×32 | RA | 2.58% | 2.33% | 10% |
|  |  | LDB | 3.08% | 2.79% | 9% |

Table 6: Coding performance of VVC with MER compared to HEVC

| Configuration | MER | VTM-8.0 vs HM-16.20 | Proposed vs HM-16.20 |
| --- | --- | --- | --- |
| RA | Disable | -34.84% | -34.84% |
|  | 8×8 | -34.17% | -34.34% |
|  | 16×16 | -33.45% | -33.72% |
|  | 32×32 | -34.17% | -34.27% |
| LDB | Disable | -30.02% | -30.02% |
|  | 8×8 | -29.18% | -29.45% |
|  | 16×16 | -28.19% | -28.58% |
|  | 32×32 | -28.94% | -29.06% |

Conclusions

It is proposed to apply normative constrains on BT and TT splits for MER. It is recommended adopting the proposed method to the next version of VVC.

Proposed changes to working draft

The suggested specification changes on top of JVET-Q2001-vE [1] is described as follows. All the newly added texts are highlighted in green.

6.4.2 Allowed binary split process

…

The variable allowBtSplit is derived as follows:

- If one or more of the following conditions are true, allowBtSplit is set equal to FALSE:

- cbSize is less than or equal to MinBtSizeY

- cbWidth is greater than maxBtSize

- cbHeight is greater than maxBtSize

- mttDepth is greater than or equal to maxMttDepth

- treeType is equal to DUAL_TREE_CHROMA and ( cbWidth / SubWidthC ) * ( cbHeight / SubHeightC ) is less than or equal to 16

- treeType is equal to DUAL_TREE_CHROMA and ( cbWidth / SubWidthC ) is equal to 4 and btSplit is equal to SPLIT_BT_VER

- treeType is equal to DUAL_TREE_CHROMA and modeType is equal to MODE_TYPE_INTRA

- cbWidth * cbHeight is equal to 32 and modeType is equal to MODE_TYPE_INTER

- btSplit is equal to SPLIT_BT_VER and cbWidth is less than or equal to (1 << Log2ParMrgLevel) and cbHeight is greater than (1 << Log2ParMrgLevel) and slice_type is not equal to I and x0 + cbWidth is less than or equal to pic_width_in_luma_samples

- btSplit is equal to SPLIT_BT_HOR and cbHeight is less than or equal to (1 << Log2ParMrgLevel) and cbWidth is greater than (1 << Log2ParMrgLevel) and slice_type is not equal to I and y0 + cbHeight is less than or equal to pic_height_in_luma_samples

- Otherwise, if all of the following conditions are true, allowBtSplit is set equal to FALSE

- btSplit is equal to SPLIT_BT_VER

- y0 + cbHeight is greater than pic_height_in_luma_samples

…

6.4.3 Allowed ternary split process

…

The variable allowTtSplit is derived as follows:

- If one or more of the following conditions are true, allowTtSplit is set equal to FALSE:

- cbSize is less than or equal to 2 * MinTtSizeY

- cbWidth is greater than Min( 64, maxTtSize )

- cbHeight is greater than Min( 64, maxTtSize )

- mttDepth is greater than or equal to maxMttDepth

- x0 + cbWidth is greater than pic_width_in_luma_samples

- y0 + cbHeight is greater than pic_height_in_luma_samples

- treeType is equal to DUAL_TREE_CHROMA and ( cbWidth / SubWidthC ) * ( cbHeight / SubHeightC ) is less than or equal to 32

- treeType is equal to DUAL_TREE_CHROMA and ( cbWidth / SubWidthC ) is equal to 8 and ttSplit is equal to SPLIT_TT_VER

- treeType is equal to DUAL_TREE_CHROMA and modeType is equal to MODE_TYPE_INTRA

- cbWidth * cbHeight is equal to 64 and modeType is equal to MODE_TYPE_INTER

- ttSplit is equal to SPLIT_TT_HOR, and cbWidth is larger than (1 << Log2ParMrgLevel) or cbHeight is larger than (1 << Log2ParMrgLevel), and cbHeight is less than or equal to 2 * (1 << Log2ParMrgLevel), and slice_type is not equal to I

- ttSplit is equal to SPLIT_TT_VER, and cbWidth is larger than (1 << Log2ParMrgLevel) or cbHeight is larger than (1 << Log2ParMrgLevel), and cbWidth is less than or equal to 2 * (1 << Log2ParMrgLevel), and slice_type is not equal to I

- Otherwise, allowTtSplit is set equal to TRUE.

…

Reference

- B. Bross, J. Chen, S. Liu, and Y. Wang, “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team, JVET-Q2001-vE.

- VTM-8.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-8.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).