| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0322-v3 |
| --- | --- |

| Title: | CCALF virtual boundary issue for 4:4:4 and 4:2:2 format |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng1, Xiaozhen Zheng2, Shanshe Wang1, Siwei Ma1 Peking University1, SZ DJI Technology Co., Ltd.2 | Tel: Email: | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd. |  |  |

_____________________________

- Abstract

In this contribution, the CCALF virtual boundary issue for 4:4:4 and 4:2:2 format is addressed. Specifically, the current processing chroma sample and its collocated luma sample may belong to different Coding Virtual Blocks (CVB, the block between two neighboring horizontal virtual boundaries) for 4:2:2 and 4:4:4 chroma format, which means that these chroma samples have to be stored until deblocking and SAO of collocated luma samples are done. This will cause extra hardware overhead.

In this proposal, a bug in CCALF covariance calculation process in VTM-8.0 is reported and fixed, firstly. Then, three methods are proposed to solve the virtual boundary issue.

Aspect 1: Bug fix of VTM-8.0 (YUV422 natural sequances)

AI: 0.00%	0.00%	0.00%; RA: 0.00%	-0.09%	-0.01%; LDB: 0.02%	-0.01%	0.07%

Aspect 2: Solution1 (Disable CCALF when a chroma sample and its collocated luma sample belong to different CVBs.)

YUV444: AI: 0.00%	0.01% 0.07%;  RA: 0.04% -0.04% 0.18%; LDB 0.03%	-0.09%	-0.01%

YUV422: AI: 0.00%	0.04% 0.09%;  RA: 0.00% 0.06% 0.07%;  LDB: 0.01% 0.07% 0.12%

Aspect 2: Solution 2 (For 4:2:2 and 4:4:4 chroma format, a chroma sample and its reference luma sample are forced to belong to the same CVB)

YUV444: AI: 0.00%	0.01%	0.05%; RA: 0.05%	-0.01%	0.02%; LDB 0.02%	-0.12%	-0.06%

YUV422: AI: 0.00%	0.05%	0.08%; RA: 0.00%	0.04%	0.13%;

Aspect 2: Solution3 (For 4:2:2 and 4:4:4 chroma format, the location of chroma VB is changed from 2 lines to 4 lines above a CTU boundary)

YUV444: AI: 0.00%	-0.02%	-0.04%; RA: 0.01%	-0.07%	0.03%; LDB 0.04%	-0.04%	-0.16%

YUV422: AI: 0.00%	0.00%	-0.02%; RA: 0.01%	-0.11%	-0.08%;LDB 0.00%	-0.15%	0.01%

Introduction

In the current VVC-8.0 [1], to reduce the line buffer requirement, Virtual Boundary (VB) is used in the current ALF and the newly adopted Cross Component ALF(CCALF[1]). To adapt to the line buffer requirement of deblocking and SAO, the VB position of ALF was set to be 4 lines (3 lines for deblocking and 1 line for SAO) and 2 lines (1 line for deblocking and 1 line for SAO) above a horizontal CTU boundary for luma and chroma component respectively, as shown in Fig. 1. When ALF or CC-ALF is applied to a sample above a VB, the samples below the VB cannot be utilized and vice versa. However, this rule is broken because of the adoption of CCALF.

- Luma                                    (b) Chroma

Fig. 1. Virtual boundary of ALF and CCALF

CCALF, aimed at improving chroma fidelity by exploiting correlations between the luma and chroma channels, was adopted since JVET 17th meeting. Specifically, CCALF applies a linear filter to the luma channel to determine a chroma residual that is added to a previously reconstructed chroma signal. As shown in Fig. 2, 4x3 filter shape is used for CCALF luma component, where C0~C7 represents the filter coefficients. The relative location of chroma being filtered and reference luma samples is shown in Fig. 3.

Fig. 2. Filter shape of CCALF

Fig. 3. Relative location of chroma sample being filtered and

its reference luma samples for 4:2:0 format

For 4:2:0 chroma format, the current processing chroma sample and its collocated luma sample always belong to the same Coding Virtual Block (CVB, the block between two neighboring horizontal virtual boundaries), as shown in Fig. 4. (Truncated filter for luma samples belonging to the lines close to the virtual boundaries [3] is not shown in Fig. 4 ~ Fig. 8 ).

For 4:2:2 and 4:4:4 chroma format, the current processing chroma sample and its collocated luma sample may belong to different CVBs, as shown in Fig. 5 and Fig. 6. Specifically, chroma samples located on the chroma VB upper two rows have luma collocated samples belong to the different CVBs, which means that these chroma samples have to be stored until deblocking and SAO of collocated luma samples are done. This will cause extra hardware overhead.

Fig. 4. CCALF for 4:2:0 format

Fig. 5. CCALF for 4:2:2 format

Fig. 6. CCALF for 4:4:4 format

Proposed Method

In this contribution, a bug in CCALF of VTM-8.0 is reported and fixed, firstly. Then, three methods are proposed to solve the problem mentioned above.

Aspect 1: bug fix of VTM-8.0

In function “getBlkStatsCcAlf” of VTM-8.0 software, the calculation of “vbDistance” is wrong. As “i” is the row number, the result of “getComponentScaleX” is corresponding to col number. For 4:2:2 sequences, the line 61,62 and 126 will be considered as the VB neighboring lines, and line 122, 123 and 124 are ignored in the current VTM-8.0

We propose to change the “getComponentScaleX” to “getComponentScaleY” for the calculation of “vbDistance”. As the results of “getComponentScaleX” and “getComponentScaleY” are the same for 4:2:0 and 4:4:4 chroma format, the proposed bug fix only affects the performance of 4:2:2 sequences.

int ELocal[MAX_NUM_CC_ALF_CHROMA_COEFF][1];

for (int i = 0; i < compArea.height; i++)

{

int vbDistance = ((i << getComponentScaleX(compID, m_chromaFormat)) % vbCTUHeight) - vbPos;

…

}

Aspect 2: solutions of CCALF line buffer issue

Solution 1

Disable CCALF when a chroma sample and its collocated luma sample belong to different CVBs.

Solution 2

For 4:2:2 and 4:4:4 chroma format, a chroma sample and its reference luma sample are forced to belong to the same CVB. Specifically, if a chroma sample and its reference luma sample belong to different CVBs, the location of reference luma sample is forced to be its nearest luma sample in the chroma sample located CVB, which is shown in Fig. 7.

Fig. 7. Solution 2

Solution 3

For 4:2:2 and 4:4:4 chroma format, the location of chroma VB is changed from 2 lines to 4 lines above a CTU boundary, as shown in Fig. 8.

Fig. 8. Proposed chroma virtual boundary location for 4:2:2 chroma format in solution 3

Simulation Results

The experiments were performed based on VTM-8.0 with VTM configuration files under test condition for 4:2:2 and 4:4:4 natural sequences [4]. And, the experiments were conducted under Ubuntu Linux. The encoding time is unreliable.

Aspect 1: bug fix of VTM-8.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | 0.00% | 0.00% | -0.01% | 106% | 173% |
| Kimono | 0.00% | 0.00% | 0.00% | 91% | 95% |
| EBUHorse | 0.00% | 0.00% | 0.00% | 174% | 159% |
| EBUGraphics | 0.00% | 0.00% | 0.00% | 174% | 165% |
| EBUWaterRocksClose | 0.00% | 0.00% | 0.00% | 148% | 165% |
| EBUKidsSoccer | 0.00% | -0.01% | 0.00% | 179% | 141% |
| Seeking | 0.00% | -0.01% | 0.02% | 99% | 93% |
| Overall | 0.00% | 0.00% | 0.00% | #NUM! | #NUM! |
|  | Random access Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | 0.02% | -0.03% | 0.11% | 100% | 96% |
| Kimono | -0.01% | -0.14% | -0.05% | 102% | 96% |
| EBUHorse | 0.01% | 0.04% | 0.07% | 100% | 63% |
| EBUGraphics | 0.01% | -0.01% | 0.00% | 98% | 95% |
| EBUWaterRocksClose | -0.01% | -0.02% | -0.04% | 99% | 99% |
| EBUKidsSoccer | -0.01% | -0.37% | -0.10% | 187% | 150% |
| Seeking | -0.01% | -0.10% | -0.07% | 100% | 92% |
| Overall | 0.00% | -0.09% | -0.01% | 112% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic |  | -0.07% | 0.19% |  | 0.12% |  | 103% |  | 101% |
| Kimono |  | -0.01% | -0.19% |  | -0.02% |  | 100% |  | 92% |
| EBUHorse |  | 0.19% | 0.30% |  | 0.38% |  | 60% |  | 98% |
| EBUGraphics |  | -0.01% | -0.03% |  | 0.00% |  | 101% |  | 95% |
| EBUWaterRocksClose |  | 0.00% | -0.08% |  | -0.01% |  | 101% |  | 93% |
| EBUKidsSoccer |  | 0.05% | -0.24% |  | 0.15% |  | 101% |  | 117% |
| Seeking |  | -0.01% | -0.01% |  | -0.10% |  | 98% |  | 94% |
| Overall |  | 0.02% | -0.01% |  | 0.07% |  | 97% |  | 94% |

Aspect 2: solutions of CCALF line buffer issue

Solution 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | 0.00% | 0.01% | 0.07% | 88% | 85% |
| YUV 4:2:2 | 0.00% | 0.04% | 0.09% | 92% | 96% |
|  | Random access Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | 0.04% | -0.04% | 0.18% | 94% | 89% |
| YUV 4:2:2 | 0.00% | 0.06% | 0.07% | 103% | 91% |
|  | Low delay B Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 |  | 0.03% | -0.09% |  | -0.01% |  | 100% |  | 84% |
| YUV 4:2:2 |  | 0.01% | 0.07% |  | 0.12% |  | 96% |  | 94% |

Solution 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | 0.00% | 0.01% | 0.05% | 87% | 83% |
| YUV 4:2:2 | 0.00% | 0.05% | 0.08% | 93% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | 0.05% | -0.01% | 0.02% | 99% | 90% |
| YUV 4:2:2 | 0.00% | 0.04% | 0.13% | 99% | 87% |
|  | Low delay B Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 |  | 0.02% |  | -0.12% | -0.06% |  | 103% |  | 84% |  |
| YUV 4:2:2 |  | #VALUE! |  | #VALUE! | #VALUE! |  | #NUM! |  | #NUM! |  |

Solution 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | 0.00% | -0.02% | -0.04% | 87% | 85% |
| YUV 4:2:2 | 0.00% | 0.00% | -0.02% | 93% | 94% |
|  | Random access Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | 0.01% | -0.07% | 0.03% | 92% | 88% |
| YUV 4:2:2 | 0.01% | -0.11% | -0.08% | 99% | 87% |
|  | Low delay B Main10 |  |  |  |  |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 |  | 0.04% | -0.04% |  | -0.16% |  | 101% |  | 83% |
| YUV 4:2:2 |  | 0.00% | -0.15% |  | 0.01% |  | 97% |  | 94% |

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q2001, Jan. 2020

- K. Misra, F. Bossen, A. Segall, et.al, “CC-ALF: Integrated Text for the Cross Component Adaptive Loop Filter,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q0795, Jan. 2020

- A. M. Kotra, S. Esenlik, J. Chen, B. Wang, H. Gao, “CE5-2: Loop filter line buffer reduction,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N0180, Mar. 2019

- Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q2013, Jan. 2020

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).