| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: By teleconference, 15–24 April 2020 | Document: JVET-R0326-v1 |
| --- | --- |

| Title: | AHG15: On Chroma Quantization Matrix Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Zhang, Xiang Li, Guichun Li, Ling Li, Shan Liu | Tel: Emal: | hobingzhang@tencent.com |
| Source: | Tencent |  |  |

_____________________________

Abstract

This contribution proposes two fixes for the issue reported in ticket #926 that prediction may be from chroma quantization matrix which is not signaled.

- Problem statements

An issue in VVC draft 8 (JVET-Q2001 [1]) was reported in the ticket #926 that current syntax allows a quantization matrix (QM) to be predicted from a chroma quantization matrix which is not signaled. To fix the issue, two methods are proposed in this contribution.

- Proposed methods

Two simple methods are proposed as below.

- Method #1 Values in non-signaled chroma QM is set to 16.

- Method #2 It is proposed that the value of scaling_list_copy_mode_flag[id] is inferred to be equal to 1 when it is not present. Then chroma QM which is not signaled can be implicitly inferred as the default value of 16. scaling_list_pred_id_delta, scaling_list_dc_coef and scaling_list_delta_coef are not signaled for non-signaled chroma QMs. According to JVET-Q2001, they are inferred to be equal to zeros. Furthermore, all the elements of its prediction QMs are derived as 16 when scaling_list_pred_id_delta is zero, and their residual elements are derived as zero when scaling_list_dc_coef and scaling_list_delta_coef are zeros. Consequently, all elements of non-signaled chroma QM are derived as 16.

With one of proposed constraint, the values of chroma QMs can be set properly. Consequently, luma QMs can be decoded successfully even when referencing a non-signaled chroma QM. Furthermore, both methods have minor changes on the current spec by only adding 1 sentence.

The spec texts of the two constrain are included in section 4.

- Conclusions

Two methods are proposed to fix the issue reported in ticket #926 by setting default values for chroma QMs which are not signaled so that they can be further referenced by luma QMs. It is proposed to adopt one of the proposed methods.

- Spec text

The following proposed text modifications are based on JVET-Q2001-vE. The added texts are highlighted in yellow.

  - Spec text of Method #1

scaling_list_chroma_present_flag equal to 1 specifies that chroma scaling lists are present in scaling_list_data( ). scaling_list_chroma_present_flag equal to 0 specifies that chroma scaling lists are not present in scaling_list_data( ). It is a requirement of bitstream conformance that scaling_list_chroma_present_flag shall be equal to 0 when ChromaArrayType is equal to 0, and shall be equal to 1 when ChromaArrayType is not equal to 0. When chroma scaling list are not present, its ScalingMatrixDCRec and ScalingMatrixRec are inferred as 16.

  - Spec text of  Method#2

scaling_list_copy_mode_flag[ id ] equal to 1 specifies that the values of the scaling list are the same as the values of a reference scaling list. The reference scaling list is specified by scaling_list_pred_id_delta[ id ]. scaling_list_copy_mode_flag[ id ] equal to 0 specifies that scaling_list_pred_mode_flag is present. When not present, the value of scaling_list_copy_mode_flag[id] should be inferred to be equal to 1.

- Reference

- Benjamin Bross, Jianle Chen, Shan Liu and Ye-Kui Wang, “Versatile Video Coding (Draft 8)” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET- Q2001, 17th Meeting: Brussels, BE, 7–17 January 2020.

- Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).