| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0339-v9 |
| --- | --- |

| Title: | Agenda and report of the category 1 AHG pre-meeting of the 18th JVET meeting |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the coordinators of the category 1 AHG pre-meeting |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Ye-Kui Wang Bytedance Inc. 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +1 908 803 3888 yekui.wang@bytedance.com |
| Source: | Coordinators of the category 1 AHG pre-meeting |  |  |

_____________________________

# Summary

The category 1 AHG pre-meeting of the 18th JVET meeting, for addressing high-level syntax (HLS) topics, mostly in the scopes of AHGs 8, 9, and 12, held the following teleconference meeting sessions on each day of April 6 to 8 (dates and time slots are all in UTC time):

- 1300-1500 1st "afternoon" session [break after 2 hours]

- 1515-1715 2nd "afternoon" session

- ["dinner" break – nearly 4 hours – enough for some side work or a nap]

- 2100-2300 1st "night" session [break after 2 hours]

- 2315-0115 2nd "night" session

- [end of day – nearly 12 hours to the next session]

Approximately XXX people attended the AHG pre-meeting, and approximately XXX input documents, including X AHG reports were discussed.

The following recommendations were made to the JVET:

- JVET-Rxxxx:

- ...

# Attendance

The list of participants in the AHG pre-meeting meeting can be found in Annex A of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

# AHG reports (5)

These reports were discussed Tuesday 7 January 2020 during 1010-1215 (chaired by GJS) and Wednesday 8 January during 0940–1300 and 1430–XX (chaired by GJS and JRO), except as otherwise noted.

JVET-Q0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, J. Chen, J. Boyce, S. Kim, S. Liu, Y.-K. Wang, Y. Ye]

This AHG report was discussed Wednesday 8 January 2020 at 1000 (chaired by GJS & JRO).

JVET-Q0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring]

This AHG report was discussed Wednesday 8 January 2020 at 1020 (chaired by GJS & JRO).

JVET-Q0008 JVET AHG report: Layered coding and resolution adaptivity (AHG8) [S. Wenger, A. Segall, M. M. Hannuksela, Hendry, S. McCarthy, Y.-C. Sun, P. Topiwala, M. Zhou]

This AHG report was discussed Tuesday 7 January 2020 at 1010 (chaired by GJS).

JVET-Q0009 JVET AHG report: High-level syntax (AHG9) [R. Sjöberg, J. Boyce, B. Choi, S. Deshpande, M. M. Hannuksela, R. Skupin, A. Tourapis, Y.-K. Wang, W. Wan]

This AHG report was discussed Tuesday 7 January 2020 at 1020 (chaired by GJS).

JVET-Q0012 JVET AHG report: High-level parallelism and coded picture regions (AHG12) [S. Deshpande, B. Choi, M. M. Hannuksela, R. Sjöberg, R. Skupin, W. Wan, Y.-K. Wang]

This AHG report was discussed Tuesday 7 January 2020 at 1110 (chaired by GJS).

# High-level syntax (HLS) proposals (251)

## AHG9: General high-level syntax (172)

### Combinations of subpictures and other features (3)

JVET-R0043 AHG9: On subpicture interaction with other tools [J. Li, K. Abe (Panasonic)]

JVET-R0058 AHG8/AHG9/AHG12: On the combination of RPR, subpictures, and scalability [Y.-K. Wang, L. Zhang, K. Zhang, Z. Deng (Bytedance)]

JVET-R0184 AHG9/AHG12: On reference picture wraparound for subpictures [S. Paluri, Hendry, S. Kim (LGE)]

### High level tool control (58)

#### Chroma deblocking tc and β offsets signalling (13)

JVET-R0338 AHG9: A summary of proposals on chroma deblocking tc and β offsets signalling [Y.-K. Wang (Bytedance)]

Discussed Monday 6 April 1315 (GJS & YKW)

This contribution intends to provide a summary of the 12 proposals on signalling of chroma deblocking tc and β offsets submitted to this JVET meeting by the 3 April 2020 submission deadline.

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

- Skip the signalling of the chroma tc and β deblocking offset syntax elements (SEs) in the PPS when the chroma format is ( 4:0:0 or (4:4:4 and the separate color plane coding mode is in use) ) and/or when the parameter values for chroma are the same as for luma?

  - Yes, (R0077, R0078, R0095, R0106, R0152, R0172, R0206, R0218, R0232), and condition the SEs on

    - A new PPS flag for controlling the presence of chroma deblocking parameters (R0077, R0078, R0081, R0106, R0206)

    - The existing pps_chroma_tool_offsets_present_flag currently for controlling the presence of the QP offsets in the PPS (R0078, R0095, R0106, R0152, R0172, R0206, R0218, R0232) AHG Recommendation (cleanup): Recommended by AHG. (The editor may also consider renaming the flag.) If the flag is zero, the chroma offsets (if needed) are inferred from the luma offsets.

    - ChromaArrayType (R0172)

  - No. (R0048, R0079, R0081, R0232)

    - Impose semantics constraints that values shall be equal to 0 when ChromaArrayType is equal to 0. (R0079, R0081, R0232)

- Skip the signalling of the chroma tc and β deblocking offset syntax elements (SEs) in the PH and the SH when the chroma format is (4:0:0 or (4:4:4 and the separate color plane coding mode is in use) ) and/or when the parameter values for chroma are the same for luma?

  - Yes, (all the 12 contributions), and condition the SEs on

    - ChromaArrayType (R0048, R0078, R0079, R0081, R0095, R0106, R0152, R0172, R0206, R0218, R0232)

    - A new PPS flag for controlling the presence of chroma deblocking parameters (R0077, R0078, R0106)

    - The existing pps_chroma_tool_offsets_present_flag currently for controlling the presence of the QP offsets in the PPS (R0078, R0152, R0232) AHG Recommendation (cleanup): Recommended by AHG. (The editor may also consider renaming the luma beta and tc offset control syntax elements.) If the flag is zero, the chroma offsets (if needed) are inferred from the luma offsets.

    - A new PH flag and a new SH flag (R0081, R0206)

  - No, but impose semantics constraints that values shall be equal to 0 when ChromaArrayType is equal to 0. (R0079, R0232)

JVET-R0048 AHG9: On chroma deblocking parameters [C.-M. Tsai, C.-W. Hsu, S.-T. Hsiang, Y.-W. Huang, S.-M. Lei (MediaTek)]

In this contribution, it is proposed to signal chroma tc and β deblocking offset syntax elements in picture header (PH) and slice header (SH) depending on ChromaArrayType. Note that, the chroma deblocking syntax elements in picture parameter set (PPS) are not changed and still signalled for 4:0:0.

JVET-R0077 AHG9: On chroma deblocking parameters signalling [J. Xu, L. Zhang, Y.-K. Wang, K. Zhang, Z. Deng (Bytedance)]

This contribution proposes the following changes on signalling of chroma deblocking parameters:

- Add a PPS flag pps_chroma_deblocking_params_present_flag, which is required to be equal to 0 when ChromaArrayType is equal to 0.

- Skip the signalling of the Cb/Cr deblocking tc/beta offsets in PPS/PH/SH when pps_chroma_deblocking_params_present_flag is equal to 0.

JVET-R0078 AHG9: On signalling of deblocking parameters for coding monochrome pictures [H.-W. Sun, H.-B. Teo, C.-S. Lim (Panasonic)]

This contribution proposes three alternative solutions to remove the signalling of chroma deblocking offset parameters in PPS, PH and SH when coding in monochrome.

In solution 1, the signalling of the offset parameters in PPS is disabled if pps_chroma_tool_offsets_present_flag is equal to 0 and the signalling of the offset parameters in PH and SH is disabled if ChromaArrayType is equal to 0.

In solution 2, the signalling of the offset parameters in PPS, PH and SH is disabled if pps_chroma_tool_offsets_present_flag is equal to 0.

In solution 3, the signalling of the offset parameters in PPS, PH and SH is disabled if the newly introduced flag pps_deblocking_filter_chroma_offset_present_flag is equal to 0.

JVET-R0079 AHG9: On signalling of chroma deblocking filter parameters for monochrome [T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)]

Option#1:

- Apply semantic constraints that beta_offset_div2 and tc_offset_div2 for chroma deblocking filter in PPS, PH and SH shall be equal to 0 when chromaArrayType is equal to 0.

Option#2:

- Apply semantic constraints that beta_offset_div2 and tc_offset_div2 for chroma deblocking filter in PPS shall be equal to 0 when chromaArrayType is equal to 0.

- Skip signalling of beta_offset_div2 and tc_offset_div2 for chroma deblocking filter in PH/SH when chromaArrayType is equal to 0.

JVET-R0081 AHG9: Chroma deblocking strength signalling [Z. Zhang, M. Pettersson, M. Damghanian, J. Enhorn, K. Andersson, J. Ström, R. Sjöberg (Ericsson)]

This contribution contains two proposals related to deblocking strength control for the Cb and Cr components.

Proposal 1 makes the presence of chroma β and tC offset syntax elements in the picture header (PH) and slice header (SH) conditional such that they are not present when ChromaArrayType is equal to 0. It is also proposed to add bitstream constraints on the chroma β and tC offset syntax elements in the picture parameter set (PPS) to enforce that they are equal to 0 when ChromaArrayType is equal to 0.

Proposal 2 is to add one flag to each of the PPS, PH and SH for controlling the presence of chroma β and tC offset syntax elements. The three new flags are proposed to be called pps_deblocking_chroma_offsets_present_flag, ph_deblocking_chroma_offsets_present_flag and slice_deblocking_chroma_offsets_present_flag, respectively. The proposed behaviour of the flags is as follows:

- When one of the proposed flags is equal to 1, chroma β and tC offset values are signalled as in the current VVC draft.

- When one of the proposed flags is equal to 0, the chroma β and tC offset values are not signalled but inferred to be equal to the values of the luma β and tC offset values signaled in the same syntax table.

- When one of the proposed flags is not present, the chroma β and tC offset values are inferred as in the current VVC draft.

The proponents claim that proposal 2 provides a short-cut for the claimed common case when the chroma β and tC offset values are identical to the luma β and tC offset values.

JVET-R0095 AHG9: Clean-up of chroma deblocking control parameter signalling [M. G. Sarwer, Y. Ye, J. Luo, J. Chen (Alibaba)]

In VVC draft 8, the chroma deblocking control parameters (i.e. beta_offset_div2 and tc_offset_div2) are signalled in case of all chroma formats including monochrome. In order to avoid unnecessary signaling, this contribution proposes not to signal beta_offset_div2 and tc_offset_div2 of Cb and Cr component if there is no chroma component in the sequence. In VVC, chroma deblocking parameter may be signalled in PPS, picture header or slice header.

In the proposed method, the chroma deblocking control parameters in PPS are signalled if pps_chroma_tool_offsets_present_flag is equal to 1. The chroma deblocking control parameters in picture header and slice headers are signaled if ChromaArrayType is not equal to 0.

JVET-R0106 AHG9: On Deblocking Control [S. Deshpande, J. Samuelsson, A. Segall, T. Zhou, T. Ikai (Sharp)]

Item 2 of this contribution belongs to this category.

The item 2 proposes the following:

- Proposal 2: It is proposed to conditionally signal chroma related deblocking filter control parameters only when chroma array type is not equal to 0.

JVET-R0152 AHG9: On signalling of chroma deblocking offsets in monochrome picture [J. Choi, J. Choi, J. Heo, S. Yoo, J. Lim, S. Kim (LGE)]

In this proposal, signalling of chroma deblocking offsets is suggested to be removed for monochrome picture as follows:

- Method 1: if pps_chroma_tool_offsets_present_flag is equal to 0, chroma deblocking offsets are not signaled in PPS, PH, and SH.

- Method 2: if pps_chroma_tool_offsets_present_flag is equal to 0, chroma deblocking offsets are not signaled in PPS. If ChromaArrayType is equal to 0, chroma deblocking offsets are not signaled in PH and SH.

JVET-R0172 AHG9: Removed Redundant Coding of Chroma Deblocking Filter Parameters [K. Naser, F. Le Léannec, T. Poirier (InterDigital)]

In JVET-Q2001, the chroma related HLS elements are coded depending on the variable ChromaArrayType. When it is zero, all chroma tools are deactivated and chroma syntax shall not be coded. However, the chroma deblocking filter parameters are coded at PPS, PH and SH regardless of ChromaArrayType. This leads to redundant coding as well as inconsistency with the rest of the design. This contribution proposes to remove this redundant coding by conditioning the presence of these parameters on ChromaArrayType.

JVET-R0206 AHG9: Modified signalling of Chroma deblocking control parameters [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei)]

This contribution proposes two aspects to reportedly improve the signalling of Chroma deblocking control parameters. The first aspect removes the signalling of Chroma deblocking control parameters for Monochrome sequences. Therefore in the PPS, the Chroma deblocking parameters, namely the syntax elements pps_cb_beta_offset_div2, pps_cb_tc_offset_div2, pps_cr_beta_offset_div2 and pps_cr_tc_offset_div2 are conditionally signalled based on the syntax element pps_chroma_tool_offsets_present_flag. In the picture header (PH) and slice header (SH), the syntax elements ph_cb_beta_offset_div2, ph_cb_tc_offset_div2, ph_cr_beta_offset_div2, ph_cr_tc_offset_div2, slice_cb_beta_offset_div2, slice_cb_tc_offset_div2, slice_cr_beta_offset_div2 and slice_cr_tc_offset_div2 are conditionally signalled based on the ChromaArrayType.

The second aspects reportedly reduces the parsing overhead of Chroma deblocking control parameters by introducing new flags namely pps_chroma_offsets_same_as_luma, ph_chroma_offsets_same_as_luma, slice_chroma_offsets_same_as_luma in the PPS, PH and SH respectively. When these syntax elements are true, the Chroma Cb, Cr Beta and Tc offsets are not signalled anymore and they share the same values as the Luma Beta and Tc offsets.

JVET-R0218 AHG9: Decoding conditions of deblocking control parameters for chroma [K. Unno, K. Kawamura, S. Naito (KDDI)]

In this contribution, it is proposed that the deblocking control parameters are decoded only if chroma data exists in the bitstream (i.e. in the case of ChromaArrayType != 0).

JVET-R0232 AHG9: APS, LMCS, deblocking and PPS constraints [N. Hu, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

The deblocking aspect of this contribution belongs this this category.

The deblocking aspect of this contribution belongs this this category.

This contribution proposes to add a constraint for APS type based on the enabled tools in SPS. If a tool, which uses an APS, is disabled then the APS with the corresponding APS type should not be present in a bitstream. Secondly, the contribution proposes to add a constraint for CCALF filters in an ALF APS based on CCALF SPS enable flag. If CCALF is disabled in SPS, an ALF_APS cannot contain any CCALF filters. Thirdly, the contribution proposes to add constraints for chroma deblocking and chroma LMCS scaling parameter for monochrome content, similar constraints are already in place for other chroma related tools and syntax elements. At last, as another option, to decouple constraints in both PPS and APS from SPS, ChromaArrayType and BitDepth constraints are removed from both PPS and APS semantics.

#### Deblocking control signalling - other aspects (5)

[Began here session 8 Tuesday 7 April 2320 UTC (GJS & YKW).]

JVET-R0072 AHG9: On signalling of deblocking control [Z. Deng, Y.-K. Wang, L. Zhang, K. Zhang, J. Xu (Bytedance)]

It is asserted that the deblocking control mechanism in the latest VVC text is pretty complicated, not straightforward, not easy to understand, and consequently prone to errors. This contribution proposes to change the deblocking signalling based on a 2-bit deblocking mode indicator in the PPS, summarized as follows:

- Replace the three PPS flags for deblocking signalling with a 2-bit deblocking mode indicator that specifies the following four modes: a) deblocking fully disabled and not used for all slices; b) deblocking used for all slices using 0-valued β and tC offsets; c) deblocking used for all slices using β and tC offsets explicitly signalled in the PPS; and d) deblocking further controlled at either picture or slice level.

- The two flags in PH/SH are renamed to be ph/slice_deblocking_filter_used_flag and ph/slice_deblocking_parameters_override_flag, with the use flag specifying whether deblocking is used for the current picture/slice, and the override flag specifying whether the β and tC offsets are overridden by the values signalled in the PH/SH.

There was discussion of the various cases described in the document.

It was commented that deblocking is the only case where a disabling in the PPS can be overridden at the picture level. Some participants said this is OK, as it enables PPS sharing, and noted that we have such an override in HEVC. Others said this is the only place in the text that allows a disabling to be overridden.

It was agreed that there is no clear bug in the current design, although it does appear that there are a number of editorial bugs in the current semantics of the text.

No clear need for action was identified by the AHG on this, so the AHG did not recommend action.

JVET-R0183 AHG9: On deblocking control signalling [S. Paluri, Hendry, S. Kim (LGE)]

This contribution asserted that the current signalling for deblocking filter control is complex and may be simplified. For example, the current design allows the deblocking to be disabled at the PPS level and then later to be enabled either in picture header or the slice header. It is asserted that such design may be confusing and make the signalling more difficult to understand.

This contribution proposed the following changes:

- Signal a flag in the PPS, i.e., pps_deblocking_enabled_flag to specify whether or not deblocking is enabled / applied to pictures that refer to the PPS.

- When deblocking is enabled (e.g., the value of pps_deblocking_enabled_flag is equal to 1, additional flags can be signalled as follows:

  - pps_deblocking_override_enabled_flag, which is an existing flag.

  - pps_deblocking_parameter_present_flag to specify whether the PPS deblocking parameter is present.

This is very similar in spirit to R0072, and is a smaller change.

No clear need for action was identified by the AHG on this, so the AHG did not recommend action.

JVET-R0159 AHG9: On high level syntax of deblocking filter [J. Chen, J. Luo, Y. Ye, R.-L. Liao (Alibaba)]

This contribution includes two syntax and semantics changes of deblocking filter (DBF) as follows.

- It is proposed to signal an SPS enabled flag for DBF as done for other loop filters. It was asked whether there are any other features that do not have an SPS-level enabling flag; CU QP delta and CU chroma QP delta enabling were noted. It was commented that the reason there is no such flag is because it was expected that bitstreams would generally have the DBF enabled (and there is no constraint flag for this). Another participant commented that we may have wanted to have control flags for things either in the SPS or PPS but not both. It was commented that in other in-loop filter cases, there is syntax in the PH that is gated by the SPS flag. No clear need for action was identified by the AHG on this, so the AHG did not recommend action.

- Fixing basically editorial bugs of the semantics of DBF control related syntax elements. AHG Recommendation (editorial BF): Adopt (with editor discretion on exact form of expression).

JVET-R0106 AHG9: On Deblocking Control [S. Deshpande, J. Samuelsson, A. Segall, T. Zhou, T. Ikai (Sharp)]

Item 1 of this contribution belongs to this category.

Proposal 1: It is proposed to move the signalling location of syntax element dbf_info_in_ph_flag to locate it near the other deblocking control parameters signalling.

This is a small proposed change to group together and logically nest the deblocking syntax in the PPS. AHG Recommendation (cleanup): Adopt.

[Stopped here for session 8 Wednesday 8 April 0115 UTC.]

JVET-R0388 AHG9: Cleanups on deblocking signalling [Z. Deng, Y.-K. Wang, L. Zhang, K. Zhang, J. Xu (Bytedance)] [late]

TBP

#### Quantization control signalling (6)

JVET-R0050 AHG9: HLS on dependent quantization and sign data hiding [S.-T. Hsiang, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Item 7 of this contribution belongs to this category.

JVET-R0073 AHG9: Some cleanups on QP delta signalling [Z. Deng, L. Zhang, Y.-K. Wang, J. Xu, K. Zhang (Bytedance)]

JVET-R0076 AHG9: Chroma QP mapping table cleanups [J. Xu, L. Zhang, Y.-K. Wang, K. Zhang, Z. Deng (Bytedance)]

JVET-R0272 AHG9: On chroma QP offsets in picture header [K. Misra, J. Samuelsson, S. Deshpande, F. Bossen, A. Segall (Sharp)]

JVET-R0302 AHG12: On signalling of chroma QP [L. Li, X. Li, B. Choi, S. Wenger, S. Liu (Tencent)]

#### High level control of LMCS, scaling list, and ALF (21)

JVET-R0404 AHG9: A summary of proposals on high level control of LMCS, Scaling list, ALF and SAO [L. Zhang, Y.-K. Wang (Bytedance)] [late]

JVET-R0051 AHG9: HLS on LMCS and scaling list [S.-T. Hsiang, Z.-Y. Lin, C.-Y. Lai, O. Chubach, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0063 AHG9: Signalling of LMCS control [L. Zhang, Y.-K. Wang, K. Zhang (Bytedance)]

JVET-R0064 AHG9: Signalling of scaling list control [Y.-K. Wang, L. Zhang, K. Zhang (Bytedance)]

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Item 4 of this contribution belongs to this category.

JVET-R0089 AHG9: On slice level control of LMCS and explicit scaling list [J. Jung, D. Kim, G. Ko, J.-H. Son, J. S. Kwak (WILUS)]

JVET-R0096 AHG9: On signalling of chroma residual scaling [M. G. Sarwer, Y. Ye, J. Luo, J. Chen (Alibaba)]

JVET-R0098 AHG9: On Slice Header Signalling of LMCS and Scaling Lists Information [	S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-R0070 AHG9: On repetition and update of non-VCL data units [Y.-K. Wang, L. Zhang, Z. Deng (Bytedance)]

Items 4 and 5 of this contribution belongs to this category.

Relates to R0149 and R0201 aspects 2 and 3.

JVET-R0149 AHG9/AHG12: Relaxing an APS constraint [M. M. Hannuksela, M. Homayouni, A. Hallapuro, A. Aminlou (Nokia)]

Relates to R0070 and R0201 aspects 2 and 3.

JVET-R0201 AHG9: On prefix and suffix APSs [N. Ouedraogo, G. Laroche, P. Onno (Canon)]

JVET-R0160 AHG9: High level syntax cleanup for LMCS, ALF and SAO [J. Chen, J. Luo, M. G. Sarwer, Y. Ye, R.-L. Liao (Alibaba)]

JVET-R0171 AHG9: Slice-Level Chroma Residual Scaling Flag [K. Naser, E. François, F. Hiron, C. Chevance (InterDigital)]

JVET-R0180 AHG9: On ALF, LMCS and Scaling List Parameters Signalling [K. Naser, F. Le Léannec, T. Poirier, P. de Lagrange (InterDigital)]

JVET-R0200 AHG9: APS information signalling in Slice Header [G. Laroche, N. Ouedraogo, P. Onno (Canon)]

The first aspect (on slice_lmcs_enabled_flag) of item 2 of this contribution belongs to this category.

JVET-R0202 AHG9: Syntax cleanups when Picture Header is in the Slice Header [G. Laroche, N. Ouedraogo, P. Onno (Canon)]

The slice_lmcs_enabled_flag and slice_explicit_scaling_list_used_flag aspects of item 1 of this contribution belong to this category.

JVET-R0210 AHG9: Cleanup of Picture Header Syntax Structure in Slice Header [S. Esenlik, B. Wang, A. Kotra, E. Alshina (Huawei)]

The slice_lmcs_enabled_flag aspect this contribution belongs to this category.

JVET-R0225 AHG9: On ALF/CC-ALF high level syntax [X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU)]

JVET-R0232 AHG9: APS, LMCS, deblocking and PPS constraints [N. Hu, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

All aspects excluding the deblocking aspect of this contribution belong this this category.

JVET-R0251 AHG9: Fixes related to the picture header [M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

Item 2 of this contribution belongs to this category.

JVET-R0393 AHG9: On LMCS for GDR [L. Wang, S. Hong, K. Panusopone, M. M. Hannuksela (Nokia)] [late]

#### High level control of other tools (13)

JVET-R0049 AHG9: HLS on disabling TSRC [S.-T. Hsiang, C.-W. Hsu, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

#### To be revisited for HLS review. This and related contributions were initially discussed in a different AHG meeting, as reported in JVET-R0340.

JVET-R0150 AHG9/AHG12: Moving joint chroma coding sign flag from picture header to slice header [M. M. Hannuksela, J. Lainema (Nokia)]

JVET-R0175 AHG9: An SPS Flag for IBC-AMVR [K. Naser, M. Kerdranvat, T. Poirier, A. Robert (InterDigital)]

JVET-R0214 AHG9: MMVD syntax modifications [R. Yu, M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

JVET-R0215 AHG9: Max num of subblock merge candidate signalling [R. Yu, M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

JVET-R0216 AHG9: Signalling the parallel merge level relative to the minimum coding block size [R. Yu, M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn, J. Ström (Ericsson)]

JVET-R0237 AHG9: Constraints based on the minimum coding block size [K. Zhang, L. Zhang, Y.-K. Wang, Z. Deng, Y. Wang, J. Xu, H. Liu (Bytedance)]

JVET-R0252 AHG9: On high-level signalling of mvd_l1_zero_flag [M. Pettersson, R. Yu, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn, D. Liu (Ericsson)]

JVET-R0137 On mvd_l1_zero_flag and NoBackwadPredFlag [T. Chujoh, E. Sasaki, T. Ikai (Sharp)]

Item 2 of this contribution belongs to this category.

JVET-R0258 AHG9: Reduce redundant signalling in picture header [J. Enhorn, M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang (Ericsson)]

All other aspects, excluding the ph_dep_quant_enabled_flag aspect of item 1, of this contribution belong to this category.

JVET-R0287 AHG9: On high level control parameters [H. Huang, Y.-J. Chang, M. Coban, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-R0371 AHG2/9: On max num of subblock merge candidates [H. Huang, J. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)] [late]

JVET-R0373 AHG9: On Maximum Number of Subblock Merge Candidates Y.-C. Yang, C.-Y. Teng (Foxconn) [late]

### General and misc. HLS topics (9)

[Started here Wednesday 8 April 1300 UTC GJS & YKW]

JVET-R0041 AHG8/AHG9: On picture types and related constraints [Y.-K. Wang (Bytedance)]

This contribution proposes some changes related to definitions of the terms "associated IRAP picture", "associated GDR picture", and "trailing picture" and the constraints regarding different types of pictures and their relationships in terms of decoding order, output order, and prediction relationship.

The proposed changes are summarized as follows:

- The definition of associated GDR picture is added and the definition of associated IRAP picture is updated, such that each picture of a layer, except the first picture in the layer in the bitstream, is specified to be associated with the previous IRAP or GDR picture of the same layer in decoding order, whichever is closer.

- The definition of trailing picture is updated, such that a trailing picture may also be associated with a GDR picture.

- The following existing constraints are updated such that they only impose restrictions to pictures within the same layer:

  - On the output order of pictures preceding an IRAP picture in decoding order

  - On the decoding order of pictures associated with an IRAP picture and some non-leading pictures

  - On RPLs for a CRA picture

- Constraints for an STSA picture, in terms of relative decoding order, output order, and prediction relationship with the associated IRAP picture and the same-layer pictures in the preceding and succeeding AUs, are specified, similarly as a trailing picture.

- Similar constraints for IRAP pictures and the same-layer pictures in the preceding and succeeding AUs in terms of relative decoding order, output order, and prediction relationship are specified for GDR pictures.

It was suggested to explicitly distinguish a “single-layer bitstream” from a “multi-layer bitstream”. As proposed, there may not be a need for such a distinction.

Some action is needed in this area; this is basically filling in gaps and proposing bug fixes for expression of the existing technical intent; no new functionalities are introduced.

Contribution R0226 is related.

It was commented that there may need to be some further cleanup regarding mixed NAL unit types; there are other contributions on that issue.

It was commented that it may be desirable to find a way to simplify / unify the discussions of GDR and IRAP if feasible (esp. if the recovery POC count is zero for the GDR).

It was noted that GDR characteristics are less restricted than IRAP, e.g. in regard to IRAP. The proponent said they had not intended to add constraints to GDR pictures, beyond what is necessary.

It was noted that various definitions depend on NAL unit types, and this could make the definitions difficult to comprehend outside the context of the document (a characteristic preferred by ISO editing guidelines).

AHG Recommendation: Adopt. (Further offline review is encouraged, and interacting aspects with other contributions remain in need of consideration.)

JVET-R0226 AHG9: Proposal to change the definition of trailing picture [R. Sjöberg, M. Pettersson, M. Damghanian, Z. Zhang, J. Enhorn, R. Yu (Ericsson)]

The proponents of this proposal assert that STSA pictures are trailing pictures in HEVC but not in VVC, and that the rules for trailing pictures therefore do not apply for STSA pictures. It is proposed to include STSA pictures in the group of trailing pictures so that the rules do apply also for STSA pictures.

The proponents also claim that bitstreams starting with a GDR picture may contain pictures that do not have any associated IRAP picture and that the current VVC draft disallows the use of TRAIL_NUT pictures for this case. The proponents believe that TRAIL_NUT pictures should be allowed for pictures that do not have any associated IRAP picture.

The proposal is summarized as:

- Include STSA pictures and pictures with no associated IRAP picture into the trailing picture type.

- Only allow pictures having an associated IRAP picture to be “leading pictures”.

- Condition the conformance requirements regarding relationships between trailing pictures and associated IRAP pictures to only apply to trailing pictures that do have an associated IRAP picture.

- Add the following rule: “Any picture that precedes a GDR picture in decoding order shall precede the GDR picture in output order”

It was questioned whether “trailing picture” is a good term if there are cases where this picture is not trailing anything else.

A difference with R0041 is that this associates a trailing picture with the most recent IRAP, whereas R0041 associates it with the most recent picture that is either an IRAP or GDR picture. The intent here is to have as few restrictions as possible that apply around a GDR picture. Revisit for this question.

Can a trailing picture reference a RADL picture (a RADL of an IRAP)? No (although there is a trick for one picture corresponding to a field coding case). This is the same in R0041 and R0226 (and HEVC).

Both contributions allow a picture that follows a GDR picture in output order to refer to another picture that follows the GDR picture in decoding order that precedes the GDR picture in output order (in a way that is not allowed if the GDR picture was an IRAP picture).

Both this contribution and R0041 constrain a picture that precedes a GDR picture in decoding order to also precede it in output order. It was discussed whether such a constraint should apply only after the recovery point is reached. Revisit for this question.

JVET-R0065 AHG8/AHG9: On IRAP and GDR AUs [Y.-K. Wang (Bytedance)]

Item 1 of this contribution belongs to this category.

This contribution proposes the following changes related to IRAP and GDR AUs:

- Each GDR AU is required to be “complete” (i.e., to have a picture for each of the layers present in the CVS). That means, an incomplete AU consisting of GDR pictures is not a GDR AU, similarly as in the current VVC text that an incomplete AU consisting of IRAP pictures is not an IRAP AU. This aspect seems to be straightforward completion of the specification of GDR to account for multi-layer usage.

- Add a flag named irap_or_gdr_au_flag to the AUD to specify whether the AU is an IRAP or GDR AU, and mandate the presence of an AUD NAL unit in each IRAP or GDR AU when vps_max_layers_minus1 is greater than 0.

The following issues were reportedly observed in the existing scalability design in the latest VVC text (in JVET-Q2001-vE/v15):

- Currently, an IRAP AU may start a new CVS and is required to be complete (i.e., to have a picture for each of the layers present in the CVS), while a GDR AU may also start a new CVS but is NOT required to be complete. It is believed that this is an oversight, as otherwise some layer-wise startup decoding process would have to be specified for such GDR case.

- A CVSS AU, which starts a new CVS, is required to be complete (i.e., to have a picture for each of the layers present in the CVS). However, according to the current design, the decoder is not able to check whether an AU includes a picture "for each of the layers present in the CVS" before it receives the last picture of the CVS, while on the other hand, even the last picture of the CVS is not easy to be determined because it is not easy to determine the start of any of CVS except for the very first CVS of the bitstream. Basically, that means, the decoder can only figure out the boundaries of CVSs after receiving the entire bitstream.

This contribution tries to address the above issues.

An OLS may contain more layers than are present in the bitstream.

It was discussed to what extent there may (or should) be out-of-band signalling to inform the decoder of the start of a new CVS or the establishment of a new target OLS. It was commented that the bitstream should at least be decodable without needing to rely on some such external means.

Another contribution R0274 proposes to not require an IRAP AU to be complete.

Revisit after offline study.

JVET-R0070 AHG9: On repetition and update of non-VCL data units [Y.-K. Wang, L. Zhang, Z. Deng (Bytedance)]

Items 1-3, and 6 of this contribution belong to this category.

This contributions proposes the following aspects regarding repetition and update of non-VCL data units and sharing of APSs across layers:

- VPS, SPS, and DCI NAL units are proposed to be disallowed to be present in an AU that has no nal_unit_type in the range of IDR_W_RADL to GDR_NUT, inclusive. It was asked why we would do this – e.g., an SPS could be sent early before it is used. No need to impose such a constraint was identified, so no action was recommended on this.

- A PU is proposed to be disallowed to have more than one VPS, SPS, or PPS NAL unit with a particular VPS, SPS, or PPS ID, and is proposed to be disallowed to have more than one DCI NAL unit. No need to impose such a constraint was identified, so no action was recommended on this.

- A slice unit (SU) is proposed to be defined as a set of NAL units that are consecutive in decoding order and contain exactly one coded slice and all its associated non-VCL NAL units.

  - Within an SU it is proposed to be disallowed to have more than one APS NAL unit with a particular APS ID and of particular APS type.

  - Within an SU it is proposed to be disallowed to have more than one SEI payload with particular type and a particular content.

- Update to the content of an ALF APS NAL unit within a PU is proposed to be allowed. This aspect should be discussed as a matter of tool control (section 4.1.2.4), as it is not just a matter of HLS, and a similar change is proposed in R0149.

- Sharing of an APS NAL unit across layers is proposed to be disallowed. This aspect is discussed in section 4.1.2.4.

- The same types of APSs are proposed to share the same value space for the APS ID, regardless of whether the APSs are prefix or suffix APS NAL units. It was commented that this has been the existing design intent. AHG Recommendation (expression of existing intent): Adopt this aspect.

[Stopped here for session 9, with item 3 yet to be reviewed]

JVET-R0082 AHG9: Byte alignment modifications [Z. Zhang, M. Pettersson, M. Damghanian, J. Enhorn, J. Ström, R. Sjöberg (Ericsson)]

JVET-R0122 AHG9: On picture output for non-reference pictures [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-R0147 AHG9: On picture order count and output order [M. M. Hannuksela (Nokia)]

JVET-R0263 AHG9: On TemporalId and sublayer [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

### Profile, tier, level (PTL) (6)

[Began here session 5 Tuesday 7 April, 2020, 13:00 (Gary & YKW & JRO & JB)].

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Item 2 of this contribution belongs to this category.

Discussed UTC Tuesday 7 April, 2020, 13:00. Chaired by JRO and JB.

It was commented that the encoder could use TemporalId 0 for IRAP pictures. However, it was commented that will make one less sublayer to be used.

It was commented that in intra based trick play, some IRAP pictures would be output/displayed multiple times.

It was commented that trick play can also use non-intra pictures, and scene cuts may appear in arbitrary positions and can be coded as IRAP pictures.

It was commented that such information, if useful, should use an SEI message.

It was commented that the information is useful, somewhat like marking non-reference pictures etc.

It was commented that DASH already makes use of this information, as a bitstream property.

It was commented that the information may be hard for the encoder to figure it out and set it. However, it was counter argued that this is similar for setting the level for temporal scalable layers.

What do people think about having an SEI message for this?

Revisit.

JVET-R0108 AHG9: Decoding Capability Information and PTL Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

Discussed UTC Tuesday 7 April, 2020, 14:10.

Following is proposed related to DCI and PTL signalling:

- Proposal 1: It is proposed that dci_max_sublayers_minus1 syntax element be removed and instead those bits and the reserved zero bit be used for the syntax element dci_num_ptls_minus1.

- Proposal 2: It is proposed to rearrange the syntax elements in profile-tier-level signalling structure such that general_level_idc syntax element, which is unconditionally signalled, is first in the structure and the other conditional signalling, which is all based on profileTierPresentFlag is together, thus requiring only a single if check.

- Proposal 3: It is proposed to conditionally signal sps_ptl_dpb_hrd_params_present_flag only when sps_video_parameter_set_id is not equal to 0.

Proposal 1:

It was commented that the added semantics constraint on vps_num_ptls_minus1 has a problem.

It was suggested to use 4 reserved bits instead of having 8 bits for the number of PTL structures.

Comment: The current extension mechanism for DCI is a bit heavy.

Suggestion: Reserve a value of dci_num_ptls_minus1, e.g., 15.

AHG recommends to remove the dci_max_sublayers_minus1 SE, but to use 4 reserved bits (at the begin of the DCI syntax) instead of having 8 bits for the number of PTL structures (as proposed), and reserve the value 15 of dci_num_ptls_minus1.

Proposal 2:

Comments: This makes level goes before profile and tier, while the interpretation of level typically depends on profile.

Comments: The GCI syntax structure can be of variable length. The proposed change makes level at fixed position.

No action on this item was taken.

Proposal 3: See notes in 4.1.6.1.

JVET-R0243 AHG9: 4:4:4 vs. 4:2:0 bit-rate in VTM [S. Keating, A. Browne, K. Sharman (Sony)]

This contribution compares bit-rates for 4:4:4 and 4:2:0 encoding. It is for information/discussion only.

Another contribution, JVET-R0244, proposes changes to the CpbVclFactor and MinCrScaleFactor for Main 4:4:4 10 profile. JVET-R0244 proposes to specify that the maximum bit-rate of 4:4:4 should be twice the maximum bit-rate of 4:2:0 (instead of 2.5 as currently specified, and as in AVC and HEVC).

Comments:

In some cases, unless low-pass filtering is applied, the bit rate is higher. However, on the other hand, low-pass filtering to chroma seems not good as it blurs the chroma.

Chroma QP offset or lambda adjustment are another ways of adjusting the bit-rate balance.

A reason for having some extra bit rate header room for 4:4:4 is that the quality expectation for 4:4:4 is higher, and the GOP length or intra refresh period may be shorter.

Sometimes there is RGB coding for 4:4:4, which is generally less efficient than YCbCr.

[Stopped session 5 here Tuesday 7 April 1500 UTC]

JVET-R0244 AHG9: Coded Picture Buffer sizes and MinCr in VVC [S. Keating, A. Browne, K. Sharman (Sony)]

JVET-R0245 AHG9: Level coding in VVC [S. Keating, A. Browne, K. Sharman (Sony)]

JVET-R0246 AHG9: Max Luma Picture Size in VVC [S. Keating, A. Browne, K. Sharman (Sony)]

### General constraints information (GCI) (9)

JVET-R0086 AHG9: Modification of general constraints flags [W. Lim, G. Bang (ETRI)]

JVET-R0173 AHG9: Cleanup of Constraint Flags [K. Naser, F. Le Léannec, M. Kerdranvat, P. de Lagrange (InterDigital)]

JVET-R0178 AHG9: On APS and GDR constraint Flags [K. Naser, F. Le Léannec, T. Poirier, P. de Lagrange (InterDigital)]

See also 4.1.6.1 regarding GDR.

JVET-R0179 AHG9: Constraint Flag for TSRC [K. Naser, F. Le Léannec, T. Poirier, M. Kerdranvat (InterDigital)]

JVET-R0191 AHG9: On miscellaneous updates for HLS signalling [Hendry, S. Paluri, S. Kim (LGE)]

Item 5 of this contribution belongs to this category.

JVET-R0207 AHG9: General constraint information for LFNST [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

JVET-R0227 AHG9: General constraint information semantics constraints and a flag for PH in SH [R. Sjöberg, R. Yu, M. Pettersson, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

JVET-R0286 AHG9: On general constraint information syntax [Y.-J. Chang, V. Seregin, Y. He, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0341 AHG9: on constraint flag for local chroma QP control [Philippe de Lagrange, Karam Naser, Philippe Bordes, Fabrice Le Léannec (interdigital)] [late]

### Parameter sets cleanups (21)

#### General (1)

JVET-R0343 AHG9: A summary of proposals on parameter sets cleanups [Hendry (LGE)] [late]

This contribution intended to provide a summary of the proposals on parameter sets cleanups submitted to this JVET meeting by the 3 April 2020 submission deadline.

It is suggested that this summary is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

Summary of proposals on SPS cleanups:

This section discussed Monday 6 April at 1440 UTC (GJS & YKW).

- New condition for signalling of syntax elements

  - When sps_ptl_dpb_hrd_params_present_flag is equal to 1, inter_layer_ref_pics_present_flag is not present and inferred to be equal to 0 (JVET-R0156 proposal 2)

Is it possible for such a SPS to be referred to by a layer that has reference layers?

It was commented that JVET-R0205 is related.

This item was moved to 4.1.10.

  - Condition the presence of sps_sublayer_dpb_params_flag on the value of sps_ptl_dpb_hrd_params_present_flag, in addition to sps_max_sublayer_minus1 (JVET-R0156 proposal 3) (JVET-R0170) (JVET-R0222 proposal 2) AHG Recommendation: Adopt.

  - Condition that sps_independent_subpics_flag is present only when there are at least two subpictures. (JVET-R0156 proposal 4)

MC wrap-around was discussed.

It was commented that item 1 of R0284 and item 1 of R0071 are identical or similar to this.

This item was moved to 4.2.1.1.

[Stopped session 2 here Monday 6 April at 1500 UTC; Resumed the discussion at the start of session 4 Monday 6 April at 2330 UTC]

  - Condition the presence of subpic_info_present_flag by res_change_in_clvs_allowed_flag (JVET-R0266 proposal 3). These cannot be used together currently, although there had been proposals to allow them to be used together. No support was expressed by non-proponents for this.

  - Add new flag in SPS to indicate that intra-only (i.e., whether inter-coding is allowed). Use this flag to condition the presence of inter-coding related tools. (JVET-R0283 proposal 1) (JVET-R0335). The proponent said this could skip about 40 syntax elements (more than 5 times more than in HEVC), and drew an analogy to monochrome. It was commented that low-resolution still-picture coding might be the strongest argument for this. Another participant suggested that skipping the irrelevant syntax would ease encoder design. After seeing the syntax table, several participants expressed support for this, while others said this adds extra syntax that is not necessary for video and the bit savings seems too small to make a special provision for it. It was asked whether the syntax structure logic should be different if we do this. Data on this (with the encoder minimizing the necessary amount) was requested. An initial estimate was 23 bits per SPS.

It was later confirmed (Wednesday 8 April) that 23 bits in the SPS (and 4 in the PPS and 1 in the PH) could be saved in such a case. The quantity of data is not compelling; the argument is more a matter of the analogy to monochrome. It was commented whether the analogy is really apt, since there was already the chroma array type information in the SPS to use for that.

There is an all-intra constraint flag in the PTL structure, which currently controls only the slice level.

After discussion, there was not a consensus for this change.

See also R0332 on syntax grouping.

  - In similar train of thought as point above, do the same for PPS (JVET-R0283 proposal 2). Revisit for data.

  - Add a constraint that sps_ptl_dpb_hrd_params_present_flag shall be equal to 1 when there is at least one OLS which has only one layer or VPS is not present? (JVET-R0275)?

It was said that this relates to some other proposals on SPS cleanup (R0191, R0156 aspect 1, R0108 proposal 3).

If VPS is not present, PTL would be absent if the flag is 0. R0156 aspect 1 and R0108 proposal 3 propose that if the sps_video_parameter_set_id is equal to 0, not to signal the flag and infer it to be equal to 1. It was commented that this could affect byte alignment of the PTL information. JVET-R0275 propose to constrain the flag for this. The motivation for not sending it was said to be to prevent the possibility of not having the PTL information at all. AHG Recommendation: To avoid changing byte alignment, the constraint approach was agreed in this case.

When the VPS is present and there is some OLS that has only one layer and the layer ID is the current layer’s ID, this case is proposed to be constrained in R0191 and R0275. AHG Recommendation: The constraint was also agreed to apply in this case.

V. Seregin agreed to provide text in an update of R0275 and to provide the software.

- Infer the value of sps_ccalf_enabled_flag to be equal to 0 when not present (i.e., when sps_alf_enabled_flag is equal to 1 and ChromaArrayType is equal to 0) (JVET-R0105). This seemed to be what was already the intended behaviour. AHG Recommendation (expression of existing intent): Adopt.

- Constraint the semantics of subpic_info_present_flag such that when it is equal to 1, at least either pic_width_max_in_luma_samples or the value of pic_height_max_in_luma_samples shall be larger than the CTB size CtbSizeY. (JVET-R0156 part 2 of item 4). After study, this might affect extraction, so no action was taken on this.

- Change sps_reserved_zero_4bits to sps_reserved_one_4bits to prevent the SPS start code emulation. (JVET-R0266 proposal 1). The case would be with an SPS ID of zero and monochrome and PTL info not present (only in a dependent layer). It was commented that our other reserved bits are zero and this seems somewhat ad hoc. There was no non-proponent interest in this, so no action was taken on it.

[Stopped AHG session 4 here Tuesday 7 April 0115 UTC.]

[Began AHG session 11 here Wednesday 8 April 2100 UTC (GJS & YKW)]

- Consolidate two syntax elements, sps_poc_msb_flag and poc_msb_len_minus1 into a single syntax element poc_msb_len (JVET-R0266 proposal 2)

It was commented that this seems almost editorial. Another participant commented that having a flag could be desired editorially for having a clear way to disable the feature, and that there may have been a similar prior situation.

It was questioned whether the current semantics are really correct, i.e., whether the signalled MSBs are intended to be all of the MSBs of the POC or only some of them. Others confirmed that all “missing” MSBs are inferred to be 0, and that this is intentional. AHG Recommendation (ed.): The editor is asked to review whether this aspect of the semantics of poc_msb_val is sufficiently clear.

It was noted that with the proposal, the value 0 would be overloaded to have a different and special meaning. It would mean more than the name of the syntax element would imply. (The proposed semantics would need some clarification in this regard.) Since this could be confusing, no action was recommended on the proposal.

- gdr_enabled_flag value constrained by no_gdr_constraint_flag (JVET-R0266 proposal 5, JVET-R0178)

There was discussion of the possibility of having some NUTs that are GDR and some that are not. This is already disallowed.

AHG Recommendation (editorial expression of existing intent): Specify that no_gdr_constraint_flag equal to 1 specifies that gdr_enabled_flag shall be equal to zero. no_gdr_constraint_flag equal to 0 does not impose such a constraint.

- Grouping syntax elements in SPS based on slice type (i.e. intra or inter) (JVET-R0332)

It was discussed whether we would want to do such a rearrangement regardless of whether we want to gate presence on whether inter pictures are present or not (see JVET-R0283 proposal 1 and JVET-R0335). Some participants said that some minor rearrangements might be OK, but wholesale restructuring would be undesirable. It was commented that software implementation would be desirable to make sure there are no overlooked dependencies. The proponent said they did implement it and could provide software for checking. It was asked for such software to be provided in a revision of the contribution. Support for this was expressed, as a more logical structuring of the syntax – the prior syntax may have been rather randomly ordered. Revisit after offline checking of the software.

Summary of proposals on PPS cleanups:

- Require the value of pps_conformance_window_flag to be equal to 0 when the picture width and height are the maximum picture width and height, and infer the values of the PPS conformance window syntax elements to be the same as those signalled in the SPS if the picture width and height are the maximum picture width and height and to be equal to 0 otherwise. (JVET-R0068 proposal 6) (JVET-R0262 proposal 1 and 2)

It was commented that there is already a constraint that in this case the window at the PPS level needs to be the same as the one at the SPS level; the question is only whether to require the flag to be 0 and to infer from the SPS level in this case. In the current draft, in this case, the window parameters are required to be sent in every PPS when non-zero and are required to always be the same.

There is also already a requirement that if the picture size in two PPSs is the same, their cropping windows must be the same. (These constraints are intended to ease RPR operation.)

One participant commented that having an inference rule that is conditional on a particular special case seems potentially confusing to implementers. Others said it only makes sense that if the parameters are required to have a particular value, that is the value that should be inferred and there shouldn’t be syntax capable of violating that constraint.

It was noted that this inference from the SPS prevent complete self-contained interpretation of the PPS content, although we already have some such dependencies.

AHG Recommendation: Adopt.

- Add a new syntax element pps_res_change_allowed_flag in the PPS and use it to condition the presence of the conformance window and scaling window syntax elements (JVET-R0262 proposal 3). This is related to the previous item above. With the action taken on the previous item, this reduces to adding a flag that would skip two flags in the PPS. No action was taken on this, due to the action taken on the previous item.

- Add a constraint for the cropping window offsets and scaling window offsets that at least one of the offsets is different than its default value when the flag that controls their presence is equal to 1 (JVET-R0115). It was commented that this extra constraint doesn’t really seem necessary, so no action was recommended on this.

- Change the signalling for wraparound offset. Signal “picture width minus wraparound offset” instead of “wraparound offset” (JVET-R0162 proposal 1). In the 360° ERP case, this corresponds to sending the padding width rather than the pre-padded picture width. For the 360° CTC, this would save about 16 bits per PPS (sending a value of 16 instead of 4448). The current syntax seems obviously inefficient, so this was supported without any expressed misgivings. AHG Recommendation: Adopt.

- Change the signalling of the PPS ID from ue(v) to u(6), as proposed in R0266 proposal 4. It was noted that this is the only parameter set ID that uses ue(v) coding for its ID. AHG Recommendation: Adopt.

To resume at 0031 UTC.

Summary of proposals on APS cleanups:

- Handling chroma related syntax elements in APS when ChromaArrayType is equal to 0

  - To avoid having APS semantics depend on the SPS, move the constraints from the APS semantics to the PH and SH semantics of the relevant APS ID, in, such that the value of scaling_list_chroma_present_flag shall be equal to 0 when the value of ChromaArrayType is equal to 0.

It is also proposed to similarly move the constraint for alf_chroma_filter_signal_flag, alf_cc_cb_filter_signal_flag, alf_cc_cr_filter_signal_flag, and add a similar constraint for lmcs_delta_abs_crs (JVET-R0074).

This is asserted to remove APS-to-SPS dependency in the semantics.

This is related to parts of contribution JVET-R0232.

It was asked whether there is really a problem with constraining APS content based on the SPS content. It was commented that this is probably not desirable, although only an editorial matter.

The contribution proposes a constraint move for the scaling list and ALF (purely editorial), and adding a constraint for LMCS.

LMCS contains only two variables (three bits) relevant to chroma. ALF contains only three flags relevant to chroma.

There was a comment about this approach forcing crs_offset not to be 0.

It was commented that the constraints are not really necessary, as the presence of the chroma data in the APS is not necessarily harmful (although we have been trying to avoid sending irrelevant chroma syntax).

  - Add flags (i.e., alf_chroma_present_flag and lmcs_chroma_present_flag) to the APS and constraint them to be equal to 0 when ChromaArrayType is equal to 0 (JVET-R0177 proposal 1)

The LMCS part of this is related to part of contribution JVET-R0232.

  - Repurpose the chroma scaling list presence flag in the APS (i.e., aps_chroma_present_flag) and use this flag to condition the presence of chroma presence flags in the APS. (JVET-R0177 proposal 2 and JVET-R0301)

The difference between “b” and “c” is basically only editorial.

Revisit for further discussion.

[Stopped here Thursday 9 April at approximately 0115 UTC.]

- Change the constraint on APS NAL units to have the same content within a picture unit to apply within a subpicture (JVET-R0149 proposal 1)

- Disallow interleaving of APS NAL units of different subpictures (JVET-R0149 proposal 2)

- Constrain suffix APS NAL units to be located after the last VCL NAL unit of the PU (JVET-R0210)

- Allow prefix and suffix APS NAL units with particular APS identifier and type to have different content (JVET-R0210)

- Constrain prefix APS NAL unit to be located before the first VCL NAL unit of the PU(JVET-R0210)

- Add a mode in PH to allow APS to be signalled within PH. Just like the mode of signalling PH in SH (JVET-R0273)

Later-added SPS cleanups:

- Change the value range of sps_max_sublayers_minus1 from 0..vps_max_sublayers_minus1 to 0..(sps_video_parameter_set_id ? vps_max_sublayers_minus1 : 6). (JVET-R0125)

- Add a constraint on the value of sps_max_sublayers_minus1 such that when sps_video_parameter_set_id is greater than 0 and vps_all_layers_same_num_sublayers_flag is equal to 1, sps_max_sublayers_minus1 shall be equal to vps_max_sublayers_minus1. (JVET-R0125)

#### SPS cleanups (10)

JVET-R0105 AHG9: On CC-ALF Signalling in SPS [S. Deshpande, A. Segall, J. Samuelsson, P. Cowan (Sharp)]

JVET-R0125 AHG8/AHG9: On signalling max number of sublayers [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-R0156 AHG8/AHG9: Signalling cleanup on SPS [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

Items 1 and 3 this contribution belong to this category.

JVET-R0170 AHG9: Removed Coding Redundant DPB Related Flag [K. Naser, F. Le Léannec, T. Poirier (InterDigital)]

JVET-R0191 AHG9: On miscellaneous updates for HLS signalling [Hendry, S. Paluri, S. Kim (LGE)]

Item 1 of this contribution belongs to this category.

JVET-R0222 AHG9: SPS sublayer syntax cleanup [J. Luo, J. Chen, Y. Ye (Alibaba)]

JVET-R0266 AHG9: Miscellaneous HLS topics [Y. He, Y-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Items 1-3, 5 of this contribution belong to this category.

JVET-R0283 AHG9: Cleanup of inter predication HLS syntax elements [K. Naser, F. Le Léannec, M. Kerdranvat, P. de Lagrange (InterDigital)]

JVET-R0332 AHG9: On syntax signalling order in SPS [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-R0335 AHG9: On SPS inter slice related syntaxes [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

#### PPS cleanups (5)

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Item 6 of this contribution belongs to this category.

JVET-R0115 AHG9: On signalling of cropping windows and scaling windows [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

JVET-R0162 AHG9: PPS and SH syntax cleanup [J. Chen, J. Luo, Y. Ye, R.-L. Liao (Alibaba)]

Item 1 (wraparound offset signalling) of this contribution belongs to this category.

JVET-R0262 AHG9: On PPS syntax [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0266 AHG9: Miscellaneous HLS topics [Y. He, Y-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Item 4 of this contribution belongs to this category.

#### APS cleanups (5)

JVET-R0074 AHG9: Removal of APS semantics dependencies on SPS [Z. Deng, L. Zhang, Y.-K. Wang, K. Zhang (Bytedance)]

JVET-R0177 AHG9: APS Cleanup [K. Naser, F. Le Léannec, T. Poirier, P. de Lagrange (InterDigital)]

JVET-R0301 AHG12: on scaling_list_chroma_present_flag in APS [L. Li, X. Li, B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-R0273 AHG9: APS signalled in picture header [V. Seregin, M. Coban, Y. He, M. Karczewicz (Qualcomm)]

JVET-R0132 On signalling of chroma related APS [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)] [late]

### Syntax for one slice per picture (14)

JVET-R0406 AHG9: A summary of proposals on syntax for one slice per picture [Y.-K. Wang (Bytedance)] [late]

JVET-R0060 AHG9/AHG12: On CLVSs with one slice per picture [Y.-K. Wang, Z. Deng, L. Zhang, K. Zhang, J. Xu (Bytedance)]

JVET-R0104 AHG9: On Raster-scan Slice Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-R0113 AHG9: On Picture Parameter Set [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

Item 2 of this contribution belongs to this category.

Item 2 proposes that, when there is no picture partitioning, not to signal the PPS-level flags that indicate whether parameters are signalled in picture header or in slice header (e.g., rpl_info_in_ph_flag, sao_info_in_ph_flag, etc. – six flags) always send these in the SH. Infer them to be equal to 0. (JVET-R0113 proposal 2).

This is related to R0251 item 3.

JVET-R0118 AHG9/AHG12: On signalling of subpicture partitioning in SPS [B. Choi, S. Wenger, S. Liu (Tencent)]

Item 1 of this contribution belongs to this category.

JVET-R0162 AHG9: PPS and SH syntax cleanup [J. Chen, J. Luo, Y. Ye, R.-L. Liao (Alibaba)]

Item 3 (skip signalling of slice_address) of this contribution belongs to this category.

JVET-R0163 AHG9: On Picture Header [J. Chen, J. Luo, Y. Ye, R.-L. Liao (Alibaba)]

JVET-R0189 AHG9: On picture_header_in_slice_header_flag syntax element [Hendry, S. Kim, J. Nam, H. Jang, J. Lim (LGE)]

JVET-R0200 AHG9: APS information signalling in Slice Header [G. Laroche, N. Ouedraogo, P. Onno (Canon)]

The second aspect (on alf_info_in_ph_flag) of item 2 of this contribution belongs to this category.

JVET-R0202 AHG9: Syntax cleanups when Picture Header is in the Slice Header [G. Laroche, N. Ouedraogo, P. Onno (Canon)]

Items 2-4, and the num_tiles_in_slice_minus1 and slice_address aspects of item 1 of this contribution belong to this category.

JVET-R0210 AHG9: Cleanup of Picture Header Syntax Structure in Slice Header [S. Esenlik, B. Wang, A. Kotra, E. Alshina (Huawei)]

The num_tiles_in_slice_minus1 and slice_address aspects of this contribution belong to this category.

JVET-R0220 AHG9: Weight prediction syntax cleanup [J. Luo, J. Chen, Y. Ye (Alibaba)]

JVET-R0251 AHG9: Fixes related to the picture header [M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

Item 3 of this contribution belongs to this category.

JVET-R0124 AHG9: Clean-ups on picture header [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

Items 1 and 2 of this contribution belong to this category.

### Picture header and slice header (12)

JVET-R0410 AHG9: A summary of proposals on PH and SH syntax [Y.-K. Wang, L. Zhang (Bytedance)] [late]

JVET-R0052 AHG9: Overhead reduction for picture header and slice header [S.-T. Hsiang, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0061 AHG9: On allowed slice types in a picture [L. Zhang, Y.-K. Wang, K. Zhang, Z. Deng (Bytedance)]

JVET-R0250 AHG9: On B-slice signalling in the PH and derivation of slice_type [M. Pettersson, R. Yu, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

JVET-R0112 AHG9: On picture header [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

JVET-R0192 AHG9: On signalling recovery point picture [Hendry (LGE)]

JVET-R0198 AHG9: On signalling of IDR or GDR picture flag in picture header [J. Nam, H. Jang, J. Lim, Hendry, S. Kim (LGE)]

JVET-R0251 AHG9: Fixes related to the picture header [M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

Item 1 of this contribution belongs to this category.

JVET-R0324 AHG9: On syntax signalling conditions in picture header [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, W. Chen, X. Wang (Kwai Inc.)]

JVET-R0165 AHG12: Entry point offsets avoiding start code emulation prevention byte [K. Abe, T. Toma, V. Drugeon (Panasonic)]

JVET-R0298 AHG9: On order of syntax elements for entry point offsets [K. Suehring (HHI), R. Foray (Allegro DVT)]

JVET-R0200 AHG9: APS information signalling in Slice Header [G. Laroche, N. Ouedraogo, P. Onno (Canon)]

Item 1 of this contribution belongs to this category.

JVET-R0124 AHG9: Clean-ups on picture header [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

### Mixed NAL unit types within a coded picture (11)

JVET-R0414 AHG9: A summary of proposals on mixed NAL unit types within a coded picture [Y.-K. Wang (Bytedance)] [late]

JVET-R0042 AHG8/AHG9/AHG12: On mixed subpicture types within a picture [Y.-K. Wang (Bytedance)]

JVET-R0136 AHG9/AHG12: Improvements on sps_independent_subpics_flag and nal_unit_type constraint [M. Katsumata, M. Hirabayashi, T. Suzuki (Sony)]

Item 2 of this contribution belongs to this category.

JVET-R0276 AHG9: On IRAP NAL constraint for reordered subpictures [V. Seregin, Y. He, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0085 AHG9: On signalling the mixed NAL unit type flag [L. Chen, S.-T. Hsiang, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0203 AHG9/AHG12: On combination of NAL unit types in a picture [Hendry, S. Kim (LGE)]

JVET-R0267 AHG9/AHG12: On mixed NAL unit types [Y. He, M. Coban, V. Seregin, A.K. Ramasubramonian, M. Karczewicz (Qualcomm)]

JVET-R0270 AHG9: On mixing of RASL and RADL NAL unit types [Hendry, S. Kim (LGE), R. Skupin, Y. Sanchez, K. Suehring (HHI)]

JVET-R0315 AHG9: On mixed nal unit type signalling and PPS cleanup [M. Coban, V. Seregin, Y. He, Y.-J. Chang, M. Karczewicz (Qualcomm)]

JVET-R0120 AHG9: On mixed NAL unit types [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

JVET-R0124 AHG9: Clean-ups on picture header [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

Item 3 of this contribution belongs to this category.

### RPL, WP, and collocated picture signalling (10)

JVET-R0411 AHG9: A Summary of Proposals Related to Reference Picture Lists, Weighted Prediction, and Collocated Picture Signalling [S. Deshpande (Sharp)] [late]

JVET-R0059 AHG9: Cleanups on RPL and related signalling [Y.-K. Wang, Z. Deng, L. Zhang, K. Zhang, J. Xu (Bytedance)]

JVET-R0323 AHG9: On TMVP enabling flag in picture header [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, W. Chen, X. Wang (Kwai Inc.)]

JVET-R0220 AHG9: Weight prediction syntax cleanup [J. Luo, J. Chen, Y. Ye (Alibaba)]

JVET-R0102 AHG9: On Reference Picture List Override Signalling [S. Deshpande, T. Chujoh, T. Ikai, J. Samuelsson, A. Segall (Sharp)]

JVET-R0138 AHG9: Some constraints of num_ref_entries [T. Chujoh, T. Ikai (Sharp)]

JVET-R0156 AHG8/AHG9: Signalling cleanup on SPS [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

Item 2 of this contribution belongs to this category.

JVET-R0205 AHG9: On signalling of inter_layer_ref_pics_present_flag [T. Nishi, K. Abe, V. Drugeon (Panasonic)]

JVET-R0253 AHG9: Three restrictions when RPL is present in PH [R. Yu, M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

JVET-R0255 AHG9: Fixes related to RPL [M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

JVET-R0277 AHG9: On reference picture list signalling [V. Seregin, M. Coban, Y. He, M. Karczewicz (Qualcomm)]

### Signalling of virtual boundaries (4)

JVET-R0121 AHG9/AHG12: On virtual boundary signalling with subpictures [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-R0191 AHG9: On miscellaneous updates for HLS signalling [Hendry, S. Paluri, S. Kim (LGE)]

Item 4 of this contribution belongs to this category.

JVET-R0256 AHG9: Virtual boundaries in increasing order using u(v) [M. Damghanian, M. Pettersson, R. Sjöberg, Z. Zhang, J. Enhorn, R. Yu, J. Ström (Ericsson)]

JVET-R0266 AHG9: Miscellaneous HLS topics [Y. He, Y-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Item 6 of this contribution belongs to this category.

### Hypothetical reference decoder (HRD) (9)

JVET-R0342 AHG9: A Summary of Proposals Related to HRD S. [Deshpande (Sharp)] [late]

JVET-R0094 AHG9: DPB output time offsets for temporal sublayers [V. Drugeon, K. Abe (Panasonic)]

JVET-R0100 AHG9: On Decoding Unit Information Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-R0101 AHG9: On Alternative Timing Information Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-R0103 AHG9: On Picture Timing Information Signalling and HRD [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-R0264 AHG9: On sub-bitstream extraction [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0295 AHG12: On subpicture conformance [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

JVET-R0297 AHG9: HRD bug-fixes and editorial clarifications [Y. Sanchez, R. Skupin, K. Suehring, T. Schierl (HHI)]

JVET-R0413 AHG9: On Parameters for HRD Timing Information [S. Deshpande (Sharp)] [late]

### DCI, VUI, and SEI (6)

JVET-R0090 AHG9: On Video Usability Information [V. Drugeon (Panasonic)]

JVET-R0190 Post-filter hint based on ALF classification [H.-B. Teo, H.-W. Sun, C.-S. Lim (Panasonic)]

JVET-R0242 AHG9/AHG12: Decoded subpicture hash SEI message [J. Boyce, L. Xu (Intel)]

JVET-R0260 AHG9: On decoding capability information [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0307 AHG8/AHG9: Positioning information SEI message of output independent layers [E. Thomas (TNO)]

JVET-R0308 AHG8: Implementation of multi-layer decoding and output independent layer composition in VTM [E. Thomas (TNO)]

### HLS editorial inputs (1)

JVET-R0249 AHG9: Proposed structural text changes to HLS in the VVC specification [M. Pettersson, R. Sjöberg, M. Damghanian, Z. Zhang, J. Enhorn (Ericsson)]

## AHG12: high-level parallelism and coded picture regions (50)

### Subpictures (24)

#### General (1)

JVET-R0415 AHG12: A summary of proposals on subpictures [Hendry (LGE)] [late]

#### General and misc. subpicture aspects (10)

JVET-R0071 AHG12: Some cleanups on subpicture signalling [Z. Deng, Y.-K. Wang, L. Zhang, K. Zhang (Bytedance)]

JVET-R0091 AHG9: Issue with slice indexing [V. Drugeon (Panasonic)]

JVET-R0156 AHG8/AHG9: Signalling cleanup on SPS [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

Item 4 of this contribution belongs to this category.

JVET-R0238 AHG12: A fix on subpicture-level slice indexing [K. Zhang, L. Zhang, Y.-K. Wang, Z. Deng, K. Fan, J. Xu, H. Liu (Bytedance)]

JVET-R0093 AHG12: Subpictures and conformance cropping window [V. Drugeon (Panasonic)]

Item 1 of this contribution belongs to this category.

JVET-R0136 AHG9/AHG12: Improvements on sps_independent_subpics_flag and nal_unit_type constraint [M. Katsumata, M. Hirabayashi, T. Suzuki (Sony)]

Item 1 of this contribution belongs to this category.

JVET-R0186 AHG12: On misc updates for picture partitioning signalling [Hendry, S. Paluri, S. Kim (LGE)]

JVET-R0257 AHG12: Raster scan order flag for subpictures [M. Damghanian, R. Sjöberg, M. Pettersson, Z. Zhang, J. Enhorn, J. Ström, R. Yu (Ericsson)]

JVET-R0284 AHG12/AHG9: On independent subpicture signalling [Y.-J. Chang, Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0337 AHG12: Subpicture with filler slice for merged stream [K. Kawamura, S. Naito (KDDI)]

#### Subpicture layout signalling (4)

JVET-R0117 AHG9/AHG12: On signalling of subpicture and slice in PPS [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-R0118 AHG9/AHG12: On signalling of subpicture partitioning in SPS [B. Choi, S. Wenger, S. Liu (Tencent)]

Item 2 of this contribution belongs to this category.

JVET-R0135 AHG12: On subpicture layout signalling [M. Katsumata, M. Hirabayashi, T. Suzuki (Sony)]

JVET-R0239 AHG9: Cleanups on signalling of tiles, slices, and subpictures [K. Zhang, L. Zhang, Y.-K. Wang, Z. Deng, J. Xu, H. Liu (Bytedance)]

#### Subpicture ID signalling (4)

JVET-R0087 AHG12: Modification of subpicture information in slice header [W. Lim, G. Bang (ETRI)]

JVET-R0088 AHG12: Modification of subpicture information in PPS [W. Lim, G. Bang (ETRI)]

Item 1 of this contribution belongs to this category.

JVET-R0126 AHG9/AHG12: On signalling of subpicture ID [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-R0265 AHG9/AHG12: On subpicture ID mapping signalling [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

#### Subpicture based bitstream extraction and merging (5)

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Item 5 of this contribution belongs to this category.

JVET-R0092 AHG12: Subpicture size calculation for subpicture extraction [V. Drugeon (Panasonic)]

JVET-R0093 AHG12: Subpictures and conformance cropping window [V. Drugeon (Panasonic)]

Item 2 of this contribution belongs to this category.

JVET-R0294 AHG12: On subpicture extraction [K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

JVET-R0148 AHG12: Subpicture merging experiments [A. Hallapuro, M. Homayouni, A. Aminlou, M. M. Hannuksela (Nokia)]

### Slices and tiles (20)

#### Tile signalling (7)

JVET-R0053 AHG9: Signalling tile partitioning [S.-T. Hsiang, C.-M. Tsai, Y.-W. Huang, S.-M. Lei (MediaTek)]

Items 1 and 2 of this contribution belong in this category.

Discussed Monday 1600 UTC (GJS & YKW).

This contribution proposes three high-level syntax modifications related to signalling tile partitioning of the coded picture, summarized as follows:

- The number of explicitly provided tile column widths or tile row heights is proposed to be allowed to be equal to 0. When no_pic_partition_flag is equal to 0 and the coded picture contains only one tile, the proposed method can infer the tile column width and the tile row height to be equal to the picture width and the picture height, respectively, without signalling the syntax elements tile_column_width_minus1[ 0 ] and tile_row_height_minus1[ 0 ] for deriving the tile column width and the tile row height.

R0285 and R0080 are said to contain the same proposal. It is a syntax optimization to avoid signalling tile width or height if the picture is not split into tiles in the corresponding dimension. It was commented that this is consistent with another use of avoiding explicit signalling. It was commented that for uniform tile size signalling, this would increase the signalling that is needed. No action on this was recommended; although it saves a few bits in the PPS in some cases, it adds more in another case that some participants expect to be more common.

- pps_log2_ctu_size_minus5 is proposed to be signalled in the picture parameter set (PPS) only when rect_slice_flag is equal to 0, single_slice_per_subpic_flag is equal to 0 and num_slices_in_pic_minus1 is greater than 0.

A participant commented that it is nice to know pps_log2_ctu_size_minus5, even in other conditions where it is not necessary for parsing. Since it is only two bits, it does not seem important to avoid sending this. No action was recommended on this.

- loop_filter_across_tiles_enabled_flag is proposed to be signalled only when the number of the tiles in the coded picture is greater than 1.

At the previous meeting, it was planned that we would move this flag to the SPS (see notes for JVET-Q0120).

R0113 and an aspect of R0197 are proposing this in principle as well.

It was asked whether bitstream merging would be affected by this, and it was commented that PPSs need to be rewritten in that case anyway.

AHG Recommendation (cleanup/previous plan): confirm the move to the SPS.

JVET-R0062 AHG12: A cleanup on uniform tile and rectangular slice partitioning [L. Zhang, Z. Deng, K. Zhang, Y.-K. Wang (Bytedance)]

Discussed Monday 1515 UTC (GJS & YKW)

This contribution proposes the following changes related to uniform tile and rectangular slice partitioning:

- In the equation (Eqn. 23) for derivation of tile columns parameters, replace the loop count "i < num_exp_tile_columns_minus1" with "i  <=  num_exp_tile_columns_minus1", such that the value of the last explicitly signalled tile_column_width_minus1[ i ] specifies the width of at least one tile column. The semantics of tile_column_width_minus1[ i ] is updated accordingly.

- In the equation (Eqn. 24) for derivation of tile rows parameters, replace the loop count "j < num_exp_tile_rows_minus1" with "j  <=  num_exp_tile_rows_minus1", such that the value of the last explicitly signalled tile_row_height_minus1[ i ] specifies the height of at least one tile row. The semantics of tile_row_height_minus1[ i ] is updated accordingly.

- In the equation (Eqn. 30) for derivation of in-tile rectangular slices parameters, replace the loop count "j < num_exp_slices_in_tile[ i ] − 1" with "j < num_exp_slices_in_tile[ i ]", such that for each value of i, the value of the last explicitly signalled exp_slice_height_in_ctus_minus1[ i ][ j ] specifies the height of at least one rectangular slice in the tile containing the i-th rectangular slice.

The basic desire is to prevent the ability of the encoder to express syntax that seems strange and confusing. It was said that the strange case is not uncommon.

This proposal was viewed favourably. One participant said this is essentially editorial. There is not really a bug – just a lack of prohibiting something strange that is asserted to be useless. The proposal would just prohibit the encoder from signalling silly values that would result in the same decoded result as sensible values. It was said there are other related proposals. AHG Recommendation (cleanup): Adopt.

JVET-R0080 AHG12: On signalling of tile and slice [Y.-U. Yoon, D. H. Park (KAU), J. H. Do (ETRI), J.-G. Kim (KAU)]

Discussed Monday 1650 UTC (GJS & YKW)

VVC (Draft 8) includes signalling for tile and slices in PPS.

This contribution proposes two modifications on signalling of tile and slice information in the PPS as follows.

- Proposal 1: It is proposed to change the condition for signalling the syntax element of tile_idx_delta_present_flag. When the value of num_slices_in_pic_minus1 is greater than 1 instead of 0, the syntax element of tile_idx_delta_present flag is signalled.

This would save one bit when there are only two rectangular slices in the entire picture. Although the savings is very minor, the change is trivial. JVET-R0211 item 1 proposes this as well.

AHG Recommendation: Adopt.

- Proposal 2: It is proposed to replace the syntax element of num_exp_tile_columns_minus1 and num_exp_tile_rows_minus1 as num_exp_tile_columns and num_exp_tile_rows, respectively. When a picture is not partitioned into multiple tiles in rows or columns, the value of num_exp_tile_columns or num_exp_tile_rows is signaled as 0. Then, the syntax element of tile_column_width_minus1 or tile_rows_height_minus1 is not signaled and inferred to be equal to PicWidthInCtbsY-1 or PicHeightInCtbsY-1.

Proposal 2 is the same as item 1 of R0053; see the notes for that item.

JVET-R0054 AHG12: On combination of wavefront parallel processing and tile partitioning [C.-M. Tsai, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Discussed Monday 1700 UTC (GJS & YKW)

[Not really an HLS proposal.]

In VVC Draft 8, parallel processing can be achieved by using wavefront parallel processing (WPP) or tile partitioning, and they are allowed to be simultaneously used within the same picture. However, allowing the combination of WPP and tile partitioning not only increases the effort of decoder verification but also introduces functionality redundancy between WPP and horizontal tile partitioning. In order to reduce the verification effort and remove the functionality redundancy, two methods are proposed in this contribution.

- In Method 1, if WPP is used in the current picture, the number of tile rows in the current picture shall be equal to 1. As a result, the functionality redundancy between WPP and horizontal tile partitioning is removed, and the behavior of CABAC context variable inheritance is also simplified.

- In Method 2, same as in most HEVC profiles (those other than the high-throughput profiles), WPP and tile partitioning are disallowed to be simultaneously used within the same picture. It is claimed that Method 2 is simpler than Method 1 and simplifies the decoder verification significantly.

The proposed restriction is motivated by verification effort. It is a functionality change and was suggested to be too substantial to be able to agree to in the AHG. One participant said that “method 1” would be undesirably restrictive in the case of rectangular slices with wavefronts.

Left open by AHG.

JVET-R0157 AHG9/AHG12: Signalling cleanup on PPS [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

Item 2 of this contribution belongs to this category.

Discussed Tuesday 1520 UTC (GJS & YKW).

It is proposed to skip signalling of tile width and height when the picture width or height is less than or equal to the CTU size. It was noted that a similar provision is applied for subpicture signalling and in the SPS syntax. This would save two bits in each relevant dimension.

It was remarked that something this is also proposed in the 2nd aspect of proposal R0239, with a somewhat simpler editorial expression.

One participant said this seemed like an unnecessary complication for a corner case. In the subpicture case the syntax element is a u(v) rather than ue(v), and it was a somewhat different circumstance.

However, another participant said it was strange to send something and have semantics saying it shall be in the range of 0 to 0.

Software had been provided and the proponent said they had tested it.

No action was recommended on this since the issue is for a very minor corner case.

An editorial bug fix is also proposed for when num_exp_tile_columns_minus1 is equal to 0; however this aspect was no longer relevant due to an action taken on R0062.

JVET-R0221 AHG9: Clean-up of tile signalling [J. Luo, J. Chen, Y. Ye (Alibaba)]

Discussed Tuesday 1555 UTC (GJS & YKW).

In VVC draft 8, the syntax allows indicating a sum of tile widths/heights that is wider than the picture width/height. It is asserted that the constraints of tile partitioning is not straightforward. If the sum of signalled tile widths/heights is larger than picture width/height, the current derivation could cause invalid CTU addresses being added to a slice. In this contribution, it is proposed to add two conformance constraints to make the conformance requirements on tile partitioning cleaner, such that invalid CTU address would not be included in a slice. In the second version, the conformance requirement on tileColBd and tileRowBd is added.

Q0359 was a related proposal of the last meeting. The proponent reported that there was still an editorial error in the constraint expression.

This is an editorial bug fix proposal.

AHG Recommendation (expression of existing intent): The editor is asked to ensure that the text adequately expresses the necessary constraints, such that tiles, slices, and subpictures are a proper partitioning of the picture (no overlaps, no gaps, no CTUs that are outside the picture).

JVET-R0285 AHG12: On tile information signalling [Y.-J. Chang, Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Discussed Tuesday 1625 UTC (GJS & YKW).

In this contribution, there are two proposed changes to the signalling of tile information:

- Replace num_exp_tile_columns_minus1 and num_exp_tile_rows_minus1 with num_exp_tile_columns and num_exp_tile_rows.

- Change the range of num_exp_tile_columns_minus1 to be 0 to PicWidthInCtbsY − 2, inclusive, and the range of num_exp_tile_rows_minus1 to be 0 to PicHeightInCtbsY − 2, inclusive.

The first aspect is the same as in R0053 item 1 and R0080; see notes elsewhere on that.

The second aspect has a somewhat similar spirit to R0062. It is intended to prohibit the encoder from sending something explicitly that could be inferred instead. However, it would not be a strictly necessary change.

“Method 1” would tighten the constraint on num_exp_tile_columns(rows)_minus1. It was commented that “Method 2” seemed like unnecessary complication for a corner case. In “Method 2”, a shortcut is proposed for when the number of tiles columns or rows is equal to the number of CTUs in the picture width or height.

No clear need for action was identified, so no action was taken.

#### Rectangular slice signalling (11)

JVET-R0088 AHG12: Modification of subpicture information in PPS [W. Lim, G. Bang (ETRI)]

Item 2 of this contribution belongs to this category.

Discussed Tuesday 1655 UTC (GJS & YKW).

This contribution proposes to modify syntax elements related to subpicture and slice in PPS. The number of subpictures and slices are dependent according to the current VVC specification. In PPS, those syntax elements are signalled regardless of each other. The following two proposals are described in this document.

Proposal 1) Subpicture-related syntax elements are signalled in PPS when a picture partitioned which refers to the PPS.

Proposal 2) Signaling difference between the number of slices and the number of subpictures instead of the number of slices.

Proposal 2 is to save some bits for signalling in the PPS. It was commented that this is for something sent only once per PPS. It was commented that the number of subpictures, which this uses, is not always available in the PPS.

Proposal 2 was said to be the same as the second item of R0117, in which item 1 proposes to make the number of subpictures unconditionally present in the PPS.

Several participants commented that it seems undesirable to couple the subpicture and slice signalling and use differential signalling, e.g., as the number of bits saved is minimal. Even without the issue of whether the number of subpictures is always present or not, this was expressed.

Another contribution R0162 was said to also be related, which proposes to change num_slices_in_pic_minus1 to num_slices_in_pic_minus2.

Q0332 was also somewhat similar, and it was concluded at the time that it would not provide a bit savings or be substantially beneficial.

The AHG recommended no action on R0088 item 2.

JVET-R0162 AHG9: PPS and SH syntax cleanup [J. Chen, J. Luo, Y. Ye, R.-L. Liao (Alibaba)]

Item 2 (num_slices_in_pic_minus2 signalling) of this contribution belongs to this category.

Discussed Tuesday 1710 UTC (GJS & YKW).

This proposes to change num_slices_in_pic_minus1 to num_slices_in_pic_minus2 in PPS. Aside from a small bit savings (which is not the main motivation), this would prevent a duplicate way of expressing the same thing.

A participant commented that the semantics of single_slice_per_subpic_flag is “one way”, and if this was adopted that would need to be changed, so there would be only one way to express that. It was commented that this seems similar in spirit to the action on each_layer_is_an_ols_flag. Both of these flags are intended as shortcuts for particular cases and making the constraint two-way might make it easier to understand. However, another participant said that although we wanted to have the shortcut, we should not force it to be used just because it is applicable. Another participant said there was a difference between the situation for the each_layer_is_an_ols_flag.

At least one subpicture in all pictures referring to the PPS would need to have at least two slices in it if this is adopted. It is noted that the slice layout is determined in the PPS, so this may not be a significant burden.

There seemed to be no clear need for action, and some participants disliked the removal of flexibility of expression. Others (including the original proponent of the shortcut) thought the two-way constraint would be more sensible and consistency with each_layer_is_an_ols_flag is desirable.

[Stopped session 6 here Tuesday 7 April 1715 UTC.]

[Resumed here with session 10 on Wednesday 8 April at 1530 UTC]

After offline study and further discussion, there continued to be mixed opinions.

JVET-R0111 AHG9/AHG12: Vertical slice boundaries [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

This contribution provides an analysis of using subpictures for server-side composition. It is asserted that it would be beneficial to be able to merge bitstreams from several different sources into a combined bitstream composed of multiple different subpictures. The contribution details some of the parameters and settings that needs to be aligned between different sources in order to support this use case and includes a proposal for one modification asserted to improve feasibility and reduce implementational burden; to allow a tile to include multiple slices either vertically or horizontally (which is different from the current VVC draft where only horizontally structured slices are allowed within a tile).

In summary, the following changes are proposed:

- A new syntax element, vertical_slice_boundaries_flag, for indicating if a tile is split vertically or horizontally.

- Modification to CTU scan derivation to include two cases depending on the value of the proposed flag.

- Update to syntax and decoding process since a vertical slice boundary is no longer required to be aligned with a vertical tile boundary.

- Replace the level limit on number of vertical tile boundaries with a limit on the sum of vertical tile boundaries and vertical slice boundaries.

The proposal would avoid having the encoder need to pre-segment the content into more tiles when authoring the content (which also has a coding efficiency penalty).

This would be a significant design change, as it would require the decoder to track vertical boundary positions that would be different for different rows. There had been an objection to this at the previous meeting as a burden on decoder implementation. It was commented that this would have a significant impact on hardware design.

It was commented that the previously proposed dependent slices concept of N0497 is an alternative approach to provide some degree of similar functionality.

It was commented that there has been usage of MCTSs and a need for highly coordinated encoding in past practice.

Especially given the late stage at which we are in this design process, there were strong objections to the proposed change, so no action was taken on this.

JVET-R0129 AHG9/AHG12: On CTU row based slice chunks of a slice within a tile [L. Chen, C.-W. Hsu, C.-C. Chen, Y.-L. Hsiao, C.-Y. Chen, T.-D. Chuang, C.-M. Tsai, Y.-W. Huang, S.-M. Lei (MediaTek)]

Discussed 8 April at 1630 UTC (GJS & YKW).

This contribution proposes to specify CTU row-based slice chunks of a slice within a tile in decoding order. The slice chunks in sequence of a slice are delivered and decoded sequentially. Each slice chunk is proposed to be contained in a single NAL unit. It is claimed that no essential new requirements are needed in the decoding process. It is claimed that the proposed slice chunks can meet ultra-low latency requirements with better coding efficiency than conventional slices. It is reported the BD-rate savings of the proposed slice chunks compared against the conventional slices are 4.52%, 3.55%, 3.64%, and 4.93% for short length (1 sec) Class A1 (4K), Class A2 (4K), Class B (1080p), and Class E (720p), respectively, when the encoding latency is one CTU row. Proposed reference software was claimed to be ready and cross-checked and was attached in the uploaded proposal package.

This would be something like a special case of the dependent slice segment concept of HEVC. The proponent said dependent slice segments was a useful feature of HEVC, while another participant said they had seldom encountered this feature in practice.

Especially given the late stage at which we are in this design process, there were strong objections to the proposed change, so no action was taken on this.

JVET-R0157 AHG9/AHG12: Signalling cleanup on PPS [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

Item 1 of this contribution belongs to this category.

Item 1 proposes the following cleanups for the PPS, when considering mixed NAL unit types: When mixed_nalu_types_in_pic_flag is equal to 1, no_pic_partition_flag is proposed not to be signalled but inferred to be equal to 0 and rect_slice_flag is proposed not to be signalled but inferred to be equal to 1.

The motivation was said to be primarily to ensure that an invalid combination is not indicated in the PPS.

There was discussion of the desired relative order of mixed_nalu_types_in_pic_flag and no_pic_partition_flag.

It was commented that mixed_nalu_types_in_pic_flag does not affect the decoding process, and this would make the syntax depend on it. It was also commented that no_pic_partition_flag is an important property and it would be undesirable to omit it even when this is hypothetically possible. It was commented that it would be common in a BEAM application to not need to change any aspect of the PPS other than to flip that flag, so semantic constraints were suggested to be sufficient, so no action was recommended on this.

Editor action item: The editor was asked to check and make sure that the constraints (which we believe are already expressed in some form) are sufficiently clear to the reader.

JVET-R0187 AHG12: On signalling for picture with one tile and multiple slices [Hendry, S. Paluri, J. Zhao, S. Kim (LGE)]

It is asserted that in the current picture partitioning signalling scheme when there is only one tile and the slices are rectangular slices, the syntax elements num_slices_in_pic_minus1 and tile_idx_delta_present_flag are not needed. In such situation, the number of slices in the picture can easily be known from the derived variable NumSlicesInTile[ 0 ] and the value of tile_idx_delta_present_flag is never be used.

Furthermore, it is asserted that by not signalling num_slices_in_pic_minus1 in the described scenario above, it would be possible to avoid having the encoder signal an incorrect value for num_slices_in_pic_minus1), i.e. a value that is different from the derived value (i.e., NumSlicesInTile[ 0 ]).

This contribution proposed to omit the signalling of num_slices_in_pic_minus1 and tile_idx_delta_present_flag when no_pic_partition_flag is equal to 0, NumTilesInPic is equal to 1, and rect_slice_flag is equal to 1.

It was commented that this might prevent having tiles that are split into a signalled number of slices.

No action was planned on this unless offline study determines otherwise.

[Stopped here for session 10 on Wednesday 8 April at 1715 UTC.]

JVET-R0188 AHG12: On signalling of rectangular slice height and width [Hendry, S. Kim, S. Paluri (LGE)]

JVET-R0209 AHG12/AHG9: On signalling of rectangular slices [S. Esenlik, B. Wang, A. M. Kotra, E. Alshina (Huawei)]

JVET-R0211 AHG12: Cleanups on rectangular slices signalling [B.-K. Lee (Xris)]

JVET-R0241 AHG12: A direct signalling method of rectangular slice partitioning [K. Zhang, L. Zhang, Y.-K. Wang, Z. Deng, J. Xu, H. Liu (Bytedance)]

JVET-R0247 AHG9: Signalling rectangular slice partitioning [S.-T. Hsiang, C.-W. Hsu, O. Chubach, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

#### Raster-scan slices (2)

JVET-R0047 AHG9/AHG12: On slice address for raster scan slices in a picture [L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0104 AHG9: On Raster-scan Slice Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-R0248 AHG9: Supporting multiple slices within one tile for raster-scan slice mode [S.-T. Hsiang, L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

### Control of loop filtering across subpicture/tile/slice boundaries (6)

JVET-R0044 AHG9: On subpicture boundary handling [J. Li, K. Abe (Panasonic)]

JVET-R0053 AHG9: Signalling tile partitioning [S.-T. Hsiang, C.-M. Tsai, Y.-W. Huang, S.-M. Lei (MediaTek)]

Item 3 of this contribution belongs to this category.

JVET-R0113 AHG9: On Picture Parameter Set [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

Item 1 of this contribution belongs to this category.

JVET-R0069 AHG12: Control of loop filtering across subpicture/tile/slice boundaries [L. Zhang, Y.-K. Wang, K. Zhang (Bytedance), Hendry, N. Park, H. Jang, J. Nam, S. H. Kim, J. Lim (LG Electronics)]

JVET-R0109 AHG9/AHG12: On tile, slice, and related loop filter control flags [L. Chen, C.-W. Hsu, C.-M. Tsai, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0197 AHG12: On signalling of loop filter across tiles and slices enabled flags [N. Park, J. Nam, H. Jang, J. Lim, Hendry, S. Kim (LGE)]

## AHG8: layered coding and resolution adaptivity (29)

### Scalability specific HLS (27)

#### General scalability HLS topics (10)

JVET-R0046 AHG8: Temporal sublayer requirements for multi-layer referencing [C.-Y. Lai, O. Chubach, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-R0065 AHG8/AHG9: On IRAP and GDR AUs [Y.-K. Wang (Bytedance)]

Item 2 of this contribution belongs to this category.

JVET-R0274 AHG8: On CVSS AU [V. Seregin, Y. He, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0066 AHG8/AHG9: On DPB memory allocation and derivation of NoOutputOfPriorPicsFlag [Y.-K. Wang (Bytedance)]

JVET-R0067 AHG8/AHG9: On the derivation of PictureOutputFlag [Y.-K. Wang (Bytedance), M. M. Hannuksela (Nokia)]

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Item 1 of this contribution belongs to this category.

JVET-R0278 AHG8: On SPS sharing and slice type constraint [V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0070 AHG9: On repetition and update of non-VCL data units [Y.-K. Wang, L. Zhang, Z. Deng (Bytedance)]

Item 5 of this contribution belongs to this category.

JVET-R0194 AHG8/AHG9: On parameter set sharing in multi-layered bitstream [Hendry (LGE)]

JVET-R0123 AHG9: On derivation of picture output flag [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

#### Scalability information signalling and related (17)

JVET-R0344 AHG9: A Summary of Proposals Related to Scalability Information Signalling [S. Deshpande (Sharp)] [late]

Discussed Monday 7 April at 1900 (GJS, YKW & JRO).

This contribution intends to provide a summary of proposals on scalability information signalling.

Seventeen proposals from the category “4.3.1.2 Scalability information signalling and related” listed in a revision of JVET-R0339-v4 are included in this summary. Thus in v3, summary is added for JVET-R0158, JVET-R0199, and JVET-R0222 aspect 1.

It is suggested that this summary be used for the reviewing of these proposals, such that the discussions may be done in a more structured and efficient manner.

List of design questions

Related to PTL information signalling

- Omit signaling of index to the list of PTL structures for output layer sets when number of signalled PTL structures is equal to total number of output layer sets and instead infer its value? (JVET-R0161 PTL part of proposal 2, JVET-R0185 proposal 4, JVET-R0204, JVET-R0275 aspect 4)

Note: There may not be an OLS that contains all layers (regardless).

Something similar is in the draft for HRD.

Is the number of OLSs always less than or equal to the number of PTL structures? There is a constraint like that already (regardless of this proposal).

It was commented that this usage seems like it would be common. This is a syntax shortcut for that case.

AHG Recommendation (cleanup): Adopt. Text and software were provided by R0161 proponent (J. Chen).

Related to DPB information signalling

- Modify the upper range of vps_num_dpb_params to allow signalling of DPB parameters for all OLSs from current fixed upper value of 16:

  - upper limit is equal to total number of OLSs minus the number of single-layer OLSs (JVET-R0099 Proposal 1, JVET-R0191 aspect 3). AHG Recommendation (expression of existing intent): Adopt. Text was provided by Hendry and he is also to supply software.

  - upper limit is equal to total number of OLSs − 1 (JVET-R0196)

- Update the range value for num_ols_hrd_params_minus1

  - similarly as in (a) in previous item? (JVET-R0191 aspect 3) AHG Recommendation (expression of existing intent): Adopt. Text was provided by Hendry and he is also to supply software.

  - to total number of OLSs − 2 (JVET-R0204)

- Don't signal and instead infer the index of the dpb_parameters( ) syntax structure that applies to the i-th OLS when a condition is met (JVET-R0099 Proposal 2, JVET-R0204, JVET-R0275 aspect 4)

  - The condition is total number of output layer sets minus number of single layer output layer sets is equal to number of signalled dpb parameters (JVET-R0099 proposal 2). AHG Recommendation (expression of existing intent): Adopt, and also apply to HRD parameters. Text was to be provided by S. Deshpande and he is also to supply software.

  - The condition is total number of output layer sets is equal to number of signalled dpb parameters (JVET-R0161 proposal 2, JVET-R0275 aspect4)

  - The condition is total number of output layer sets is equal to number of signalled dpb parameters + 1 (JVET-R0204)

- Start the for loop which signals ols_dpb_pic_width[ i ], ols_dpb_pic_height[ i ], and ols_dpb_params_idx[ i ] to start at 1 instead of at 0, since 0-th OLS is single layer? (JVET-R0099 Proposal 1, JVET-R0196). AHG Recommendation (expression of existing intent): Adopt (unless affected by proposals to redefine the 0-th OLS).

- Replace if( !vps_all_independent_layers_flag ) condition on vps_num_dpb_params syntax element with if(!each_layer_is_an_ols_flag) (JVET-R0185 proposal 1, JVET-R0196, JVET-R0275 aspect 3). AHG Recommendation (bug fix): Adopt.

  - If above main item is agreed, additionally change vps_num_dpb_params to vps_num_dpb_params_minus1? (JVET-R0185 proposal 2, JVET-R0196, JVET-R0275 aspect 3). It was commented the semantics of each_layer_is_an_ols_flag is a “one-way” constraint. It was asked why we would want to allow the flag to be 0 and still have each layer be an OLS – all of this is in the VPS, so the encoder should know what it is doing when it writes the VPS. AHG Recommendation (bug fix): Adopt and change the semantics to a “two-way” constraint (so if the flag is zero, there must be at least one multilayer OLS specified by the VPS).

  - Additionally signal DPB parameters for OLS in this case only if(!each_layer_is_an_ols_flag) (JVET-R0185 proposal 3). AHG Recommendation (cleanup): Adopt.

Text was provided by Hendry (to be modified for the “two-way” constraint) and he is also to supply software.

Related to HRD information signalling

- Allow control separately if HRD parameters are signalled for an OLSs or not on individual basis? (JVET-R0195).

The proponent indicated that this was motivated by the syntax allowing some HRD parameters to be present and some not for single-layer OLSs. This flexibility is not provided for other cases.

It was commented that the situation for single-layer OLSs was just a consequence of where the data is sent, and noted that each single-layer OLS could be extracted and become a single-layer stand-alone bitstream. So no action was recommended by the AHG on this.

  - If want separate control then signal a separate new flag for each OLS to specify if index to HRD parameters structure is signalled or not? OR

  - Designate 0-th index to mean HRD parameters are not specified for an OLS?

- Change condition for omitting signalling of ols_hrd_idx[ i ] from “num_ols_hrd_params_minus1 + 1  !=  TotalNumOlss” to “num_ols_hrd_params_minus1 + 2  !=  TotalNumOlss”? (JVET-R0204). This item no longer needed to be considered due to the action taken on item 4.a.

- Add a constraint that vps_general_hrd_params_present_flag shall be equal to 1 when more than one layer is included into any OLS? (JVET-R0275). It was commented that HRD parameters presence is optional in the single-layer case and has been optional in AVC and HEVC and their extensions and should be optional, so no action was taken on this.

Common or Combination aspects of PTL, DPB, HRD signalling:

- Constrain that each DPB, HRD, parameter structure signalled in VPS shall be associated with at least one OLS (in the VPS) that contains more than one layer and each PTL structure that is signalled is associated with at least one OLS? (JVET-R0191 Aspect 3). These are just “sensibility” constraints. AHG Recommendation (expression of existing intent): Adopt. Text was provided by Hendry and he is also to supply software.

- Define and use a common gating flag vps_dpb_hrd_params_present_flag and use this to condition presence of dpb_parameters() and ols_hrd_parameters()? (JVET-R0275 aspect 2). The proposal is motivated by a desire for a consistent approach in the VPS and SPS. However, it was commented that the circumstances in the VPS and SPS are different. This item no longer needed to be considered due to the action taken on item 6.a.

- Include PTL signalling in VPS under a common gating flag along with DPB and HRD signalling in VPS? (JVET-R0275 aspect 3). This is similar in spirit to item 11, so no action was taken on this.

Related to max_tid and number of sublayers:

- Signal the syntax elements max_tid_ref_present_flag[ i ], max_tid_il_ref_pics_plus1[ i ] only when ols_mode_idc is not equal to 1 and each_layer_is_an_ols_flag is not equal to 1? (JVET-R0107 Proposal 1). This is a “sensibility” issue – avoiding sending information that is not used. A participant questioned the aspect about ols_mode_idc, and it was discussed whether this information is intended to be metadata or only for sub-bitstream extraction. The proponent said that the syntax in the VPS is intended only to be non-metadata syntax. It was commented that the two syntax elements are already gated by a !vps_independent_layer_flag[ i ] condition, and each_layer_is_an_ols_flag can only be true if all layers are independent. No action seemed needed unless offline study indicates otherwise.

[Stopped session 3 here Monday 6 April 2300 UTC.]

[Resumed the discussion at the start of session 7 Tuesday 7 April 2100 UTC.]

- Assertedly simplify the condition checking for signalling ptl_max_temporal_id[ i ], dpb_max_temporal_id[ i ], and hrd_max_tid[ i ] to only use the flag vps_all_layers_same_num_sublayers_flag instead of using the flag vps_all_layers_same_num_sublayers_flag and vps_max_sublayers_minus1 syntax element. Also assertedly simplify the inference rules for ptl_max_temporal_id[ i ], dpb_max_temporal_id[ i ], and hrd_max_tid[ i ], when not present? (JVET-R0107 Proposal 2)

It was commented that this appears purely editorial – it is just removing checks that are unnecessary.

AHG Recommendation (editorial simplification): The editor is asked to confirm this and remove checks that are unnecessary.

- Change the inferred value of max_tid_il_ref_pics_plus1[] when not present from 7 to vps_max_sublayers_minus1 + 1, to avoid an asserted wrong derivation case for the value of the variable NumSubLayersInLayerInOLS? (JVET-R0119 item 1)

AHG Recommendation (cleanup): Adopt this item. Text was provided by B. Choi, and he is also to supply the software.

- Don't derive the NumSubLayersInLayerInOLS[] and layerIncludedInOlsFlag[][] values, when vps_all_independent_layers_flag is equal to 1? (JVET-R0119 item 2).

It was commented that this appears purely editorial – it is just removing an unnecessary derivation.

AHG Recommendation (editorial simplification): The editor is asked to confirm this and remove the derivation if confirmed editorially undesirable.

- Fix an asserted bug in the iteration loop in eq. (40)? (JVET-R0119 item 3)

AHG Recommendation (editorial bug fix): Correct the error.

- Signal max_tid_il_ref_pics_plus1 value separately for each direct reference layer of a layer, i.e. max_tid_il_ref_pics_plus1[ i ][ j ] for each direct reference layer j less than i, instead of single  max_tid_il_ref_pics_plus1[ i ] as currently? (JVET-R0193)

It was commented that HEVC has a two-dimensional array similar to what is proposed. With the one-dimensional approach, in some cases there may be unnecessary sublayers present after operation of the specified extraction process. The issue is whether the maximum number of sublayers used for interlayer prediction could be different for different layers.

It was commented that if some kind of hypothetical extra metadata is available (e.g. in a system environment or some SEI message), it could provide a more highly optimized extraction capability.

It was commented that the one-dimensional approach was chosen at the previous meeting (see the notes for Q0398), with an understanding that it involved some loss of generality, although there had not been much careful consideration of the question at the time. The amount of complication needed for supporting the greater generality did not seem substantial.

In HEVC, the generality is present in the syntax, and this functionality is used for reference picture list construction but it is not used in the extraction process.

It was commented that the HRD parameters in the bitstream are for the “thin” bitstream – i.e., the bitstream from which all pictures not needed for an OLS have been removed.

AHG Recommendation (cleanup): Adopt. Text was provided in the contribution, and the authors are to supply the software.

- Signal a flag in VPS to indicate that all dependent layers share the same value of max_tid_il_ref_pics_plus1. If the flag is set, signal a common vps_max_tid_il_ref_pics_plus1 for all layers. Otherwise conditionally signal separate values for max_tid_il_ref_pics_plus1[ i ]? (JVET-R0261/ aspect 2)

This is a proposed signalling shortcut in the VPS. We have shortcuts for “vps_all_layers_same_num_sublayers_flag” and “vps_all_independent_layers_flag”. This proposes an additional shortcut “vps_all_layers_same_tid_il_flag” to save repetition of values of max_tid_ref_present_flag[ i ] and max_tid_il_ref_pics_plus1[ i ]. With the adoption of JVET-R0193, this would save some more max_tid_il_ref_pics_plus1 values, since that becomes two-dimensional.

It was commented that we should have conformance bitstreams to test the shortcuts.

The proponent said this does address a common case. Others thought this was unnecessary complication, and the most common case would not use this part of the syntax at all. No action was taken on this.

- Fix an asserted bug for semantics of max_tid_il_ref_pics_plus1[ i ] for special value 0? (JVET-R0107 proposal 3, JVET-R0296 aspect 1)? AHG Recommendation (editorial text bug): Adopt.

Additionally define the semantics for special value 0 to include GDR pictures with recovery_poc_cnt equal to 0 (JVET-R0107 Proposal 3)? It was said that such a GDR picture is functionally equivalent to an IRAP picture. Another participant commented that RPL constraints are different for such a GDR picture, and there was discussion of whether difference is appropriate or not. AHG Recommendation (editorial text bug): Adopt (assuming we don’t disallow GDR pictures with recovery_poc_cnt equal to 0).

- Modify the sub-bitstream extraction process to account for GDR pictures with recovery_poc_cnt equal to 0? (JVET-R0107 Proposal 3) AHG Recommendation (editorial text bug): Adopt (assuming we don’t disallow GDR pictures with recovery_poc_cnt equal to 0).

- Fix an asserted bug in the derivation of NumSubLayersInLayerInOLS by separating the cases for each_layer_is_an_ols_flag is equal to 1 and ols_mode_idc is equal to 0? (JVET-R0296 aspect2). AHG Recommendation (bug fix): Adopt.

Related to Output layer sets and layer dependency:

- Re-define 0-th OLS to include all independent layers when present and every included layer is output? (JVET-R0261 aspect 3). Currently, the 0-th OLS is conceptually a base layer, and there did not seem to be a strong need to change that, so no action was recommended on this by the AHG.

[Stopped session 7 here at 2300 UTC Tuesday 7.]

- Keep the design that the 0-th OLS contains only the lowest layer when each_layer_is_an_ols_flag is equal to 1, the output layer set mode is equal to 0 or the output layer set mode is equal to 1, but relax this when output layer set mode equal to 2 and if so modify the loop and derivation? (JVET-R0306)

- Change vps_all_independent_layers_flag to 2-bit vps_layer_dependency_idc to indicate common layer dependency to align with VPS OLS mode signaling (0 means all layers independently coded, 1 means all non-base layers use ILP, with immediate lower layer as direct reference layer, 2 means general referencing, 3 is reserved)? (JVET-R0261 aspect 1)

- Add a constraint that for each independent layer (i.e., vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1), there shall be an OLS that contains that layer only? (JVET-R0191 item 2).

Other VPS clean-ups:

- Change the coding of ols_ptl_idx[ i ] from u(8)? (JVET-R0161 proposal 1)

  - Option 1: Change to u(v) with length equal to Ceil(Log2(vps_num_ptls_minus1+1))

  - Option 2: Change to ue(v)

- Change the coding of num_output_layer_sets_minus1 from u(8) to u(v) with length eqaul to min( 8, vps_max_layers_minus1 + 1 ) (JVET-R0161 proposal 3)

- Infer vps_layer_id [0] to be equal to nuh_layer_id of the first VCL NAL unit in a bitstream when vps_layer_id [0] is not signaled? (JVET-R0158 aspect 1)

- When VPS is not present:

  - Directly require sps_max_sublayers_minus1 to be in the range of 0 to 6, inclusive? (JVET-R0158 aspect 2)

  - Infer vps_max_sublayers_minus1 to be equal to 6 when sps_video_parameter_set_id is equal to 0 (i.e. VPS is not present). (JVET-R0222 aspect 1)

  - If DCI is present infer vps_max_sublayers_minus1 to be dci_max_sublayers_minus1 or 6 otherwise. (JVET-R0199 aspect 2)

- Constrain the maximum value of vps_max_sublayers_minus1 to be less than or equal to dci_max_sublayers_minus1? (JVET-R0199 aspect 1)

JVET-R0099 AHG8/AHG9: On Output Layer Sets Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-R0107 AHG8/AHG9: On Temporal Sublayers Information [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-R0119 AHG8/AHG9: On derivation of sublayer number in output layer set [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-R0158 AHG9: Semantic bug fixes for syntax elements in VPS and SPS [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

JVET-R0161 AHG8/AHG9: On VPS syntax signalling [J. Chen, J. Luo, Y. Ye, R.-L. Liao (Alibaba)]

JVET-R0185 AHG9: On syntax elements signalling in VPS [S. Paluri, Hendry, S. Kim (LGE)]

JVET-R0191 AHG9: On miscellaneous updates for HLS signalling [Hendry, S. Paluri, S. Kim (LGE)]

Items 2, 3 of this contribution belong to this category.

JVET-R0193 AHG8/AHG9: On signalling of syntax element max_tid_il_ref_pics_plus1 [Hendry, S. Paluri, S. Kim (LGE)]

JVET-R0195 AHG8/AHG9: On HRD structure and OLS mapping signalling in VPS [Hendry (LGE)]

JVET-R0196 AHG8: On signalling of DPB parameters in the VPS [T. Nishi, K. Abe, V. Drugeon (Panasonic)]

JVET-R0199 AHG9: On vps_max_sublayers_minus1 [D. Kim, J. Jung, G. Ko, J. Son, J. Kwak(WILUS)]

JVET-R0204 AHG8: On inference of index for PTL/DPB/HRD parameters in the VPS [T. Nishi, K. Abe, V. Drugeon (Panasonic)]

JVET-R0261 AHG9: On VPS syntax [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-R0275 AHG8: On PTL, HRD, and DPB structures signalling in VPS and SPS [V. Seregin, M. Coban, Y. He, M. Karczewicz (Qualcomm)]

JVET-R0296 AHG9: On sublayer references [Y. Sanchez, R. Skupin, K. Suehring, T. Schierl (HHI)]

JVET-R0306 AHG8/AHG9: On the 0-th OLS for multi-layer bitstream [E. Thomas (TNO)]

### Reference picture resampling (RPR) specific HLS (2)

JVET-R0217 AHG8: On signalling PH RPR scaling window offsets [T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen (Dolby), J. Boyce (Intel), J. N. Shingala (Ittiam)]

JVET-R0114 AHG9: On scaling window offsets [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

Initially discussed in session 12 8 April 2340 UTC in PPS syntax discussion.

It is proposed to allow signalling of negative scaling window offsets so that negative vertical and horizontal offsets can be derived even when the referenced picture did not include a scaling window (JVET-R0114).

It was asserted that this would improve support of zooming. An example use of this was illustrated in the contribution. The change to the text would be just changing ue(v) to se(v) and having a different value range.

It was asked whether this would increase the bit width needed for reference picture referencing. The proponent indicated that this should not be an issue.

It was commented that R0217 is related to the use cases for this.

Revisit with R0217.

# Annex A:
List of meeting participants

The participants of the seventeenth meeting of the JVET, according to an attendance sheet circulated during the meeting sessions (approximately XXX people in total), were as follows: