| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0354-v3 |
| --- | --- |

| Title: | AHG14: BDPCM for Inter/IBC-predicted residuals |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In HEVC, RDPCM, which is a similar coding tool to BDPCM in VVC, can be applied to Intra-predicted residuals, Inter-predicted residuals and IBC-predicted residuals. On the other hand, in current VVC, BDPCM can be applied to only intra-predicted residuals.

This contribution proposes to apply to both Inter-predicted residuals and IBC-predicted residuals to make the design of BDPCM consistency.

Simulation results show that:

For 444/lossless condition (DualTreeOn),

- Average bitrate differences for YUV are -0.53% for AI, -1.14% for RA and -1.32% for LB

- Average bitrate differences for RGB are -0.72% for AI, -1.40% for RA and -1.93% for LB

- Runtimes (Enc, Dec) for YUV are (105%, 102%) for AI, (108%, 98%) for RA and (113%, 98%) for LB

- Runtimes (Enc, Dec) for RGB are (102%, 97%) for AI, (107%, 99%) for RA and (108%, 96%) for LB

For 444/SCC/common QP condition (QP=22, 27, 32, 37, DualTreeOff),

- Average BD-rate differences (Y, U, V) are (-0.07%, 0.12%, -0.01%) for AI, (-0.03%, 0.14%, 0.12%) for RA and (0.04%, 0.14%, 0.19%) for LB

- Average BD-rate differences (R, G, B) are (-0.09%, -0.11%, -0.10%) for AI, (-0.15%, -0.16%, -0.14%) for RA and (-0.19%, -0.20%, -0.21%) for LB

- Runtimes (Enc, Dec) for YUV are (105%, 99%) for AI, (110%, 100%) for RA and (111%, 97%) for LB

- Runtimes (Enc, Dec) for RGB are (102%, 98%) for AI, (107%, 99%) for RA and (109%, 98%) for LB

For 444/SCC/low QP condition (QP=2, 7, 12, 17, DualTreeOff),

- Average BD-rate differences (Y, U, V) are (-0.17%, -0.13%, -0.18%) for AI, (-0.59%, -0.57%, -0.57%) for RA and (-0.33%, -0.32%, -0.33%) for LB

- Average BD-rate differences (R, G, B) are (-0.27%, -0.27%, -0.27%) for AI, (-0.60%, -0.50%, -0.64%) for RA and (-0.44%, -0.45%, -0.44%) for LB

- Runtimes (Enc, Dec) for YUV are (106%, 100%) for AI, (115%, 102%) for RA and (120%, 100%) for LB

- Runtimes (Enc, Dec) for RGB are (103%, 100%) for AI, (108%, 100%) for RA and (113%, 99%) for LB

Introduction

In HEVC [1], RDPCM, which is a similar coding tool to BDPCM in VVC [2], can be applied to Intra-predicted residuals, Inter-predicted residuals and IBC-predicted residuals. On the other hand, in VVC, BDPCM can be applied to only Intra-predicted residuals. At the last meeting, Inter BDPCM was proposed in JVET-Q0460 [3], which provided additional gains in lossless coding.

Table 1-1 shows the summary of RDPCM/BDPCM supported in HEVC, VVC, JVET-Q0460 and JVET-R0354.

It would be desirable to support BDPCM for Inter/IBC predicted residuals like RDPCM in HEVC.

Table 1-1: Summary of supported coding tools

| Method | Supported RDPCM/BDPCM |  |  |
| --- | --- | --- | --- |
|  | Intra-predicted block | Inter-predicted block | IBC-predicted block |
| HEVC [1] | Implicit RDPCM | Explicit RDPCM | Explicit RDPCM |
| VVC WD 8 [2] | Intra BDPCM | N/A | N/A |
| JVET-Q0460 [3] | Intra BDPCM | Inter BDPCM | N/A |
| JVET-R0354 | Intra BDPCM | Inter BDPCM | IBC BDPCM |

Proposal

To make the design of BDPCM consistency, it is proposed to apply BDPCM to both Inter-predicted residuals and IBC-predicted residuals.

Summary of proposed changes are as below:

- Rename intra_bdpcm_{luma, chroma}_flag and intra_bdcpm_dir_{luma, chroma}_flag to bdpcm_{luma, chroma}_flag and bdcpm_dir_{luma, chroma}_flag

- Signal bdpcm_{luma, chroma}_flag for Inter/IBC-predicted residuals if following conditions are satisfied:

- CuPredMode is equal to MODE_INTER or MODE_IBC

- SBT is off (cu_sbt_flag==0)

- sps_bdpcm_enabled_flag is equal to 1

- CU size is less than or equal to MaxTsSize

- ACT is off (cu_act_enabled_flag==0) for Chroma BDPCM for Inter/IBC

- Further signal bdcpm_dir_{luma, chroma}_flag to specify the residual prediction direction if bdpcm_{luma, chroma}_flag is equal to 1

- Add one additional context for each bdpcm_{luma, chroma}_flag and bdcpm_dir_{luma, chroma}_flag

- Reuse entire BDPCM modules (Inverse BDPCM, Forward BPDCM) for Inter/IBC BDPCM

- To reduce encoder runtimes, following fast search is applied:

  - Search Chroma BDPCM for Inter/IBC if either of coded_flag for Cb or Cr is equal to 1

  - Reuse Chroma BDPCM mode in non-JCCR mode for JCCR mode

Highlighted syntax changes are as below:

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( cu_coded_flag ) { |  |
| … |  |
| if( sps_act_enabled_flag && CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && treeType = = SINGLE_TREE ) |  |
| cu_act_enabled_flag | ae(v) |
| if( (CuPredMode[ chType][x0][y0] == MODE_INTER && !cu_sbt_flag) ¦¦ CuPredMode[ chType][x0][y0] == MODE_IBC ){ |  |
| if( sps_bdpcm_enabled_flag && cbWidth <= MaxTsSize && cbHeight <= MaxTsSize) |  |
| bdpcm_luma_flag | ae(v) |
| if( bdpcm_luma_flag ) |  |
| bdpcm_luma_dir_flag | ae(v) |
| if( cbWidth / SubWidthC <= MaxTsSize && cbHeight / SubHeightC <= MaxTsSize && sps_bdpcm _enabled_flag && ChromaArrayType != 0 && ! cu_act_enabled_flag ) |  |
| bdpcm_chroma_flag | ae(v) |
| if( bdpcm_chroma_flag ) |  |
| bdpcm_chroma_dir_flag | ae(v) |
| } |  |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| … |  |

sps_bdpcm_enabled_flag equal to 1 specifies that bdpcm_luma_flag and bdpcm_chroma_flag may be present in the coding unit syntax for coding units with intra prediction mode, or inter prediciton mode, or IBC prediction. sps_bdpcm_enabled_flag equal to 0 specifies that bdpcm_luma_flag and bdpcm_chroma_flag are not present in the coding unit syntax for coding units with intra prediction mode, or inter prediciton mode, or IBC prediction. When not present, the value of sps_bdpcm_enabled_flag is inferred to be equal to 0.

bdpcm_luma_flag equal to 1 specifies that BDPCM is applied to the current luma coding block at the location ( x0, y0 ), i.e. the transform is skipped, the intra luma prediction mode is specified by bdpcm_luma_dir_flag. bdpcm_luma_flag equal to 0 specifies that BDPCM is not applied to the current luma coding block at the location ( x0, y0 ).

When bdpcm_luma_flag is not present it is inferred to be equal to 0.

The variable BdpcmFlag[ x ][ y ][ cIdx ] is set equal to bdpcm_luma_flag for x = x0..x0 + cbWidth − 1, y = y0..y0 + cbHeight − 1 and cIdx = 0.

bdpcm_luma_dir_flag equal to 0 specifies that the BDPCM prediction direction is horizontal. bdpcm_luma_dir_flag equal to 1 specifies that the BDPCM prediction direction is vertical.

The variable BdpcmDir[ x ][ y ][ cIdx ] is set equal to bdpcm_luma_dir_flag for x = x0..x0 + cbWidth − 1, y = y0..y0 + cbHeight − 1 and cIdx = 0.

bdpcm_chroma_flag equal to 1 specifies that BDPCM is applied to the current chroma coding blocks at the location ( x0, y0 ), i.e. the transform is skipped, the chroma prediction mode is specified by bdpcm_chroma_dir_flag. bdpcm_chroma_flag equal to 0 specifies that BDPCM is not applied to the current chroma coding blocks at the location ( x0, y0 ).

When bdpcm_chroma_flag is not present it is inferred to be equal to 0.

The variable BdpcmFlag[ x ][ y ][ cIdx ] is set equal to bdpcm_chroma_flag for x = x0..x0 + cbWidth − 1, y = y0..y0 + cbHeight − 1 and cIdx = 1..2.

bdpcm_chroma_dir_flag equal to 0 specifies that the BDPCM prediction direction is horizontal. bdpcm_chroma_dir_flag equal to 1 specifies that the BDPCM prediction direction is vertical.

The variable BdpcmDir[ x ][ y ][ cIdx ] is set equal to bdpcm_chroma_dir_flag for x = x0..x0 + cbWidth − 1, y = y0..y0 + cbHeight − 1 and cIdx = 1..2.

Detail text changes relative to JVET-Q2001vE are found in JVET-R0354_ DraftText.docx.

Simulation Results

The proposed method is implemented on the VTM-8.0 and tested under following test conditions:

- 444/lossless common test condition [4]

- 444/SCC/common QP condition [5]

- 444/SCC/low QP condition (QP=2,7,12,17) with first 100 frames following [5]

Table 3-1: Coding Performance under 444/lossless condition

| RGB | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 10.9 | 10.9 | -0.59% |
| TGM 720p | 5.0 | 5.1 | -0.27% |
| A | 3.0 | 3.1 | -2.29% |
| M | 3.0 | 3.0 | -0.37% |
| CC | 0.9 | 0.9 | -0.04% |
| Overall | 5.2 | 5.3 | -0.72% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 97% |  |  |
| YUV | All Intra |  |  |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 10.7 | 10.7 | -0.62% |
| TGM 720p | 6.4 | 6.4 | -0.35% |
| A | 3.5 | 3.6 | -0.96% |
| M | 3.9 | 3.9 | -0.52% |
| CC | 1.0 | 1.0 | -0.06% |
| Overall | 5.8 | 5.8 | -0.53% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 102% |  |  |

| RGB | Random Access |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 173.2 | 174.9 | -1.15% |
| TGM 720p | 51.5 | 51.9 | -1.62% |
| A | 8.6 | 8.9 | -1.87% |
| M | 19.1 | 19.5 | -1.74% |
| CC | 0.9 | 0.9 | -0.21% |
| Overall | 61.5 | 62.1 | -1.40% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 98% |  |  |
| YUV | Random Access |  |  |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 174.7 | 176.5 | -1.28% |
| TGM 720p | 64.5 | 65.0 | -1.50% |
| A | 7.9 | 8.0 | -0.94% |
| M | 26.3 | 26.7 | -1.28% |
| CC | 1.1 | 1.1 | -0.22% |
| Overall | 66.3 | 67.0 | -1.14% |
| Enc Time[%] | 111% |  |  |
| Dec Time[%] | 98% |  |  |

| RGB | Low delay B |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 251.2 | 254.2 | -1.47% |
| TGM 720p | 90.1 | 90.9 | -1.92% |
| A | 7.1 | 7.7 | -3.45% |
| M | 21.0 | 21.5 | -2.12% |
| CC | 0.9 | 0.9 | -0.32% |
| Overall | 90.7 | 91.8 | -1.93% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 96% |  |  |
| YUV | Low delay B |  |  |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 252.9 | 256.1 | -1.60% |
| TGM 720p | 115.8 | 116.6 | -1.75% |
| A | 7.9 | 7.9 | -0.68% |
| M | 28.9 | 29.4 | -1.64% |
| CC | 1.1 | 1.1 | -0.35% |
| Overall | 99.2 | 100.3 | -1.32% |
| Enc Time[%] | 113% |  |  |
| Dec Time[%] | 98% |  |  |

Table 3-2: Coding performance under 444/common QP condition

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.15% | -0.22% | -0.21% | 104% | 99% |
| TGM 720p | -0.10% | -0.10% | -0.08% | 100% | 96% |
| Animation | 0.04% | 0.05% | 0.05% | 103% | 100% |
| Mixed content | -0.11% | -0.14% | -0.11% | 101% | 98% |
| Overall | -0.09% | -0.11% | -0.10% | 102% | 98% |
|  | Random access Main10 |  |  |  |  |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.26% | -0.27% | -0.31% | 108% | 100% |
| TGM 720p | -0.13% | -0.15% | -0.12% | 106% | 99% |
| Animation | 0.05% | 0.00% | 0.07% | 109% | 98% |
| Mixed content | -0.23% | -0.18% | -0.14% | 107% | 101% |
| Overall | -0.15% | -0.16% | -0.14% | 107% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.36% | -0.36% | -0.37% | 112% | 99% |
| TGM 720p | -0.13% | -0.03% | -0.16% | 112% | 101% |
| Animation | -0.06% | 0.06% | 0.03% | 105% | 95% |
| Mixed content | -0.17% | -0.47% | -0.30% | 107% | 98% |
| Overall | -0.19% | -0.20% | -0.21% | 109% | 98% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.10% | 0.00% | 0.01% | 105% | 97% |
| TGM 720p | -0.08% | 0.26% | -0.08% | 105% | 98% |
| Animation | -0.02% | 0.04% | -0.03% | 106% | 101% |
| Mixed content | -0.07% | 0.16% | 0.08% | 104% | 99% |
| Overall | -0.07% | 0.12% | -0.01% | 105% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.13% | 0.06% | 0.01% | 112% | 99% |
| TGM 720p | -0.04% | 0.21% | 0.18% | 109% | 99% |
| Animation | 0.05% | 0.10% | 0.04% | 112% | 101% |
| Mixed content | 0.03% | 0.22% | 0.25% | 108% | 100% |
| Overall | -0.03% | 0.14% | 0.12% | 110% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.18% | -0.08% | -0.01% | 112% | 97% |
| TGM 720p | 0.06% | 0.26% | 0.28% | 112% | 98% |
| Animation | 0.12% | 0.20% | 0.13% | 112% | 98% |
| Mixed content | 0.25% | 0.23% | 0.39% | 107% | 97% |
| Overall | 0.04% | 0.14% | 0.19% | 111% | 97% |

Table 3-3: Coding performance under 444/low QP condition

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.50% | -0.49% | -0.51% | 105% | 101% |
| TGM 720p | -0.20% | -0.24% | -0.20% | 103% | 100% |
| Animation | -0.06% | -0.01% | -0.05% | 102% | 101% |
| Mixed content | -0.26% | -0.29% | -0.28% | 102% | 99% |
| Overall | -0.27% | -0.27% | -0.27% | 103% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.72% | -0.76% | -0.79% | 111% | 101% |
| TGM 720p | -0.75% | -0.84% | -0.86% | 107% | 100% |
| Animation | -0.24% | -0.19% | -0.18% | 107% | 100% |
| Mixed content | -0.58% | -0.50% | -0.60% | 107% | 98% |
| Overall | -0.60% | -0.60% | -0.64% | 108% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.81% | -0.84% | -0.80% | 114% | 100% |
| TGM 720p | -0.42% | -0.46% | -0.39% | 112% | 100% |
| Animation | -0.12% | -0.13% | -0.15% | 114% | 96% |
| Mixed content | -0.28% | -0.24% | -0.31% | 110% | 100% |
| Overall | -0.44% | -0.45% | -0.44% | 113% | 99% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.32% | -0.29% | -0.29% | 108% | 101% |
| TGM 720p | -0.16% | -0.07% | -0.16% | 107% | 101% |
| Animation | 0.08% | 0.07% | -0.03% | 103% | 98% |
| Mixed content | -0.21% | -0.18% | -0.20% | 106% | 101% |
| Overall | -0.17% | -0.13% | -0.18% | 106% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.70% | -0.69% | -0.71% | 115% | 100% |
| TGM 720p | -0.86% | -0.57% | -0.46% | 113% | 100% |
| Animation | -0.04% | 0.12% | -0.04% | 115% | 101% |
| Mixed content | -0.64% | -1.10% | -1.04% | 115% | 101% |
| Overall | -0.59% | -0.57% | -0.57% | 114% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.64% | -0.56% | -0.63% | 120% | 100% |
| TGM 720p | -0.33% | -0.30% | -0.25% | 119% | 99% |
| Animation | 0.01% | 0.04% | -0.02% | 122% | 101% |
| Mixed content | -0.23% | -0.36% | -0.34% | 119% | 100% |
| Overall | -0.33% | -0.32% | -0.33% | 120% | 100% |

Conclusion

This contribution proposes to support BDPCM for both Inter-predicted residuals and IBC-predicted residuals to make the design of BDPCM consistency. Simulation results shows that the proposed method has coding performance improvements under 444/lossless coding, 444/SCC/low QP condition and 444/SCC/common QP condition.

It is recommended to adopt the proposed method to the next VTM/VVC WD or further study it toward the future extension of VVC.

Reference

[1] ISO/IEC 23008-2 | ITU-T Rec. H.265, “High efficiency video coding”, ISO/IEC | ITU-T standard, 2019.

[2] B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Jan. 2020.

[3] G. Kulupana, S. Blasi, “Non-CE3: Inter BDPCM for lossless video coding”, JVET-Q0460, Jan. 2020.

[4] T.-C. Ma, A. Nalci, T. Nguyen, “JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding”, JVET-Q2014, Apr. 2020.

[5] Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats”, JVET-Q2013, Apr. 2020.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).