| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0355_r1 |
| --- | --- |

| Title: | On clipping input residuals to IACT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu1, Yi-Wen Chen1, Tsung-Chuan Ma1, Hong-Jheng Jhu1, Xianglin Wang1 Jie Zhao2 Seung Hwan Kim2 Jizheng Xu3, Li Zhang3, Weijia Zhu3, Kai Zhang3 | Email: | xiaoyuxiu@kwai.com jie.zhao@lge.com xujizheng@bytedance.com |
| Source: | Kwai Inc. 1, LG Electronics2, Bytedance Inc.3 |  |  |

_____________________________

- Abstract

In the VVC draft 8, a clipping operation is applied to the residuals of three color components before the inverse adaptive colour transform (IACT) is applied. In the current specification, the clipping range is [-2BitDepth, 2BitDepth-1]. Given that the ACT transform increases the bit-depth of Cg and Co residuals by 1-bit, this contribution proposes to increase the clipping range of Cg and Co by one more bit, i.e., [-2BitDepth+1, 2BitDepth+1-1]. Simulation results show that for RGB sequences the proposed change has negligible BD-rate performance impact on lossy coding and bit-exact bit-rates for lossless coding.

Problem statement and proposal

At the 18-th JVET meeting, reversible YCgCo transform, namely YCgCo-R, was adopted in VVC draft 8 [1] to support both lossless and lossy coding for adaptive color transform (ACT). When the YCgCo-R transform is applied, the bit-depth of the output Cg and Co residuals from the forward ACT transform is increased by 1-bit while the bit-depth of the Y residuals is kept unchanged. In VVC draft 8 [1], the residuals of three colour components are clipped to the range [-2BitDepth, 2BitDepth-1] before the inverse adaptive colour transform (IACT) is applied. However, in the VTM-8.0 [2], such clipping range is [-2BitDepth+1, 2BitDepth+1-1]. Therefore, there is one mismatch between the current specification text and the reference software on the residual clipping range to the IACT. One ticket #993 was submitted in bug tracker for reporting the same mismatch

In this contribution, it is proposed to clip the input residuals to the IACT to the corresponding full dynamic range of each colour component. Specifically, for Cg and Co residuals, the clipping range is increased by one more bit, i.e., [-2BitDepth+1, 2BitDepth+1-1]. For Y residuals, given that their bit-depth is not changed, the corresponding clipping range is [-2BitDepth, 2BitDepth-1].

Proposed specification changes

8.7.4.6 Residual modification process for blocks using colour space conversion

……

The (nTbW)x(nTbH) arrays of residual samples rY, rCb and rCr are modified as follows:

rY[ x ][ y ] = Clip3( −( 1 << BitDepth ), ( 1 << BitDepth ) − 1, rY[ x ][ y ] )	(1220)

rCb[ x ][ y ] = Clip3( −( 1 << (BitDepth + 1) ), ( 1 << (BitDepth + 1) ) − 1, rCb[ x ][ y ] )	(1221)

rCr[ x ][ y ] = Clip3( −( 1 << (BitDepth + 1) ), ( 1 << (BitDepth + 1) ) − 1, rCr[ x ][ y ] )	(1222)

tmp = rY[ x ][ y ] − ( rCb[ x ][ y ] >> 1 )			(1223)

rY[ x ][ y ] = tmp + rCb[ x ][ y ]                             	(1224)

rCb[ x ][ y ] = tmp − ( rCr[ x ][ y ] >> 1 )                      	(1225)

rCr[ x ][ y ] += rCr[ x ][ y ]                                                     	(1226)

It is noted that the proposed change is necessary to support the lossless coding for the ACT mode.

Simulation results

Lossy coding results (non-420 CTC)

Table 1 BD-rate performance of the proposed method on RGB SCC sequences

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p |  | 0.00% | 0.00% |  | 0.00% |  | 102% |  | 100% |
| TGM 720p |  | 0.00% | 0.00% |  | 0.00% |  | 98% |  | 98% |
| Animation |  | 0.00% | 0.01% |  | 0.00% |  | 104% |  | 101% |
| Mixed content |  | -0.02% | 0.00% |  | 0.00% |  | 106% |  | 106% |
| Overall |  | 0.00% | 0.00% |  | 0.00% |  | 102% |  | 101% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p |  | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 100% |
| TGM 720p |  | 0.00% | 0.00% |  | 0.01% |  | 100% |  | 99% |
| Animation |  | 0.00% | 0.00% |  | 0.00% |  | 104% |  | 103% |
| Mixed content |  | 0.01% | 0.00% |  | -0.01% |  | 100% |  | 101% |
| Overall |  | 0.00% | 0.00% |  | 0.00% |  | 101% |  | 101% |

Table 2 BD-rate performance of the proposed method on RGB natural sequences

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | 0.00% | 0.00% | 0.00% | 94% | 98% |
| Kimono | 0.00% | 0.00% | 0.00% | 107% | 105% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 98% | 102% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 100% | 82% |
| VenueVu | 0.00% | 0.00% | 0.00% | 80% | 84% |
| DucksAndLegs | 0.00% | 0.00% | 0.00% | 107% | 117% |
| OldTownCross | 0.00% | 0.00% | 0.00% | 101% | 110% |
| ParkScene | 0.00% | 0.00% | 0.00% | 101% | 110% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 98% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | 0.00% | 0.00% | 0.00% | 96% | 93% |
| Kimono | 0.00% | 0.00% | 0.00% | 100% | 99% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 100% | 99% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 96% | 104% |
| VenueVu | 0.00% | 0.00% | 0.00% | 99% | 98% |
| DucksAndLegs | 0.00% | 0.00% | 0.00% | 99% | 101% |
| OldTownCross | 0.00% | 0.00% | 0.00% | 116% | 109% |
| ParkScene | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |

Lossy coding results (non-420 CTC + MR1488/1492)

Table 3 BD-rate performance of the proposed method on RGB SCC sequences

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM 720p | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Mixed content | 0.00% | -0.01% | 0.00% | 102% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.01% | 0.00% | 0.00% | 98% | 100% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Animation | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Mixed content | 0.01% | 0.00% | 0.01% | 99% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |

Table 4 BD-rate performance of the proposed method on RGB natural sequences

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | 0.00% | 0.00% | 0.00% | 110% | 127% |
| Kimono | 0.00% | 0.00% | 0.00% | 107% | 116% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 104% | 102% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 104% | 100% |
| VenueVu | 0.00% | 0.00% | 0.00% | 98% | 97% |
| DucksAndLegs | 0.00% | 0.00% | 0.00% | 93% | 94% |
| OldTownCross | 0.00% | 0.00% | 0.00% | 94% | 84% |
| ParkScene | 0.00% | 0.00% | 0.00% | 112% | 117% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | #VALUE! | #VALUE! | #VALUE! | 0% | 0% |
| Kimono | 0.00% | 0.00% | 0.00% | 96% | 97% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 99% | 100% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 98% | 97% |
| VenueVu | 0.00% | 0.00% | 0.00% | 103% | 98% |
| DucksAndLegs | 0.00% | 0.00% | 0.00% | 102% | 101% |
| OldTownCross | 0.00% | 0.00% | 0.00% | 100% | 105% |
| ParkScene | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Lossless coding results

| RGB | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-7.0 | VTM-8.0 |  | VTM-7.0 | VTM-8.0 |  | VTM-7.0 | VTM-8.0 |  |
| TGM 1080p | 21.7 | 21.7 | 0.00% | 43.3 | 43.3 | 0.00% | 62.8 | 62.8 | 0.00% |
| TGM 720p | 10.1 | 10.1 | 0.00% | 12.9 | 12.9 | 0.00% | 22.5 | 22.5 | 0.00% |
| A | 6.0 | 6.0 | 0.00% | 2.2 | 2.2 | 0.00% | 1.8 | 1.8 | 0.00% |
| M | 6.0 | 6.0 | 0.00% | 4.8 | 4.8 | 0.00% | 5.3 | 5.3 | 0.00% |
| CC | 1.7 | 1.7 | 0.00% | 0.2 | 0.2 | 0.00% | 0.3 | 0.3 | 0.00% |
| Overall | 10.4 | 10.4 | 0.00% | 15.4 | 15.4 | 0.00% | 22.7 | 22.7 | 0.00% |
| Enc Time[%] | 103% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 99% |  |  | 100% |  |  |

Reference

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Jan. 2020, Brussels, Belgium.

VTM-8.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-8.0.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).