| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April. 2020 | Document: JVET-R0369-v2 |
| --- | --- |

| Title: | Combination of JVET-R0367 and JVET-R0368 for GPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yanzhuo Ma, Qihong Ran, Ruipeng Qiu, Junyan Huo, Shuai Wan, Fuzheng Yang, Yuanfang Yu, Yang Liu | Email: | yzma@mail.xidian.edu.cn qhRan@stu.xidian.edu.cn rpqiu@stu.xidian.edu.cn jyhuo@mail.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

# Abstract

Proposed is a combination of the methods proposed in JVET-R0367[1] and JVET-R0368[2].

The experimental results are as below:

For RA configuration: -0.10 %, -0.09%, and -0.15%.

For LB configuration: x.xx %, x.xx%, and x.xx%.

Introduction

In VVC Draft 8[3], The variable shiftHor is calculated by the following equations:

hwRatio = nH / nW		                                              (1)

shiftHor = ( angleIdx % 16  = =  8  | |  ( angleIdx % 16  !=  0  &&  hwRatio > 0 ) ) ? 0 : 1 (2)

And to maximize the common operations shared by different merge modes, the regular merge list is constructed and its motion vectors are used for geometric prediction. Once the regular merge list is constructed, for each merge candidate in the regular merge candidate list, one and only one of its L0 or L1 motion vector is used for GPM. In addition, the order of selecting the L0 vs. L1 motion vector is based on its merge index parity as shown in Figure 1.

Figure 1. Uni-prediction MV selection for GPM in VTM8.0

In JVET-R0367[2], it is proposed that the calculation method of shiftHor is modified as follow:

hwRatio = nH / nW

shiftHor = ( angleIdx % 16  = =  8  | |  ( angleIdx % 16  !=  0  &&  hwRatio > 0 ) ) ? 0 : 1

shiftHor = (nH << (angleIdx%16 > 8?(16 - angleIdx%16) : angleIdx%16)) < (nW<<4) ?1:0 (3)

In JVET-R0368[3], it is proposed to construct the GPM unidirectional merge list with all the unique candidates in regular merge list.

For each merge candidate in the regular merge candidate list, its L0 and L1 motion vectors are filled into the unidirectional GPM merge candidate list in the merge index increasing order. In addition, the unavailable unidirectional candidates and repeated candidates should be skipped.

Figure 2 gives an example, the blue area in the regular merge candidate list represents the available unidirectional candidates, the white area represents unavailable unidirectional candidates, and the colors with different shade represents different unidirectional MV values. At last, unidirectional zero motion vectors may be added to make the merge list full.

Figure 2. A unidirectional GPM merge candidate list construction example with MaxNumGpmMergeCand=5

Proposed methods

In this contribution, a combination of JVET-R0367 and JVET-R0368 is proposed, that is,

- the calculation method of shiftHor was modified as follow:

hwRatio=nH/nW

shiftHor=(angleIdx%16==8|angleIdx%16!=0&&hwRatio>0?0:1

shiftHor=nH≪angleIdx%16>8 ? 16-angleIdx%16 :angleIdx%16<nW≪4?1:0

- to construct the GPM unidirectional merge list with all the unique unidirectional candidates in regular merge list.

Experimental results

The proposed method is integrated into VTM-8.0 and the results of the proposed implementation under CTC [4] are shown below against VTM8.0[5].

Table 1. Results of proposed method under Random Access configuration

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.04% | -0.12% | 100% | 100% |
| Class A2 | -0.07% | -0.05% | -0.03% | 100% | 100% |
| Class B | -0.07% | -0.09% | -0.10% | 100% | 100% |
| Class C | -0.20% | -0.15% | -0.33% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.10% | -0.09% | -0.15% | 100% | 100% |
| Class D | -0.13% | -0.13% | -0.26% | 100% | 100% |
| Class F | -0.08% | -0.10% | -0.12% | 101% | 100% |

Table 2. Results of proposed method under Low delay B configuration

|  |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.22% | -0.35% | -0.08% | 101% | 91% |
| Class E | -0.17% | 0.07% | 0.03% | 94% | 90% |
| Overall |  |  |  |  |  |
| Class D | -0.15% | -0.76% | -0.47% | 102% | 98% |
| Class F | -0.19% | 0.06% | -0.34% | 89% | 88% |

Conclusion

This contribution presents a combination of JVET-R0367 and JVET-R0368 for GPM variable shiftHor calculation and GPM unidirectional merge candidate list construction. The performance of the combined method is -0.10% (RA) and -0.xx% (LDB) over VTM8.0 with similar encoding and decoding time. It is recommended to adopt this method in the next version of VVC.

References

- Y.-Z. Ma, Q.-H. Ran, R.-P. Qiu et al .“Adjustment of shiftHor calculation in GPM”, Document JVET-R0367, by teleconference, April 2020.

- Y.-Z. Ma, Q.-H. Ran, R.-P. Qiu et al .“GPM merge list construction modification”, Document JVET-R0368, by teleconference, April 2020.

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, Verstatile Video Coding (Draft 8), Document JVET-Q2001, Version 15 , Brussels, Belgium, January 2020

- F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, Document JVET-N1010, Geneva, Switzerland, March 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-8.0

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).