| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0380-v3 |
| --- | --- |

| Title: | Scaling list for adaptive colour transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya Karam Naser Philippe de Lagrange Fabrice Le Léannec Philippe Bordes | Tel: Email: | iwamura.s-gc@nhk.or.jp karam.naser@interdigital.com |
| Source: | NHK, InterDigital |  |  |

_____________________________

- Abstract

In this contribution, modifications of scaling matrix derivation are proposed. In VVC Draft 8, adaptive colour transform (ACT) for 4:4:4 can be controlled at the CU level flag (cu_act_enabled_flag). Although the energy distribution of transform coefficients in RGB and YCgCo domain would be different, in VVC draft 8, user defined scaling lists are always shared for G and Y, for B and Cg, for R and Co, which may cause undesirable behaviour of quantization process. To avoid the undesirable behaviour, this contribution proposes to introduce additional flag (scaling_matrix_act_disabled_flag) in APS to disable scaling matrix for the block coded with ACT.

V2 document add solution #2 which repurposes the existing “scaling_matrix_for_lfnst_disabled_flag” to disable scaling list for both LFNST and ACT coded block.

V3 document add solution #3 and #4, which add a new SPS flag instead of APS.

Introduction

In VVC Draft 8, adaptive colour transform (ACT) is introduced for 4:4:4 RGB video signals. ACT can be controlled at the CU level flag (cu_act_enabled_flag). When ACT is applied to the current CU, colour space of the residual signals is converted from RGB (GBR) to YCgCo. In general, correlation between residual signals of three components in RGB domain is stronger than that in YCgCo domain, which means that the energy distribution of transform coefficients in RGB and YCgCo domain would be different.

However, in VVC draft 8, user defined scaling lists are always shared for G and Y, for B and Cg, for R and Co, which may cause undesirable behaviour of quantization process.

In similar to scaling_matrix_lfsnt_disabled_flag, new flag in APS, or reusing LFNST flag, for disabling scaling matrix for the block coded with ACT is proposed in this contribution. It alternatively proposes adding a new in SPS instead of APS to reduce the signaling overhead.

Proposed solution 1

Proposed solution simply disables scaling matrix for block coded with ACT by signalling scaling_matrix_for_act_disabled_flag in scaling_list_data(). When scaling_matrix_for_act_disabled_ flag is set equal to 1, the intermediate scaling factor m[ x ][ y ] is set equal to 16 even if scaling_list_enbled flag is equal to 1.

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| scaling_matrix_for_lfnst_disabled_flag | u(1) |
| scaling_list_chroma_present_flag | u(1) |
| if ( scaling_list_chroma_present_flag ) |  |
| scaling_matrix_for_act_disabled_flag | u(1) |
| for( id = 0; id < 28; id ++ ) |  |
| … |  |

scaling_matrix_for_act_disabled_flag equal to 1 specifies that scaling matrices are not applied to blocks coded with ACT. scaling_matrix_for_act_disabled_flag equal to 0 specifies that the scaling matrices may apply to the blocks coded with ACT.

In the scaling process for transform coefficients, one condition for deriving intermediate scaling factor m[ x ][ y ] is added as follows:

If one or more of the following conditions are true, m[ x ][ y ] is set equal to 16.

- slice_explicit_scaling_list_used_flag is equal to 0.

- transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1.

- scaling_matrix_for_lfnst_disabled_flag is equal to 1 and ApplyLfnstFlag is equal to 1.

- scaling_matrix_for_act_disabled_flag is equal to 1 and cu_act_enabled_flag[ xTbY ][ yTbY ] is equal to 1.

Proposed solution 2

Instead of adding a new flag in APS, the existing LFNST flag “scaling_matrix_for_lfnst_disabled_flag” can be repurposed for disabling the scaling list for CU’s employing ACT and LFNST. This provides the encoder with the option to disable scaling list for “non-typical” residuals resulting from LFNST or ACT. The corresponding specification changes are:

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| scaling_matrix_for_lfnst_act_disabled_flag | u(1) |
| … |  |

In the scaling process for transform coefficients, one condition for deriving intermediate scaling factor m[ x ][ y ] is added as follows:

If one or more of the following conditions are true, m[ x ][ y ] is set equal to 16.

- slice_explicit_scaling_list_used_flag is equal to 0.

- transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1.

- scaling_matrix_for_lfnst_act_disabled_flag is equal to 1 and ApplyLfnstFlag is equal to 1.

- scaling_matrix_for_lfnst_act_disabled_flag is equal to 1 and cu_act_enabled_flag[ xTbY ][ yTbY ] is equal to 1.

Proposed solution 3

Instead of controlling the scaling list for ACT at APS level, it is proposed here to perform it at SPS level. That is, when ACT and scaling lists are activated, an SPS flag is signaled to specify whether to use scaling list for ACT blocks. The corresponding specification changes are:

| seq_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| ... |  |  |
| sps_explicit_scaling_list_enabled_flag | u(1) |  |
| ... |  |  |
| sps_act_enabled_flag | u(1) |  |
| if ( sps_act_enabled_flag && sps_explicit_scaling_list_enabled_flag ) |  |  |
| sps_scaling_matrix_for_act_disabled_flag | u(1) |
| ... |  |  |

The semantics of this flags are:

sps_scaling_matrix_for_act_disabled_flag equal to 1 specifies that scaling matrices are not applied to blocks coded with ACT. sps_scaling_matrix_for_act_disabled_flag equal to 0 specifies that the scaling matrices may be applied to the blocks coded with ACT. When not present, the value of scaling_matrix_for_act_disabled_flag is inferred to be equal to 1.

The decoding process is modified as follows:

- The intermediate scaling factor m[ x ][ y ] is derived as follows:

- If one or more of the following conditions are true, m[ x ][ y ] is set equal to 16:

- slice_explicit_scaling_list_used_flag is equal to 0.

- transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1.

- scaling_matrix_for_lfnst_disabled_flag is equal to 1 and ApplyLfnstFlag is equal to 1.

- sps_scaling_matrix_for_act_disabled_flag is equal to 1 and cu_act_enabled_flag[ xTbY ][ yTbY ] is equal to 1.

Proposed solution 4

Similar to Proposed solution 2, it is proposed here to have a single control for both LFNST and ACT scaling lists. However, instead of APS control, SPS control is proposed here. This is based on the modifications specified in JVET-R0064, where the APS flag “scaling_matrix_for_lfnst_disabled_flag” is moved to SPS level. The main motivation is to avoid redundant signaling for this flag when the encoder typically chose a constant option whether to use or not scaling lists for LFSNT. The combined specification changes for both LFNST and ACT are:

| seq_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| ... |  |  |
| sps_lfnst_enabled_flag | u(1) |  |
| ... |  |  |
| sps_act_enabled_flag | u(1) |  |
| if ( (sps_lfnst_enabled_flag ¦¦ sps_act_enabled_flag) && sps_explicit_scaling_list_enabled_flag ) |  |  |
| sps_scaling_matrix_for_lfnst_act_disabled_flag | u(1) |
| ... |  |  |

The semantics of this flags are:

sps_scaling_matrix_for_lfnst_act_disabled_flag equal to 1 specifies that scaling matrices are not applied to blocks coded with LFNST or ACT. sps_scaling_matrix_for_lfnst_act_disabled_flag equal to 0 specifies that the scaling matrices may be applied to the blocks coded with LFNST or ACT. When not present, the value of scaling_matrix_for_lfnst_act_disabled_flag is inferred to be equal to 1.

The decoding process is modified as follows:

- The intermediate scaling factor m[ x ][ y ] is derived as follows:

- If one or more of the following conditions are true, m[ x ][ y ] is set equal to 16:

- slice_explicit_scaling_list_used_flag is equal to 0.

- transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1.

- sps_scaling_matrix_for_lfnst_act_disabled_flag is equal to 1 and ApplyLfnstFlag is equal to 1.

- sps_scaling_matrix_for_lfnst_act_disabled_flag is equal to 1 and cu_act_enabled_flag[ xTbY ][ yTbY ] is equal to 1.

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).