| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Alpbach, AT, 15–24 April 2020 | Document: JVET-R0435-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-R0281 (AHG16: Cleanup MIP flag signalling) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Pfaff | Tel: Email: | +49 jonathan.pfaff@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the crosscheck results for JVET-R0281 (AHG16: Cleanup MIP flag signalling). In this document, three modification of the context modelling for the matrix-based intra prediction (MIP) –flag are proposed. The first method proposes to not use an above CU for the context-modelling of the MIP-flag if the above CU belongs to a different CTU. The second method proposes to never use an above CU for the context-modelling of the MIP-flag. The third method proposes to never use an above CU and to never use a left CU for the context modelling of the MIP-flag.  The crosschecker verified that the software matches the description and the proposed changes in the draft text. The simulation results reportedly match those provided by the proponents.

# Experimental results

## Experimental results of method 1

|  |  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  | 0,01% |  | 0,08% |  | -0,04% |  | 99% |  | 99% |  |
| Class A2 |  | 0,01% |  | 0,01% |  | -0,03% |  | 100% |  | 100% |  |
| Class B |  | 0,00% |  | 0,05% |  | 0,06% |  | 100% |  | 99% |  |
| Class C |  | 0,00% |  | -0,09% |  | 0,02% |  | 100% |  | 98% |  |
| Class E |  | 0,00% |  | -0,04% |  | -0,08% |  | 101% |  | 97% |  |
| Overall |  | 0,00% |  | 0,00% |  | 0,00% |  | 100% |  | 98% |  |
| Class D |  | 0,01% |  | -0,03% |  | 0,07% |  | 101% |  | 101% |  |
| Class F |  | 0,00% |  | 0,00% |  | 0,00% |  | 100% |  | 99% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 | -0,01% |  | 0,07% |  | -0,11% |  | 101% |  | 103% |  |
| Class A2 | -0,01% |  | -0,08% |  | 0,07% |  | 101% |  | 100% |  |
| Class B | -0,02% |  | -0,04% |  | -0,08% |  | 101% |  | 100% |  |
| Class C | -0,02% |  | -0,04% |  | -0,15% |  | 101% |  | 102% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0,01% |  | -0,02% |  | -0,07% |  | 101% |  | 101% |  |
| Class D | 0,00% |  | -0,05% |  | 0,04% |  | 103% |  | 102% |  |
| Class F | 0,06% |  | -0,04% |  | -0,04% |  | 99% |  | 98% |  |

## Results of Method 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | 0,03% | 0,04% | 98% | 99% |
| Class A2 | 0,00% | 0,06% | 0,01% | 99% | 99% |
| Class B | 0,00% | 0,01% | 0,06% | 99% | 99% |
| Class C | 0,01% | -0,10% | 0,03% | 100% | 99% |
| Class E | 0,00% | -0,02% | 0,01% | 97% | 95% |
| Overall | 0,00% | -0,01% | 0,04% | 99% | 98% |
| Class D | 0,01% | -0,05% | -0,04% | 101% | 99% |
| Class F | -0,01% | 0,05% | -0,03% | 97% | 98% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | -0,05% | -0,03% | 102% | 105% |
| Class A2 | -0,03% | -0,06% | 0,10% | 103% | 99% |
| Class B | -0,03% | -0,03% | 0,03% | 101% | 100% |
| Class C | 0,02% | 0,01% | -0,05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,01% | -0,03% | 0,01% | 101% | 101% |
| Class D | 0,00% | 0,10% | 0,25% | 102% | 103% |
| Class F | 0,03% | -0,24% | -0,01% | 100% | 98% |

## Results of Method 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | 0,07% | -0,01% | 98% | 100% |
| Class A2 | 0,00% | 0,00% | 0,01% | 99% | 99% |
| Class B | 0,00% | 0,02% | 0,01% | 98% | 98% |
| Class C | 0,01% | -0,05% | -0,03% | 99% | 101% |
| Class E | 0,02% | 0,02% | -0,05% | 99% | 98% |
| Overall | 0,01% | 0,01% | -0,01% | 99% | 99% |
| Class D | 0,00% | -0,07% | -0,07% | 99% | 102% |
| Class F | -0,02% | 0,05% | 0,00% | 100% | 99% |
| Class TGM | #WERT! | #WERT! | #WERT! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | -0,02% | -0,14% | 101% | 104% |
| Class A2 | -0,03% | 0,02% | 0,14% | 100% | 100% |
| Class B | 0,01% | -0,02% | -0,07% | 101% | 100% |
| Class C | 0,01% | -0,07% | -0,22% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0,00% | -0,03% | -0,08% | 101% | 101% |
| Class D | 0,05% | 0,31% | -0,13% | 101% | 102% |
| Class F | -0,01% | -0,20% | -0,11% | 99% | 98% |
| Class TGM | #WERT! | #WERT! | #WERT! | #DIV/0! | #DIV/0! |