| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0460 |
| --- | --- |

| Title: | Crosscheck of JVET-R0331 (Cleanup of LFNST signalling) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kyohei Unno Kei Kawamura 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-6887-2592 yo-kidani@kddi.com ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution is a cross-check report of JVET-R0331. The proposal introduces three alternative methods of LFSNT index signalling as follows:

- Method 1 proposes to remove redundant checking regarding significant coefficient position at only DC component region from single tree chroma coding blocks.

- Method 2 proposes to move the LFNST index parsing position from CU-based to TU-based signalling processes, and to remove the redundant checking of the significant coefficient position at both DC component and zero-out regions and transform skip flag for single tree chroma coding blocks regarding LFNST index signalling.

- Method 3 proposes to disable LFNST application for dual tree chroma coding blocks.

Introduction

In WD8 and VTM-8.0, the signalling of LFNST index is conditioned by significant coefficient positions of both luma and chroma coding blocks in single tree even though there is no need to check those of chroma coding blocks since LFSNT is prohibited in chroma coding blocks. JVET-R0331 [1] proposes three alternative methods of LFNST index signalling as follows:

- Method 1 proposes to remove redundant checking regarding significant coefficient position at only DC component region from single tree chroma coding blocks.

- Method 2 proposes to move the LFNST index parsing position from CU-based to TU-based signalling processes, and to remove the redundant checking of the significant coefficient position at both DC component and zero-out regions and transform skip flag for single tree chroma coding blocks regarding LFNST index signalling. Furthermore, along with the signalling modification, redundant checking in RDO is removed.

- Method 3 proposes to disable LFNST application for dual tree chroma coding blocks.

An implementation of the provided software is consistent with the description in the contribution.

The anchor result was generated by using the common test condition [2]. AI and RA configuration are evaluated. The provided software was compiled on 64-bit CentOS environment. The experiments were performed on the cluster PCs.

Method 1

Table 1 All intra configuration results of JVET-R0331 Method 1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 2 Random access configuration results of JVET-R0331 Method 1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.08% | 0.00% | 100% | 100% |
| Class A2 | -0.01% | 0.10% | 0.13% | 100% | 100% |
| Class B | -0.01% | -0.04% | 0.00% | 100% | 100% |
| Class C | -0.01% | 0.04% | 0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.04% | 100% | 100% |
| Class D | 0.00% | 0.05% | -0.01% | 100% | 101% |
| Class F | 0.00% | 0.05% | -0.04% | 100% | 100% |

Method 2

Table 3 All intra configuration results of JVET-R0331 Method 2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 4 Random access configuration results of JVET-R0331 Method 2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.07% | 0.03% | 100% | 99% |
| Class A2 | 0.01% | -0.03% | 0.10% | 100% | 100% |
| Class B | 0.01% | -0.02% | 0.04% | 100% | 100% |
| Class C | -0.02% | 0.09% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | 0.05% | 100% | 100% |
| Class D | -0.01% | 0.04% | -0.01% | 100% | 100% |
| Class F | 0.01% | 0.03% | 0.00% | 100% | 100% |

Method 3

Table 5 All intra configuration results of JVET-R0331 Method 3

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 1.56% | 1.97% | 95% | 100% |
| Class A2 | -0.04% | 1.58% | 1.60% | 92% | 99% |
| Class B | 0.12% | 1.05% | 1.47% | 97% | 100% |
| Class C | 0.25% | 0.66% | 0.92% | 94% | 100% |
| Class E | 0.22% | 1.08% | 0.98% | 96% | 101% |
| Overall | 0.12% | 1.14% | 1.37% | 95% | 100% |
| Class D | 0.26% | 0.86% | 0.98% | 94% | 99% |
| Class F | 0.23% | 0.54% | 0.49% | 97% | 100% |

Table 6 Random access configuration results of JVET-R0331 Method 3

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | 0.39% | 0.38% | 100% | 99% |
| Class A2 | 0.01% | 0.38% | 0.54% | 100% | 99% |
| Class B | 0.04% | 0.58% | 0.76% | 100% | 100% |
| Class C | 0.14% | -0.46% | -0.49% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.22% | 0.30% | 100% | 100% |
| Class D | 0.18% | 0.58% | 0.69% | 100% | 100% |
| Class F | 0.23% | -0.16% | 0.20% | 100% | 100% |

Conclusion

We had verified results reported in JVET-Q0331. The results of the cross-check matched with proponent’s report.

This work was conducted under the project "Research and Development of Next generation video transmission and communication technology for ultra-high definition video application under versatile purpose and environment" of the Ministry of Internal Affairs and Communications (MIC), Japan.

References

- X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang, “Cleanup of LFNST signalling”, JVET-R0331, by teleconference, 15–24 April 2020.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

KDDI does not have any current or pending patent rights relating to the technology described in this contribution.