| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0461 |
| --- | --- |

| Title: | AHG4: Candidate test sequences for verification tests |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Mathias Wien RWTH Aachen University Vittorio Baroncini VABTECH ltd | Tel: Email: | +49-241-80-27867 wien@lfb.rwth-aachen.de baroncini@gmx.com |
| Source: | RWTH Aachen University, VABTECH |  |  |

_____________________________

# Abstract

This document discussed the suitability of SDR test sequences in UHD resolution for usage in the VVC verification tests according to the draft verification test plan described in JVET-R2009. Available test sequences are collected and characterized by means of RD measurements based on HM16.20 in RA configuration and visual inspection. A number of about seven sequences is identified to be inspected further. A method is suggested to perform online viewing sessions in order to proceed in the selection process during the meeting. It is noted that the collection of sequences reported in this document only addresses the SDR UHD test case so far. Contributions and suggestions from JVET experts are solicited, including potential test sequences, but also support with VTM simulations.

# Introduction

This document discussed the suitability of SDR test sequences in UHD resolution for usage in the VVC verification tests according to the draft verification test plan described in JVET-R2009. Available test sequences are collected and characterized by means of RD measurements and visual inspection to narrow down the list of candidate sequences for the verification tests. A further goal is to converge towards rate points suitable for the visual tests. Simulations with the HEVC test model software HM-16.20 have been performed for an initial assessment of the rate-distortion behaviour of potential candidate test sequences. At a physical meeting, experts viewing sessions could have been performed on this material to collect insight from subjective assessment. A potential method to conduct “Visual Inspection” sessions under the given online meeting conditions is proposed.

It is noted that this collection of sequences only addresses the SDR UHD test case so far. It does not claim to be complete and further interesting material, such as the sequences presented in JVET- Q0791 is expected to be provided and included in the selection process. Contributions and suggestions from JVET experts are solicited, including potential test sequences, but also support with VTM simulations.

# Online viewing for visual inspection of coded test sequences

Under the given Corona situation, convention visual assessment sessions are not possible anymore. In order to proceed with the identification of a suitable verification test set, a simple and informal visual assessment procedure with JVET experts may be conducted as described below. It is noted that local self-isolation rules should not be violated by the participants for this activity.

A few preparatory steps are necessary to proceed with a visual check of the proposed test sequences.

- All participants must be equipped with a suitable computer able to play in a fluid way video clip (in mp4 file format) up to UHD resolution. The sequences to be assessed will be provided as HEVC bitstreams in mp4-containers.

- All participants must have a monitor (or TV set) of at least 32” size; the recommended set-up includes a UHD 55” OLED TV set (regardless the brand).

- Use a suitable player.

- Set up a clean room with protection from light and external noise, equipped with a desk, the monitor and the PC.

- Be logged in to the corresponding Zoom meeting (to be announced)

- Agree with the test manager the schedule and the video content to assess.

The application of the above simple rules will make possible an effective visual check of the video content under consideration.

# Test sequences

Test sequences available on the JVET ftp site or available for download elsewhere are listed below. In a first pass, the sequences have been screened for general suitability for the intended purpose. The result of this assessment is listed under “Comments” in the tables below. A further iteration was performed considering a first visual screening of the HM simulation results. Based on this assessment colour coding was added to the tables indicating the suitability using red (not recommended), yellow (borderline), and green (interesting).

## JVET FTP site: /ahg/candidates/JVET-G0096/

| Sequence | Resolution | Framerate | Duration | Comments |
| --- | --- | --- | --- | --- |
| BeachMountain_3840x2160_30fps_420_10bit.yuv | 3840x2160 | 30fps | 10sec | not considered |
| MountainBay_3840x2160_30fps_420_10bit.yuv | 3840x2160 | 30fps | 10sec | may be interesting |

## JVET FTP site: /ahg/candidates/netflix/Chimera/

| Sequence | Resolution | Framerate | Duration | Comments |
| --- | --- | --- | --- | --- |
| Netflix_Aerial_4096x2160_60fps_10bit_420 | 4096x2160 | 60 | 20sec | not considered |
| Netflix_BarScene_4096x2160_60fps_10bit_420 | 4096x2160 | 60 | 20sec | not considered |
| Netflix_Dancers_4096x2160_60fps_10bit_420 | 4096x2160 | 60 | 20sec | could be interesting when starting later. Too easy to code |
| Netflix_DinnerScene_4096x2160_60fps_10bit_420 | 4096x2160 | 60 | 20sec | tbd |
| Netflix_DrivingPOV_4096x2160_60fps_10bit_420 | 4096x2160 | 60 | 20sec | Interesting |
| Netflix_PierSeaside_4096x2160_60fps_10bit_420 | 4096x2160 | 60 | 20sec | may be interesting, artifacts on windows fo building? |
| Netflix_RollerCoaster_4096x2160_60fps_10bit_420 | 4096x2160 | 60 | 20sec | not considered |
| Netflix_WindAndNature_4096x2160_60fps_10bit_420 | 4096x2160 | 60 | 20sec | not considered |

Note: For inspection, a 10sec section was extracted from Netflix_Dancers_4096x2160_60fps_10bit_420, starting at frame 540 of the sequence (called Dancers2 in the RD results below). From Netflix_DrivingPOV, the first 10sec were extracted (called DrivingPOV2 in the RD results below).

## JVET FTP site: /ahg/candidates/netflix/ElFuente/

| Sequence | Resolution | Framerate | Duration | Comments |
| --- | --- | --- | --- | --- |
| Netflix_Boat_4096x2160_60fps_10bit_420 | 4096x2160 | 60 | 5sec | not considered, high bitrate |
| Netflix_BoxingPractice_4096x2160_60fps_10bit_420 | 4096x2160 | 60 | 4.3sec | interesting but to short |
| Netflix_Crosswalk_4096x2160_60fps_10bit_420 | 4096x2160 | 60 | 5sec | may be interesting |
| Netflix_Narrator_4096x2160_60fps_10bit_420 | 4096x2160 | 60 | 5sec | rate range appears to be small, probably not suitable |
| Netflix_SquareAndTimelapse_4096x2160_60fps_10bit_420 | 4096x2160 | 60 | 10sec | Timelapse, not considered |
| Netflix_TunnelFlag_4096x2160_60fps_10bit_420 | 4096x2160 | 60 | 10sec | not considered |

Note: For inspection, Netflix_BoxingPractice_4096x2160_60fps_10bit_420 was “mirror-looped” to double the sequence duration (called BoxingPracticeFlip in the RD results below).

## JVET FTP site: /ahg/candidates/sjtu/

| Sequence | Resolution | Framerate | Duration | Comments |
| --- | --- | --- | --- | --- |
| BundNightscape_3840x2160_30fps_8bit_420pf.yuv | 3840x2160 | 30 | 10sec | Timelapse, not considered |
| ConstructionField_3840x2160_30fps_8bit_420pf | 3840x2160 | 30 | 10sec | not considered |
| Fountains_3840x2160_30fps_8bit_420pf | 3840x2160 | 30 | 10sec | May be interesting, dynamic texture sequence |
| Library_3840x2160_30fps_8bit_420pf | 3840x2160 | 30 | 10sec | not considered |
| Marathon_3840x2160_30fps_8bit_420pf | 3840x2160 | 30 | 10sec | may be interesting |
| ResidentialBuilding_3840x2160_30fps_8bit_420pf | 3840x2160 | 30 | 10sec | not considered |
| Runners_3840x2160_30fps_8bit_420pf.yuv | 3840x2160 | 30 | 10sec | may be interesting |
| RushHour_3840x2160_30fps_8bit_420pf.yuv | 3840x2160 | 30 | 10sec | motion only in lower half. Interesting? |
| Scarf_3840x2160_30fps_8bit_420pf.yuv | 3840x2160 | 30 | 8.4sec | not considered |
| TallBuildings_3840x2160_30fps_8bit_420pf | 3840x2160 | 30 | 10sec | very low motion, maybe with aliasing issues, interesting? |
| TrafficAndBuilding_3840x2160_30fps_8bit_420pf.yuv | 3840x2160 | 30 | 10sec | motion only in lower half. Some camera movement. Interesting? |
| TrafficFlow_3840x2160_30fps_8bit_420pf | 3840x2160 | 30 | 10sec | static camera, cars may trigger interesting artifacts |
| TreeShade_3840x2160_30fps_8bit_420pf.yuv | 3840x2160 | 30 | 10sec | not considered |
| Wood_3840x2160_30fps_8bit_420pf | 3840x2160 | 30 | 10sec | not considered |

## From http://ultravideo.cs.tut.fi/#testsequences

| Sequence | Resolution | Framerate | Duration | Comments |
| --- | --- | --- | --- | --- |
| Beauty_3840x2160_120fps_420_10bit_YUV | 3840x2160 | 120fps | 5sec | not considered |
| Bosphorus_3840x2160_120fps_420_10bit_YUV | 3840x2160 | 120fps | 5sec | may be interesting, too short |
| CityAlley_3840x2160_50fps_10bit.yuv | 3840x2160 | 50fps | 12sec | not considered |
| FlowerFocus_3840x2160_50fps_10bit | 3840x2160 | 50fps | 12sec | not considered |
| FlowerKids_3840x2160_50fps_10bit | 3840x2160 | 50fps | 12sec | backlight -> hdr? otherwise maybe interesting |
| FlowerPan_3840x2160_50fps_10bit | 3840x2160 | 50fps | 12sec | not considered |
| HoneyBee_3840x2160_120fps_420_10bit_YUV | 3840x2160 | 120fps | 5sec | not considered |
| Jockey_3840x2160_120fps_420_10bit_YUV | 3840x2160 | 120fps | 5sec | maybe too fast |
| Lips_3840x2160_120fps_10bit | 3840x2160 | 120fps | 5sec | not considered |
| ReadySetGo_3840x2160_120fps_420_10bit_YUV | 3840x2160 | 120fps | 5sec | better than Jockey too short |
| RaceNight_3840x2160_50fps_10bit | 3840x2160 | 50fps | 12sec | may be interesting |
| ShakeNDry_3840x2160_120fps_420_10bit_YUV | 3840x2160 | 120fps | 2.5sec | too short, too few motion |
| SunBath_3840x2160_50fps_10bit.yuv | 3840x2160 | 50fps | 6sec | not considered |
| Twilight_3840x2160_50fps_10bit | 3840x2160 | 50fps | 12sec | visually interesting. enough motion? too easy to code |
| YachtRide_3840x2160_120fps_420_10bit_YUV | 3840x2160 | 120fps | 5sec | might be interesting, similar to Bosphorus too short |

## HEVC Verification Test Sequences

| Sequence | Resolution | Framerate | Duration | Comments |
| --- | --- | --- | --- | --- |
| HomelessSleeping_3840x2160_60_8bit_420.yuv | 3840x2160 | 60fps | 10sec | tbd |
| Sedof_3840x2160_60_8bit_420.yuv | 3840x2160 | 60fps | 15sec | may be interesting |
| Book_3840x2160_50_10bit_420.yuv | 3840x2160 | 50fps | 10sec | tbd |
| Manege_3840x2160_60_8bit_420.yuv | 3840x2160 | 60fps | 15sec | may be interesting |

# Rate-Distortion Plots

The plots below show the rate-distortion performance for sequences from the above set which may be useful for subjective evaluation. The simulations were performed with HM-16.20 using the Random-Access configuration at QP=32,36,40,44 with the IRAP interval set to 24,32,48,64,128 for sequences at 24/25Hz, 30Hz, 50Hz, 60Hz, and 120Hz, respectively. The high QP range has been selected to enable the determination of a quality range spanning from very few to significant compression artifacts.

# Conclusions

The rate-distortion plots reveal that some sequences seem to be too easy to code, although being visually interesting (e.g. Dancers2, Twilight). A duration of at least 8sec is important to allow for visual assessment with naïve subjects. From the initial screening and the reported rate-distortion results, the test sequences MountainBay, DrivingPOV2, Marathon, Runners, TallBuildings, TrafficFlow, and RaceNight seem to be suitable and should be inspected more thoroughly.

Is is suggested to identify a set of candidates which should be coded with the VTM at similar quality for rate-distortion comparison, and further, to conduct visual inspection in order to assess the suitability of the chosen coding configuration.

# Patent rights declaration

RWTH Aachen University or VABTECH ltd do not have any current or pending patent rights relating to the technology described in this contribution.

E.g. VLC, or the Vooya player (available at https://www.offminor.de/downloads.html)